3GPP TSG-RAN WG4 Meeting #110bis	R4-2404507

Changsha, China, April 15 – 19, 2024

Source:	Huawei, Hisilicon

Title:	Discussion on 3Tx SAR solution for inter-band CA with PC1.5

Agenda item:			4.2.3

Document for:	Approval

# 1. Introduction

In RAN#101, the revised WID [1] was approved to support 3Tx inter-band UL CA and EN-DC. In this topic, SAR solution for PC1.5 inter-band UL CA with 3Tx was specified in TS 38.101-1. No UE capability for maximum uplink duty cycle to inter-band CA with PC1.5 was introduced, and the maxUplinkDutyCycle-interBandCA-PC2 is reused. However, the default state for the capability of maximum uplink duty cycle to inter-band CA with PC1.5 may not be applied directly to the case with PC1.5. The paper is to discuss the remaining issue for 3Tx inter-band in PC1.5.

# 2. Discussion

For power class 1.5, 3Tx inter-band UL CA with UL MIMO and with Tx diversity, the UE capability maxUplinkDutyCycle-interBandCA-PC2 is used as a duty threshold in SAR solution.

#### 6.2H.3.1 UE maximum output power for inter-band UL CA with UL MIMO … If a UE supports power class 1.5 for the band combination listed in Table 6.2H.3.1-1: – if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and – if the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 (The exact evaluation period is no less than one radio frame); or – if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2H.3.4 is 23dBm or lower; – shall apply all requirements for the default power class and set the configured transmitted power as specified in clause 6.2H.3.4; – if the average percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5maxUplinkDutyCycle-interBandCA-PC2 but less than or equal to maxUplinkDutyCycle-interBandCA-PC2; or – if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2H.3.4 is between 23dBm and 26dBm; – shall apply all requirements for the power class 2 and set the configured transmitted power as specified in clause 6.2H.3.4; – else; – shall apply all requirements for the power class 1.5 and set the configured transmitted power as specified in clause 6.2H.3.4 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent).

#### 6.2L.3.1 UE maximum output power for inter-band UL CA with Tx Diversity … If a UE supports power class 1.5 for the band combination listed in Table 6.2L.3.1-1: – if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and – if the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 (The exact evaluation period is no less than one radio frame); or – if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2L.3.4 is 23dBm or lower; – shall apply all requirements for the default power class and set the configured transmitted power as specified in clause 6.2L.3.4; – if the average percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-interBandCA-PC2 but less than or equal to maxUplinkDutyCycle-interBandCA-PC2; or – if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2L.3.4 is between 23dBm and 26dBm; – shall apply all requirements for the power class 2 and set the configured transmitted power as specified in clause 6.2L.3.4; – else; – shall apply all requirements for the power class 1.5 and set the configured transmitted power as specified in clause 6.2L.3.4 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent).

Observation 1: For 3Tx inter-band UL CA with UL MIMO and with Tx diversity in power class 1.5, if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent, the UE shall apply all requirements for the power class 1.5.

However, in TS38.306, the UE capability maxUplinkDutyCycle-interBandCA-PC2 is defined as follows, and the default state for the capability is that UE shall work on power class 2. Apparently, it is contradicted to what is specified for 3Tx inter band UL CA in power class 1.5.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| maxUplinkDutyCycle-interBandCA-PC2-r17 Indicates the maximum average percentage of symbols during a certain evaluation period that can be scheduled for uplink transmission so as to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies. The average percentage of uplink symbols is specified in 6.2A.1.3 in TS 38101-1[2] and the capability applies to the CA combinations listed in table 6.2A.1.3-1 in TS 38101-1[2]. If the field is absent, UE shall work on power class 2 regardless of UL duty cycle and may use P-MPRc as defined in 6.2.4 in TS 38101-1[2] if necessary. Value n50 corresponds to 50%, value n60 corresponds to 60% and so on. NOTE: Specific targeted UL duty cycle percentage is not assumed if the field is absent. | BC | No | N/A | FR1 only |

Observation 2: The definition of the default state for UE capability maxUplinkDutyCycle-interBandCA-PC2 is contradicted to the case in 3Tx inter band UL CA in power class 1.5.

In addition, from the definition of the parameter, the capability can be only applicable to inter-band CA defined in 6.2A.1.3. The 3Tx operations in sub-clause 6.2H.3.1 or 6.2L.3.1 are not included.

There are two solutions to the issue. The simpler one is to modify Rel-18 RAN2 spec to accommodate RAN4 design for 3Tx inter band UL CA in power class 1.5. Either to remove ‘shall work on power class regardless of UL duty cycle’, or to add ‘For PC2 band combination,’ could be workable.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| maxUplinkDutyCycle-interBandCA-PC2-r17 Indicates the maximum average percentage of symbols during a certain evaluation period that can be scheduled for uplink transmission so as to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies. The average percentage of uplink symbols is specified in 6.2A.1.3, 6.2H.3.1 or 6.2L.3.1 in TS 38101-1[2] and the capability applies to the CA combinations listed in table 6.2A.1.3-1, table 6.2H.3.1-1 and table 6.2L.3.1-1 in TS 38101-1[2]. If the field is absent, UE may use P-MPRc as defined in 6.2.4 in TS 38101-1[2] if necessary. Value n50 corresponds to 50%, value n60 corresponds to 60% and so on. NOTE: Specific targeted UL duty cycle percentage is not assumed if the field is absent. | BC | No | N/A | FR1 only |

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| maxUplinkDutyCycle-interBandCA-PC2-r17 Indicates the maximum average percentage of symbols during a certain evaluation period that can be scheduled for uplink transmission so as to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies. The average percentage of uplink symbols is specified in 6.2A.1.3, 6.2H.3.1 or 6.2L.3.1 in TS 38101-1[2] and the capability applies to the CA combinations listed in table 6.2A.1.3-1, table 6.2H.3.1-1 and table 6.2L.3.1-1 in TS 38101-1[2]. For PC2 band combination, if the field is absent, UE shall work on power class 2 regardless of UL duty cycle and may use P-MPRc as defined in 6.2.4 in TS 38101-1[2] if necessary. Value n50 corresponds to 50%, value n60 corresponds to 60% and so on. NOTE: Specific targeted UL duty cycle percentage is not assumed if the field is absent. | BC | No | N/A | FR1 only |

The other solution is to improve RAN4 spec in sub-clause 6.2H.3.1 and 6.2L.3.1. Take the case of 3Tx inter-band UL CA with UL MIMO in PC1.5 as an example.

#### 6.2H.3.1 UE maximum output power for inter-band UL CA with UL MIMO … If a UE supports power class 1.5 for the band combination listed in Table 6.2H.3.1-1: – if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and – if the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 (The exact evaluation period is no less than one radio frame); or – if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2H.3.4 is 23dBm or lower; – shall apply all requirements for the default power class and set the configured transmitted power as specified in clause 6.2H.3.4; – if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5maxUplinkDutyCycle-interBandCA-PC2 but less than or equal to maxUplinkDutyCycle-interBandCA-PC2; or – if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2H.3.4 is between 23dBm and 26dBm; or – if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent – shall apply all requirements for the power class 2 and set the configured transmitted power as specified in clause 6.2H.3.4; – else; – shall apply all requirements for the power class 1.5 and set the configured transmitted power as specified in clause 6.2H.3.4 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent).

Observation 3: To resolve the contradiction of the applicable power class for 3Tx inter-band UL CA in PC1.5 between RAN2 and RAN4, both modifying Rel-18 RAN2 spec to accommodate RAN4 design and improve RAN4 spec in sub-clause 6.2H.3.1 and 6.2L.3.1 are workable.

We slightly prefer to modify Rel-18 RAN2 spec to accommodate RAN4 design.

Proposal 1: Modify UE capability maxUplinkDutyCycle-interBandCA-PC2 in RAN2 spec to accommodate RAN4 design for 3Tx inter-band UL CA in power class 1.5. Either to remove ‘shall work on power class regardless of UL duty cycle’, or to add ‘For PC2 band combination,’ could be workable.

# 3. Conclusions

This contribution presents the following observations and proposal:

Observation 1: For 3Tx inter-band UL CA with UL MIMO and with Tx diversity in power class 1.5, if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent, the UE shall apply all requirements for the power class 1.5.

Observation 2: The definition of the default state for UE capability maxUplinkDutyCycle-interBandCA-PC2 is contradicted to the case in 3Tx inter band UL CA in power class 1.5.

Observation 3: To resolve the contradiction of the applicable power class for 3Tx inter-band UL CA in PC1.5 between RAN2 and RAN4, both modifying Rel-18 RAN2 spec to accommodate RAN4 design and improve RAN4 spec in sub-clause 6.2H.3.1 and 6.2L.3.1 are workable.

Proposal 1: Modify UE capability maxUplinkDutyCycle-interBandCA-PC2 in Rel-18 RAN2 spec to accommodate RAN4 design for 3Tx inter-band UL CA in power class 1.5. Either to remove ‘shall work on power class regardless of UL duty cycle’, or to add ‘For PC2 band combination,’ could be workable.

# 4. Reference

[1] RP-231719, WID revision for Core part 4Rx handheld UE for low NR bands (1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC, Rel-18, Rapporteur: OPPO, RAN#101

3GPP TSG-RAN WG4 Meeting #110bis	R4-240xxxx

Changsha, China, April 15 – 19, 2024

Title:	LS on capability of maxUplinkDutyCycle-interBandCA-PC2

Response to:

Release:	Rel-18

Work Item:	4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-Core

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name: 	Hu Dan

E-mail Address: hudan11@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

For 3Tx inter-band UL CA with UL MIMO and with Tx diversity in power class 1.5, the UE capability maxUplinkDutyCycle-interBandCA-PC2 is used as a duty threshold in the SAR solution. As per the definition of the parameter, the capability maxUplinkDutyCycle-interBandCA-PC2 is applicable to inter-band CA defined in clause 6.2A.1.3, and the 3Tx operations in clause 6.2H.3.1 or 6.2L.3.1 are not included. In addition, the default state for the capability is that UE shall work on power class 2. It is contradicted to what is specified for 3Tx inter band UL CA in power class 1.5.

#### 6.2H.3.1 UE maximum output power for inter-band UL CA with UL MIMO … If a UE supports power class 1.5 for the band combination listed in Table 6.2H.3.1-1: – if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is present and – if the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 (The exact evaluation period is no less than one radio frame); or – if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2H.3.4 is 23dBm or lower; – shall apply all requirements for the default power class and set the configured transmitted power as specified in clause 6.2H.3.4; – if the average percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5maxUplinkDutyCycle-interBandCA-PC2 but less than or equal to maxUplinkDutyCycle-interBandCA-PC2; or – if 10log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2H.3.4 is between 23dBm and 26dBm; – shall apply all requirements for the power class 2 and set the configured transmitted power as specified in clause 6.2H.3.4; – else; – shall apply all requirements for the power class 1.5 and set the configured transmitted power as specified in clause 6.2H.3.4 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent).

2. Actions:

To RAN2:

ACTION: RAN4 respectfully asks RAN2 to update the capability of maxUplinkDutyCycle-interBandCA-PC2 to accommodate the SAR solution for 3Tx inter-band UL CA with UL MIMO and with Tx diversity in power class 1.5.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN4 Meeting#111						20 - 24 May 2024							Fukuoka, Japan

TSG-RAN4 Meeting#112  	 				19 – 23 August 2024				Maastricht, Netherlands