3GPP TSG-RAN WG4 Meeting #110bis	R4-2404526

Changsha, China, Apr 15 – Apr 19, 2024

Source:	Nokia

Title:	On MIMO_evo UE demodulation performance and CSI requirements - Simulations

Agenda item:	6.11.2.2

Document for:	Discussion

Introduction

This document details Nokia’s current simulation results for alignment on the WID for advanced receivers for MU-MIMO scenario. Updated agreements were made in RAN4#110 [1] for facilitating companies to provide simulation results.

The following we provide Nokia’s updated simulation results.

Simulation Assumptions

Table 1 provides an overview of the already agreed simulation assumptions taken from the WF [1] and proposed test parameters:

Table 1: Agreed simulation assumptions for Phase I

| Reference receiver for phase II simulation | R-ML, E-IRC, MMSE-IRC |
| --- | --- |
| Sub-carrier spacing, Bandwidth | FDD:15KHz, 10 MHz TDD:30 KHz, 40 MHz(TDD) |
| UL-DL Pattern | FDD: Not applicable, TDD: FR1.30-1 |
| Rank allocation for the target and co-scheduled UE, with 1 co-scheduled UE | 2Tx2Rx UE: 1+1 2Tx4Rx UE: 1+1 4Tx4Rx UE: 2+2 |
| DMRS port configurations for the target and co-scheduled UEs | - Use different CDM groups for: - rank 2 (DMRS port 0, 1) + 2 (DMRS port 2, 3) - Use the same CDM group for - rank 1 (DMRS port 0) + 1 (DMR port 1) |
| Antenna configuration | For rank 1+1: 2T2R, 2T4R For rank 2+2: 4Tx 4Rx |
| Channel model | Use TDLC300-100, TDLA30-10 when the rank of the target UE is 1 Use TDLA30-10, XP medium when the rank of the target UE is 2 |
| Antenna correlation | Rank 1+1: ULA medium and ULA low (for 2T2R,2T4R) Rank 2+2: ULA Low, XP medium |
| Precoder selection target and co-scheduled UEs | Single panel Type 1 For rank 1+1: Random and Orthogonal PMI selection For rank 2+2: Orthogonal PMI selection |
| QCL assumptions | Assume all scheduled DMRS ports have same QCL assumptions |
| Evaluation metric | Reuse the Rel-17 MMSE-IRC phase I evaluation assumptions captured in TR38.833 as a start point. |
| The number of co-scheduled UEs | Only consider 1 co-scheduled UE |
| DMRS sequence for the co-scheduled UE | Assume the scrambling ID for DMRS sequence is the same for the target UE the co-scheduled UE(s) |
| PDSCH resource allocation for the target UE | Maximum transmission bandwidth configuration.10 MHz 15KHz SCS Full OFDM symbol allocation. |
| TDRA of co-scheduled UE | Assume TDRA of target and co-scheduled UE is same |
| FDRA of co-scheduled UE | Full transmission bandwidth configuration., Partial transmission bandwidth configuration |

Simulation Results

FDD Results

Based on the information in Table 1, we provide the simulation results on the cases of interest for this meeting, listed in Table 2 for FDD cases without MO BD, and for FDD cases with MO BD in Table 3.

Table 2 Simulation results for FDD cases without MO BD, DCI index 1 to 5 indicated.

| Alignment Case Number | Duplex Mode and SCS | CHBW | Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | Nokia |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | R-ML | E-IRC | MMSE-IRC |
| 1 | FDD 15kHz SCS | 10MHz | 1+1 | 2T2R | Random | TDLC300-100 | ULA medium | MCS 13 | QPSK | 15.1 | 22.7 | 24.5 |
| 2 |  |  |  |  | Orthogonal |  |  |  |  | 14.4 | 19.9 | 21.4 |
| 3 |  |  |  | 2T4R | Random |  |  |  |  | 12.7 | 21.1 | 24.9 |
| 4 |  |  |  |  | Orthogonal |  |  |  |  | 12.1 | 18.7 | 21.5 |
| 5 |  |  | 2+2 | 4T4R | Orthogonal | TDLA30-10 | ULA Low |  |  | 10.7 | 13.9 | 13.3 |
| 6 |  |  |  |  |  |  | XP medium |  |  | 11.7 | 15.8 | 15.2 |
| 7 |  |  |  |  |  |  | ULA low | MCS 17 | 16QAM | 16.4 | 18 | 17.6 |
| 8 |  |  |  |  |  |  | XP medium |  |  | 17.9 | 20.2 | 19.5 |
| 9 |  |  |  |  |  |  |  |  | QPSK | 14.7 | 20.4 | 19.7 |
| NA Not achievable |  |  |  |  |  |  |  |  |  |  |  |  |

Table 3 Simulation results for FDD cases with MO BD, DCI index 6 indicated.

| Alignment Case Number | Duplex Mode and SCS | CHBW | Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | Nokia |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | R-ML | E-IRC | MMSE-IRC |
| 19 | FDD 15kHz SCS | 10MHz | 1+1 | 2T2R | Random | TDLC300-100 | ULA medium | MCS 13 | QPSK | 15.3 | 22.7 | 24.5 |
| 20 |  |  |  |  | Orthogonal |  |  |  |  | 14.6 | 19.9 | 21.4 |
| 21 |  |  |  |  | Random |  | ULA Low |  |  | 12.5 | 15.8 | 17.4 |
| 22 |  |  |  | 2T4R | Random |  | ULA Medium |  |  | 14.4 | 21.1 | 24.9 |
| 23 |  |  |  |  | Orthogonal |  |  |  |  | 13.6 | 18.7 | 21.5 |
| 24 |  |  |  |  | Random | TDLA300-10 | ULA Low |  |  | 8.1 | 8.9 | 9.5 |
| 25 |  |  |  | 2T2R | Random | TDLC300-100 | ULA medium | MCS 17 | 16QAM | 23.5 | NA | NA |
| 26 |  |  |  |  | Orthogonal |  |  |  |  | 21.9 | 27.3 | 26.7 |
| 27 |  |  |  |  | Random |  | ULA Low |  |  | 18.3 | 19.9 | 22.4 |
| 28 |  |  |  | 2T4R | Random | TDLC300-100 | ULA medium |  |  | 21.2 | 27.4 | 30.2 |
| 29 |  |  |  |  | Orthogonal |  |  |  |  | 19.8 | 22.6 | 25.9 |
| 30 |  |  |  |  | Random | TDLA30-10 | ULA Low |  |  | 11.8 | 12 | 14.3 |
| 31 |  |  | 2+2 | 4T4R | Orthogonal | TDLA30-10 | ULA Low | MCS 13 | QPSK |  | 13.9 | 13.3 |
| 32 |  |  |  |  |  |  | XP medium |  |  |  | 15.8 | 15.2 |
| NA Not achievable |  |  |  |  |  |  |  |  |  |  |  |  |

TDD Results

Based on the information in Table 1, we provide the simulation results on the cases of interest for this meeting, listed in Table 4 for TDD cases without MO BD, and for TDD cases with MO BD in Table 5.

Table 4 Simulation results for TDD cases without MO BD, DCI index 1 to 5 indicated.

| Alignment Case Number | Duplex Mode and SCS | CHBW | Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | Nokia |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | R-ML | E-IRC | MMSE-IRC |
| 10 | TDD 30kHz SCS | 40MHz | 1+1 | 2T2R | Random | TDLC300-100 | ULA medium | MCS 13 | QPSK | 16.2 | 23.7 | 26.4 |
| 11 |  |  |  |  | Orthogonal |  |  |  |  | 15.4 | 21.5 | 22.6 |
| 12 |  |  |  | 2T4R | Random |  |  |  |  | 13.7 | 22.5 | 29.2 |
| 13 |  |  |  |  | Orthogonal |  |  |  |  | 12.9 | 21.3 | 24.2 |
| 14 |  |  | 2+2 | 4T4R | Orthogonal | TDLA30-10 | ULA Low |  |  | 10.9 | 14.4 | 13.4 |
| 15 |  |  |  |  |  |  | XP medium |  |  | 11.9 | 16.4 | 15.2 |
| 16 |  |  |  |  |  |  | ULA low | MCS 17 | 16QAM | 17.1 | 19.1 | 18 |
| 17 |  |  |  |  |  |  | XP medium |  |  | 18.6 | 21.4 | 20.1 |
| 18 |  |  |  |  |  |  |  |  | QPSK | 15.2 | 21.4 | 20.4 |
| NA Not achievable |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5 Simulation results for TDD cases with MO BD, DCI index 6 indicated.

| Alignment Case Number | Duplex Mode and SCS | CHBW | Rank for target + Co-UE | MIMO | Precoder selection | Channel Model | Antenna correlation | MCS for the target UE (MCS Table 1) | Modulation order for the co-scheduled UE | Nokia |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | R-ML | E-IRC | MMSE-IRC |
| 33 | TDD 30kHz SCS | 40MHz | 1+1 | 2T2R | Random | TDLC300-100 | ULA medium | MCS 13 | QPSK | 16.2 | 23.7 | 26.4 |
| 34 |  |  |  |  | Orthogonal |  |  |  |  | 15.5 | 21.5 | 22.6 |
| 35 |  |  |  |  | Random |  | ULA Low |  |  | 13.2 | 16.9 | 19 |
| 36 |  |  |  | 2T4R | Random |  | ULA Medium |  |  | 14 | 22.5 | 29.2 |
| 37 |  |  |  |  | Orthogonal |  |  |  |  | 13.3 | 21.3 | 24.2 |
| 38 |  |  |  |  | Random | TDLA300-10 | ULA Low |  |  | 8 | 8.8 | 9.3 |
| 39 |  |  |  | 2T2R | Random | TDLC300-100 | ULA medium | MCS 17 | 16QAM | 26.8 | NA | NA |
| 40 |  |  |  |  | Orthogonal |  |  |  |  | 24.1 | NA | NA |
| 41 |  |  |  |  | Random |  | ULA Low |  |  | 20.5 | 22.5 | 26.8 |
| 42 |  |  |  | 2T4R | Random | TDLC300-100 | ULA medium |  |  | 26.8 | 28.3 | NA |
| 43 |  |  |  |  | Orthogonal |  |  |  |  | 22.6 | 23.9 | 30.5 |
| 44 |  |  |  |  | Random | TDLA30-10 | ULA Low |  |  | 11.5 | 12.1 | 14.5 |
| 45 |  |  | 2+2 | 4T4R | Orthogonal | TDLA30-10 | ULA Low | MCS 13 | QPSK |  | 14.4 | 13.4 |
| 46 |  |  |  |  |  |  | XP medium |  |  |  | 16.4 | 15.2 |
| NA Not achievable |  |  |  |  |  |  |  |  |  |  |  |  |

Conclusion

In this contribution we have provided Nokia's simulation results for Advanced receivers.

- References

- R4-2402864 - Way forward on [110][319] NR_demod_enh3_Part1