3GPP TSG-RAN WG4 Meeting #110bis	R4-2404529

Changsha, China, Apr 15 – Apr 19, 2024

Source:	Nokia

Title:	On Lessthan5MHz UE demod perf and CSI requirements - Simulations

Agenda item:	6.8.7.1

Document for:	Discussion

Introduction

This contribution contains Nokia’s simulation results for Lessthan5MHz topic.

Simulation results

PDCCH

Punctured PDCCH CORESET#0

Configuration used for the following results: (15PRBs, 3 symbols, non-interleaved, AL4, DCI 1_0 (35 bits for 15 PRBs); Use CCEs #4, #5, #6, and #7 to transmit PDCCH with DCI 1_0.)

Table 4 PDCCH Simulation results for CORESET#0 PDCCH (Punctured)

| Duplex | Bandwidth (MHz) | Coreset RB | Coreset duration | AL | PropCond | Ant Cfg | Interleaved | pm-dsg(%) | SNR(dB) | With Impairment SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | 3 | 15 | 3 | 4 | TDLA30-10 | 1x2 Low | no | 1 | 3.3 | 5.8 |
| FDD | 3 | 15 | 3 | 4 | TDLC300-100 | 2x2 Low | no | 1 | -0.9 | 1.6 |
| FDD | 3 | 15 | 3 | 4 | TDLA30-10 | 1x4 Low | no | 1 | -3.3 | -0.8 |
| FDD | 3 | 15 | 3 | 4 | TDLC300-100 | 2x4 Low | no | 1 | -5.2 | -2.7 |

PBCH

Table 7 PBCH simulation results for non-HST conditions

| Duplex | Bandwidth (MHz) / Subcarrier spacing (kHz) | SSB/PBCH index | Propagation condition | Antenna configuration and correlation matrix | Reference value |  | With Impairment |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) | SNR (dB) |
| FDD | 3 / 15 | Unknown | TDLC300-100 | 1x2 Low | 1 | -3.6 | -1.1 |
| FDD | 3 / 15 | Unknown | TDLC300-100 | 1x4 low | 1 | -7.4 | -5.9 |

Conclusion

We have presented Nokia's simulation results for Lessthan5MHz UE demod performance and CSI requirements.

- References

- R4-2402863 - Way forward on [110][318] NR_FR1_lessthan_5MHz_BW_demod