3GPP TSG-RAN WG4 Meeting #110-bis	R4-2404531

Changsha, China, Apr 15 – Apr 19, 2024

Source:	Nokia

Title:	On MultiRx Demodulation performance and CSI requirements – PMI reporting requirements

Agenda item:	6.3.4.3

Document for:	Discussion

Introduction

During RAN4#109 discussions, the PMI reporting configurations, parameter setup for the simulations and the test cases were agreed [2]. Agreements were made for the remaining open issues in RAN4#110 [1].

In RAN4#110 draftCR [3] was endorsed with requirements agreed in [1][2] and a request for companies to provide updated/new simulation results for the next meeting

This paper contains Nokia’s new simulation results for the result alignment of PMI reporting requirements.

Discussion

Simulation results for PMI reporting

The following is Nokia’s simulation results for PMI reporting based on the agreed DraftCR from RAN4#110 [3]:

Table 1: Results for PMI reporting rank 1+1 with MCS13 – Separate processing.

| Simulation Configuration | PTRS Port Configuration | Bandwidth (MHz) / Subcarrier spacing (kHz) | TDD UL-DL pattern | MCS | Rank | Channel Model | RB Allocation Size | Cross talk factor (rho) - dB | Performance Metric | Throughput ratio γ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| sDCI SDM Rank 1+1 (0us, 0Hz) | With 1 PTRS port | 100/120 | FR2.120-1 | MCS13 | 1+1 | TDLA30-35 | 66 | -12 | 90% | 1.30 |
| sDCI SDM Rank 1+1 (0us, 0Hz) | With 2 PTRS ports | 100/120 | FR2.120-1 | MCS13 | 1+1 | TDLA30-35 | 66 | -12 | 90% | 1.30 |

In Table 1, we have provided our updated simulation results for PMI reporting related use cases. Based on the collected results from RAN4#110 (see [4]) we see that our new results are aligned with provided results from RAN4#110, hence we see that it feasible to update the throughput ratio values for PMI reporting as provided in the endorsed draftCR [3] from RAN4#110 by considering our new simulation results.

Our new simulation results are aligned with the RAN4#110 results of other companies.

Update PMI reporting requirements in TS38.101-4 based on the latest simulation results.

Conclusion

Within this contribution we have presented our simulation results for the performance alignment of PMI reporting requirements of FR2 multi-RX downlink demodulation requirements.

We have the following observations and proposals:

- Our new simulation results are aligned with the RAN4#110 results of other companies.

- Update PMI reporting requirements in TS38.101-4 based on the latest simulation results.

- References

- R4-2402859 - WF for FR2_multiRX_DL_Demod_v00_mod (RAN4#110)

- R4-2321140 - WF on FR2_Multi-Rx_Demod (RAN4#109)

- R4-2402993 - Draft CR to 38.101-4: PMI reporting requirements for FR2 multipanel reception; MediaTek

- R4-2401139 - Simulation results summary for FR2 multi-Rx performance requirements