3GPP TSG RAN WG4 #110BIS meeting 	 R4-2404541

Changsha, China, 15th – 19th April, 2024

Agenda Item:	9.1.2

Source:	Meta Ireland

Title:	High power UE for intra/inter-band CA/DC including 2Tx/3Tx in TN

Document for:	Approval

- Introduction

In RAN #103 meeting, RAN approved new WID on UE RF enhancements for FR1 and FR2 in Rel-19 [1]. In these WID, there are three objectives in RF related issues. First objective is for the high power CA/DC combinations, 2nd objective is power boosting and MPR reduction issue for single carrier and NR intra-band UL CA. The last objective is support of 6Rx for HHUE and FWA devices.

In this contribution, we focus on the high power UE for CA/DC including 2Tx/3Tx in terrestrial networks as follows:

High power UE (HPUE) for CA in terrestrial network (TN) [1] - Specify the generic requirements of high-power UE (HPUE) for NR uplink (UL) CA in FR1 and EN-DC with NR FR1 bands - Power class 1.5 (PC1.5) UE for NR TDD intra-band UL contiguous and non-contiguous CA with 2Tx - Specify the requirements for intra-band UL contiguous CA with or without UL-MIMO - Example band combinations: - CA_n41C, CA_n78C, CA_n77C, CA_n79C for intra-band uplink contiguous CA configurations - Focus on the maximum output power (MOP), MPR/A-MPR requirements, SAR solution - Specify the requirements for intra-band UL non-contiguous CA without UL-MIMO - Example band combinations: - CA_n78(2A), CA_n77(2A) for intra-band uplink non-contiguous CA configurations - Focus on the maximum output power (MOP), MPR/A-MPR requirements, SAR solution - NOTE: leave the other band combination specific requirements to the corresponding Rel-19 basket WIs - PC1.5 UE for two band NR inter-band uplink CA with 2Tx and/or 3Tx for handheld and FWA, and PC1.5 and PC2 for two band EN-DC with 2Tx and/or 3Tx for handheld and FWA - Focus on the SAR solution - Enable power class 2 (PC2) and PC1.5 of two band inter-band uplink CA and EN-DC with 3Tx for handheld UE - Identify and update the requirements if necessary - Only PC3 is considered for LTE FDD in EN-DC - NOTE: leave the band combination specific requirements, e.g., MSD to the corresponding Rel-19 basket WIs - Investigate and if feasible, support increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band uplink CA and EN-DC HPUE with the different existing power classes which have already been specified. - Introduce the signalling to support the above objectives, if needed. - Consider release independency, if needed

In this paper, we suggest how to define the RF requirements for high power UE for intra-band uplink CA i.e. n41C, n77 and other CA combinations and inter-band CA/DC including 3Tx in the cross inter-band CA/DC operation.

- Current TS38.101-1 for intra-band CA of High power UE

  - Max. output power reduction of PC2 between 1Tx and 2Tx in single carrier

In this session, we compared the MPR values for PC2 between 1Tx and 2Tx for a single carrier.

Captured in TS38.101-1 [2] Table 6.2.2-2 Maximum power reduction (MPR) for power class 2 Modulation; MPR (dB). Edge RB allocations; Outer RB allocations; Inner RB allocations. DFT-s-OFDM; Pi/2 BPSK; ≤ 3.5; ≤ 0.5; 01. QPSK; ≤ 3.5; ≤ 1; 02. 16 QAM; ≤ 3.5; ≤ 2; ≤ 1. 64 QAM; ≤ 3.5; ≤ 2.5. 256 QAM; ≤ 4.5. CP-OFDM; QPSK; ≤ 3.5; ≤ 3; ≤ 1.5. 16 QAM; ≤ 3.5; ≤ 3; ≤ 2. 64 QAM; ≤ 3.5. 256 QAM; ≤ 6.5. Table 6.2D.2-1: Maximum power reduction (MPR) for power class 2 with dual Tx Modulation; MPR (dB). Edge RB allocations; Outer RB allocations; Inner RB allocations. DFT-s-OFDM; Pi/2 BPSK; ≤ 3.5; ≤ 1; 01. QPSK; ≤ 3.5; ≤ 2; 0.52. 16 QAM; ≤ 3.5; ≤ 2.5; ≤ 1.5. 64 QAM; ≤ 3.5; ≤ 3. 256 QAM; ≤ 5.5. CP-OFDM; QPSK; ≤ 4.0; ≤ 3.5; ≤ 2. 16 QAM; ≤ 4.0; ≤ 3.5; ≤ 2.5. 64 QAM; ≤ 4.5. 256 QAM; ≤ 8.0.

Based on the comparison, we observed as follow:

Observation 1: The required MPR values for PC2 with Dual Tx for single carrier had slightly larger MPR values due to the reverse IMD problem and other interference problems.

  - Max. output power reduction of PC2 between 1Tx and 2Tx in intra-band contiguous CA

In session, 2.2 we compare the MPR values for PC2 between 1Tx and 2Tx for intra-band contiguous CA as follows:

Captured in TS38.101-1 [2] Table 6.2A.2.1-1a: Contiguous RB allocation for Power Class 2 Modulation; MPR for bandwidth class B(dB); MPR for bandwidth class C(dB). inner; Outer1; inner; outer. DFT-s-OFDM; Pi/2 BPSK; 2.0; 4.01; 2.5; 7. QPSK; 2.0; 4.01; 2.5; 7. 16QAM; 2.5; 4.01; 2.5; 7. 64QAM; 3.0; 4.51; 5; 7. 256QAM; 5.5; 6.0; 7; 7.5. CP-OFDM; QPSK; 2.5; 5.01; 3.5; 8. 16QAM; 3.0; 5.01; 3.5; 8. 64QAM; 3.5; 5.01; 5; 8. 256QAM; 6.5; 6.5; 7; 8. NOTE 1: When 1 RB or 2 RB are allocated at the lower edge of lowest CC or upper edge of upper CC, MPR for outer is 5.5 dB.. Table 6.2A.2.1-1b: Contiguous RB allocation for Power Class 2 with dual Tx2 Modulation; MPR for bandwidth class B(dB); MPR for bandwidth class C(dB). inner; Outer1; inner; outer. DFT-s-OFDM; Pi/2 BPSK; 3.0; 5.01; 3.5; 8. QPSK; 3.0; 5.01; 3.5; 8. 16QAM; 3.5; 5.01; 3.5; 8. 64QAM; 4.0; 5.51; 6; 8. 256QAM; 6.5; 7.0; 8; 8.5. CP-OFDM; QPSK; 3.0; 5.51; 4.0; 8.5. 16QAM; 3.5; 5.51; 4.0; 8.5. 64QAM; 4.0; 5.51; 5.5; 8.5. 256QAM; 7.0; 7.0; 7.5; 8.5. NOTE 1: When 1 RB or 2 RB are allocated at the lower edge of lowest CC or upper edge of upper CC, MPR for outer is 5.5 dB. NOTE 2: UE indicating TxD supported.

Observation 2: The required MPR values for PC2 with Dual Tx for intra-band contiguous CA had slightly larger MPR values due to the reverse IMD problem and other interference problems.

  - Consideration on PC1.5 with 2Tx for intra-band contiguous CA

In TS38.101-1 in Rel-18, RAN4 defined the MPR requirements with dual Tx for single carrier as shown in Table 6.2D.2-2. The same RF architecture with separate PA and separate antenna RF architectures will be assumed for intra-band contiguous CA with dual Tx for CA_n41C, CA_n77C, CA_n78C and CA_n79C in Rel-19 Objctives.

Table 6.2D.2-2: Maximum power reduction (MPR) for power class 1.5 with dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ [2] | ≤ 0.5 |
|  | QPSK | ≤ 6.5 | ≤ [2.5] | ≤ 0.5 |
|  | 16 QAM | ≤ 6.5 | ≤ [3.5] | ≤ 1.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [4] | ≤ 3.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 6.5 | ≤ [6.5] |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ [4.5] | ≤ 2 |
|  | 16 QAM | ≤ 6.5 | ≤ [4.5] | ≤ 2.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [5] | ≤ 4.5 |
|  | 256 QAM | ≤ 8.5 | ≤ 8.5 | ≤ [8.5] |

Based on the same RF architecture, RAN4 needs to evaluate the MPR requirements to support wide intra-band contiguous CA up to 200MHz.

Proposal 1: RAN4 considers separate PA (2x26dBm) and separate antenna RF architecture for Power class 1.5 to derive the MPR/A-MPR requirements for the intra-band contiguous CA UE.

For the SAR regulation requirements, RAN4 can reuse ‘the max uplink duty cycle limitation’ of the single carrier SAR solution for intra-band contiguous CA operation. It means that intra-band contiguous CA UE will be operated by synchronous operation between 2CCs as simultaneous transmit or reception to comply with the SAR limitation.

Proposal 2: RAN4 can reuse the max uplink duty cycle limitation of the single carrier SAR solution for PC 1.5 intra-band contiguous CA combinations UE.

- Current TS38.101-1 for intra-band non-contiguous CA without UL-MIMO

  - Max. output power reduction of PC2 between 1Tx and 2Tx in intra-band non-contiguous CA

In this session, we compared the MPR values for PC2 intra-band non-contiguous CA UE between 1Tx and 2Tx RF chains.

In TS38.101-1, RAN4 specified 2 types of MPR requirements to meet the different emission requirements with -30dBm/MHz and -13dBm/MHz. In this session, we just captured the MPR requirements with -30dBm/MHz emission limit as follows:

Captured in TS38.101-1 [2] 6.2A.2.2.1 MPR to meet -30dBm/MHz 6.2A.2.2.1.2 PC2 with indicating dualPA-Architecture supported MPR in this clause is for intra-band non-contiguous CA power class 2 for UEs indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as: MPR=MAWhere MA is defined as follows MA = 15.5; 0 ≤ B < 1.44 15.0; 1.44 ≤ B < 2.88 14.0; 2.88 ≤ B < 5.76 12.0; 5.76 ≤ B < 10.8 10.5; 10.8 ≤ B < 23.04 9.0; 23.04 ≤ B <Some Sub-clauses are omitted> 6.2A.2.2.1.4 PC2 without indicating dualPA-Architecture supported MPR in this clause is for intra-band non-contiguous CA power class 2 for UEs without indicating IE dualPA-Architecture supported. The allowed maximum output power reduction is defined as: MPR=MAWhere MA is defined as follows MA = 19.5; 0 ≤ B < 1.08 19; 1.08 ≤ B < 2.16 18; 2.16 ≤ B < 5.04 16.5; 5.04≤ B < 10.08 16; 10.08 ≤ B < 36 12; 36 ≤ B < 56.88 10.5; 56.88 ≤ B

Observation 3: For the intra-band non-contiguous CA operation, the required MPR values for PC2 with single Tx had been defined with larger MPR values due to the PCB isolation, IMD problem and other interference problems.

  - Consideration on PC1.5 with 2Tx for intra-band non-contiguous CA

In PC2 high power intra-band non-contiguous CA in Rel-17, RAN4 considered 4 candidate RF architectures to support different PA capability combinations and the limitation of supporting channel Bandwidths.

However, RAN4 can only consider 2x26dBm PA + 2LO as baseline architecture to support up to 200MHz CBW and the baseline RF architecture will be considered to derive MPR/A-MPR requirements for PC1.5 intra-band non-contiguous CA operation with 2Tx.

Proposal 3: RAN4 can consider 2x26dBm PA (2LO) and separate antenna RF architecture as a baseline for power class 1.5 UE to derive the MPR/A-MPR requirements for the intra-band non-contiguous CA UE.

For the SAR regulation requirements, RAN4 can reuse ‘the max uplink duty cycle limitation’ of the single carrier SAR solution for intra-band non-contiguous CA operation. The same SAR solution will be applied for all intra-band contiguous CA and intra-band non-contiguous CA UE. It means that intra-band non-contiguous CA UE will be operated by synchronous operation between 2CCs as simultaneous transmit or reception to comply with the SAR limitation.

Proposal 4: RAN4 can reuse the max uplink duty cycle limitation of the single carrier SAR solution for PC 1.5 intra-band non-contiguous CA combinations UE.

- Current TS38.101-1 for inter-band CA/EN-DC with 2Tx and/or 3Tx

For 3Tx and 4Rx WI, RAN4 defined the detailed RF requirements for high power CA/DC combinations in Rel-18.

Captured in TS38.101-1 [2] Table 6.2H.3.1-1: UE Power Class for inter-band UL CA with UL MIMO in one frequency band NR UL CA Configuration; Class 1.5 (dBm); Tolerance (dB); Class 2 (dBm); Tolerance (dB); Class 3 (dBm); Tolerance (dB). CA_n2A-n77A; 293; +2/-3; 262; +2/-3; 23; +2/-3. CA_n5A-n77A; 293; +2/-3; 262; +2/-3; 23; +2/-3. CA_n7A-n77A; 293; +2/-3; 262; +2/-3; 23; +2/-3. CA_n8A-n78A; 262; +2/-3; 23; +2/-3. CA_n25A-n41A; 293; +2/-3; 262; +2/-3; 23; +2/-3. CA_n25A-n77A; 293; +2/-3; 262; +2/-3; 23; +2/-3. CA_n26A-n78A; 262; +2/-3; 23; +2/-3. CA_n28A-n41A; 262; +2/-3; 23; +2/-3. CA_n28A-n78A; 262; +2/-3; 23; +2/-3. CA_n41A-n66A; 293; +2/-3; 262; +2/-3; 23; +2/-3. CA_n41A-n71A; 293; +2/-3; 262; +2/-3; 23; +2/-3. CA_n41A-n77A; 264; +2/-3; 23; +2/-3. CA_n66A-n77A; 293; +2/-3; 262; +2/-3; 23; +2/-3. CA_n71A-n77A; 293; +2/-3; 262; +2/-3; 23; +2/-3. NOTE 1: An uplink CA configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: The UE supports PC3 in FDD band and PC3 or PC2 with UL MIMO in TDD band. NOTE 3: The UE supports PC3 in FDD band and PC1.5 with UL MIMO in TDD band. NOTE 4: The UE supports PC2 with UL MIMO in either one of the TDD bands and PC2 in the other TDD band. NOTE 5: Power class 3 is default power class unless otherwise stated. NOTE 6: FWA form factor is targeted unless otherwise stated..

In the Rel-19 WI, RAN4 will focus on the SAR solution for CA and EN-DC with 2Tx/3Tx and investigate whether to support new power classes to increase UE transmitted power limit up to accumulate max. output power.

  - Consideration of PC1.5 inter-band CA/DC operation with 2Tx and 3Tx

In the last RAN plenary, the detailed objectives were finalized to focus on SAR solution for 2Tx/3Tx inter-band CA/DC operation.

Also, RAN4 needs to investigate whether to define some new power classes to support different PC combinations between two inter-band CA/DC UE or not.

In the WI in Rel-19, RAN4 can define the new CA/DC band combinations as below and RAN4 can define new power classes to support the total accumulated power for inter-band CA/DC combination UE in Rel-19.

- Proposed inter-band CA/DC band combinations with 2Tx/3Tx

    - 1Tx PC2 in Band X (i.e., n77, n78, n79 without UL-MIMO) + 1Tx PC2 in Band Y

      - The power class combination already specified in TS38.101-1 and TS38.101-3

    - 1Tx PC2 in Band X  (i.e., n77, n78, n79 without UL-MIMO) + 1Tx PC3 in Band Y

Need to define new power class as like PC1.x

    - 2Tx PC2 in Band X (i.e., n77, n78, n79 with UL-MIMO) + 1Tx PC2 in Band Y

      - The power class combination already specified in TS38.101-1 and TS38.101-3

    - 2Tx PC2 in Band X  (i.e., n77, n78, n79 with UL-MIMO) + 1Tx PC3 in Band Y

Need to define new power class as like PC1.x

    - 2Tx PC1.5 in Band X  (i.e., n77, n78, n79 with UL-MIMO) + 1Tx PC2 in Band Y

Need to define new power class as like PC1.y

    - 2Tx PC1.5 in Band X  (i.e., n77, n78, n79 with UL-MIMO) + 1Tx PC3 in Band Y

Need to define new power class as like PC1.z

Proposal 5: RAN4 can define the above high power inter-band CA/DC band combinations with 2Tx/3Tx in the related Basket WIs in Rel-19. In the Rel-19 UE RF enhancement WI, RAN4 only focuses on general RF requirements for high power inter-band CA/DC UE.

Proposal 6: To support total accumulated power for inter-band CA/DC band combinations UE, RAN4 can define some new power classes in Rel-19.

Proposal 7: The power class information of high-power inter-band CA/DC UE with 2Tx/3Tx will be reported per band and per band combinations.

Proposal 8: For the SAR regulation requirements of high-power inter-band CA/DC UE, RAN4 can consider both duty cycle limitation and P-MPR approach from Rel-19 as same in FR2 MPE solutions.

- Conclusions

In this contribution, we propose some RF architectures and SAR solutions for intra-band contiguous CA or non-contiguous CA. Also, we share our preference on how to support the increased CA/DC UE power limitation with different PA capability between two Bands. Based on our initial analysis for the high power intra-band CA or inter-band CA/DC combination with 2Tx/3Tx, we proposed as follow

Proposal 1: RAN4 considers separate PA (2x26dBm) and separate antenna RF architecture for Power class 1.5 to derive the MPR/A-MPR requirements for the intra-band contiguous CA UE.

Proposal 2: RAN4 can reuse the max uplink duty cycle limitation of the single carrier SAR solution for PC 1.5 intra-band contiguous CA UE.

Proposal 3: RAN4 can consider 2x26dBm PA (2LO) and separate antenna RF architecture as baseline to derive the MPR/A-MPR requirements for the intra-band non-contiguous CA UE.

Proposal 4: RAN4 can reuse the max uplink duty cycle limitation of the single carrier SAR solution for PC 1.5 intra-band non-contiguous CA combinations UE.

Proposal 5: RAN4 can define the above high power inter-band CA/DC band combinations with 2Tx/3Tx in the related Basket WIs in Rel-19. In the Rel-19 UE RF enhancement WI, RAN4 only focuses on general RF requirements for high power inter-band CA/DC UE.

Proposal 6: RAN4 can define some new power classes to support total accumulated power for inter-band CA/DC band combinations UE in Rel-19.

Proposal 7: The power class information of high-power inter-band CA/DC UE with 2Tx/3Tx will be reported per band and per band combinations.

Proposal 8: For the SAR regulation requirements of high power inter-band CA/DC UE, RAN4 can consider both duty cycle limitation and P-MPR approach from Rel-19 as same in FR2 MPE solutions.

Reference

[1] RP-240828, “New WID on UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4,” Huawei

[2] TS38.101-1

[3] TS38.101-3