3GPP TSG-RAN4 Meeting #110bis	                            R4-2404568

Changsha, China, April 15th – 19th, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | <CR#> | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | × | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on activation deactivation delay for Pre-MG within the concurrent MGs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | <Rel-18> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The activation/deactivation time taking effective in the activation/deactivation delay requirements for multiple pre-configured measurement gaps was not clearly defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add activation/deactivation time taking effective in the activation/deactivation delay requirements for multiple pre-configured measurement gaps |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The activation/deactivation delay for multiple pre-configured measurement gaps will not be available |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.19.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--- Start of change 1 ---

8.19	Pre-configured measurement gap activation/deactivation delay

8.19.5	Activation/deactivation delay requirements for multiple pre-configured measurement gaps

The requirements in this clause apply to a UE configured with multiple pre-configured measurement gaps within concurrent gaps.

8.19.5.1	Activation/deactivation delay requirements for non-overlapped activation/deactivation of multiple pre-configured measurement gaps

The requirements in this clause only apply when the activation/deactivation procedures of the individual pre-configured measurement gaps do not overlap in time.

When multiple pre-configured measurement gaps are activated/deactivated non-overlapped upon DCI/timer-based BWP switch, upon SCell activation/deactivation or upon RRC reconfiguration, for each individual pre-configured measuremeng gap, the requirements defined in clauses 8.19.2, 8.19.3 and 8.19.4 apply.

8.19.5.2	Activation/deactivation delay requirements for fully overlapped activation/deactivation of multiple pre-configured measurement gaps

The requirements in this clause only apply when the activation/deactivation procedures of the individual pre-configured measurement gaps fully overlap in time.

Fully overlapped activation/deactivation of pre-configured measuremeng gaps can occur in the following cases:

-	Both pre-configured measurement gaps are triggered by the same event.

-	Two pre-configured measurement gaps are triggered by two events of the same type at the same time.

When multiple pre-configured measuremeng gaps are activated/deactivated simultaneously, the activation/deactivation delay equals the delay requirements defined at clause 8.19.2, 8.19.3 and 8.19.4 for DCI/timer-based BWP switch, SCell activation/deactivation or RRC reconfiguration, respectively, plus an additional 2ms post-processing time.

8.19.5.3	Pre-configured measurement gap activation/deactivation delay when colliding with a measurement gap

When the pre-configured measurement gap activation procedure is overlapped with one measurement gap occasion and the pre-configured measurement gap has higher priority, the pre-configured gap activation shall be applied 5ms after the overlapping measurement gap.

When the pre-configured measurement gap deactivation procedure is overlapped with one measurement gap occasion and the pre-configured measurement gap has higher priority, or when the pre-configured measurement gap activation/deactivation procedure is overlapped with one measurement gap occasion and the measurement gap has higher priority, requirements specified in 8.19.2 apply.

Activation of Pre-MG takes effect from the first MG occasion after the activation delay if the time difference between the last overlapping MG and first Pre-MG occasion to be changed is larger than 5ms. Otherwise, the first Pre-MG occasion shall be kept as deactivated also.

--- End of change 1 ---