3GPP TSG-RAN WG4 Meeting # 110bis	R4-2404573

# Changsha, China, April 15 – April 19, 2024

Source: 	Xiaomi

Title: 	Discussion on testability and interoperability issues for beam management

Agenda Item:	9.11.2

Document for:	Discussion

# 1 Introduction

In this contribution, we will discuss testability issues for beam management.

- Legacy non-AI based BM requirement and test - Test metric for AI based BM - Channel model for AI based BM - Ground truth for AI based BM

- Discussion

      - 2.1 Legacy non-AI based BM requirement and test

The latest agreements in RAN1 #116 meeting are as below:

Agreement For UE-sided model, at least for BM-Case1, for content in the report of inference results, support - Opt 1: Beam information on predicted Top K beam(s) among a set of beams - Opt 2: Beam information on predicted Top K beam(s) among a set of beams and RSRP of predicted Top K beam(s) among a set of beams - At least K=1 and more, FFS on max value - FFS on beam information - FFS on the definition of predicted Top K beam(s) - FFS on definition of reported RSRP when applicable - FFS on other information in the report with potential down selection among the following options - Opt 3: Beam information on predicted Top K beam(s) among a set of beams and probability information of predicted Top K beam(s) among a set of beams - FFS on the quantization method of probability information - Probability information is the probability of the beam to be the Top 1 or Top K beam - Opt 4: Beam information on predicted Top K beam(s) among a set of beams, RSRP of predicted Top K beam(s) among a set of beams, and confidence information of the RSRP - FFS on definition of reported RSRP - FFS on the definition and quantization method of confidence information - Other options are not precluded. where the set of beams is Set A, i.e., the beams for UE prediction.

From RAN1 agreement, both predicted beam index and predicted RSRP are candidate options for L1 beam reporting, similar as legacy non-AI based beam reporting.

Therefore, before we discuss AI based beam reporting requirement and test, let’s review legacy non-AI based beam reporting test metric, requirement and corresponding testcase. Then we can further identify the difference for AI based beam reporting requirement.

In legacy L1-RSRP reporting, two kinds of L1-RSRP accuracy requirement are defined:

- Absolute L1-RSRP accuracy

- Relative L1-RSRP accuracy

Observation 1: In legacy non-AI based L1-RSRP accuracy requirement, both absolute and relative accuracy requirements are defined.

      - 2.1.1 Test metric and test case for absolute L1-RSRP accuracy

For absolute accuracy requirement, the test metric is as below:

Absolute L1-RSRP accuracy = measured L1-RSRP - ideal absolute L1-RSRP

Different from FR1, FR2 test is performed by OTA. Therefore, ideal absolute RSRP is not a single value but a range which consider the minimum and maximum RX beamforming gain. RX beamforming gain range will depend on UE power class and fine beam or rough beam assumption. For L1-RSRP test, fine beam is assumed.

For the testcase defined in A.7.6.3.3.3, two RSs will be configured, e.g. CSI-RS0 and CSI-RS1. The L1-RSRP report shall include the results for both CSI-RS#0 and CSI-RS#1. For absolute accuracy of CSI-RS0 and absolute accuracy of CSI-RS1, the UE is deemed to meet the requirement if the reported L1-RSRP is in the range:

For CSI0:

CSI-RS _RP0 -δ + Gmin ≤ Reported RSRP(dBm) ≤CSI-RS _RP0 +δ + Gmax

For CSI1:

CSI-RS _RP1 -δ + Gmin ≤ Reported RSRP(dBm) ≤CSI-RS _RP1 +δ + Gmax

Where δ is absolute accuracy requirement, e.g. 6.5dB for normal condition. CSI-RS_RPn is the equivalent power received by an antenna with 0dBi gain at the center of the quiet zone configured in the test for the CSI-RS under consideration. Gmin and Gmax are the minimum and maximum UE RX beamforming gain selected according to the UE power class.

From the test procedure, it’s clear that the absolute RSRP difference is the delta between the measured L1-RSRP and ideal RSRP for the same beam index. During Rel-15 L1-RSRP accuracy performance discussion, the simulation results is also provided by assuming that RSRP difference is corresponding to the same beam index. No best beam selection functionality is tested. Therefore, the test metric in legacy absolute L1-RSRP requirement is:

Absolute L1-RSRP accuracy = measured L1-RSRP of beam index n - ideal absolute L1-RSRP of beam index n

Observation 2: For non-AI based absolute L1-RSRP accuracy test metric, ideal RSRP is not a value but a range including minimum and maximum RX beamforming gain. Ideal RSRP is determined by transmit signal power at TE side and RX beamforming gain range of UE.

In other words, it didn’t consider whether best RX beam is chosen or not. Since the whole RX beamforming range is considered.

Observation 3: For non-AI based absolute L1-RSRP accuracy test metric, RSRP difference is measured RSRP with ideal RSRP for the same beam index. No best beam selection functionality is tested.

      - 2.1.2 Test metric and test case for relative L1-RSRP accuracy

The drawback of absolute RSRP requirement is that the ideal RSRP range is too large and it’s hard to verify the UE performance. Therefore, relative RSRP accuracy requirement is also defined. In RAN4 90bis, it’s agreed that:

Agreement: - Use method 3 for testing absolute accuracy - On top of that, - Verify the relative accuracy between different cells in the same test, - Verify the relative accuracy between levels on the same cell in the same direction across different RSRP levels

Relative accuracy test is designed that two CSI-RS will be configured, i.e. CSI-RS 0 and CSI-RS 1. Two RSs are with different RSRP level. If the two RSs are from the same or similar direction, relative accuracy test can reduce or get rid of the large RX beamforming gain impact and verify UE baseband performance. For relative accuracy requirement, the test metric is as below:

Relative L1-RSRP accuracy = measured L1-RSRP of RS0 - measured L1-RSRP of RS1

For relative accuracy testcase, the L1-RSRP difference between reported CSI-RS0 and CSI-RS1 will be calculated and make sure that it will satisfy the relative accuracy requirement:

Difference of two Reported RSRPs ≤ δ

Where δ is relative accuracy requirement, i.e. 6.5dB.

Observation 4: Relative L1-RSRP accuracy is designed to verify baseband performance which can reduce or get rid of the impact of large RX beamforming gain range. Relative accuracy is the measured L1-RSRP difference between two RSs.

      - 2.2 Test metric for AI based BM

In last meeting, down selection for test metric is discussed. The test metrics are as below:

- Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%): the percentage of "the Top-1 genie-aided beam is Top-1 predicted beam" - Top-K/1 (%): the percentage of "the Top-1 genie-aided beam is one of the Top-K predicted beams" - Top-1/K (%) (Optional): the percentage of "the Top-1 predicted beam is one of the Top-K genie-aided beams" - - Where K >1 and values can be reported - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options

First, we would like to share our understanding about which purpose test metric will be used for.

For option 1, RSRP prediction accuracy focus about the RSRP difference for the same beam. RSRP prediction accuracy is more similar as legacy RSRP measurement accuracy definition. RSRP prediction accuracy metric is defined in clause 6.3.1 in TR 38.843:

For AI/ML models, which provide L1-RSRP as the model output, the accuracy of predicted L1-RSRP is to be evaluated. Companies optionally report average (absolute value)/CDF of the predicted L1-RSRP difference, where the predicted L1-RSRP difference is defined as the difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the same beam.

While both option 2 and 3 can be classified into beam prediction accuracy.

In RAN1, the beam prediction accuracy can be defined from two different aspects:

- Type 1: Based on RSRP difference between predicted Top-1 beam and Genie Top-1 beam

- Type 2: Based on beam index difference between predicted Top-1/K beam and Genie Top-1/K beam

Therefore, Option 2 refers to beam index difference while option 3 refers to RSRP difference between predicted best beam and ideal best beam.

Observation 5: For option 1, RSRP prediction accuracy focus about the RSRP difference for the same beam.

Observation 6: Both option 2 and 3 can be classified into beam prediction accuracy. Option 2 refers to beam index difference while option 3 refers to RSRP difference between predicted best beam and ideal best beam.

Besides, we also identify some issues regarding to option 3. In current option 3, it only mentioned that predicted RSRP should be larger than L1-RSRP of strongest beam - X dB. On the other side, predicted RSRP should be smaller than L1-RSRP of strongest beam + X dB. Therefore, Option 3 needs to be updated.

Proposal 1: Option 3 needs to be updated by considering both upper bound and lower bound.

- The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB and smaller than the RSRP of the strongest beam + x dB,

  - Related measurement accuracy can be considered to determine x

Next, we will discuss the relationship between these options.

In last meeting, some company mention that whether beam prediction accuracy requirement can be skipped if RSRP prediction accuracy (Option 1) requirement is defined. From our understanding, whether RSRP prediction accuracy can verify both beam prediction and RSRP accuracy or only RSRP accuracy depends on how accurate the requirement is and the RSRP difference between two ideal best TX beams.

Suppose that the ideal best two TX beam RSRP difference is 2dB. If RSRP prediction for each beam index can be accurate enough, for example, the difference is within 1dB. Then it’s easy for UE to choose the best beam index according to predicted L1-RSRPs. In this case, RSRP prediction accuracy requirement can verify both beam index prediction and RSRP accuracy performance. On the other side, if the RSRP prediction accuracy is not so accurate, e.g. 5dB, then it can’t verify the best beam prediction performance. However, the RSRP difference between two best TX beam may change depends on different scenarios. It can’t guarantee that beam prediction functionality can always been tested. Therefore, we suggest not to skip beam prediction accuracy test.

Besides, in RAN1#116 agreement, beam index reporting is the common part for all options. Therefore, at least beam prediction accuracy related requirement should be considered in RAN4. Since both option 2 and option 3 are defining beam prediction requirement from two different aspects, RAN4 needs to discuss whether to choose one or both of them to define requirement.

Observation 7: RSRP prediction accuracy requirement can verify both RSRP accuracy and beam prediction accuracy only under certain conditions.

For RSRP prediction accuracy, since RAN1 is still discussing whether L1-RSRP will be reported or not, it can be decided based on RAN1 agreement later.

Proposal 2: RAN4 needs to at least define beam prediction accuracy requirement and discuss which test metric to be chosen, e.g. based on beam index difference or RSRP difference, or both.

      - 2.3 Channel model assumption for AI based BM

In last meeting, there are discussions related to channel model assumption in AI based BM. We would like to discuss the issue first. There are two channel model assumptions:

- AWGN channel

- Fading channel

For AWGN channel, signal power will be constant for both spatial and time domain. It’s a deterministic channel.

For Fading channel, it will include variation in both spatial and time domain. It’s a random channel which satisfy certain statistic distribution in spatial and time domain. Therefore, the main difference is whether the channel should be deterministic or random for AI based BM.

For AWGN channel, received L1-RSRP for different TX beam will only depend on TX beamforming gain and RX beamforming gain. In other words, there will only be beamforming gain difference. While for signal itself, it’s constant. For a given array configuration, antenna array gain is fixed. Therefore, AWGN channel model is not suitable since there is no any uncertainty in both spatial and time domain. It’s hard to verify the prediction functionality introduced by AI.

Since AI model needs to predict both spatial and time domain statistic characteristic of channel model, the channel should be random. If everything is deterministic, prediction is not needed.

Proposal 3: AWGN channel is not suitable for beam prediction test since there is no any uncertainty in spatial domain. Random channel should be chosen to verify prediction functionality.

      - 2.4 Ground truth for AI based BM

For AI based BM, predicted best beam index or predicted L1-RSRP will be compared with a ground truth value.

In general, there are two options to define ground truth:

- Option 1: Ideal value, e.g. real best beam index or L1-RSRP value

- Option 2: Measured value, e.g. measured best beam index or L1-RSRP value

      - 2.4.1 Ideal value as ground truth

The key question is whether ideal value can be known by TE. We will discuss the issue for three options respectively.

2.4.1.1 Option 1: RSRP accuracy

For multi-path channel, ideal L1-RSRP will depend on many factors. if TE can know all the factors, then it’s possible. These factors include:

- channel spatial domain information, e.g. AOA, DOA, ZOA, DOA, power of multi-path, etc.

- channel time domain information, e.g. moving direction, speed, power, delay of multi-path, etc.

- TX beamforming gain

- UE implementation related, e.g. RX beamforming gain, which RS is used, etc.

From our understanding, since TE is generating signal, it can know all the channel model related information and TX side implementation except for UE implementation part. In the above factors, channel spatial/time domain information and TX beamforming gain can be known by TE. The unknown part is about UE implementation: which RX beam is used, RX beamforming gain and which RS is used.

RX beamforming gain

For RX beamforming gain and which RX beam is used, the issue can be solved by adding a RX beamforming gain range into ideal L1-RSRP, similar as legacy non-AI based test. The ideal L1-RSRP is a range including the minimum and maximum RX beamforming gain. Of course, the drawback is that the ideal L1-RSRP range will be large. It’s FFS whether RX beamforming gain can be reduced under some conditions or information exchange. Or it’s also possible to introduce relative accuracy test.

Which RS is used

In legacy, ideal value is assumed to be unknow in fading channel since TE didn’t know which RS UE will apply for measurement. The ideal value will change if different RS is chosen.

In RSRP predication accuracy in AI, we would like to further split the issue for BM case-1 and BM case-2.

BM case-1 mainly focuses on prediction on the signal power variation with different angle direction. Therefore, the issue can be solved by assuming a slow time-varying channel. If channel is varying slowly, L1-RSRP of adjacent RS used for L1-RSRP prediction will be similar. In other words, no matter which RS UE is chosen, the L1-RSRP in time-domain will be similar. Besides, For FR2, the main scenario is not for high speed, therefore, it may be also reasonable to assume slow time-varying channel. At least for BM case-1, slow time-varying channel assumption is fine.

BM case-2 mainly focuses on signal power prediction in future time. BM case-2 is more complicated since there may be both spatial domain and time domain variation due to UE movement. We are open to further discuss whether channel model with low speed is reasonable assumption. Again, we would like to point out that for FR2, the main scenario is not for high speed, company can discuss whether RS selection will have impact on the result under certain speed limitation,.

In summary, it’s possible for TE to know ideal L1-RSRP in test under some conditions for BM case-1.

Observation 8: For Option 1, it’s possible that TE knows ideal L1-RSRP at least for BM case-1 under some conditions.

2.4.1.2 Option 2: beam prediction accuracy in terms of beam index difference

For best beam index prediction, the test metric defined in RAN1 is as below:

- Beam prediction accuracy (%): - Top-1 (%): the percentage of "the Top-1 genie-aided beam is Top-1 predicted beam" - Top-K/1 (%): the percentage of "the Top-1 genie-aided beam is one of the Top-K predicted beams" - Top-1/K (%) (Optional): the percentage of "the Top-1 predicted beam is one of the Top-K genie-aided beams" - Where K >1 and values can be reported

The predicted best TX beam index will be compared with Top-1 genie TX beam index. Here, Genie means ideal or true value. For test, we would like to trigger discussion whether ideal Top-1 TX beam index can be known by TE or not.

Similar as analysis for RSRP accuracy, TE can know all the channel model related information and TX side implementation except for UE implementation part. The unknown part is RX beamforming gain and which RS is used. If UE is not moving very fast and channel spatial distribution is not changing rapidly, which RS to be used will not have impact on the TX beam index selection. The main issue lies in RX beam assumption.

Impact of RX beam assumption

As shown in RAN1 agreements below, ground truth is Top-1 genie-aided Tx beam which has two options related to RX beam assumption.

Top-1 genie-aided Tx beam considers the following definitions: - Option A (baseline), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams - Option B (optional), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s) - Specific Rx beam(s) are to be reported. Note: specific Rx beams are a subset of all Rx beams.

NLOS only

In NLOS only environment, there may be many clusters. Different cluster may result in different best AOA and DOA pair.

For option B, the ideal best TX beam index is chosen by specific RX beam. The ideal best TX beam is suboptimal. As shown in Fig.1. If RX beam 1 is assumed, the corresponding ideal best TX beam is TX beam with index 1. This beam pair is related to cluster 1. While if RX beam 2 is assumed, the ideal best TX beam maybe TX beam with index 2. This beam pair is related to cluster 2. If assumed RX beam index change, the ideal best TX beam index may change accordingly. Therefore, for specific RX beam case, the best TX beam index is determined by cluster location and RX beam direction.

For TE, suppose it can know cluster location, however it didn’t know detail RX beam information as it’s UE implementation. Without the information of RX beam direction, TE can’t know what’s the corresponding ideal Top-1 TX beam.

Fig.1 best TX and RX beam pair in NLOS only case

For option A, the ideal best TX beam index is chosen by considering all RX beam sweepings. The ideal best TX beam index is global optimization in theory. However, in reality, UE can only apply limited RX beam sweepings, e.g. N=8. Then there are many different RX beam angle sets from different UE implementation. Different RX angle set may also correspond to different best TX beam index. Therefore, TE can’t know what’s the ideal best TX beam index either.

In summary, in multi-path case, TE side can’t know the exact ideal best TX beam index. In this case, some margin needs to be added. Similar as legacy where ideal RSRP is a range by considering RX beamforming gain, here, ideal TX beam index can also be a TX beam index set. For example, the ideal TX beam set can be combination of several TX beam indexes.

LOS+NLOS case

In LOS+NLOS or LOS-only scenario, LOS ray will be the strongest. For LOS ray, the best TX beam direction is only determined by the location of transmitter and receiver. The best Tx beam index is not related to which RX beam is chosen. Therefore, if TE knows the UE location, TE can know the ideal best TX beam index.

Fig.2 best TX and RX beam pair in LOS case

Observation 9: For option 2, TE may not know the ideal best TX beam index for some cases.

2.4.1.3 Option 3: beam prediction accuracy in terms of RSRP difference

In option 3, the RSRP of predicted best TX beam will be compared with RSRP of the strongest TX beam index. However, as discussed for option 2, ideal strongest TX beam index may not be known by TE in some cases. Besides, TE didn’t know RX beamforming gain. For BM case-2, TE didn’t know which RS UE will use in fading channel. It’s more challenging for TE to know the ideal L1-RSRP for the strongest TX beam.

Observation 10: For option 3, it’s more challenging for TE to know the corresponding L1-RSRP for the strongest TX beam.

In summary, we suggest to discuss whether ideal value can be known for each test metric respectively. Besides, we also suggest to split the discussion for BM case-1 and BM case-2.

Proposal 4: Regarding to whether ideal value can be known or not, we propose:

- RAN4 to discuss for each test metric respectively

- RAN4 to discuss for BM case-1 and BM case-2 respectively

- RAN4 to discuss possible condition if ideal value can be known

      - 2.4.2 Measured value as ground truth

If measured value is used as ground truth, then:

- For Option 1, the ground truth will be the measured L1-RSRP of Top-1 predicted beam

- For Option 2, the ground truth will be the measured best TX beam index

- For Option 3, the ground truth will be measured L1-RSRP from the measured strongest TX beam

When measured value is used as ground truth, it’s the requirement from another aspect. Actually, it’s not conflicting with ideal value. It can reflect the difference between prediction and measurement. It’s some kind of relative accuracy.

When measured value is used as ground truth, there are pros and cons:

- Pros: Ground truth can always be obtained even in fading channel. Besides, there is no uncertainty since all UE implementation has already been included in measured value.

- Cons: It’s easy for UE to pass the test since UE can know the measured value and predicted value. Besides, it can’t reflect the difference with real value.

Observation 11: If measured value is used as ground truth, it’s the requirement from another aspect. It’s some kind of relative accuracy compared with measurement. There are both pros and cons.

Proposal 5: RAN4 to discuss whether measured value can be used as ground truth.

- Conclusion

In this contribution, we provide the following proposals:

Observation 1: In legacy non-AI based L1-RSRP accuracy requirement, both absolute and relative accuracy requirements are defined.

Observation 2: For non-AI based absolute L1-RSRP accuracy test metric, ideal RSRP is not a value but a range considering minimum and maximum RX beamforming gain. Ideal RSRP is determined by transmit signal power at TE side and RX beamforming gain range of UE.

Observation 3: For non-AI based absolute L1-RSRP accuracy test metric, RSRP difference is measured RSRP with ideal RSRP for the same beam index. No best beam selection functionality is tested.

Observation 4: Relative L1-RSRP accuracy is designed to verify baseband performance which can reduce or get rid of the impact of large RX beamforming gain range. Relative accuracy is the measured L1-RSRP difference between two RSs.

Observation 5: For option 1, RSRP prediction accuracy focus about the RSRP difference for the same beam.

Observation 6: Both option 2 and 3 can be classified into beam prediction accuracy. Option 2 refers to beam index difference while option 3 refers to RSRP difference between predicted best beam and ideal best beam.

Proposal 1: Option 3 needs to be updated by considering both upper bound and lower bound.

- The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB and smaller than the RSRP of the strongest beam + x dB,

  - Related measurement accuracy can be considered to determine x

Observation 7: RSRP prediction accuracy requirement can verify both RSRP accuracy and beam prediction accuracy only under certain conditions.

Proposal 2: RAN4 needs to at least define beam prediction accuracy requirement and discuss which test metric to be chosen, e.g. based on beam index difference or RSRP difference, or both.

Proposal 3: AWGN channel is not suitable for beam prediction test since there is no any uncertainty in spatial domain. Random channel should be chosen to verify prediction functionality.

Observation 8: For Option 1, it’s possible that TE knows ideal L1-RSRP at least for BM case-1 under some conditions.

Observation 9: For option 2, TE may not know the ideal best TX beam index for some cases.

Observation 10: For option 3, it’s more challenging for TE to know the corresponding L1-RSRP for the strongest TX beam.

Proposal 4: Regarding to whether ideal value can be known or not, we propose:

- RAN4 to discuss for each test metric respectively

- RAN4 to discuss for BM case-1 and BM case-2 respectively

- RAN4 to discuss possible condition if ideal value can be known

Observation 11: If measured value is used as ground truth, it’s the requirement from another aspect. It’s some kind of relative accuracy compared with measurement. There are both pros and cons.

Proposal 5: RAN4 to discuss whether measured value can be used as ground truth.

- Reference

- TR38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface