3GPP TSG-RAN WG4 Meeting#110bis	R4-2404630

# Changsha, China, 15 – 19 April 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | <CR#> | rev | - | Current version: | 17.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.101-1: MPR and A-MPR per serving cell with a derated per-band power class for UL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Power_Limit_CA_DC |  |  |  |  |  | Date: |  |  | 2024-04-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The applicable MPR/A-MPR for a serving cell c is unclear in case the PCMAX,f,c is reduced (derated) by a per-band power class of the band combination (BC) lower than the NR band power class. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 6.2A.2.0 (new): the MPRc for a serving cell c within an NR band of a configured band combination is determined according to - the maximum UE power for the NR band as indicated by ue-PowerClassPerBandPerBC-r17, if present, ue-PowerClass in BandNR otherwise, - the number of transmitter antenna connectors used for transmission and other conditions (PRB allocations etc) specified in the sub-clauses specifying the allowed MPR for different types of CA configurations. The MPRc is not changed by the per-BC power class (determines when the UE prioritizes the output power) when lower than the per-band power class or by a configured P-Max when configured, only by the power class for a band. Clause 6.2A.3.0 (new): the corresponding change for A-MPRc for a serving cell c. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The applicable MPR/A-MPR for a serving cell c within an NR band is unclear in case the PCMAX,f,c is reduced by a per-band power class of the band combination as indicated by the ue-PowerClassPerBandPerBC-r17 replacing the ue-PowerClass of the NR band. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2.0 (new), 6.2A.3.0 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | . |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Note to MCC: the changes in this draft CR are based on v17.12.0. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

### 6.2A.2	UE maximum output power reduction for CA

#### 6.2A.2.0	General

The allowed maximum power reduction per serving cell c (MPRc) within an NR band of a configured band combination is determined according to

-	the maximum UE power for the NR band specified in Table 6.2.1-1 without account of the tolerance as indicated by ue-PowerClassPerBandPerBC-r17, if present, ue-PowerClass/ue-PowerClass-v1610 in BandNR otherwise,

-	the number of transmitter antenna connectors used for transmission

### and the conditions specified in the following sub-clauses.

#### 6.2A.2.1	UE maximum output power reduction for Intra-band contiguous CA

< text omitted >

### 6.2A.3	UE additional maximum output power reduction for CA

#### 6.2A.3.0	General

The allowed additional maximum power reduction per serving cell c (A-MPRc) within an NR band of a configured band combination is determined according to

-	the maximum UE power for the NR band specified in Table 6.2.1-1 without account of the tolerance as indicated by ue-PowerClassPerBandPerBC-r17, if present, ue-PowerClass/ue-PowerClass-v1610 in BandNR otherwise,

-	the number of transmitter antenna connectors used for transmission

and the conditions specified in the following sub-clauses.

#### 6.2A.3.1	 UE additional maximum output power reduction for Intra-band CA

< end of changes >