3GPP TSG-RAN WG4 Meeting #110bis		R4-2404656

Changsha, China, 15th – 19th April, 2024

Agenda item:	 9.14.2

Source:	 vivo

Title:	Views on LP-WUS UE RF requirements

Document for:	 Approval

- Introduction

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

RAN plenary approved the Rel-19 LP-WUS WID with the following RAN4 RF scope [1]:

This contribution shares initial views on LP-WUS UE RF requirements.

- LP-WUS UE RF

  - New methodology/framework to specify LP-WUS receiver requirements

Issue 1-6-1: Performance metric for LP-WUR RF requirements

Agreements:

RAN4 should discuss a new the methodology for WUR requirements in WI phase. Following can be considered:

- BLER

- Misdetection ratio, including paging failure

- others

Issue 1-6-2: LP-WUR Sensitivity

Agreements:

- FFS, sensitivity requirement depends on RAN-P decision on LP-WUS coverage, which can be discussed in WI phase.

Issue 1-6-3: LP-WUR test cases

Agreements:

- The testability issue of LP-WUR RF requirements should be discussed in WI phase.

As discussed in the SI phase, given LR has no DL throughput, a new methodology to quantify receiver requirements is needed, and initial agreements have been reached in RAN4#108bis meeting [2]:

Based on large amount of link-level simulation in SI phase (also captured in TR 38.869), seems 1% BLER as new metric can be a good starting point for further discussions.

Proposal 1: Use [1%] BLER as metric to specify LP-WUS receiver requirements.

As also discussed in SI, the testability issue of LP-WUS BLER should also be resolved, according to the LP-WUR mode, there could be two general directions to go as starting point, e.g., MR idle mode and MR connection mode.

Proposal 2: For LP-WUR testability issue, RAN4 can consider the following two options, e.g.,

- MR idle mode, LP-WUS no feedback. New UE test mode for LP-WUS testing

- MR connection mode, LP-WUS testing by MR close-loop feedback

  - ASCS requirements

Conclusion for RAN4 ASCS guard RB analysis:

Based on the following analysis, RAN4 observed that for 5th order filter, the guard RB number for LP-WUS ASCS is in the range of 0RB ~ 1RBs for 30KHz SCS, or 0RBs ~2RBs for 15KHz SCS. Similar number of guard RBs could be applied also with lower filter orders as performance was observed to remain similar with 3rd order filter.

SI phase reach conclusions on quite converged number of guard RBs for ASCS requirements, in TR38.869:

The number of guard RB has been limited to 0~ 2RBs for 15kHz SCS case with 5th order filter assumption, which will be further converged to single value.

However, another issue is that SI phase does not discuss the specific ASCS value, which is a key task in WI phase. Based on SI outcome, the ASCS value (filter suppression level) with guard RB as 0RB~2RB could be: 18dB ~23dB, under the assumption of max 50ppm CFO.

Proposal 3: RAN4 should specify the ASCS requirements, the value between [18~23]dB from SI phase can be a starting point. The corresponding required min number of guard RB should also be specified.

ASCS mainly impacts on the in-channel performance, so traditional ACLR/ACS based co-existence simulation is not feasible. To further specify a dedicated value for ASCS and guard RB, ASCS related link-level simulations should be re-evaluated with more aligned parameters and RF impairments.

Proposal 4: To specify the ASCS value and number of guard RB, RAN4 should perform new link-level simulation with the aligned single-set of parameters (including RF impairments), e.g., channel structure, waveform, sampling rate, ADC bit, channel model, performance metric.

  - ACS requirements

Conclusions for RAN4 ACS guard RB analysis:

Based on the analysis above, RAN4 observed that for 5th order filter, the guard RB number for LP-WUS ACS is in the range of 1RB ~ 6RBs for 30kHz SCS, or 2RBs ~12RBs for 15kHz SCS. For lower power, i.e., lower order filters, the required guard RBs may increase. There is no recommendation on which filter order should be considered to the baseline in SI phase.

-	As the analysis from companies did not comprehensively consider all impairments, and individual results are missing the impact of one or many of the following: phase noise, non-linearity, frequency error, implementation margin. For the evaluation, companies have used OOK waveform and FSK waveform, other type of WUS signals are not used. Final conclusion on the number of guard RB is left to WI discussion.

For ACS, the SI phase study outcome presents the required large number of guard RBs, in TR38.869:

Some the of reason is the unaligned assumption and methodology among companies, the study is just initial estimation from interested companies. The evaluated number of guard RB is quite diverged and different RF impairments have been considered by companies.

Besides, in SI phase the ACS value of MR is reused for evaluation purpose, but for LR, given different architecture (RF elements), waveform and receiver type, the required actual ACS value would be different.

Proposal 5: RAN4 should specify ACS requirement for LP-WUS, FFS same as MR or not. The corresponding required min number of guard RB should also be specified.

Then RAN4 need to discuss how to specify the ACS requirement for LR, given MR ACS is traditionally specified based on system-level coexistence simulation under 5% TP loss, RAN4 needs to discuss and decide whether co-existence simulation is needed (with the new methodology of BLER if agreed) to specify required LR ACS value. Or a link-level simulation approach similar to ASCS evaluation is sufficient.

- Approach 1: Co-existence evaluation based on system-level simulation

This approach is the traditional way used in RAN4, but since WUS cannot use throughput as performance metric, a new metric needs to be defined to determine whether performance degradation due to interference is acceptable, e.g., mapping between SINR and BLER, SINR loss, etc. The following assumption can be the starting point if this approach will be performed.

Table 1 simulation assumption for system level coexistence evaluation

| Parameters |  | Indoor | Urban macro |
| --- | --- | --- | --- |
| Network layout |  | 50m x 120m, 12BSs | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| Inter-site distance |  | 20m | 500m |
| BS antenna height |  | 3 m (ceiling) | 25 m |
| UE location | Outdoor/indoor | Indoor | 20% Outdoor |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | Low/high Penetration loss ratio |  | 50% low loss, 50% high loss |
|  | UE antenna height | 1 m | Same as 3D-UMa in TR 36.873 |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS - UE distance (2D) |  | 0 m | 35 m |
| BS antenna configurations |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1) (dH, dV) = (0.5, 0.5) λ | (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) (dH, dV) = (0.5, 0.8) λ |
| UE antenna configurations |  | (M, N, P, Mg, Ng) = (1, 1, 2, 1, 1) (dH, dV) = (0.5, 0.5) λ Omnidirectional pattern |  |
| Channel model |  | Same as TR 38.901 |  |
| Channel bandwidth |  | 20 MHz |  |
| Scheduled channel bandwidth per UE (DL) |  | 20 MHz |  |
| The number of active UE (DL) |  | Same as the number of BS beam |  |
| Traffic model |  | Full buffer |  |
| DL power control |  | NO |  |
| BS max TX power in dBm |  | 24dBm | 49dBm |
| WUR Noise figure in dB |  | [9 dB/15 dB] |  |
| Handover margin |  | 3dB |  |

- Approach 2: Co-existence evaluation based on link-level simulation (using system-level simulation just to get interference distribution)

Considering the performance metric may hard to be aligned across companies, an alternative is to use link-level simulation which is similar to the ASCS evaluation. In the SI stage, the link-level simulation was also performed by some companies and the 31.5 dB higher is used as interference which comes from NR ACS test case, but it cannot represent the real interference condition in the different scenarios for LP-WUS receiver.

For this approach, a CDF of SIR (signal-interference ratio) should be derived from system-level simulation to represent the interference distribution for a dedicated scenario, then RAN4 should discuss one reasonable interference power level and put it into the link level simulation with different ACIR, to observe the performance degradation. In this approach, BLER can be the metric and most simulation assumption should be same as ASCS evaluation.

Proposal 6: RAN4 should decide whether coexistence system-level simulation (Approach 1) is needed to specify ACS requirements for LP-WUR. If needed, the simulation parameters should be aligned first, and system-level performance metric should be decided.

  - REFSENS requirements

For REFSENS, the traditional approach for MR is using the following equation with consideration of different implementations:

Sensitivity = -174dBm(kT) + 10*log(RX BW) + NF + SNR +IM – diversity gain

Where,

- NF: 9~10.5dB (MR different band)

- SNR= -1dB (at baseband, assume cell edge for QPSK 1/3 coding rate)

- IM (Implementation Margin) = ~2.5 dB

- diversity gain = 3dB for 2Rx

However, for LP-WUS receiver, before going into details, RAN4 should confirm first whether the above equation approach can be reused. If this can be confirmed then the group could focus on next-step SNR, NF and potential IM value discussions.

Proposal 7: RAN4 should discuss and decide whether reusing the same approach of MR to derive REFSENS value for LP-WUS receiver.

Proposal 8: RAN4 should discuss and decide how to specify the SNR value.

  - Other UE RF requirements

The above ACS, ASCS, REFSENS are key requirements to be defined for LR with the assuming that most of other receiver requirements can be reused from MR, however RAN4 also need to check all the receiver requirements for LP-WUS to see whether there is any impact.

Proposal 9: RAN4 further check whether other receiver requirements should be re-evaluated and specified for LR.

- Conclusion

In this contribution, we share our views on LP-WUR RF and have the following proposals:

Proposal 1: Use [1%] BLER as metric to specify LP-WUS receiver requirements.

Proposal 2: For LP-WUR testability issue, RAN4 can consider the following two options, e.g.,

- MR idle mode, LP-WUS no feedback. New UE test mode for LP-WUS testing

- MR connection mode, LP-WUS testing by MR close-loop feedback

Proposal 3: RAN4 should specify the ASCS requirements, the value between [18~23]dB from SI phase can be a starting point. The corresponding required min number of guard RB should also be specified.

Proposal 4: To specify the ASCS value and number of guard RB, RAN4 should perform new link-level simulation with the aligned single-set of parameters (including RF impairments), e.g., channel structure, waveform, sampling rate, ADC bit, channel model, performance metric.

Proposal 5: RAN4 should specify ACS requirement for LP-WUS, FFS same as MR or not. The corresponding required min number of guard RB should also be specified.

Proposal 6: RAN4 should decide whether coexistence system-level simulation (Approach 1) is needed to specify ACS requirements for LP-WUR. If needed, the simulation parameters should be aligned first, and system-level performance metric should be decided.

Proposal 7: RAN4 should discuss and decide whether reusing the same approach of MR to derive REFSENS value for LP-WUS receiver.

Proposal 8: RAN4 should discuss and decide how to specify the SNR value.

Proposal 9: RAN4 further check whether other receiver requirements should be re-evaluated and specified for LR.

# References

- RP-240801, Revised WID Low-power wake-up signal and receiver for NR (LP-WUS_WUR), vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony, RAN meeting #103, Mar 2024.

- R4-2317766, WF on UE RF part for LP_WUS, vivo, RAN4#108bis

- TR 38.869 LP-WUS RAN1 TR

# Annex

Simulation parameters (including RF impairments value) should be well-aligned

Table 1: Link-level Simulation parameters template

| NR system BW |  |
| --- | --- |
| Subcarrier spacing |  |
| Guardband of NR channel |  |
| LP-WUR Rx antenna |  |
| WUS waveform |  |
| WUS BW within NR channel |  |
| WUS signal |  |
| Guard RB size of LP-WUS |  |
| WUS placement within NR channel |  |
| Channel structure |  |
| Coding |  |
| ACS interferer signal |  |
| ACS interferer signal level |  |
| Adjacent subcarrier interference |  |
| Filter characteristic |  |
| Filter passband BW (-3 dB) |  |
| LO frequency |  |
| Center Frequency error |  |
| Phase noise |  |
| Non-linearities |  |
| Channel model |  |
| Sampling rate |  |
| ADC bit |  |
| BS Power boosting |  |
| Performance metric |  |