# 3GPP TSG-RAN WG4 Meeting #110bis				            R4-2404676

# Changsha, China, 15th – 19th April, 2024

Agenda Item:	4.2.1.1

Source: 	CMCC

Title: 			 Discussion on mandatory of enhanced channel raster

Document for:	Discussion

- Introduction

In last RAN4 meeting, the UE FG enhanced channel raster was discussed. However, whether the feature is optional or mandatory for different UE types is still FFS.

In last RAN4 meeting, good progress had been made for channel raster enhancements. It was agreed to specify a new channel raster for both BS and UE with step size 10KHz. And whether the support of new channel raster for each band is mandatory or optional depends on operators’ input.

In this contribution, we give our input on the bands that should mandatory support the new channel raster.

In this contribution, we further discuss the UE capability for enhanced channel raster.

- Discussion

In RAN4#110 meeting, it was agreed that new enhanced channel raster can be supported as mandatory in NR band n1, n2, n3, n5, n25, n28, n66, n71 and n85 from Rel-18.

Issue 2-1-1: New channel raster table format to distinguish mandatory or optional feature in TS38.101-1 Agreement: - Based on discussion papers from interested operators, RAN4 can support the enhanced channel raster as mandatory feature in NR Band n1, n2, n3, n5, n25, n28, n66, n71 and n85 from Rel-18.

However, for RedCap, the conclusion of mandatory or optional is still FFS. The main difference between RedCap UE and normal UE is that RedCap UE has limitation on the supported maximum channel bandwidth, i.e. 20MHz. Hence, we propose that enhanced channel raster should be mandatory for all RedCap UEs.

Proposal: It is proposed that enhanced channel raster is mandatory for RedCap UEs.

- Conclusions

In this contribution, we further discuss the mandatory of enhanced channel raster:

Proposal 1: It is proposed that enhanced channel raster is mandatory for RedCap UEs.

- Reference

R4-2403842, RAN4 UE feature list for Rel-18 (version 3).

R4-2321993, RAN4 UE feature list for Rel-18 (version 2).

R4-2318929, Discussion on the bands that mandatory support of new channel raster, CMCC.

R4-2317607, WF on NR channel raster enhancement.

R4-2317248, Topic summary for [108-bis][124] NR_channel_raster_enh.

R4-2314681, WF on channel raster enhancement

R4-2314211, Topic summary for [108][129] NR_channel_raster_enh

R4-2310269, WF on NR channel raster enhancement, Ericsson

R4-2310012, Topic summary for [107][129] NR_channel_raster_enh.

R4-2306598, WF on channel raster enhancement, Ericsson.

R4-2306288, Topic summary for [106-bis-e][126] NR_channel_raster_enh.

RP-230813, New WID on Channel raster enhancement, Ericsson.