3GPP TSG-RAN WG4 Meeting # 110bis	R4-2404732

# Changsha, China, 15 – 19 April, 2024

Source:	LG Electronics

Title:	Discussion on RRM requirements for LP-WUS/WUR

Agenda item:	9.14.4

Document for:	Discussion

Introduction

In agreed WID for LP-WUR/WUR [1], following objectives were captured for RAN4.

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline - Specify necessary RRM requirements

In this contribution, we provide our views on RRM core requirements and scope on NR LP-WUS/WUR based on the above objectives.

Discussion

Scope of LP-WUS/WUR for RAN4 RRM

Based on the objectives for LP-WUS/WUR, RAN4 needs to clarify the overall scope of RRM core requirements. LP-WUS design commonly applicable to both IDLE/INCATIVE and CONNECTED modes, but RAN4 needs to only focus on IDLE/INACTIVE mode related core requirements.

- Proposal 1: Focus on RRM core requirements only for IDLE/INACTIVE mode in Rel-19

For IDLE/INACTIVE mode, there are two types of LP-WUR for synchronization; one is LP-SS based synchronization and other is existing PSS/SSS based synchronization as Note in objectives below.

- Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS.

Depednin on LP-WUR receiver type, serving cell measurement based on LP-SS and SSS could be different, and LP-SS waveform is discussed in RAN1. Therefore, RAN4 needs to consider both types of LP-WUR when defining RRM core requirements if needed.

- Proposal 2: Consider both types of LP-WUR that can recive LP-SS and existing PSS/SSS when defining RRM core requirements if needed after LP-SS design is finalized in RAN1.

RRM requirements for LP-WUS/WUR in IDLE/INACTIVE mode

For entry/exit condition for LP-WUS monitoring, in SI phase, RAN2 has discussed and concluded the condition(s) could be at least based on the serving cell measurement using LR and/or MR. However, further discussion for the conditions is also needed from a RAN4 perspective.

- Proposal 3: Study other conditions for the entry/exit condition for LP-WUS monitoring other than the measurement based condition from the RAN4 pespective.

For RRM measurement relaxation, in case of UE serving cell measurement offloaded from MR to LP-WUR, the UE does not perform intra/inter-frequency and inter-RAT measurement, and only serving cell measurement is performed by LP-WUR. The condition for serving cell measurement offloaded from MR to LP-WUR would be the same condition for entry/exit for LP-WUS monitoring. However, RAN4 needs further discussion whether to be applicable for the same criterion for intra/inter-frequency and inter-RAT measurement such as Srxlev/Squal for LP-WUR since the measurement accuracy using MR and LP-WUR would be different.

- Proposal 4: Discuss if triggering conditions for intra/inter-frequency and inter-RAT measurement based on LP-WUR are possible or if other conditions should be defined

Conclusion

In this contribution, we provide views on scope and RRM core requirements on NR LP-WUS/WUR, and we propose

- Proposal 1: Focus on RRM core requirements only for IDLE/INACTIVE mode in Rel-19

- Proposal 2: Consider both types of LP-WUR that can recive LP-SS and existing PSS/SSS when defining RRM core requirements if needed after LP-SS design is finalized in RAN1.

- Proposal 3: Study other conditions for the entry/exit condition for LP-WUS monitoring other than the measurement based condition from the RAN4 pespective.

- Proposal 4: Discuss if triggering conditions for intra/inter-frequency and inter-RAT measurement based on LP-WUR are possible or if other conditions should be defined

Reference

[1] RP-240801, “Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR ),” vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony.