3GPP TSG-RAN WG4 Meeting #110bis		R4-2404741

Changsha, China, 15th – 19th April 2024

Agenda item:	6.5

Source:	Intel Corporation

Title:	Feature list proposals for measurement gap enhancements

Document for:	Discussion

# Introduction

Discussions on RAN4 Rel-18 UE feature list started from RAN4 #109 (November 2023) and this paper summarizes our views on the proposals for measurements gap enhancements Rel-18 work item.

# Rel-18 UE feature list

## NR_MG_enh2

The feature list for NR_MG_enh2 is nearly completed. In the below tables we propose to complete the feature lists for both NR and LTE features for the work item. To note that we propose to add the simultaneous reception of different numerologies between target NR 30kHz SSB and serving LTE 15kHz scheduling. This is necessary in the same way as the ones we introduced for NR to LTE measurements.

Table 1: Rel-18 NR UE features for NR_MG_enh2 WI.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_MG_enh2 | 32-1 | Concurrent gaps with Pre-MG in a FR | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) Pre-MG. Details in Clause [9.1.x.2] of TS 38.133. | 19-3-x and 19-2 x = 1 or 2 | Yes | No | UE behaviour is undefined if the network configures concurrent MGs where at least one of the gaps is a Pre-MG | Per UE | No | No | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | [32-2] | Two Pre-MG configuration with simultaneous activation/deactivation | Support configurations of two Pre-MG with simultaneous activation/deactivation in the same FR. | 32-1 | Yes | No | UE activation/deactivation time for simultaneous Pre-MG is undefined | Per UE | No | No | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | [32-3] | Support for dynamic collisions | Support RRM requirements for handling dynamic collisions between a Pre-MG and another measurement gap or Pre-MG. | 32-1 | Yes | No | UE is not expected to meet RRM requirements for dynamic collisions | Per UE | No | No | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-4 | Concurrent gaps with NCSG in a FR | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) NCSG. Details in clause [9.1.y.2] of TS 38.133. | 19-1 and 19-2 | Yes | No | UE behaviour is undefined if the network configures concurrent MGs where at least one of the gaps is a NCSG | Per UE | No | No | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | [32-5] | Two NCSG configuration in a FR | Support configurations of two NCSG in the same FR | 32-4 | Yes | No | UE behaviour is undefined if the network configures two NCSGs in the same FR | Per UE | No | No | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | [32-6] | [Inter-RAT EUTRAN measurements without gap and outside active DL BWP] | Support of requirements of inter-RAT EUTRAN measurements outside active DL BWP without gap with or without interruption. | No | Yes | NA | UE behaviour of supporting inter-RAT EUTRAN measurements without gap is known to network | Per UE | No | No | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-7 | Inter-RAT EUTRAN measurement without gap [and within active DL BWP] | Support of inter-RAT EUTRAN measurements without gap when CRS is contained within UE’s active DL BWP | No | Yes | No | Measurement gap will be needed for inter-RAT EUTRAN measurements | Per UE | No | FR1 only | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-8 | Effective measurement window for inter-RAT EUTRAN measurements | Support configuration of effective measurement window for inter-RAT EUTRAN measurements, including offset, duration and periodicity. | 32-6 or 32-7 | Yes | No | Undefined UE measurement behaviour and when to allow scheduling restriction | Per UE | No | No | N.A | - A bitmap for 6 effective measurement window (EMW) patterns defined in TS 38.133. - #0 and #1 are mandatory, if UE supports EMW feature. - FFS other conditions, e.g., UE supports Case b-1 or b-2 Other patterns are optional | Optional with capability signalling |
| 32. NR_MG_enh2 | 32-9 | Simultaneous reception of NR data and EUTRAN CRS within BWP with different numerology | Support concurrent inter-RAT measurement on EUTRAN cell in non-DSS with CRS contained within UE’s active DL BWP and PDCCH or PDSCH reception from the serving cell with a different numerology | 32-7 | Yes | No | scheduling restriction is applicable | Per UE | No | FR1 only | N.A |  | Optional with capability signalling |
| 32. NR_MG_enh2 | [32-10] | Simultaneous reception of NR data and EUTRAN CRS outside BWP with different numerology | Support concurrent inter-RAT measurement on EUTRAN cell in non-DSS with CRS not contained within UE’s active DL BWP and PDCCH or PDSCH reception from the serving cell with a different numerology. | 32-6 | Yes | No | scheduling restriction is applicable | Per UE | No | FR1 only | NA |  | Optional with capability signalling |

Table 2: Rel-18 LTE UE features for NR_MG_enh2 WI.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_MG_enh2 | [x-y] | InterRAT-BandInfoNR-r18 | Support of inter-RAT NR measurements without gap with or without interruption. This feature has a defined RRC signalling: ‘InterRAT-BandInfoNR-r18’ |  | Yes | NA | The UE does not support inter-RAT NR measurements without gap with or without interruption for performing inter-RAT NR measurement without gap | Per UE | No | No | NA | # [Component 1 candidate value: “{no-gap-with-interruption, no-gap-no-interruption}”] | Optional with capability signalling |
|  | [x-y] | Simultaneous reception of LTE data and NR SSB with 30kHz SCS | Support concurrent inter-RAT NR measurement with a different numerology than 15kHz and reception from LTE serving cell |  | Yes | NA | Scheduling restriction is applicable | Per UE | No | No | NA |  | Optional with capability signalling |

# Conclusion

This paper summarizes our proposals for general principles and the specific feature groups in Rel-18 measurement gap enhancement work item.

# References

[1] R4-2403842	Rel-18 RAN4 UE feature list for NR (version 3), CMCC