3GPP TSG-RAN4 Meeting #110bis	R4-2404844

# Changsha, China, April 15 – 19, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | draft | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR on SL-U A-MPR for NS_28 and NS_30 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Only 5 NS values are specified in Rel-18 SL-U. The remaining 12 NS values are needed to be defined based on the RAN4#110 agreement(R4-2403868) and Rel-19 SL CA approved in RAN#103(RP-240840) which targets only SL intra-band non-contigous CA and SL intra-band contiguous CA in n47. - RAN4#110 agreement - If SL is agreed as Rel-19 RAN4-led package and the remaining NS values are included, specify them in Rel-19. If not, specify them in Rel-18 maintenance. - Rel-19 SL CA - NR Sidelink: Intra-band Carrier Aggregation in ITS band And, for NS_31 A-MPR, NOTE3 and NOTE4 in Table 6.2E.3F.2-2, NOTE2 and NOTE3 in Table 6.2E.3F.2-3 are not necessary. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For NS_31 A-MPR, - NOTE3 : Void, and NOTE4 : Void in Table 6.2E.3F.2-2 - NOTE2 : Void, and NOTE3 : Void in Table 6.2E.3F.2-3 - Specify NS_28 and NS_30 A-MPR requirements for SL-U. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. For NS_31 A-MPR, some NOTEs in Table 6.2E.3F.2-2 and Table 6.2E.3F.2-3 are remained unnecessarily. 2. The remaining NS values are missed in SL-U. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.3F.2, 6.2E.3F.8, 6.2E.3F.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

### 6.2E.3F	UE additional maximum output power reduction for Sidelink Unlicensed

#### 6.2E.3F.1	General

Additional emission requirements can be signalled by the network or pre-configured radio parameters. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [7].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2E.1F-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2E.2F.

Table 6.2E.3F.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2E.3F.1-1A.

Table 6.2E.3F.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (clause) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | n46, n96 | 20, 40, 60, 80 |  | N/A |
| NS_28 | 6.5F.3.3.1 | n46 | 20, 40, 60, 80 |  | 6.2E.3F.8 |
| NS_30 | 6.5F.3.3.3 | n46 | 20, 40, 60, 80 |  | 6.2E.3F.9 |
| NS_31 | 6.5F.3.3.4 | n46 | 20, 40, 60, 80 |  | 6.2E.3F.2 |
| NS_53 | 6.5F.3.3.5 | n96 | 20, 40, 60, 80, 100 |  | 6.2E.3F.3 |
| NS_58 | 6.5F.3.3.6 | n102 | 20, 40, 60, 80 |  | 6.2E.3F.4 |
| NS_60 | 6.5F.3.3.5 | n96 | 20, 40, 60, 80, 100 |  | 6.2E.3F.5 |
| NS_61 | 6.5F.3.3.7 | n96 | 20, 40, 60, 80, 100 |  | 6.2E.3F.6 |
| NOTE 1: The A-MPR shall apply to all active 20 MHz sub-bands contiguously allocated in the channel. |  |  |  |  |  |

[The NS_01 label with the field additionalPmax [7] absent is default for all NR bands.]

Table 6.2E.3F.1-1A: Mapping of network signaling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n46 | NS_01 | NS_28 |  | NS_30 | NS_31 |  |  | Reserved |
| n96 | NS_01 | NS_53 |  |  | NS_60 | NS_61 |  | Reserved |
| n102 | NS_01 | NS_58 |  |  |  |  |  | Reserved |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [7]. |  |  |  |  |  |  |  |  |

#### 6.2E.3F.2	A-MPR for NS_31

When NS_31 is indicated by the network or pre-configured radio parameters for NR sidelink UE, this clause specifies the additional maximum output power reduction  for NR sidelink physical channels and signals due to PSCCH/PSSCH, PSFCH and S-SSB transmission.

For contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed A-MPR is specified in Table 6.2F.3F.2-1 for power class 5 NR sidelink UE.

Table 6.2E.3F.2-1 A-MPR for NS_31 NR SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation (Note 4) |  |  |  | RB Allocation (Note 3) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 6.5 | ≤ 4.5 | ≤ 6.5 | See Table 6.2E.2F-1 |
|  | 16 QAM | ≤ 5.5 | ≤ 7.0 | ≤ 4.5 | ≤ 7.0 |  |
|  | 64 QAM | ≤ 5.5 | ≤ 7.0 | ≤ 4.5 | ≤ 7.0 |  |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |  |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5745, 5765, 5785, and 5805 MHz. NOTE 4: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 5: Contiguous sub-band configuration in Table 6.2E.2F-3 applies. |  |  |  |  |  |  |

For PSFCH transmission with single RB set and multiple RB sets, the allowed A-MPR is specified in Table 6.2E.3F.2-2 for power class 5 NR sidelink UE.

Table 6.2E.3F.2-2 A-MPR for NS_31 for PSFCH transmission for NR SL-U UE power class 5

|  | RB Allocation |  |
| --- | --- | --- |
|  | Outer RB set configuration2 | Inner RB set configuration2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 12.5 | ≤ 12.5 |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Outer sub-band configuration and inner sub-band configuration in Table 6.2E.2F-3 apply. NOTE 3: Void NOTE 4: Void |  |  |

For S-SSB transmission, the allowed A-MPR is specified in Table 6.2E.3F.2-3 for power class 5 NR sidelink UE.

Table 6.2E.3F.2-3 A-MPR for NS_31 for S-SSB transmission for NR SL-U UE power class 5

|  | RB Allocation |  |  |  |
| --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Void NOTE 3: Void |  |  |  |  |

< END OF CHANGE #1 >

< START OF CHANGE #2 >

#### 6.2E.3F.7	A-MPR for SL-U con-current operation

For NR SL-U inter-band con-current operation, the allowed additional maximum power reduction (A-MPR) for the maximum output power shall be applied per each component carrier. The A-MPR requirements in clause 6.2.3 apply for NR Uu operation in licensed band, and the A-MPR requirements in clause 6.2E.3F apply for NR sidelink operation in unlicensed band, n46, n96 and n102.

#### 6.2E.3F.8	A-MPR for NS_28

When NS_28 is indicated by the network or pre-configured radio parameters for NR sidelink UE, this clause specifies the allowed Maximum Power Reduction (MPR) power for NR sidelink physical channels and signals due to PSCCH/PSSCH, PSFCH and S-SSB transmission.

For contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed A-MPR is specified in Table 6.2F.3F.8-1 for power class 5 NR sidelink UE.

Table 6.2E.3F.8-1: A-MPR for NS_28 NR SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation (Note 3) |  |  |  | RB Allocation (Note 4) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 7.0 | ≤ 4.0 | ≤ 5.0 | Table 6.2E.2F-1 |
|  | 16 QAM | ≤ 6.0 | ≤ 7.5 | ≤ 4.0 | ≤ 5.0 |  |
|  | 64 QAM | ≤ 6.5 | ≤ 7.5 | ≤ 4.5 | ≤ 5.5 |  |
|  | 256 QAM | ≤ 7.5 | ≤ 8.0 | ≤ 7.5 | ≤ 7.5 |  |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, 5480, and 5700 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, 5490, and 5510 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5300, 5320, 5500, 5520, 5680 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 4: Applicable for all valid channels other than those enumerated under NOTE 3. NOTE 5: Contiguous outer sub-band configuration and contiguous inner sub-band configuration in Table 6.2E.2F-3 apply. NOTE 6: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |  |

For PSFCH transmission with single RB set and multiple RB sets, the allowed A-MPR is specified in Table 6.2E.3F.8-2 for power class 5 NR sidelink UE.

Table 6.2E.3F.8-2: A-MPR for NS_28 for PSFCH transmission for NR SL-U UE power class 5

|  | RB Allocation2 |  | RB Allocation3 |
| --- | --- | --- | --- |
|  | Outer RB set configuration | Inner RB set configuration | Outer/Inner RB sets |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 15.0 | ≤ 12.5 | Table 6.2E.2F-4(TS38.101-1) |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, and 5480 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, and 5490 MHz, 60 MHz hannels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5680 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, 5530, and 5610 MHz. NOTE 3: Applicable for all valid channels other than those enumerated under NOTE 3. NOTE 5: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |

For S-SSB transmission, the allowed A-MPR is specified in Table 6.2E.3F.8-3 for power class 5 NR sidelink UE.

Table 6.2E.3F.8-3: A-MPR for NS_28 for S-SSB transmission for NR SL-U UE power class 5

|  | RB Allocation2 |  |  |  | RB Allocation3 |
| --- | --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  | Outer/Inner RB sets |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |  |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 10.0 | ≤ 10.0 | ≤ 8.5 | Table 6.2E.2F-5(TS38.101-1) |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5985 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated under NOTE 2. NOTE 5: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |

#### 6.2E.3F.9	A-MPR for NS_30

When NS_30 is indicated by the network or pre-configured radio parameters for NR sidelink UE, this clause specifies the allowed Maximum Power Reduction (MPR) power for NR sidelink physical channels and signals due to PSCCH/PSSCH, PSFCH and S-SSB transmission.

For contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed A-MPR is specified in Table 6.2F.3F.9-1 for power class 5 NR sidelink UE.

Table 6.2E.3F.9-1: A-MPR for NS_30 NR SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation3 |  |  |  | RB Allocation4 |  |  |  | RB Allocation (Note 5) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration |  | Inner RB set configuration |  | Outer RB set configuration |  | Inner RB set configuration |  |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| CP-OFDM | QPSK | ≤ 10.0 | ≤ 12.0 | ≤ 7.0 | ≤ 7.5 | ≤ 6.0 | ≤ 7.0 | ≤ 3.0 | ≤ 2.0 | Table 6.2E.2F-1 |
|  | 16 QAM | ≤ 10.0 | ≤ 12.5 | ≤ 7.0 | ≤ 7.5 | ≤ 6.0 | ≤ 7.0 | ≤ 3.0 | ≤ 2.0 |  |
|  | 64 QAM | ≤ 10.0 | ≤ 13.0 | ≤ 7.0 | ≤ 7.5 | ≤ 6.0 | ≤ 7.0 | ≤ 4.0 | ≤ 3.0 |  |
|  | 256 QAM | ≤ 10.5 | ≤ 13.5 | ≤ 7.5 | ≤ 8.0 | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |  |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, and 5480 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, and 5490 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5300, 5320, 5500, and 5520 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 4: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180 and 5320 MHz, and 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5510 MHz, and 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5220, 5280, 5540, and 5680 MHz. NOTE 5: Applicable for all valid channels other than those enumerated under NOTE 3 and NOTE 4. NOTE 6: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |  |  |  |  |  |

For PSFCH transmission with single RB set and multiple RB sets, the allowed A-MPR is specified in Table 6.2E.3F.9-2 for power class 5 NR sidelink UE.

Table 6.2E.3F.9-2: A-MPR for NS_30 for PSFCH transmission for NR SL-U UE power class 5

| RB set configuration | RB Allocation /Centre frequency of CBW (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | RB Allocation2 |  | RB Allocation3 |  |
|  | Outer RB set configuration | Inner RB set configuration | Outer RB set configuration | Inner RB set configuration |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 36.0 | ≤ 16.0 | ≤ 15.5 | ≤ 12.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, and 5480 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, 5490, and 5510 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5300, 5320, 5500, and 5520 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 3: Applicable for all valid channels other than those enumerated under NOTE 2. NOTE 4: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |

For S-SSB transmission, the allowed A-MPR is specified in Table 6.2E.3F.9-3 for power class 5 NR sidelink UE.

Table 6.2E.3F.9-3: A-MPR for NS_30 for S-SSB transmission for NR SL-U UE power class 5

|  | RB Allocation2 |  |  |  | RB Allocation3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/ Non-contiguous sub-band RB sets | ≤ 35.5 | ≤ 28.0 | ≤ 13.5 | ≤ 11.0 | ≤ 13.0 | ≤ 11.0 | ≤ 10.5 | ≤ 8.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, and 5480 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, and 5490 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5300, 5320, 5500, and 5520 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 3: Applicable for all valid channels other than those enumerated under NOTE 2. NOTE 4: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |  |  |  |

< END OF CHANGE #2 >