3GPP TSG-RAN WG4 Meeting#109					R4-2404899

15th Apr, 2024 – 19th Apr, 2024

Source: 	Qualcomm Incorporated

Title: 	Views on 8Rx CPE/FWA simplified receiver along with PDSCH demod. simulation results

Agenda item:	 6.1.3

Document for:	Discussion

# 1 Introduction

In this paper we propose a new description of simplified 8Rx receiver and submit simulation results for alignment for both single carrier and Carrier Aggregation (CA) cases.

# 2 PDSCH Demodulation Requirements

## 2.1 Simplified Receiver

Issue 1-1: UE feature definition

- Agreement

  - Define two optional without UE capability signalling UE feature for the support of different 8Rx receiver types

    - Baseline SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector

    - Simplified SU-MIMO 8Rx receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 4 layers with two joint 4Rx MIMO detectors

In [1], simplified receiver is described as:

To have clear simulation assumptions for the simplified receiver, the above description was agreed and captured in the previous meeting for the purpose of collecting and aligning simulation results. Nevertheless, in our view, once the SNR numbers are set, a UE can use any means possible to meet the spec. requirement. Therefore, our proposal is to have a generic capability description in spec. so that no implementation that can meet the spec. requirement is precluded.

We propose two wordings for simplified receiver capability description:

Proposal1: 8Rx receives for SU-MIMO transmissions with support of up to 4 layers without joint 8Rx MIMO detector (e.g., two joint 4Rx MIMO detectors)

Or

Proposal1: 8Rx receives for SU-MIMO transmissions with support of up to 4 layers with two joint 4Rx MIMO detectors or any other detector without joint 8Rx MIMO.

## 2.2 Single carrier PDSCH demod requirements

### 2.2.1 TDD

### The following table has the latest simulation results of both baseline and simplified receivers for alignment purposes with simulation settings in Annex A of [1].

| Layers | Channel | MCS (Table1) | Simplified | Baseline |
| --- | --- | --- | --- | --- |
| 2 | TDLC300-100, MediumB | 19 | 13.6 | 11.6 |
| 4 | TDLA30-10 | 17 | 14 | 10.3 |
| 8 | TDLA30-10 | 17 |  | 21 |

### 2.2.2 FDD

Following table has the latest simulation results.

| Layers | Channel | MCS (Table1) | Simplified SNR (dB) | Baseline SNR (dB) |
| --- | --- | --- | --- | --- |
| 2 | TDLC300-100, MediumB | 19 | 13.6 | 11.5 |
| 4 | TDLA30-10 | 17 | 13.5 | 10.2 |
| 8 | TDLA30-10 | 17 |  | 20.3 |

## 2.3 CA PDSCH demod requirements

### 2.3.1 Rank8 requirements

#### 2.3.1.1 TDD

### The following table has the latest simulation results for CA cases with Rank8 with settings in Annex A of [1].

| Bandwidth | Channel | MCS (Table1) | Baseline SNR (dB) |
| --- | --- | --- | --- |
| 100 | TDLA30-10 | 17 | 22.0 |
| 90 | TDLA30-10 | 17 | 22.1 |
| 80 | TDLA30-10 | 17 | 21.7 |
| 60 | TDLA30-10 | 17 | 21.5 |
| 50 | TDLA30-10 | 17 | 20.9 |
| 40 | TDLA30-10 | 17 | 21.0 |
| 20 | TDLA30-10 | 17 | 20.5 |
| 10 | TDLA30-10 | 17 | 20.5 |

#### 2.3.1.2 FDD

### The following table has the latest simulation results for CA cases with Rank8 with settings in Annex A of [1].

| Bandwidth | Channel | MCS (Table1) | Baseline SNR (dB) |
| --- | --- | --- | --- |
| 50 | TDLA30-10 | 17 | 21.6 |
| 45 | TDLA30-10 | 17 | 22.0 |
| 40 | TDLA30-10 | 17 | 22.0 |
| 35 | TDLA30-10 | 17 | 21.6 |
| 30 | TDLA30-10 | 17 | 21.4 |
| 25 | TDLA30-10 | 17 | 20.9 |
| 20 | TDLA30-10 | 17 | 21.0 |
| 10 | TDLA30-10 | 17 | 20.3 |
| 5 | TDLA30-10 | 17 | 20.1 |

### 2.3.2 Rank2 requirements

#### 2.3.2.1 TDD

### The following table has the latest simulation results for CA cases with Rank2 with settings in Annex A of [1].

| Bandwidth | Channel | MCS (Table1) | Simplified SNR (dB) | Baseline SNR (dB) |
| --- | --- | --- | --- | --- |
| 100 | TDLC300-100, MediumB | 19 | 14.2 | 12.1 |
| 90 | TDLC300-100, MediumB | 19 | 14.4 | 12.3 |
| 80 | TDLC300-100, MediumB | 19 | 13.8 | 11.8 |
| 60 | TDLC300-100, MediumB | 19 | 13.9 | 11.8 |
| 50 | TDLC300-100, MediumB | 19 | 13.9 | 11.8 |
| 40 | TDLC300-100, MediumB | 19 | 13.6 | 11.6 |
| 20 | TDLC300-100, MediumB | 19 | 13.9 | 11.7 |
| 10 | TDLC300-100, MediumB | 19 | 13.9 | 11.6 |

#### 2.3.2.2 FDD

### The following table has the latest simulation results for CA cases with Rank2 with settings in Annex A of [1].

| Bandwidth | Channel | MCS (Table1) | Simplified SNR (dB) | Baseline SNR (dB) |
| --- | --- | --- | --- | --- |
| 50 | TDLC300-100, MediumB | 19 | 14 | 11.9 |
| 45 | TDLC300-100, MediumB | 19 | 13.8 | 11.8 |
| 40 | TDLC300-100, MediumB | 19 | 13.7 | 11.8 |
| 30 | TDLC300-100, MediumB | 19 | 13.6 | 11.6 |
| 20 | TDLC300-100, MediumB | 19 | 13.5 | 11.3 |
| 10 | TDLC300-100, MediumB | 19 | 13.6 | 11.5 |
| 5 | TDLC300-100, MediumB | 19 | 13.5 | 11.5 |

# 3. Conclusion

1. Alternatives for the simplified receiver capability are:

Proposal1: 8Rx receives for SU-MIMO transmissions with support of up to 4 layers without joint 8Rx MIMO detector (e.g., two joint 4Rx MIMO detectors)

Or

Proposal1: 8Rx receives for SU-MIMO transmissions with support of up to 4 layers with two joint 4Rx MIMO detectors or any other detector without joint 8Rx MIMO.

2. Simulation results of single carrier and CA cases for alignment purposes are submitted.

# References

[1] WF R4-2402857, “WF for UE 8Rx performance requirements”, 3GPP TSG RAN WG4 meeting, #110, Feb. 26 – Mar. 1, 2024, Athens, Greece.