# 3GPP TSG-RAN WG4 Meeting #110bis	R4-2404913

Changsha, CN, April 15 – April 19, 2024

Title: 	Discussion on RRM test case design for FR2 multi-Rx

Source: 	OPPO

Agenda item:	6.3.3.1

Document for:	Discussion

Introduction

In RAN4#110 meeting, a WF on RRM requirements for FR2 multi-Rx DL receptions was approved [1].

In this contribution, we will provide the discussion on test case deign for FR2 multi-Rx DL receptions.

Discussion

In R18, RRM enhancement requirements due to FR2 multi-Rx DL receptions can be listed as below. The test cases can be defined to verify the listed enhanced features.

Test case for L1 measurements

#### Issue 2-4: Test case(s) for fast beam sweeping - Option 1: - TC1: Define test case for fast beam sweeping with non-GBBR L1-RSRP measurement with non-DRX. - TC2: Define test case for fast beam sweeping with [RLM] with non-DRX. - Note: We may further discuss the conditions of how fast beam sweeping are applicable in the test configuration. - FFS AoA setup. - Option 2: - Introduce new tests for fast beam sweeping by revising all relevant legacy tests. The UE passes multi-Rx test should not be tested with [same] legacy test.

We think there is no need to verify the faster beam sweeping for each type of L1 measurements. We suggest to verify the capability of faster beam sweeping in one test of each type of L1 measurements. For CSI-RS based L1 measurement, there is also no need to define new test case for the reduced sweeping factor.

Proposal 1: RAN4 to test faster beam sweeping for SSB based measurements for R18 multi-Rx reception:

- TC1: SSB based RLM Out-of-sync Test with faster beam sweeping for FR2 PCell in non-DRX mode

- TC2: SSB based non-GBBR L1-RSRP measurement with faster beam sweeping in non-DRX mode

Test case for scheduling/measurement restriction relaxation

One test case is agreed to introduced for scheduling restriction. The details can be discussed in CR review.

#### Issue 2-5: Test case(s) for scheduling restriction - Introduced on test case - Combine GBBR with scheduling restriction, i.e., GBBR is always configured in the test. - Scheduling restriction relaxation during the L1-RSRP non-GBBR is tested in the follow-up L1-RSRP non-GBBR measurement. - Only L1-RSRP GBBR measurement delay requirements are tested in this test, i.e., no accuracy is tested in this test. - Measurement restriction requirements are also tested in this test.

Test case for dual TCI state switching

- For TCI state switching test - Define test at least for Single TCI to dual TCI ([RS1] to [RS2, RS3]) - Further discuss dual-to-dual active TCI state switching if the testability is confirmed. - Option a: T1: Two TCI [RS1, RS3], with non-overlapping PDSCH T2: Two TCI [RS1, RS2], with RS1 and RS2 are a beam pair - FFS Option b (feasibility to be further confirmed): T1: Two TCI [RS1, RS3] (source), with RS1 and RS3 are a beam pair T2: Two TCI [RS2, RS4] (target), with RS2 and RS4 are a beam pair The offset of beam pair in T2 is not the same of the offset in T1.

For dual TCI state switching from dual TCI to dual TCI, 4 active probes are needed. However, based on current RF agreement for the test set up, specific AoA offset is needed to satisfy the corresponding RF requirements and it is very likely that at most 1 qualified AoA pairs can be found for each test point. Thus, it is very difficult to test dual TCI switching from one AOA pair to another AOA pair due to limited probes in the test environment. Before the testability issue was solved, we prefer not to test dual-to-dual active TCI state switching in R18.

Proposal 2: RAN4 not to test dual-to-dual active TCI state switching in R18.

Conclusions

In this paper we provided our analysis on the impacts on RRM performance for FR2 multi-Rx DL receptions. The followings are provided.

Proposal 1: RAN4 to test faster beam sweeping for SSB based measurements for R18 multi-Rx reception:

- TC1: SSB based RLM Out-of-sync Test with faster beam sweeping for FR2 PCell in non-DRX mode

- TC2: SSB based non-GBBR L1-RSRP measurement with faster beam sweeping in non-DRX mode

Proposal 2: RAN4 not to test dual-to-dual active TCI state switching in R18.

Reference

- R4-2403540, WF on FR2 multi-RX operation RRM requirements