# 3GPP TSG-RAN WG4 Meeting #110bis	R4-2404921

# Changsha, China, 15th – 19th April, 2024

Agenda item:	6.20.3

Source:	OPPO

Title:	Discussion on performance requirements for Rel-18 SL

Document for:	Approval

- Introduction

Based on the agreements on Rel-18 SL performance requirements achieved during the last meeting, this contribution will provide our consideration on the leftover issues captured in [1].

- Discussion

#### Issue 2-5: Accuracy requirements – RSSI Agreement: - Reuse SL-RSSI accuracy requirements for SL-U congestion control - FFS: Define RSSI accuracy requirements for SL-U LBT measurement, e.g., reuse NR-U RSSI accuracy - FFS how to capture the requirements in the specification (e.g., add it in existing table or introduce new clause)

Last meeting agreed to reuse the legacy SL-RSSI accuracy requirements for SL-U congestion control. And how to define RSSI accuracy requirements for SL-U LBT measurement will be further discussed in this meeting. In our understanding, SL-U LBT measurement results are used for UE itself to determine whether the channel is clear so that it could transmit data. And no report is introduced for SL-U LBT measurement in RAN1 or RAN2. Therefore, we propose to not define accuracy requirement for SL-U LBT measurement in RAN4.

Observation 1: No report is defined for SL-U LBT measurement.

Proposal 1: Not define accuracy requirements for SL-U LBT measurement in RAN4.

#### Issue 2-12: Test case list for SL-U RRM Agreement: - Introduce the TC for Initiation/Cease of SLSS transmission. - Do not introduce the TC for Selection/Reselection of synchronization Reference source - Do not introduce the TC for UE Transmit timing - Do not introduce the TC for L1 SL-RSRP measurement - Do not introduce the TC for Congestion control measurement - Do not introduce the TC for Interruption

#### Issue 2-8: Whether to define new RMCs for PSCCH and PSSCH due to CCA. Way forward: - Come back next meeting

As shown above, only one TC for initiation/cease of SLSS transmission will be introduced for SL-U. After reviewing the test cases defined in A.9.1.2, it is identified that V2X sidelink communication (pre-)configuration and RMC for PSCCH/PSSCH for SL-U are needed in the test case. So, the related configuration should be introduced and the accompanied draft CR is provided in [2].

Proposal 2: Define new V2X sidelink communication and new RMCs for PSCCH and PSSCH due to CCA.

- Conclusion

This contribution gave the following proposals on the RRM performance requirements for Rel-18 SL enhancements.

Observation 1: No report is defined for SL-U LBT measurement.

Proposal 1: Not define accuracy requirements for SL-U LBT measurement in RAN4.

Proposal 2: Define new V2X sidelink communication and new RMCs for PSCCH and PSSCH due to CCA.

References

[1] R4-2403472, WF for [110][228] NR_SL_enh2_part2, OPPO, RAN4 #110

[2] R4-2404923, Draft CR on new RMC for PSCCH and PSSCH for SLU, OPPO, RAN4 #110bis