3GPP TSG-RAN WG4 Meeting #110bis	R4-2404922

# Changsha, China,  15th - 19th April 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on new CCA model for SLU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on R4-2403472 WF for NR_SL_enh2_part2, new CCA models for SL-U should be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce new CCA models for SL-U. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will be incompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.26.1 (new)A.3.26.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------ START OF CHANGE 1--------------

A.3.26.1	Introduction

The CCA model is used in some RRM test cases with at least one cell on a carrier frequency with CCA. The intention with the CCA model is to emulate in the test equipment the behaviour of a gNB or UE which performs channel measurement to check that the channel is clear prior to performing one or more downlink or uplink or sidelink transmissions.

A.3.26.3	Sidelink CCA model for operation on a carrier frequency with CCA

A.3.26.3.1	Sidelink CCA model for SyncRef UE

For the SyncRef UE transmitting S-SSB in sidelink CCA, the modelling approach is based on probability PCCA_SL_ref of successful CCA.

If the CCA attempt is successful for a S-SSB transmission, then the test equipment shall transmit also other remaining S-SSB transmission occassions, according to the configuration.

If the CCA attempt is not successful for a S-SSB transmission, the test equipment shall determine whether the CCA attempt is successful for the next configured transmission, based on probability PCCA_SL_ref.

The probability can be different in different time intervals Ti during a test case. One probability value applies at any time point during a test; one or more probability values can be configured in the entire test, one value PCCA_SL_ref per time interval Ti where I ≥ 1, and the multiple time intervals (when I > 1) do not overlap (e.g., PCCA_SL_ref = 1.0 in T1 and PCCA_SL_ref = 0.75 in T2).

To decide whether the CCA attempt for the first S-SSB occasion within each of S-SSB period, TE shall:

1 - Generate a uniform random variable p from the range [0, 1].

2 - If p > PCCA_SL_ref,

- TE models CCA failure in this S-SSB occasion and the other S-SSB occasions within the same period.

- This S-SSB period is considered as unavailable to the UE under test.

2 - If p ≤ PCCA_ref,

- TE transmits this S-SSB occasions.

- This S-SSB period is considered as available to the UE under test.

The above steps are repeated for each S-SSB periods in each time interval Ti of the test.

In many test cases, the requirement under a test depends on the number of S-SSB periods which are not available during the test due to CCA failure, so the test equipment shall track how many such S-SSB periods are not transmitted by TE during the test period.

A.3.26.3.2	Sidelink CCA model for DUT

For the device under test (DUT) in sidelink CCA, the modelling approach is based on probability PCCA_SL_DUT of successful CCA. Probability PCCA_SL_DUT is configured in the corresponding test case.

Consistent sidelink CCA failures are modelled by configuring a low value for PCCA_SL_DUT, e.g., PCCA_SL_DUT = 0%.

The probability can be different in different time intervals Ti during a test case. One probability value applies at any time point during a test; one or more probability values can be configured in the entire test, one value PCCA_SL_DUT per time interval Ti where I ≥ 1, and the multiple time intervals (when I > 1) do not overlap (e.g., PCCA_SL_DUT = 1.0 in T1 and PCCA_SL_DUT = 0.75 in T2).

TCCA µs prior to each sidelink transmission in the test, the test equipment (TE) shall generate a uniform random variable p from the range [0, 1]. If p>PCCA_SL_DUT, the TE transmits an OCNG noise pattern with an energy level X within the DUT BW scheduled/configured for the sidelink transmission for at-least TCCA µs. Where TCCA µs is energy detection time for accessing the sidelink channel as defined in section 5.1.1 of TS 37.106 [36]. Where:

- X is 3 dB above the energy detection threshold defined in section 5.1.1 of TS 37.106 [36].

- TCCA is the channel sensing period depending on CCA category for the next sidelink transmission.

For each sidelink CCA failure generated by the model, the TE shall monitor the corresponding sidelink resource for the desired sidelink signal, and based on when and/or whether the TE received the desired sidelink signal, it deems the test case to pass or fail.

In many cases, the requirement under a test depends on the number of S-SSB periods which are not available during the test, so the test equipment shall track how many such S-SSB peirods are not transmitted in V2X sidelink carrier during the test period.

------------ END OF CHANGE 1--------------