3GPP TSG RAN WG4 Meeting #110bis	R4-2404953

Changsha, China, 15 – 19 April, 2024

Agenda item:			6.6.1

Source:	vivo

Title:	On remaining issues for BWP operation without restriction

Document for:	Discussion

- Introduction

In the RAN4#110 meeting, there were little discussions on core maintenance part of BWP operation without restriction. Following agreements were captured in the WF [1].

<Core Maintenance> #### Issue 1-2: Requirements/UE behaviour for UE supporting both option C and A - For UE supporting both option C and option A, UE shall perform L1 measurement according to network configuration, no additional requirements/ UE behaviour will be defined. - Both NCD-SSB and CSI-RS within active BWP can be configured by network for L1 measurement. #### Issue 1-5: Configuration of SSB resources within CSI-ResourceConfig settings for option B-1-1 - FFS: LS RAN2 to ask whether SSB resources outside a BWP can be configured within CSI-ResourceConfig settings for the BWP if UE supports option B-1-1.

Other open issues were also captured in the WF. In this contribution, we further provide our views on remaining open issues for BWP operation without restriction.

- Discussion

- 2.1	Combination of options

There were discussions in the past RAN4 meetings on whether or not and how to define requirements for UE supporting multiple options for BWP operation without restriction.

Issue 1-1: Requirements/UE behaviour for UE supporting both option B-1-1 and A

- FFS following options - Option 1a: - For UE supporting both option B-1-1 and option A, UE shall perform L1 measurement according to network configuration, there is no need to define additional requirements/ UE behaviour. - Option 1b: - For UE supporting both option B-1-1 and A, if configured by NW, UE should perform L1 measurement on both CD-SSB outside active BWP and CSI-RS within active BWP. No additional requirements will be defined for such scenario. - Option 2: - For UE supporting both option B-1-1 and A, if CSI-RS is configured within the active BWP for L1 measurements, then UE is NOT expected to perform RLM/BFD/BM based on CD-SSB outside active BWP. - It is clarified in the SSB based L1 measurement requirements that if UE supports both option B-1-1 and option A then the UE is not required to measure the SSB if it is outside active BWP.

Whether and how to define requirements/UE behaviour for UE supporting both option B-1-1 and A

Following UE capabilities for option B-1-1 and option A were introduced in Rel-18.

rlm-BM-BFD-CSI-RS-OutsideActiveBWP-r18 Indicates whether the UE supports RLM/BM/BFD measurements based on CSI-RS, when CD-SSB is outside active DL BWP. bwpOperationMeasWithoutInterrupt-r18 Indicates whether the UE supports RLM/BM/BFD and gapless L3 intra-frequency measurements based on CD-SSB outside active BWP without interruptions.

Thus, support of both option B-1-1 and option A is possible from UE implementation perspective. In existing specification, UE should perform RLM/BFD/BM measurements over both CD-SSB outside active BWP and CSI-RS within active BWP if NW configures so.

- 8.1 Radio Link Monitoring - 8.1.1 Introduction UE is not required to perform RLM outside the active DL BWP unless the UE supports [FG53-1], provided that the SSB is within the configured UE-specific CBW. - 8.1.3 Requirements for CSI-RS based radio link monitoring The requirements in this clause apply for each CSI-RS based RLM-RS resource configured for PCell, PSCell or deactivated PSCell, provided that the CSI-RS configured for RLM is actually transmitted within UE active DL BWP during the entire evaluation period specified in clause 8.1.3.2.

However, if a UE supports both option B-1-1 by indicating bwpOperationMeasWithoutInterrupt-r18 and option A by indicating rlm-BM-BFD-CSI-RS-OutsideActiveBWP-r18, it is not desirable that when option A is supported in practical network, the UE still needs to work with option A and option B-1-1 together. It brings no obvious gain by operating with the two options simultaneously.

If UE supports option A only, it only measures CSI-RSs within active BWP for L1 measurements and measures CD-SSB outside active BWP for L3 intra-frequency measurements. If UE supports option B-1-1 only, it only measures CD-SSB outside active BWP for both L1 measurements and L3 intra-frequency measurements.

Based on existing specification, if UE supports both option A and option B-1-1 and the two features works together, then the UE needs to measure both CSI-RS and CD-SSB outside active BWP for L1 measurements depending on NW configuration, and CD-SSB outside active BWP L3 intra-frequency measurements. It will complicate UE implementation, especially it will bring additional power consumption due to working with larger channel bandwidth, but the gain of additional CSI-RS based L1 measurements over SSB based L1 measurements is not obvious. In addition, option B-1-1 is supposed to be a replacement of option A considering commercialization in industry. Thus, it would be better not to consider the two options working simultaneously.

For UE supporting both option B-1-1 and A, if CSI-RS is configured within the active BWP for L1 measurements, then UE is not expected to perform RLM/BFD/BM based on CD-SSB outside active BWP.

Proposal 1: For UE supporting both option B-1-1 and A, if CSI-RS is configured within the active BWP for L1 measurements, then UE is NOT expected to perform RLM/BFD/BM based on CD-SSB outside active BWP.

Proposal 2: It is clarified that if UE supports both option B-1-1 and option A then the UE is not required to measure the SSB outside active BWP.

Issue 1-3: Requirements/UE behaviour for UE supporting both option C and B-1-1

- FFS following options - Option 1: - For UE supporting both option B-1-1 and C, if both CD-SSB and NCD-SSB are configured outside active BWP, then UE is expected to perform RLM/BFD/BM following NW configuration based on TS 38.331. No additional requirements will be defined for such scenario. - Option 2: - For UE supporting both option B-1-1 and C, if the active BWP that does not contain CD- or NCD-SSB, UE should perform L1 measurement on CD-SSB. No additional requirements will be defined for such scenario.

In RAN4#108 meeting, conclusion on requirements/UE behaviour was made for UE supporting both option B-1-1 and option C. It was captured in the WF [2].

#### Issue 1-3: Requirements/UE behaviour for UE supporting both option B-1-1 and C - It is RAN4 understanding that for UE supporting both option B-1-1 and C, if NCD-SSB is configured within the active BWP, then UE is expected to perform RLM/BFD/BM and L3 intra-frequency measurements based on NCD-SSB based on TS 38.331. No additional requirements will be defined for such scenario.

There are generally two cases, which are depicted in Fig 1, that can be considered for UE supporting both option B-1-1 and C.

Fig 1. Cases for combination of option B-1-1 and C

The conclusion can address case 1 when there is only NCD-SSB being configured in the active BWP. UE is expected to perform RLM/BFD/BM based on the NCD-SSB.

Case 2 was raised in the past meetings. Both CD-SSB and NCD-SSB are configured outside active BWP. If UE doesn’t support option C, then the UE can only be configured with CD-SSB for L1 measurements. If UE supports option C, then it is possible for NW to configure either CD-SSB or NCD-SSB for L1 measurements. In such case, UE is also feasible to perform measurements over either SSB. Our view is that UE should follow NW configuration. No additional requirements are needed from RAN4 perspective.

Proposal 3: For UE supporting both option B-1-1 and C, if both CD-SSB and NCD-SSB are configured outside active BWP, then UE is expected to perform RLM/BFD/BM following NW configuration based on TS 38.331. No additional requirements will be defined for such scenario.

- 2.2	L1-RSRP/L1-SINR reporting requirements

Issue 1-5: Configuration of SSB resources within CSI-ResourceConfig settings for option B-1-1

FFS: LS RAN2 to ask whether SSB resources outside a BWP can be configured within CSI-ResourceConfig settings for the BWP if UE supports option B-1-1

Following requirements are specified for L1-RSRP/L1-SINR reporting and the highlighted parts for UE supports option B-1-1 are not finalized yet.

9.5.1 Introduction …… The UE shall be able to measure all CSI-RS resources and/or SSB resources of the nzp-CSI-RS-ResourceSet and/or csi-SSB-ResourceSet within the CSI-ResourceConfig settings configured for L1-RSRP for the active BWP, [or for other candidate BWP if the UE supports [FG53-1]], provided that the number of resources, including the number of SSB resources of the cell with PCI different from serving cell configured for L1-RSRP measurements in 9.13, does not exceed the UE capability indicated by beamManagementSSB-CSI-RS. The UE shall report the measurement quantity (reportQuantity) and send periodic, semi-persistent or aperiodic reports, according to the reportConfigType according to the CSI reporting configuration(s) (CSI-ReportConfig) for the active BWP, [or for other candidate BWP if the UE supports [FG53-1]]. The measurement reporting delay can be longer for the measurement reporting requirements in this clause when IDC autonomous denial is configured. 9.8.1 Introduction …… The UE shall be able to measure all CSI-RS resources and/or SSB resources and/or CSI-IM resources of the nzp-CSI-RS-ResourceSet and/or csi-SSB-ResourceSet and/or CSI-IM-ResourceSet within the CSI-ResourceConfig settings for L1-SINR for the active BWP [or for other candidate BWP if the UE supports [FG53-1]], and measure interference on corresponding NZP CSI-RS or CSI-IM resources if configured, provided that the number of resources does not exceed the UE capability indicated by beamManagementSSB-CSI-RS. The UE shall report the measurement quantity (reportQuantity) and send periodic, semi-persistent or aperiodic reports, according to the reportConfigType according to the CSI reporting configuration(s) (CSI-ReportConfig) for the active BWP [or for other candidate BWP if the UE supports [FG53-1]]. The measurement reporting delay can be longer for the measurement reporting requirements in this clause when IDC autonomous denial is configured.

In current requirements, it is clarified that SSB resources within the CSI-ResourceConfig settings configured for L1-RSRP for other candidate BWP if the UE supports option B-1-1. However, it is not correct as UE only measures CSI resources for the active BWP. From RRC configuration perspective, CSI-ResourceConfig settings is associated to a BWP. However, it is not clear whether SSB resources outside the BWP can be configured within CSI-ResourceConfig settings if UE supports option B-1-1. It should be allowed otherwise option B-1-1 may not work.

-- ASN1START

-- TAG-CSI-RESOURCECONFIG-START

CSI-ResourceConfig ::=      SEQUENCE {

csi-ResourceConfigId        CSI-ResourceConfigId,

csi-RS-ResourceSetList      CHOICE {

nzp-CSI-RS-SSB              SEQUENCE {

nzp-CSI-RS-ResourceSetList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need R

csi-SSB-ResourceSetList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF CSI-SSB-ResourceSetId  OPTIONAL  -- Need R

},

csi-IM-ResourceSetList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId

},

bwp-Id                      BWP-Id,

resourceType                ENUMERATED { aperiodic, semiPersistent, periodic },

...,

[[

csi-SSB-ResourceSetListExt-r17      CSI-SSB-ResourceSetId                                                  OPTIONAL  -- Need R

]]

}

-- TAG-CSI-RESOURCECONFIG-STOP

-- ASN1STOP

In current TS 38.331, there is no clarification on whether SSB resources outside the BWP can be configured within CSI-ResourceConfig settings if UE supports option B-1-1 or there is no restriction on SSB resources configuration at all. RAN4 needs to inform RAN2 about the issue.

From RAN1 spec perspective, it is neither clear if SSB outside BWP can be configured as CSI resource. In TS 38.214,

5.2.1.2 Resource settings Each CSI Resource Setting CSI-ResourceConfig contains a configuration of a list of S≥1 CSI Resource Sets (given by higher layer parameter csi-RS-ResourceSetList), where the list is comprised of references to either or both of NZP CSI-RS resource set(s) and SS/PBCH block set(s) or the list is comprised of references to CSI-IM resource set(s). Each CSI Resource Setting is located in the DL BWP identified by the higher layer parameter BWP-id, and all CSI Resource Settings linked to a CSI Report Setting have the same DL BWP.

Thus, it would be necessary to ask RAN2 and RAN1 to clarify the issue. RAN4 requirements can be finalized after there is feedback from RAN2/RAN1.

Proposal 4: LS RAN2 and RAN1 to ask whether SSB resources outside a BWP can be configured within CSI-ResourceConfig settings for the BWP if UE supports option B-1-1.

- 2.3	Reply LS

In the RAN2#124 meeting, a reply LS [3] was approved and sent to RAN4. One question was raised and is needed to be answered by RAN4 regarding applicability of requirements for UE supporting option C.

RAN2 would like to thank RAN4 on the LS on BWP_Wor. RAN2 has discussed the Option C (i.e. NCD-SSB) and concluded: 1 Capture the behavior that UE shall report no gap and no interruption/no NCSG for intra-frequency measurement in RRC specification. 2 Capture the restriction for NCD-SSB measurement to be only applicable for PCell. Besides, regarding RAN4 agreement below: - The following scenario is supported from RAN4 requirement perspective: - For UE supporting option C and configured with CA, NCD-SSB based L1 and L3 intra-frequency measurement requirements are only applicable for the PCell.. RAN2 would like to check whether it is also applicable for the PSCell.

In the last RAN4 meeting, there were discussions on the question but no consensus was reached.

Issue 1-4: Applicability of NCD-SSB based L1 and L3 intra-frequency measurement requirements

- FFS: NCD-SSB related requirements also apply for PSCell when SCG deactivation is not enabled for the UE. - FFS requirements are specified for EN-DC only or for both EN-DC and NR-DC.

According to existing requirements, RLM measurements should be performed for PCell and PSCell. BFD/BM measurements should be performed for PCell, PSCell and SCell(s). Suppose EN-DC is configured, the NCD-SSB should be allowed for UE to perform L1 measurements on NR PSCell. Even when NR-DC is configured, the NCD-SSB could be allowed for UE to perform L1 measurements for both PCell and PSCell. It will provide flexibility of NW deployment and BWP configuration. Moreover, there is no additional complexity being introduced compared to CD-SSB based L1 measurement. Therefore, NCD-SSB can be used for L1 measurements for PSCell in dual connectivity.

For L3 measurements, there is no reason that NCD-SSB cannot be used for intra-frequency measurements for PSCell.

SCG activation/deactivation is only relevant to L3 intra-frequency measurements. In our understanding, the intra-frequency measurements based on NCD-SSB should always be performed without measurement gaps, regardless of whether the SCG is activated or not. It seems no additional complexity if NCD-SSB is supported on PSCell even if SCG activation/deactivation is supported.

Though we don’t see there is technical reason not to support NCD-SSB for PSCell in NR-DC, we would be fine to consider EN-DC only, i.e., requirements are defined for EN-DC only, if it is the majority view.

Proposal 5: For UE supporting option C and configured with dual connectivity, NCD-SSB based L1 and L3 intra-frequency measurement requirements are applicable for both PCell and PSCell.

Issue 1-6: Requirements for L3 intra-f measurement without gaps for option C

- FFS following option - Option 1: - Add one bullet in section 9.2.1 that the target measurement SSB is completely contained in the active downlink BWP of the UE provided the UE supports [FG53-3] and servingCellMO is present in the corresponding BWP-DownlinkDedicated.

The intention is to make requirements for intra-frequency measurement without gaps applicable for NCD-SSB based measurement. It was already agreed and captured in the spec that the SSB and SMTC in the requirements for intra-frequency measurement applies for both CD-SSB and NCD-SSB if the UE supports ncd-SSB-BWP-Wor-r18. With this applicability, it seems no further clarification is needed.

Observation 1: With previous agreements being captured in the spec, no further clarification is needed on requirements for L3 intra-f measurement without gaps for option C.

- Summary

In this contribution, we provided our views on remaining issues for BWP operation without restriction. Following proposals are provided.

Proposal 1: For UE supporting both option B-1-1 and A, if CSI-RS is configured within the active BWP for L1 measurements, then UE is NOT expected to perform RLM/BFD/BM based on CD-SSB outside active BWP.

Proposal 2: It is clarified that if UE supports both option B-1-1 and option A then the UE is not required to measure the SSB outside active BWP.

Proposal 3: For UE supporting both option B-1-1 and C, if both CD-SSB and NCD-SSB are configured outside active BWP, then UE is expected to perform RLM/BFD/BM following NW configuration based on TS 38.331. No additional requirements will be defined for such scenario.

Proposal 4: LS RAN2 and RAN1 to ask whether SSB resources outside a BWP can be configured within CSI-ResourceConfig settings for the BWP if UE supports option B-1-1.

Proposal 5: For UE supporting option C and configured with dual connectivity, NCD-SSB based L1 and L3 intra-frequency measurement requirements are applicable for both PCell and PSCell.

Observation 1: With previous agreements being captured in the spec, no further clarification is needed on requirements for L3 intra-f measurement without gaps for option C.

- References

- R4-2403544	WF on core maintenance and performance part for BWP operation without restriction, vivo, Vodafone

- R4-2314339 WF on support for bandwidth part operation without restriction, vivo

- R2-2313951 Reply LS to RAN4 on BWP operation without restriction, vivo