# 3GPP TSG-RAN WG4 Meeting # 110bis	R4-2404966

# Changsha, China, 15th – 19th April, 2024

Agenda Item:	6.5.1

Source:	Ericsson

Title:	Rel-18 NeedForGaps capability

Document for:	Discussion

# Introduction

In last meeting, RAN4 discussed the remaining issues of Rel-16 NeedForGaps capability. The issues were identified in WF [1]. In this paper, we discuss the relation between Rel-18 NeedForInterruption capability and Rel-16 NeedForGaps capability.

# Justification

The WID “Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps” clearly captures that RAN4 will implement the requirement for NeedForGaps feature and identify additional interruption when UE reporting NeedForGapsInfoNR.

## RP-222337 New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps … ## 3 Justification … (2) RRM requirements for measurements without gaps The NeedForGaps feature can bring gains for system throughput and/or mobility performance under certain conditions, where gap-less measurements can be applied. The existing RRM requirements for UEs supporting NeedForGaps feature are specified for intra-frequency SSB based measurements without gaps, while requirements for inter-frequency measurements without gaps are missing. Also, RRM requirements for intra-frequency SSB based measurements without gaps do not assume interruptions, which may be required. … ### Objective of SI or Core part WI or Testing part WI … (2)Define RRM requirements for measurement without gaps for the following cases - NR SSB-based inter-frequency and intra-frequency measurements without gaps for UEs reporting NeedForGapsInfoNR IE [RAN4] - Study whether the additional interruption is allowed when UE reporting ‘NeedForGapsInfoNR'. Further define the interruption length, occasion and ratio, if the interruption is allowed - Define related requirements, such as CSSF, measurement period, scheduling restriction etc.

Observation 1: In Rel-18 WID, it is clearly captured that the intention to introduce NeedForGaps-r18 is to solve the additional interruption issue of NeedForGaps-r16.

# The relation between Rel-16 and Rel-18 NeedForGaps capability

Background

In RAN2, the Rel-18 NeedForGaps capability was agreed as follow.

-- ASN1START -- TAG-NeedForInterruptionInfoNR-START NeedForInterruptionInfoNR-r18 ::= SEQUENCE { intraFreq-needForInterruption-r18 NeedForInterruptionIntraFreqList-r18, interFreq-needForInterruption-r18 NeedForInterruptionBandListNR-r18 } NeedForInterruptionIntraFreqList-r18 ::= SEQUENCE (SIZE (1.. maxNrofServingCells)) OF NeedForInterruptionNR-r18 NeedForInterruptionBandListNR-r18 ::= SEQUENCE (SIZE (1..maxBands)) OF NeedForInterruptionNR-r18 NeedForInterruptionNR-r18 ::= SEQUENCE { interruptionIndication-r18 ENUMERATED {no-gap-with-interruption, no-gap-no-interruption} OPTIONAL } -- TAG-NeedForInterruptionInfoNR-STOP -- ASN1STOP NeedForInterruptionInfoNR field descriptions. intraFreq-needForInterruption Indicates the interruption requirement information for NR intra-frequency measurement. Each entry in the list is associated to the entry in list intraFreq-needForGap-r16 with the same index. This field shall be set to no-gap-no-interruption for the corresponding band(s) where bwpOperationMeasWithoutInterrupt-r18 is supported by the UE.. interFreq-needForInterruption Indicates the interruption requirement information for NR inter-frequency measurement. Each entry in the list is associated to the entry in list interFreq-needForGap-r16 with the same index.. NeedForInterruptionNR field descriptions. interruptionIndication Indicates whether interruption is needed for the UE to perform SSB based measurements without measurement gap. Value no-gap-with-interruption indicates that interruption is needed. Value no-gap-no-interruption indicates interruption is not needed..

In the Rel-18 RAN2 signalling design, a new IE is introduced for the network to configure the flag needForInterruptionConfigNR, to which the UE replies with nr-NeedForInterruptionReport-r18. The RAN2 design enables the NeedForGaps capability for determining whether interruption is needed by the UE when performing the gapless measurements, which is associated with the entries in list NeedForGapsInfoNR-r16.

| Nr-NeedForGap-Reporting-r16 Indicates whether the UE supports reporting the measurement gap requirement information for NR target in the UE response to a network configuration RRC message. | UE | No | No | No |
| --- | --- | --- | --- | --- |
| nr-NeedForInterruptionReport-r18 Indicates whether the UE supports reporting the interruption requirement information for SSB based measurement towards NR target without gap in the UE response to a network configuration RRC message. The UE supporting this feature shall also indicate support of nr-NeedForGap-Reporting-r16. | UE | No | No | No |

Observation 2: In Rel-18, UE supporting nr-NeedForInterruptionReport-r18 shall also indicate support of nr-NeedForGap-Reporting-r16 in TS 38.306.

In last meeting, RAN4 triggered some discussion about Rel-16 NeedForGaps capability with the following agreements.

Issue 1-1-1: Scenario 1, LTE – NR inter-RAT measurement Tentative agreements - interRAT-NeedForGaps-r16=FALSE means that the UE support measurement without gaps - The UE may or may not cause interruption. - if UE causes interruptions when performing measurements without gaps: - Support early implementation of Rel-18 NeedForInterruption: - Optional since R17 - FFS mandatory report of the UE capability R18 interRAT-NeedForIntrNR-r18 from Rel-18 UE if the UE reports interRAT-NeedForGaps-r16=FALSE - FFS the UE behavior if the Rel-18 UE does not support interRAT-NeedForIntrNR-r18 Issue 1-1-2: Scenario 2, NR measurements without gaps Tentative agreements - “no-gap” as part of NeedForGapsInfoNR-r16=FALSE means that the UE support measurement without gaps - The UE may or may not cause interruption. - if UE causes interruptions when performing measurements without gaps: - Support early implementation of Rel-18 NeedForInterruption: - Optional since R17 - FFS the UE behavior if the Rel-18 UE does not support NeedForInterruptionNR-r18

It can be seen that whether Rel-18 NeedForGaps capability is conditional mandatory based on Rel-16 NeedForGaps capability is FFS.

Capability

In our understanding, currently Rel-16 UE which supports Rel-16 NFG but not supporting Rel-18 NFI can achieve the performance gain due to no gap request from the UEs. However, if we mandatorily require a Rel-18 UE to report both Rel-16 NFG and Rel-18 NFI, the Rel-18 UE which has implemented Rel-16 NFG only but not support Rel-18 NFI cannot report to support Rel-16 NFG. In this case, the performance degradation will be observed for Rel-18 UEs which supports Rel-16 NFG capability only. Thus, we propose that UE is allowed to optional report Rel-18 NFI capability in Rel-18.

Observation 3: Rel-16 UE which supports Rel-16 NFG but not supporting Rel-18 NFI can achieve the performance gain due to no gap request from UE.

Observation 4: The performance degradation will be observed for the Rel-18 UEs which only supports Rel-16 NFG capability provided that Rel-18 UE is required to support both Rel-16 NFG and Rel-18 NFI as a pair.

UE behaiour

Secondly, RAN4 needs to clarify UE’s behaviour under different scenarios. In our understanding, when UE only reports Rel-16 NFG capability but not reports Rel-18 NFI capability, the UE’s behaviour is the same as Rel-16 UE which means the UE’s measurement behaviour is undefined in spec if UE reports ‘no gap’.

The next issue is to clarify the UE’s behaviour when a UE supports both Rel-16 NFG and Rel-18 NFI capabilities in a Rel-16 NW. We understand that if a NW only supports Rel-16 NFG, it cannot identify the Rel-18 NFI capability and won’t request UE to report Rel-18 NFI status. Thus, the UE’s behaviour is also the same as Rel-16 UE in such Rel-16 NW.

Proposal 1: In Rel-18, UE is allowed to optional report Rel-18 NFI capability for both interRAT-NeedForIntrNR-r18 and NeedForInterruptionNR-r18.

Proposal 2: When a Rel-18 UE only supports Rel-16 NFG capability but not supports Rel-18 NFI capability, the UE’s behaviour is the same as Rel-16 UE.

Proposal 3: When a Rel-18 UE supports both Rel-16 NFG and Rel-18 NFI capabilities, but NW doesn’t configure Rel-18 needForInterruptionConfigNR, the UE’s behaviour is the same as Rel-16 UE.

# Summary and Conclusion

In the contribution, we discuss the relation between Rel-16 NeedForGaps capability and Rel-18 NeedForInterruption with the following proposals:

Observation 1: In Rel-18 WID, it is clearly captured that the intention to introduce NeedForGaps-r18 is to solve the additional interruption issue of NeedForGaps-r16.

Observation 2: In Rel-18, UE supporting nr-NeedForInterruptionReport-r18 shall also indicate support of nr-NeedForGap-Reporting-r16 in TS 38.306.

Observation 3: Rel-16 UE which supports Rel-16 NFG but not supporting Rel-18 NFI can achieve the performance gain due to no gap request from UE.

Observation 4: The performance degradation will be observed for the Rel-18 UEs which only supports Rel-16 NFG capability provided that Rel-18 UE is required to support both Rel-16 NFG and Rel-18 NFI as a pair.

Proposal 1: In Rel-18, UE is allowed to optional report Rel-18 NFI capability for both interRAT-NeedForIntrNR-r18 and NeedForInterruptionNR-r18.

Proposal 2: When a Rel-18 UE only supports Rel-16 NFG capability but not supports Rel-18 NFI capability, the UE’s behaviour is the same as Rel-16 UE.

Proposal 3: When a Rel-18 UE supports both Rel-16 NFG and Rel-18 NFI capabilities, but NW doesn’t configure Rel-18 needForInterruptionConfigNR, the UE’s behaviour is the same as Rel-16 UE.

# References

- R4-2403559, “WF on Interruption requirements for Rel-16 measurements without gaps”, Nokia