# 3GPP TSG-RAN WG4 Meeting # 110bis	R4-2404978

# Changsha, China, 15th – 19th April, 2024

Agenda Item:	6.6.2

Source: 	Ericsson

Title:	RRM test cases on BWP operation without restriction

Document for:	Discussion

# Introduction

The RRM core requirements for BWP operation without restriction for different options were finalized and agreed in the last meeting [1].

In this paper we provide our analysis of the required RRM performance requirements, which will cover necessary RRM test cases.

# Test cases design

The remaining issues for test cases design are as follow.

Issue 2-1: Test cases list for RLM/BFD/BM requirements for option B-1-1 - FFS: selected set of test cases from the existing test cases for RLM/BFD/L1-RSRP/L1-SINR measurements Issue 2-2: Test cases list for intra-frequency measurements without gaps for option B-1-1 - FFS: selected set of test cases from the existing test cases for intra-frequency measurements without gaps measurements Issue 2-4: Test cases list for intra-frequency measurements and handover for option C - Further discuss whether Test #1 and 4 are needed. Issue 2-6: Test cases list for RLM/BFD/BM requirements for option A - FFS: selected set of test cases from the existing test cases for RLM/BFD/L1-RSRP/L1-SINR measurements

To avoid additional test efforts, RAN4 needs to agree some general rules to downselect the overall test cases for BWP operation without restriction.

- Only test non-DRX cases

- Test both FR1 and FR2

- Not to test SBI reporting

Proposal 1: RAN4 to agree the following general rules to define test cases.

- Test both FR1 and FR2

- Only define test case under non-DRX

- Define test case without SBI reporting

The main aim of the test cases should be to ensure that the UE performs intra-frequency L1/L3 measurements and meet the corresponding requirements based on NW’s configuration in different options. To simplify the test, only non-DRX test cases shall be considered in option A/B-1-1/C.

Proposal 2: For UE supporting option B-1-1, the following RLM/BFD/L1-RSRP/L1-SINR and intra-frequency L3 measurements test cases will be tested.

- TC1: Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode

- TC2: Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode

- TC3: Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode

- TC4: Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode

- TC5: Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode

- TC6: Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode

- TC 7: SSB based L1-RSRP measurement for FR1 when DRX is not used

- TC 8: SSB based L1-RSRP measurement for FR2 when DRX is not used

- TC 9: L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used

- TC 10: L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used

- TC 11: SA event triggered reporting tests without gap for FR1 under non-DRX

- TC 12: SA event triggered reporting tests without gap for FR2 under non-DRX

Proposal 3: For UE supporting option A/option C, the following RLM/BFD/L1-RSRP/L1-SINR test cases will be tested.

- TC1: Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode

- TC2: Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode

- TC3: Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode

- TC4: Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode

- TC5: Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode

- TC6: Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode

- TC 7: SSB based L1-RSRP measurement for FR1 when DRX is not used

- TC 8: SSB based L1-RSRP measurement for FR2 when DRX is not used

- TC 9: L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used

- TC 10: L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used

# Summary

In this contribution, we have discussed the test cases design on BWP operation without restriction requirements. Based on the discussions, the following are the proposals:

Proposal 1: RAN4 to agree the following general rules to define test cases.

- Test both FR1 and FR2

- Only define test case under non-DRX

- Define test case without SBI reporting

Proposal 2: For UE supporting option B-1-1, the following RLM/BFD/L1-RSRP/L1-SINR and intra-frequency L3 measurements test cases will be tested.

- TC1: Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode

- TC2: Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode

- TC3: Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode

- TC4: Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode

- TC5: Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode

- TC6: Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode

- TC 7: SSB based L1-RSRP measurement for FR1 when DRX is not used

- TC 8: SSB based L1-RSRP measurement for FR2 when DRX is not used

- TC 9: L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used

- TC 10: L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used

- TC 11: SA event triggered reporting tests without gap for FR1 under non-DRX

- TC 12: SA event triggered reporting tests without gap for FR2 under non-DRX

Proposal 3: For UE supporting option A/option C, the following RLM/BFD/L1-RSRP/L1-SINR test cases will be tested.

- TC1: Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode

- TC2: Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode

- TC3: Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode

- TC4: Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode

- TC5: Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode

- TC6: Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode

- TC 7: SSB based L1-RSRP measurement for FR1 when DRX is not used

- TC 8: SSB based L1-RSRP measurement for FR2 when DRX is not used

- TC 9: L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used

- TC 10: L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used

# References

- R4-2403544, WF on core maintenance and performance part for BWP without restriction, vivo, Vodafone

- R2-2313951, Reply LS to RAN4 on BWP operation without restriction, RAN2 reply LS to RAN4

- R4-2317430, LS on conclusion on BWP operation without restriction, RAN4 LS to RAN2