3GPP TSG-RAN WG4 Meeting # 110bis	R4-2404986

# Changsha, China, 15th - 19th, April, 2024

Source: 	Samsung

Title: 	Views on HPUE for CA in terrestrial network

Agenda Item:	9.1.2

Document for:	Discussion

- Introduction

In RAN#103, a new WID [1] for UE RF enhancement was approved with three objectives, one of them is High power UE (HPUE) for CA in terrestrial network (TN) (including 3Tx), with the following details.

High power UE (HPUE) for CA in terrestrial network (TN)

- Specify the generic requirements of high power UE (HPUE) for NR uplink (UL) CA in FR1 and EN-DC with NR FR1 bands

  - Power class 1.5 (PC1.5) UE for NR TDD intra-band UL contiguous and non-contiguous CA with 2Tx

    - Specify the requirements for intra-band UL contiguous CA with or without UL-MIMO

      - Example band combinations:

        - CA_n41C, CA_n78C, CA_n77C, CA_n79C for intra-band uplink contiguous CA configurations

      - Focus on the maximum output power (MOP), MPR/A-MPR requirements, SAR solution

    - Specify the requirements for intra-band UL non-contiguous CA without UL-MIMO

      - Example band combinations:

        - CA_n78(2A), CA_n77(2A) for intra-band uplink non-contiguous CA configurations

      - Focus on the maximum output power (MOP), MPR/A-MPR requirements, SAR solution

    - NOTE: leave the other band combination specific requirements to the corresponding Rel-19 basket WIs

  - PC1.5 UE for two band NR inter-band uplink CA with 2Tx and/or 3Tx for handheld and FWA, and PC1.5 and PC2 for two band EN-DC with 2Tx and/or 3Tx for handheld and FWA

    - Focus on the SAR solution

    - Enable power class 2 (PC2) and PC1.5 of two band inter-band uplink CA and EN-DC with 3Tx for handheld UE

      - Identify and update the requirements if necessary

    - Only PC3 is considered for LTE FDD in EN-DC

    - NOTE: leave the band combination specific requirements, e.g., MSD to the corresponding Rel-19 basket WIs

  - Investigate and if feasible, support increasing UE transmission power limit up to the sum of maximum output power per band for NR inter-band uplink CA and EN-DC HPUE with the different existing power classes which have already been specified

  - Introduce the signaling to support the above objectives, if needed.

  - Consider release independency, if needed

Generally there are three targets for HPUE in this WID to our understanding,

- Specifying general requirements for PC1.5 TDD intra-band UL contiguous CA w/ and w/o UL MIMO and specifying general requirements for PC1.5 TDD intra-band UL NC CA w/o MIMO;

- Specifying general requirements for PC1.5 two band NR CA, PC2 and PC1.5 two band EN-DC, with 2Tx and/or 3Tx for handheld UE and FWA;

- Investigate and if feasible, support increasing UE transmission power up to the sum of maximum output per band

We discuss them one by one in the following clauses.

- 3. Discussion

3.1 PC1.5 for intra-band contiguous UL CA w/o UL MIMO

Intra-band contiguous UL CA w/o UL MIMO was introduced in Rel-17, so far PC2 has been specified for n41C, n77C and n78C, but n79C not yet, the following table is excerpted from 38.101-1 v18.4.0.

Similarly for PC1.5 single band, PC1.5 intra-band ULCA shall be achieved through dual Tx, same note as for single band PC1.5 can be added to PC1.5 intra-band ULCA as a clarification, i.e., “Note X: Achieved via dual Tx”. Regarding MOP tolerance in our view it can be +2/-3.

Proposal 1: PC2 for UL CA n79C should be specified as well, can be done in this WI.

Proposal 2: MOP tolerance can be +2/-3 dB, in addition adding the following note into MOP table for PC1.5.

- Note X: Achieved via dual Tx

Regarding SAR compliance, usually the relevant description is under the MOP table, for single band, intra-band contiguous ULCA, intra-band NC ULCA and inter-band ULCA+UL MIMO, the methodology is quite straightforward and can be borrowed.

In addition, please note that there was discussion during Feb meeting on whether to remove the relevant description under MOP table, and further shift some necessary part to configured transmitted power clause. It is expected the discussion would continue under “power class” agenda in this meeting. Therefore, the SAR description would be present in which clause should be TBD but we can first discuss the applicableΔPPowerClass,CA and its condition.

Observation 1: The description for SAR mechanism would be present in which clause should be TBD, depending on the outcome of “power class’ relevant discussion. We can first discuss the applicableΔPPowerClass,CA and its condition.

maxUplinkDutyCycle-PC1dot5-MPE-FR1 was originally introduced for FWA, but in the field actually there is no solution to prohibit handheld UE to utilize it, thus this IE should be deemed as applicable for both handheld UE and FWA. Given intra-band CA reuse single band duty cycle UE capability, it is proposed:

Proposal 3:

For a PC1.5 UE:

ΔPPowerClass,CA = 6dB

- When 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.1 is 23 dBm or lower;

- or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50%;

- or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 ;

- or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306.

ΔPPowerClass,CA = 3dB

- When 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.1 is between 23 dBm and 26dBm;

- or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is between 25% and 50%;

- or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is between 0.5*maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306;

- or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 but less than 2* maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306.

ΔPPowerClass,CA = 0dB otherwise

Note: The exact evaluation period is no less than one radio frame.

In terms of MPR requirements, For PC2 intra-band contiguous ULCA, as can be observed in clause 6.2A.2.1, different architectures are considered for requirements specification:

- 1 PA+1 Lo (TxD does not support, dualPA-Architecture is absent)

- 2 PA+2 Lo (dualPA-Architecture is indicated)

- 2 PA+1 Lo (TxD support, dualPA-Architecture is absent)

For PC1.5 single band single CC operation, given so far from spec perspective, dual Tx is still assumed for implementation and requirements specification, we think it is fair to assume 2Tx/2PA for PC1.5 intra-band ULCA requirements evaluation.

Proposal 4: Consider the following architectures for MPR evaluation.

- 2 Lo +2 PA (dualPA-Architecture is indicated)

- 1 Lo +2 PA (TxD support is indicated)

For PCMAX tolerance, it is proposed:

Proposal 5: PCMAX tolerance for PC1.5 intra-band contiguous UL CA without UL MIMO:

- TLOW=3dB

- THIGH=2dB

Proposal 6：For PC 1.5, NR ALCR as 31dB, UTRA ALCR not needed for the example combos.

3.2 PC1.5 for intra-band contiguous ULCA with UL MIMO

Intra-band contiguous UL CA with UL MIMO was specified in Rel-17, with a new suffix H introduced, and so far only n41C and n78C with PC2 are specified.

The example combos are n41C, n78C, n77C, n79C per the WID, while to our understanding PC2 and PC3 for n77C+UL MIMO and n79C+UL MIMO needs to be specified as well.

Proposal 7: PC2 and PC3 for UL CA n77C with UL MIMO, ULCA n79C with UL MIMO should be specified, can be done in this WI.

Proposal 8: MOP tolerance can be +2/-3 dB, in addition adding the following note into MOP table for PC1.5.

- Note X: Achieved via dual Tx

For SAR compliance, there is no SAR relevant description under MOP table in clause 6.2H.1.1, while as stated in clause 6.2H.1.4 “ΔPPowerClass,CA and ∆TC,c are specified in clause 6.2A.4 unless otherwise stated;” it can be found that intra-band ULCA w/ UL MIMO reuse the same methodology as for intra-band ULCA w/o MIMO, in terms of SAR mechanism and ΔPPowerClass,CA.

Proposal 9: The applicable ΔPPowerClass,CA and its conditions for PC1.5 intra-band contiguous CA w/o UL MIMO can be reused for PC1.5 intra-band contiguous CA w/ UL MIMO.

As stated in clause 6.2H.1.2 “the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2H.1.1-1 is specified in Table 6.2A.2.1-1, Table 6.2A.2.1-2 for power class 3 CA; Table 6.2A.2.1-1b, Table 6.2A.2.1-4 for power class 2 CA”, it can be observed that intra-band contiguous ULCA w/ UL MIMO reuse the MPR  requirements of intra-band contiguous ULCA w/o UL MIMO for which dual Tx(TxD) is assumed for PC2, in our view same methodology can be applied for PC1.5.

Proposal 10: For PC1.5, intra-band contiguous ULCA with UL MIMO can reuse the MPR requirements of intra-band contiguous ULCA without UL MIMO and with dual Tx(TxD).

Proposal 11: PCMAX tolerance for PC1.5 intra-band contiguous UL CA with UL MIMO can be:

- TLOW=3dB

- THIGH=2dB

3.3 PC1.5 for intra-band NC ULCA w/o UL MIMO

The discussion points are quite similar with intra-band contiguous ULCA without UL MIMO, which are not duplicated here.

Proposal 12: MOP tolerance can be +2/-3, in addition adding the following note into MOP table for PC1.5.

- Note X: Achieved via dual Tx

Proposal 13:

For a PC1.5 UE:

ΔPPowerClass,CA = 6dB

- When 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.2 is 23 dBm or lower;

- or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50%;

- or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 ;

- or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306.

ΔPPowerClass,CA = 3dB

- When 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.2 is between 23 dBm and 26dBm;

- or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is between 25% and 50%;

- or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is between 0.5*maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306;

- or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 but less than 2* maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306.

ΔPPowerClass,CA = 0dB otherwise

Note: The exact evaluation period is no less than one radio frame.

Proposal 14: Consider the following architectures for MPR discussion.

- 2 Lo +2 PA (dualPA-Architecture is indicated)

- 1 Lo +2 PA (TxD support is indicated)

Proposal 15: PCMAX tolerance for PC1.5 intra-band non-contiguous UL CA without UL MIMO

- TLOW=3dB

- THIGH=2dB

Proposal 16：For PC1.5, NR ALCR as 31dB, UTRA ALCR not needed for the example combos.

3.4 PC1.5 for 2-band Inter-band UL NR-CA

As directed by the WID, for inter-band UL NR-CA, only PC1.5 is under consideration, and both 2Tx and 3Tx for handheld UE and FWA are in the scope.

Particularly, in Rel-18, 3Tx for PC2 and PC1.5 is specified for some selected scenarios (reproduced as below) and for FWA only in Rel-18 with a high level clarification in clause 4.3 as following.

For Rel-19 3Tx, we think we can first remove the limitation in clause 4.3, moreover the scope/scenarios can be limited to Rel-18 scenarios unless new requests/scenarios raised from operators.

Proposal 17: Same requirements of 3Tx FWA are applicable to 3Tx handheld UE, and to support 3Tx operation for handheld UE in Rel-19, remove the following limitation in clause 4.3 of TS 38.101-1.

“For a terminal that supports inter-band Carrier Aggregation (CA) with UL MIMO or Tx diversity operation, the requirements are targeted for FWA form factor in current version of specification.”

Proposal 18: For Rel-19 3Tx operation, the scenarios can be the same with Rel-18 scenarios unless new requests/scenarios raised from operators. Depending on the new requests/scenarios, some general requirements including the Note in MOP table and SAR requirements can be updated if needed.

In addition, for PC1.5, 3Tx (ULCA+TxD or ULCA+UL MIMO, 23+29=29) was introduced in Rel-18 while PC1.5 2Tx (26+26=29) was postponed to Rel-19 due to the time limitation of Rel-18. But from requirements perspective, 2Tx and 3Tx share some general requirements such as MOP tolerance.

Proposal 19: For 2Tx PC1.5 MOP tolerance, reuse the 3Tx PC1.5 MOP tolerance, i.e., +2/-3dB.

For PC1.5 2Tx MSD, though it is usually band combination specific requirement, we think it is necessary to set some high level guidance for it in this WI.

More specific, for PC1.5 3Tx MSD evaluation, the PCMAX for each band is agreed and captured in Note 1 of Table 7.3A.5-1b, which corresponds to the Rel-18 scenario, i.e., 23dBm for single Tx band, and 27.8dBm for the band with 2Tx, in total 29dBm. While for PC1.5 2Tx, 26 PA +26 PA architecture is assumed, thus the 3Tx framework for IMD evaluation is not suitable anymore, thus it is suggested to introduce a new MSD due to IMD table for 2Tx PC1.5 assuming min (+26dBm, PCMAX_L,f,c) for each band.

Proposal 20: For 2Tx PC1.5 IMD, it is suggested to introduce a new MSD table assuming min (+26dBm, PCMAX_L,f,c) for each band for MSD evaluation.

Proposal 21: For 2Tx PCMAX tolerance, it was already automatically specified when 3Tx PC1.5 was introduced in Rel-18, since 3Tx configured transmitted power clause refer to 2Tx configured transmitted power clause.

For PC1.5 2Tx SAR compliance, similar mechanism for PC1.5 3Tx SAR mechanism (specified in clause 6.2H.3.1 and clause 6.2L.3.1) can be referenced, while please note the description is limited to Rel-18 3Tx scenario (i.e., PC3+PC1.5=PC1.5), thus some update is needed for PC1.5 2Tx.

Proposal 22: For 2Tx PC1.5 SAR compliance, 3Tx PC1.5 SAR methodology can be referred while the restriction on the scenarios needs to be removed/updated.

3.5 PC1.5 and PC2 for two-band EN-DC

As instructed by the WID, both PC2 and PC1.5, handheld UE and FWA are considered. The discussion points are similar as for 2Tx PC1.5 NR-CA.

Proposal 23: Same requirements of 3Tx FWA are applicable to 3Tx handheld UE, and to support 3Tx operation for handheld UE in Rel-19, remove the following limitation in clause 4.2 of TS 38.101-3.

“For a terminal that supports inter-band Dual-Connectivity (DC) with UL MIMO or Tx diversity operation, the requirements are targeted for FWA form factor in current specification.”

PC1.5 3Tx for EN-DC has not been specified in Rel-18, so it is suggested to first clarify the 3Tx scenarios to be considered in Rel-19, as well as if there is any new request/demand for PC2 3Tx EN-DC scenarios.

Proposal 24: For Rel-19 3Tx PC2/PC1.5 EN-DC, It is suggested to first collect the demand/request on the following aspects before specifying the requirements.

- PC1.5 scenarios

- PC2 scenarios, Whether there is new scenarios to be considered in Rel-19 compared with the Rel-18 scenarios

3.6 Increasing UE transmission power

If understood correctly of the objective, it intends for leveraging the existing increasing UE high power limit feature (higherPowerLimit-r17, higherPowerLimitMRDC-r17) to cover more “power classes”. However, it is not crystal clear to us what does “investigate and if feasible” intends for, probably it means study the applicable cases.

Increasing high power limit feature was introduced in Rel-17 in order to maximize PA’s capability to achieve full power. Due to the limited time, only PC3+PC2(=27.8dBm in total) which is based on PC2 was specified in Rel-17 for both NR-CA and EN-DC with single carrier for each individual band, other scenarios were dropped. In Rel-18, the feature was further extended to PC3+PC5 (=24.6 dBm in total) which is based on PC3, one of the two bands is NR-U band with PC5 as default PC, and both single carrier for each band(2CC in total) and inter+intra band (3CC UL) are covered.

Moreover, in Rel-18 3Tx WI, it is clarified “Note 2: Increase UE power high limit feature is not included”. Accordingly though PC1.5 is introduced in this WI, PC3+PC1.5 or PC2+PC1.5 has not been discussed yet.

Observation 2: The following table summarize the specified scenarios of increasing high power limit feature in Rel-17/18.

| Indicated PC for A-B (2Tx in total) | PC for band A of A-B | PC for band B of A-B | From which release increasing high power limit feature supported | Note |
| --- | --- | --- | --- | --- |
| PC2 | PC3 | PC2 | Support from Rel-17 | One CC per band |
| PC3 | PC5(NRU band) | PC3 | Support from Rel-18 | One CC per band; One CC on band B, 2CC on band A(NRU band) |

In the following part we share the possible scenarios that can be considered in Rel-19, please note that the analysis is based on the already specified existing combos and/or the combos/scenarios would certainly be specified within Rel-19. Thus for 2Tx, we think there is no new cases to be considered.

Proposal 25: For 2Tx, if based on existing specified combos, there seems no new scenarios to be considered so far.

Given 3Tx and 2Tx CA are in different suffix, they can be considered separately. The following table summarize the scenarios that may could be considered/discussed in Rel-19.

Proposal 26: For 3Tx, the following scenarios may could be considered.

(Note the analysis is based on the specified combos and the combos/scenarios would certainly be specified within Rel-19)

| Indicated PC for A-B (3Tx in total) | PC for band A of A-B (1Tx) | PC for band B of A-B (2Tx) | The total power (dBm) | Note |
| --- | --- | --- | --- | --- |
| PC2 | PC3 | PC2 | 27.8 | One CC per band |
| PC1.5 | PC3 | PC1.5 | 30.8 | One CC per band; For FWA only |

- 3. Conclusion

For 3.1 PC1.5 for intra-band contiguous UL CA w/o UL MIMO

Proposal 1: PC2 for UL CA n79C should be specified as well, can be done in this WI.

Proposal 2: MOP tolerance can be +2/-3 dB, in addition adding the following note into MOP table for PC1.5.

- Note X: Achieved via dual Tx

Observation 1: The description for SAR mechanism would be present in which clause should be TBD, depending on the outcome of “power class’ relevant discussion. We can first discuss the applicableΔPPowerClass,CA and its condition.

Proposal 3:

For a PC1.5 UE:

ΔPPowerClass,CA = 6dB

- When 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.1 is 23 dBm or lower;

- or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50%;

- or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 ;

- or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306.

ΔPPowerClass,CA = 3dB

- When 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.1 is between 23 dBm and 26dBm;

- or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is between 25% and 50%;

- or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is between 0.5*maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306;

- or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 but less than 2* maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306.

ΔPPowerClass,CA = 0dB otherwise

Note: The exact evaluation period is no less than one radio frame.

Proposal 4: Consider the following architectures for MPR evaluation.

- 2 Lo +2 PA (dualPA-Architecture is indicated)

- 1 Lo +2 PA (TxD support is indicated)

Proposal 5: PCMAX tolerance for PC1.5 intra-band contiguous UL CA without UL MIMO:

- TLOW=3dB

- THIGH=2dB

Proposal 6：For PC 1.5, NR ALCR as 31dB, UTRA ALCR not needed for the example combos.

For 3.2 PC1.5 for intra-band contiguous ULCA with UL MIMO

Proposal 7: PC2 and PC3 for UL CA n77C with UL MIMO, ULCA n79C with UL MIMO should be specified, can be done in this WI.

Proposal 8: MOP tolerance can be +2/-3 dB, in addition adding the following note into MOP table for PC1.5.

- Note X: Achieved via dual Tx

Proposal 9: The applicable ΔPPowerClass,CA and its conditions for PC1.5 intra-band contiguous CA w/o UL MIMO can be reused for PC1.5 intra-band contiguous CA w/ UL MIMO.

Proposal 10: For PC1.5, intra-band contiguous ULCA with UL MIMO can reuse the MPR requirements of intra-band contiguous ULCA without UL MIMO and with dual Tx(TxD).

Proposal 11: PCMAX tolerance for PC1.5 intra-band contiguous UL CA with UL MIMO can be:

- TLOW=3dB

- THIGH=2dB

For 3.3 PC1.5 for intra-band NC ULCA w/o UL MIMO

Proposal 12: MOP tolerance can be +2/-3, in addition adding the following note into MOP table for PC1.5.

- Note X: Achieved via dual Tx

Proposal 13:

For a PC1.5 UE:

ΔPPowerClass,CA = 6dB

- When 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.2 is 23 dBm or lower;

- or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50%;

- or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 ;

- or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306.

ΔPPowerClass,CA = 3dB

- When 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.2 is between 23 dBm and 26dBm;

- or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is between 25% and 50%;

- or when the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is between 0.5*maxUplinkDutyCycle-PC2-FR1 and maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306;

- or when the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 but less than 2* maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306.

ΔPPowerClass,CA = 0dB otherwise

Note: The exact evaluation period is no less than one radio frame.

Proposal 14: Consider the following architectures for MPR discussion.

- 2 Lo +2 PA (dualPA-Architecture is indicated)

- 1 Lo +2 PA (TxD support is indicated)

Proposal 15: PCMAX tolerance for PC1.5 intra-band non-contiguous UL CA without UL MIMO

- TLOW=3dB

- THIGH=2dB

Proposal 16：For PC1.5, NR ALCR as 31dB, UTRA ALCR not needed for the example combos.

For 3.4 PC1.5 for 2-band Inter-band UL NR-CA

Proposal 17: Same requirements of 3Tx FWA are applicable to 3Tx handheld UE, and to support 3Tx operation for handheld UE in Rel-19, remove the following limitation in clause 4.3 of TS 38.101-1.

“For a terminal that supports inter-band Carrier Aggregation (CA) with UL MIMO or Tx diversity operation, the requirements are targeted for FWA form factor in current version of specification.”

Proposal 18: For Rel-19 3Tx operation, the scenarios can be the same with Rel-18 scenarios unless new requests/scenarios raised from operators. Depending on the new requests/scenarios, some general requirements including the Note in MOP table and SAR requirements can be updated if needed.

Proposal 19: For 2Tx PC1.5 MOP tolerance, reuse the 3Tx PC1.5 MOP tolerance, i.e., +2/-3dB.

Proposal 20: For 2Tx PC1.5 IMD, it is suggested to introduce a new MSD table assuming min (+26dBm, PCMAX_L,f,c) for each band for MSD evaluation.

Proposal 21: For 2Tx PCMAX tolerance, it was already automatically specified when 3Tx PC1.5 was introduced in Rel-18, since 3Tx configured transmitted power clause refer to 2Tx configured transmitted power clause.

Proposal 22: For 2Tx PC1.5 SAR compliance, 3Tx PC1.5 SAR methodology can be referred while the restriction on the scenarios needs to be removed/updated.

For PC1.5 and PC2 for two-band EN-DC

Proposal 23: Same requirements of 3Tx FWA are applicable to 3Tx handheld UE, and to support 3Tx operation for handheld UE in Rel-19, remove the following limitation in clause 4.2 of TS 38.101-3.

“For a terminal that supports inter-band Dual-Connectivity (DC) with UL MIMO or Tx diversity operation, the requirements are targeted for FWA form factor in current specification.”

Proposal 24: For Rel-19 3Tx PC2/PC1.5 EN-DC, It is suggested to first collect the demand/request on the following aspects before specifying the requirements.

- PC1.5 scenarios

- PC2 scenarios, Whether there is new scenarios to be considered in Rel-19 compared with the Rel-18 scenarios

For 3.6 Increasing UE transmission power

Observation 2: The following table summarize the specified scenarios of increasing high power limit feature in Rel-17/18.

| Indicated PC for A-B (2Tx in total) | PC for band A of A-B | PC for band B of A-B | From which release increasing high power limit feature supported | Note |
| --- | --- | --- | --- | --- |
| PC2 | PC3 | PC2 | Support from Rel-17 | One CC per band |
| PC3 | PC5(NRU band) | PC3 | Support from Rel-18 | One CC per band; One CC on band B, 2CC on band A(NRU band) |

Proposal 25: For 2Tx, if based on existing specified combos, there seems no new scenarios to be considered so far.

Proposal 26: For 3Tx, the following scenarios may could be considered.

(Note the analysis is based on the specified combos and the combos/scenarios would certainly be specified within Rel-19)

| Indicated PC for A-B (3Tx in total) | PC for band A of A-B (1Tx) | PC for band B of A-B (2Tx) | The total power (dBm) | Note |
| --- | --- | --- | --- | --- |
| PC2 | PC3 | PC2 | 27.8 | One CC per band |
| PC1.5 | PC3 | PC1.5 | 30.8 | One CC per band; For FWA only |

- 4. Reference

[1] RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4