3GPP TSG-RAN WG4 Meeting #110bis	R4-2404989

Changsha, China, 15-19 Apr, 2024

Title:	LS on further clarification for ue-PowerClassPerBandPerBC-r17 (R4 16-8)

Response to:

Release:	Rel-17

Work Item:	Power_Limit_CA_DC

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact Person:

Name:  Yuanyuan Zhang

E-mail Address:  tina55.zhang@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- Overall Description:

In RAN4#106 meeting, RAN4 replied RAN2’s LS on the received clarification questions for the interaction between ue-PowerClassPerBandPerBC-r17 (R4 16-8) and other existing different power class parameters. After extended discussion during subsequent meetings, RAN4 would like to have following further clarification and updated information with RAN2.

- ue-PowerClassPerBandPerBC-r17 is applicable to all NR-CA configurations. ue-PowerClassPerBandPerBC-r17 is not applicable to MR-DC band combinations.

- ue-PowerClassPerBandPerBC-r17 indicates the power class that a UE supports over the carrier(s) for each individual band within a given band combination, while powerClass/powerClass-v1610 indicates the power class the UE supports when operating according to this band combination.

- ue-PowerClassPerBandPerBC-r17 if indicated for a band shall determine the power class the UE supports over the carrier(s) for this constituent band of a band combination, otherwise ue-PowerClass/ue-PowerClass-v1610/ue-PowerClass-v1700 applies.

- ue-PowerClassPerBandPerBC-r17 does not modify powerClass/powerClass-v1610 for a band combination and conversely. The capability definition of powerClass/powerClass-v1610 requires update to include ue-PowerClassPerBandPerBC-r17, i.e., if the power class indicated by powerClass/powerClass-v1610 is higher than the power class that the UE supports on the individual bands of this band combination (ue-PowerClassPerBandPerBC-r17 if indicated or ue-PowerClass in BandNR otherwise), the latter determines maximum TX power available in each band.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully ask RAN2 to take above RAN4 clarification into consideration.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #111	20 – 24 May, 2024		              Fukuoka, Japan

TSG-RAN WG4 Meeting #112	19 - 23 Aug, 2024		               Maastricht, NL