3GPP TSG-RAN WG4 Meeting # 110bis		R4-2405011

# Changsha, China, 15th – 19th April, 2024

Agenda item:	10.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on missing parameters for different power class

Document for:	Discussion

# Introduction

The missing parameters for different power classes were discussion in last RAN4 meeting triggered by LS from RAN5 [1]. The corresponding agreements are captured in [2]. In this contribution, we further provide our views on the remaining issues.

# Discussion

Based on the discussion in last meeting, the gain difference between rough beam and fine beam in peak direction and spherical coverage directions were agreed as follows:

Issue 2-1-6: How to consider Gain difference Y between fine and rough beams at Rx beam peak direction? - Agreement: Value “Y” in dB, for each UE power class. 1; 2; 3; 4; 5; 6; 7. [18]; 9.0; 7.0; FFS; [15.5]; [15.5]; FFS. Issue 2-1-7: How to consider Gain difference Z between fine and rough beams at Spherical coverage directions? - Agreement: Value “Z” in dB, for each UE power class. 1; 2; 3; 4; 5; 6; 7. [18]; 9.0; 7.0; FFS; [15.5]; [15.5]; FFS.

The values were derived based on the assumptions of antenna elements for different power class as follows:

PC 1: 64

PC 5: 36

PC 6: 36

Observation 1: The value of “Y” and “Z” are defined based on the assumption that the number element for PC1/5/6 are 64/36/36.

For the remaining parameters, we present our views/analysis based on the same assumptions.

For the UE gain G, the status is summarized as follows:

Issue 2-1-11: UE gain G - Proposals - Option 1: Qualcomm UE gain G, Rx beam peak direction UE Power class. 1; 2; 3; 4; 5; 7. Minimum, dBi; 0; FFS; -10; FFS; -5; FFS. Maximum, dBi; 57; FFS; +20; FFS; 57; FFS. - Option 2: Samsung There is no need to define the UE gain (B 2.1.5 and B 2.1.6) for PC6. - Option 2: Nokia UE gain is needed for PC6. Exact value FFS. - Agreement: - FFS PC6 shall have the same UE gain as PC5 - FFS the value of UE gain for PC1, 5, and 6

Regarding the Gmin and Gmax, we suggest to define the requirements for different PC taking PC3 as the reference. For Gmax, the worst case is that the gain of the rough beam is same as that for fine beam. Based on the observation 1, since the more antenna elements are assumed for PC1/5/6 compared with PC3, larger antenna gain can be assumed. Though there is no agreed assumptions on number of elements for PC3 when defining Gmax, it is recalled that 4/8 elements are mentioned by companies during the discussion. Thus, for Gmax for PC 1, Gmax can be derived as 20 dB +10*lg (64/8) = 29 dB. Similarly, for PC 5 and 6, Gmax can be derived as 20 dB + 10*lg(36/8) = 26.5 dB.

Observation 2: Taking PC3 as reference, Gmax for PC1/PC5/PC6 are proposed to be 29dB/26.5dB/26.5dB.

For Gmin, though more antenna elements are assumed for Gmax and also REFSENS requirements, Gmin are cover the worst case. Following the same logic when defining “Y” and “Z”, where the values are derived assuming that only one element is active for rough beam. Thus, there is no significant difference between different power classes for Gmin.

Observation 3: Considering the worst case for the minimum gain for rough beam, Gmin are similar for different power class.

Based on observation 2 and 3, the missing value for UE gain G are proposed as follows:

UE gain G, Rx beam peak direction

|  | UE Power class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 |
| Minimum, dBi | -10 | FFS | -10 | FFS | -10 | -10 |
| Maximum, dBi | 29 | FFS | +20 | FFS | 26.5 | 26.5 |

Proposal 1: The UE gain G for PC1/5/6 are defined as follows:

UE gain G, Rx beam peak direction

|  | UE Power class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 |
| Minimum, dBi | -10 | FFS | -10 | FFS | -10 | -10 |
| Maximum, dBi | 29 | FFS | +20 | FFS | 26.5 | 26.5 |

Another issue is about the rough beam gain reduction “D”. The status is summarized as follows:

Issue 2-1-13: the values of Rough Beam gain reduction “D” - Proposals - Option 1: check with RAN5 whether the values of Rough Beam gain reduction “D” needs to be defined in RAN4 in B.2.1.5. - Option 2: other，please specify - Agreement: - RAN4 continue to discuss on the values of rough beam gain reduction “D”

The definition of D is defined as follows:

#### B.2.1.5.3 Alignment of Rough beam to Rx beam Peak The definition of Rx Beam Peak in TS 38.101-2 [19] clause 7.3.2 is based on Throughput at Reference sensitivity power level, and assumes use of Fine beams. In many RRM scenarios the UE can use Rough beams, but the largest Rough beam gain direction may not be aligned to the Fine beam Peak direction. When the Rx Beam Peak is selected and defined based on Fine Beams, the rough beam gain in that direction may be lower than the largest rough beam gain in another direction within Spherical Coverage. The term “D” is the maximum allowed rough beam gain reduction, and is specified in Table B.2.1.5.3-1 for each power class. Table B.2.1.5.3-1: Rough Beam gain reduction “D” in Rx Beam Peak direction UE Power class. 1; 2; 3; 4; 7. Maximum gain reduction, dB; FFS; FFS; 5.5; FFS; FFS.

The intention of involving D is the relative accuracy TC is that: The Rough Peak beam (Beam 1) is selected based on the Fine beam peak direction, and the coverage rough beam (Beam 2) is selected from the sets for fine coverage rough beam. Companies stated that there could be misalignment between the rough peak and fine peak. Then, for the worst case, Beam 2 happens to be the peak direction while the gain of Beam 1 is lower than that of Beam 2. Thus, additional margin is needed.

For power class 1/5/6 with more antenna elements, the situation could be severer. Since the beam is narrower for PC1/5/6 than PC3, the beam misalignment between fine and rough beam could result in more significant gain reduction. Also, it could be observed from RF spec that, PC3 UE shall have better spherical coverage performance than other PCs (e.g. 50% CCDF vs 85 CCDF).

Observation 4: The beam misalignment between fine beam and rough beam will lead to severer rough beam gain reduction for PC1/5/6 since:

- Narrower beam with more antenna elements

- Looser requirements for spherical coverage performance

Thus, the rough beam gain reduction for PC1/5/6 shall be larger than that defined for PC 3. However, as discussed in R15 maintenance for PC3, this value is highly related to UE implementation, it is hard to converge on a concrete value. It is proposed to add 3 dB margin for PC 5/6 and another 3 dB for PC1.

Proposal 2: The rough beam gain reduction for PC1/5/6 are defined as:

Rough Beam gain reduction “D” in Rx Beam Peak direction

|  | UE Power class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 |
| Maximum gain reduction, dB | 10.5 | FFS | 5.5 | FFS | 8.5 | 8.5 |

# Conclusions

Observation 1: The value of “Y” and “Z” are defined based on the assumption that the number element for PC1/5/6 are 64/36/36.

Observation 2: Taking PC3 as reference, Gmax for PC1/PC5/PC6 are proposed to be 29dB/26.5dB/26.5dB.

Observation 3: Considering the worst case for the minimum gain for rough beam, Gmin are similar for different power class.

Proposal 1: The UE gain G for PC1/5/6 are defined as follows:

UE gain G, Rx beam peak direction

|  | UE Power class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 |
| Minimum, dBi | -10 | FFS | -10 | FFS | -10 | -10 |
| Maximum, dBi | 29 | FFS | +20 | FFS | 26.5 | 26.5 |

Observation 4: The beam misalignment between fine beam and rough beam will lead to severer rough beam gain reduction for PC1/5/6 since:

- Narrower beam with more antenna elements

- Looser requirements for spherical coverage performance

Proposal 2: The rough beam gain reduction for PC1/5/6 are defined as:

Rough Beam gain reduction “D” in Rx Beam Peak direction

|  | UE Power class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 |
| Maximum gain reduction, dB | 10.5 | FFS | 5.5 | FFS | 8.5 | 8.5 |

# References

[1] R5-237837 LS on defining the missing relative angular offsets and UE gain-related parameters for different power classes

[2] R4-2403446 Ad-hoc minutes on Reply LS

[3] R4-2403526 WF on RRM requirements for Netw_Energy_NR