3GPP TSG-RAN WG4 Meeting # 110-bis					R4-2405044

China, Changsha, April 15 – April 19, 2024

Title:	TP to TR 37.718-11-21: Addition of DC_28A_n78A-n105A

Source:	Nokia, Spark

Agenda item:	5.6.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 37.718-11-21 to add DC_28A_n78A-n105A as specified in WID [1]. All fallbacks have been analysed.

************************************* Start of TP*****************************************

## 6.x	DC_28_n78-n105

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_28_n78-n105 | 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

Table 6.x.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_28A_n78A-n105A | DC_28A_n78A |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_28A_n78A-n105A | 28 | 15 |  | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 155 |
|  | n78 | 15 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | n105 | 15 |  | 5 | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |  |

### 6.x.3	Co-existence studies

For UE coexistence study of UL DC_28_n78, the 2nd, 3rd, 4th, and 5th order harmonics and the 2nd, 3rd, 4th, and 5th order inter-modulation products are calculated and presented in Table 6.x.3-1.

Based on the calculation, we identify the following interference impact:

- The IMD5 of UL DC_28_n78 may have an impact on DL Band n105.

Table 6.x.3-1: The harmonic and IMD products caused by UL DC_28_n78

| UE DL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 803 | 3300 | 3800 |
| 3rd Band DL | 612 | 652 |  |  |

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2552 | 3097 | 4003 | 4548 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2394 | 1804 | 5852 | 6897 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4706 | 5296 | 7303 | 8348 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1691 | 1056 | 9152 | 10697 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6194 | 5104 | 8006 | 9096 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5409 | 6044 | 10603 | 12148 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14497 | 12452 | 308 | 988 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 9994 | 8404 | 4356 | 5491 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 13903 | 15948 | 6112 | 6792 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11306 | 12896 | 8709 | 9844 |

### 6.x.4	∆TIB and ∆RIB values

For DC_28_n78-n105, ΔTIB,c values are as follows (from DC_12-71_n77).

Table 6.x.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_28_n78-n105 | 0.4 | 0.5 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

There are no ΔRIB,c values for in DC_12-71_n77 table Table 7.3B.3.3.2-1. Hence, as per 7.3B.3.0, the ΔRIB,c values are set to zero.

### 6.x.5	MSD requirements

The MSD values for IMD5 of UL DC_28_n78 impact on DL Band n105 is re-used from DC_12A-71A_n77A.

The MSD requirements can be specified as follows:

Table 6.x.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands) NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_28A_n78A-n105A | 28 | 705.5 | 5 | 25 | 760.5 | N/A | N/A |
|  | n78 | 3450 | 10 | 50 | 3450 | N/A | N/A |
|  | n105 | 674 | 5 | 25 | 628 | 3.9 | IMD5 |

************************************* End of TP******************************************

# References

[1]	RP-233488，Rel-18 Dual Connectivity (DC) of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)，LG Electronics，RAN#102, December 2023