3GPP TSG-RAN WG4 Meeting # 110-bis					R4-2405053

China, Changsha, April 15 – April 19, 2024

Title:	TP to TR 38.898: Addition of PC2 for DC_40A_n40A

Source:	Nokia, nbn

Agenda item:	5.15.2

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.898 to add PC2 for DC_40A_n40A. DC_40A_n40A PC3 is completed as a fallback.

************************************* Start of TP*****************************************

## 5.x	DC_40_n40

### 5.x.1	Configurations

When requested EN-DC configuration is 2DL2UL, it is not needed to update the inter-band EN-DC configuration table. By referring to the maximum output power table, it can be checked whether these configurations support PC2. This band combination for PC3 is already specified in TS 38.101-3, so this section is omitted.

### 5.x.2	Maximum output power

Table 5.x.2-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| DC_40A_n40A4 | 26 | +2/-3 | 23 | +2/-3 |
| NOTE 4: Only single switched UL is supported. |  |  |  |  |

### 5.x.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is not needed due to higher power UL DC. Since DC_40_n40 is to support only single switched UL, there should be no MSD interference issues occurred. Hence, additional MSD is not needed.

### 5.x.4	∆TIB and ∆RIB values

ΔTIB,c and ΔRIB,c are not applicable for intra-band non-contiguous EN-DC, so this section is omitted.

************************************* End of TP*****************************************