3GPP TSG-RAN WG4 Meeting #110bis	R4-2405089

Changsha, China, 15 April – 19 April 2024

Agenda Item:	6.7.3.2

Source:	Ericsson

Title:	UE demodulation requirements for FR2 HST with CA

Document for:	Discussion

# 1	Introduction

RAN4#110 agreed with the WF on the UE demodulation requirements for Rel-18 FR2 HST [1]. This contribution discusses the UE feature on UE demodulation performance for FR2 HST CA.

# 2	UE feature

One remaining open issue is the UE demodulation performance features for FR2 HST CA [2].

Issue 4-1-1: UE feature lists for FR2 PDSCH requirements with CA WF - Following the UE feature list in main session discussion on Rel-18 FR2 HST, the components of support of the enhanced demodulation requirements specified for CA for HST FR2 UE is not captured into the UE feature list of 34-2

Rel-18 CA demodulation requirements for FR2 HST is based on the single carrier UE demodulation requirements for Rel-17 FR2 HST, where RAN4 has introduced the UE feature FG 22-1 “Support of FR2 HST operation”. Since Rel-18 CA demodulation requirements are simple extension to CA, we don’t think additional UE feature is necessary, that is, the combination of Rel-17 FG 22-1 (PC6) and CA capabilities is sufficient to define this feature. Therefore, we don’t think a new UE feature on “Enhanced demodulation requirements for CA HST FR2” is necessary.

Proposal: Not to introduce new UE feature components on enhanced demodulation requirements for CA HST FR2.

# 3	Summary

Proposal: Not to introduce new UE feature components on enhanced demodulation requirements for CA HST FR2.

# References

- R4-2402861, “Way forward on [110][316] NR_HST_FR2_enh_Demod”, Samsung.

- R4-2403842, “Rel-18 RAN4 UE feature list for NR (version 3)”, CMCC.

# Appendix

## A.1	UE feature on NR_HST_FR2_enh [2]

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34.NR_HST_FR2_enh | 34-1 | Support of NR FR2 HST with simultaneous DL reception with two different QCL TypeD RSs | 1) Support of enhanced RF requirement to support FR2-1 PC6 UEs with simultaneous DL signals reception with two different QCL TypeD RSs 2) Support of enhanced RRM requirement to support FR2-1 PC6 UEs with simultaneous DL signals reception associated with two different QCL TypeD RSs | 22-1 | Yes | N/A | UE does not support FR2 high speed train scenario with simultaneous DL reception with two different QCL TypeD RSs | Per Band | No | FR2 only | N/A | A single indication element is used to indicate for the components for 34-1. The UE capability reported in this feature is applied when highSpeedDeploymentTypeFR2-r17 is configured by network as bidirectional | Optional with capability signaling |
| 34.NR_HST_FR2_enh | 34-2 | Enhanced FR2 HST RRM requirements for intra-band CA and inter-frequency measurements in connected mode | 1) Support of the RRM requirement for intra-band CA operation in connected mode to support FR2 high speed up to 350 km/h, as specified in TS 38.133 2) Support of the RRM requirement for enhanced inter-frequency measurements in connected mode to support FR2 high speed up to 350 km/h, as specified in TS 38.133 | 22-1 | Yes | N/A | The performance of intra-frequency measurement on SCC and/or inter-frequency measurements in connected mode for NR FR2 HST scenario cannot be guaranteed | Per band | No | FR2 only | N/A |  | Optional with capability signaling |
| 34.NR_HST_FR2_enh | 34-3 | Enhanced FR2 HST RRM requirements for inter-frequency measurement in Idle and Inactive mode | Support of the RRM requirement for inter-frequency measurements in idle and Inactive mode to support FR2 high speed up to 350 km/h, as specified in TS 38.133 | 22-1 | No | N/A | The performance of inter-frequency measurement in idle and Inactive mode for FR2 HST scenario cannot be guaranteed | N/A | No | FR2 only | N/A |  | Optional without capability signaling |
| 34.NR_HST_FR2_enh | 34-4 | Support of enhanced MAC CE for TCI state switch indication for FR2 HST | 1. Support of enhanced one-shot large UL transmit timing adjustment requirement to support FR2-1 PC6 UEs, as specified in TS 38.133 based on [the cross-RRH TCI state indication for UE-specific PDCCH MAC CE] 2. Support of enhanced TCI state switching delay requirewments based on new MAC CE for TCI state switch indication named as [the cross-RRH TCI state indication for UE-specific PDCCH MAC CE] in HST FR2 scenario, as specified in TS 38.133. | 22-1 | Yes | N/A | UE does not support enhanced MAC CE for TCI state switch indication for FR2 HST | Per Band | No | FR2 only | N/A | The naming of MAC CE for [the cross-RRH TCI state indication for UE-specific PDCCH] depends on RAN2 | Optional with capability signalling |