3GPP TSG-RAN WG4 Meeting #110bis	R4-2405096

Changsha, China, 15 April – 19 April 2024

Agenda Item:	6.24.4

Source:	Ericsson

Title:	Discussion on UE demodulation requirements for NES

Document for:	Discussion

# 1	Introduction

RAN4#110 agreed with the way forward on the UE demodulation and CSI reporting requirements for the NR network energy saving [1]. We continue to discuss the open issues.

# 2	Discussion

## 2.1	UE demodulation requirements for SSB-less SCell operation for inter-band CA in FR1

Issue 1-1: Whether to introduce SSB less Scell requirements for inter-band CA Agreement - RAN4 to evaluate the performance of SSB-less SCells CA performance, if the observed performance constitutes a practical operating point of the feature, or if SSB-less SCell compliant UE shall be tested with normal CA requirements

RAN4#110 discussed whether to define PDSCH demodulation requirements for the SSB-less SCell operation and agreed to evaluate the PDSCH demodulation performance of SSB-less SCell with received time difference (RTD) from PCell. Table 1 shows the simulation parameters we evaluated according to the WF [1].

Table 1	Simulation parameters for PDSCH on SSB-less SCell.

| Parameter | Unit | Value |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Active DL BWP index |  | 1 |
| Channel model |  | TDLA30-10 |
| Antenna configuration |  | 2x2, ULA Low |
| SCS/CBW |  | 30kHz/40MHz (106 PRB) |
| MCS |  | 16QAM, 0.48 (MCS13) |
| Channel estimation |  | DM-RS based: non-ideal/practical |
| TO Estimation & Compensation |  | As per implementation |
| TO of SCell from PSCell | CP | [0, 0.1, 0.4, 0.9, 1.0] x CP |
| FFT window shift target |  | {As per implementation} |
| SSB/TRS |  | PCell: SSB and TRS (optional) SCell: TRS only |
| Precoder |  | Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 |
| Rank |  | 1 and 2 |

| (a) Rank 1 | (b) Rank 2 |
| --- | --- |

Figure 1	SSB-less SCell PDSCH simulation results.

Figure 1 shows our PDSCH simulation results with several RTD values. It is observed that the results with 0 < RTD < 1.0 x CP show slight better performance than the result with RTD = 0. We think it comes from the TDL channel model and FFT start position. Figure 2 illustrates the FFT start point for SCell with RTD>0. We usually set the FFT start position at the peak of path delay profile (PDP). However, a part of signals is also come before the FFT start position in the case of TDLA30. We think RTD > 0 effectively could capture more wanted signals compared with RTD = 0, and this the performance could be improved slightly.

Figure 2 FFT start point for SCell with RTD.

Observation: PDSCH performance for SCell with 0 < RTD < 1.0CP is same or slight better than that with RTD=0.

As we commented in RAN4#110, this is co-located FR1 CA scenario, we don’t expect huge RTD which is close to or exceeds a CP, although the RRM requirements set the MRTD to 1.0 x CP. Moreover, RAN5 conformance test starts to measure the SCell PDSCH throughput after UE complete the SCell activation procedure. We therefore don’t think it is necessary to define new SSB-less SCell PDSCH demodulation requirements under Rel-18 network energy saving WI.

Proposal: RAN4 does not define new PDSCH demodulation requirements on SSB-less SCell.

# 3	Summary

Observation: PDSCH performance for SCell with 0 < RTD < 1.0CP is same or slight better than that with RTD=0.

Proposal: RAN4 does not define new PDSCH demodulation requirements on SSB-less SCell.

# References

- R4-2402872, “WF on [109][330] Netw_Energy_NR_demod”, Huawei, HiSilicon.