3GPP TSG-RAN WG4 Meeting #110bis	R4-2405101

# Changsha, China, April 15-19, 2024

Source:	Ericsson

Title:	On UE RF architecture in work plan

Agenda item:	9.7.1

Document for:	Discussion

# 1	Introduction

RAN plenary #103 approved a new WID was approved [1].

# 2	Discussion

The UE types are defined in R4-2217734 (WF on NonCol_intraB_ENDC_NR_CA for New Type UE [2]):

Table1: UE Types

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 3b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |  |

The table provides some information regarding number of antennas, LNA etc, but it would be beneficial to have an early agreement of UE RF architecture in work plan. A UE architecture is at least on architecture for which the requirements would ably, but not the only one.

Observation 1: It would be beneficial to have an early agreement of UE RF architecture in work plan. A UE architecture is at least on architecture for which the requirements would ably.

The WID [1] states.

“Specify the core requirements for type 4 FWA UE that supports non-co-located scenarios for both FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and FR1 intra-band non-contiguous NR-CA for 4-layer DL MIMO, where MRTD>CP is assumed.

From Table 1, type 4b fulfills the requirements of WI, that is ENDC and NRCA with up to 4-layer DL MIMO for FWA. Type 4a is restricted to ENDC and was perhaps initially considered to fit advanced smartphone form-factor as well.

Observation 2: Do we need two type 4, i.e., type 4a and 4b, or is it enough with type 4b?

Observation 3: If we keep simplified type 4a, why restrict to ENDC?

# 3 	Summary

Observation 1: but it would be beneficial to have an early agreement of UE RF architecture in work plan. A UE architecture is at least on architecture for which the requirements would ably.

Observation 2: Do we need two type 4, i.e., type 4a and 4b, or is it enough with type 4b?

Observation 3: If we keep simplified type 4a, why restrict to ENDC?

# References

[1] 		RP-240728, New WID on Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s), Apple.

[2]	R4-2217734, WF on NonCol_intraB_ENDC_NR_CA for New Type UE, KDDI