3GPP TSG-RAN WG4 Meeting #110b	R4-2405144

# Changsha, China, 15th - 19th Apr, 2024

Title: 	Discussion on UE demodulation requirements for NR NTN enhancements

Source: 	Huawei, HiSilicon

Agenda item:	6.16.8.2

Document for:	Discussion

Background

During RAN4#110 meeting, WF [1] on NR_NTN_enh_SAN_UE_demod was approved. In this contribution, we share our views about UE demodulation requirements for NR NTN enhancements.

Discussion

Channel model

Doppler for DL

- Way forward - Option 1 (Apple, Huawei): 600Hz - Option 2 (Qualcomm): 1200Hz - Option 3 (Ericsson): 2000Hz

In previous meeting, the agreements have been reached that 120kHz SCS is used for both downlink and uplink for FR2 NTN. For the downlink, the maximum tracking range using TRS at 120kHz SCS is about 14kHz. For the uplink, the maximum tracking range using DMRS with pos1 configuration at 120kHz SCS is about 7kHz. So it is feasible for both downlink and uplink to handle the Doppler shift value up to 0.1ppm, i.e. 2000Hz for UE side. However, the Doppler used for the NTN cases is Doppler spread rather than Doppler shift, so the performance will be degraded with the larger Doppler value, as our evaluation results shown in following Figure 2.1.2-1. Therefore, we propose to select 600Hz for UE side for FR2 NTN.

Figure 2.1.2-1 Simulation results for different Doppler value

Select 600Hz for UE side for FR2 NTN.

Test scope

How to define requirements for GSO and NGSO for above 10 GHz bands

- Way forward - Option 1: Consider 32 HARQ processes for GSO scenarios and 16 HARQ processes for NGSO scenarios. - Option 2: Consider one set of requirements for both NGSO and GSO.

From our understanding, the only difference between LEO and GEO is the K_offset value that has no impact on the demodulation processing. So we propose to consider one set of requirements for both NGSO and GSO.

Consider one set of requirements for both NGSO and GSO.

For the K_offset selection, here we derived the satellite-UE RTT delay for tilt angle α=30° (illustrated in Figure 2.2.1-1) under different scenario as showing in Table 2.2.2-1.

Table 2.2.2-1 Satellite-UE RTT delay for tilt angle α=30° under different scenario

|  |  | Unit |  |  |
| --- | --- | --- | --- | --- |
|  |  | ms | K_offset | slot |
| Delay/ms | LEO-600 | 7.17 | 8 | 64 |
|  | LEO-1200 | 13.34 | 14 | 112 |
|  | GEO | 257.58 | 258 | 2064 |

Figure 2.2.2-1 Illustration of the tilt angle α

We can see that even with 32 HARQ processes, there is still HARQ stalling for all candidate scenarios. To reduce the simulation time and the test time, we propose to only consider K_offset = 8 that corresponding to 64 slots for 120kHz SCS.

Even with 32 HARQ processes, there is still HARQ stalling for all candidate scenarios.

Only consider K_offset = 8 that corresponding to 64 slots for 120kHz SCS

Test scope

General

Antenna configuration

- Way forward - Option 1: 1Tx1Rx - Option 1a: Take 1Tx1Rx for parabolic VSAT antenna configuration for initial demodulation discussion and input from satellite companies is needed - Option 2: 1Tx2Rx - Option 2a: Need further clarification on impact to demodulation performance with parabolic VSAT antenna configuration - Option 3: Both 1Tx1Rx and 1Tx2Rx, with antenna type not limit to parabolic, but also phase antenna array

For the antenna configuration, we think both 1Tx1Rx and 1Tx2Rx are feasible to be considered for FR2 NTN. In addition, the antenna type should not limit to parabolic, but also phase antenna array, although there is no impact on baseband processing.

Consider both 1Tx1Rx and 1Tx2Rx for FR2-NTN, with antenna type not limit to parabolic, but also phase antenna array, although there is no impact on baseband processing.

Applicability rule

- Way forward - Option 1: Adding similar applicability rule for FR2 NTN UE optional capabilities as in Rel-17 FR1 NTN UE UE-NR-Capability-v1700; Applicability. nonTerrestrialNetwork-r17; ntn-ScenarioSupport-r17. Supported; GSO only; UE needs to pass the test in Section x of TS38.101-5. NGSO only; UE needs to pass the test in Section x of TS38.101-5. N/A Note: N/A means UE supports both GSO and NGSO; UE needs to pass the test in Section x of TS38.101-5. - Option 2: Discuss applicability rules once we have concluded the discussion on scenarios and test setup.

Considering that there are optional NTN features that may be supported FR2 NTN UE, there is necessity to adding similar applicability rule for FR2 NTN UE optional capabilities as in Rel-17 FR1 NTN UE.

Adding similar applicability rule for FR2 NTN UE optional capabilities as in Rel-17 FR1 NTN UE.

PDCCH

Aggregation level

- Agreement - Option 1: 8 as baseline - Option 2: 2 and 4 - Option 3: 4, 8 and 16

Since that the link budget is not very good for the downlink, we think higher aggregation level has benefit for above 10 GHz bands. Considering the test coverage, we propose to consider aggregation level 4, 8 and 16 for FR2 NTN demodulation requirements.

Consider aggregation level 4, 8 and 16 for FR2 NTN demodulation requirements as Table 2.3.2-1.

Table 2.3.2-1 Proposed PDCCH cases for NTN

| Bandwidth (MHz) | SCS (kHz) | CORESET RB | CORESET duration | Aggregation level | CCE to REG mapping type | REG bundle size | Interleaver size | Shift index | DCI format | Payload (without CRC) | Propagation Condition | Antenna configuration and correlation Matrix | Test metric |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100 | 120 | 60 | 1 | 4 | Interleaved | 6 | 2 | 0 | 1_1 | 56 | NTN-TDLC5-600 | 1x1, 1x2 | 1% of Pm-dsg |
| 100 | 120 | 60 | 1 | 8 | Interleaved | 2 | 3 | 0 | 1_1 | 56 | NTN-TDLC5-600 | 1x1, 1x2 | 1% of Pm-dsg |
| 100 | 120 | 60 | 2 | 16 | Interleaved | 2 | 3 | 0 | 1_0 | 40 | NTN-TDLC5-600 | 1x1, 1x2 | 1% of Pm-dsg |

Proposals

In this contribution, we discuss on UE demodulation requirements for NR NTN enhancements. Our observations and proposals are:

- Select 600Hz for UE side for FR2 NTN.

Consider one set of requirements for both NGSO and GSO.

- Even with 32 HARQ processes, there is still HARQ stalling for all candidate scenarios.

Only consider K_offset = 8 that corresponding to 64 slots for 120kHz SCS

Consider both 1Tx1Rx and 1Tx2Rx for FR2-NTN, with antenna type not limit to parabolic, but also phase antenna array, although there is no impact on baseband processing.

Adding similar applicability rule for FR2 NTN UE optional capabilities as in Rel-17 FR1 NTN UE.

Consider aggregation level 4, 8 and 16 for FR2 NTN demodulation requirements as Table 2.3.2-1.

Table 2.3.2-1 Proposed PDCCH cases for NTN

| Bandwidth (MHz) | SCS (kHz) | CORESET RB | CORESET duration | Aggregation level | CCE to REG mapping type | REG bundle size | Interleaver size | Shift index | DCI format | Payload (without CRC) | Propagation Condition | Antenna configuration and correlation Matrix | Test metric |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100 | 120 | 60 | 1 | 4 | Interleaved | 6 | 2 | 0 | 1_1 | 56 | NTN-TDLC5-600 | 1x1, 1x2 | 1% of Pm-dsg |
| 100 | 120 | 60 | 1 | 8 | Interleaved | 2 | 3 | 0 | 1_1 | 56 | NTN-TDLC5-600 | 1x1, 1x2 | 1% of Pm-dsg |
| 100 | 120 | 60 | 2 | 16 | Interleaved | 2 | 3 | 0 | 1_0 | 40 | NTN-TDLC5-600 | 1x1, 1x2 | 1% of Pm-dsg |

Reference

R4-2402865, WF on NR_NTN_enh_SAN_UE_demod, RAN4#110, Huawei, HiSilicon