3GPP TSG-RAN WG4 Meeting #110	  R4-2405150

Changsha, China, 15 – 19 April, 2024

Agenda Item:	9.11.3

Source:	Huawei, HiSilicon

Title:	Testability and interoperability issues for positioning accuracy enhancement

Document for:	Discussion and Decision

# Introduction

In [1], a new WID is approved, where RAN4 is required to specify core requirements for AIML-enabled beam management and positioning accuracy enhancement.

According to the TR 38.843 [2], potential test metrics are identified for positioning accuracy enhancement. The testability and necessity of each identified test metric needs further justification.

In this contribution, both the positioning accuracy enhancement specific core requirement and testability of test metrics are analyzed.

# Core requirements for LCM procedures

R19 SID on AIML [1]: - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above two use cases. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring.

It is noticed that RAN1/2 will specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the positioning accuracy enhancement use cases, if any. RAN4 can start investigating associated core requirements after achieving sufficient input from other WGs.

# Test metrics for positioning accuracy enhancement

TR 38.843 [2]: For metrics for positioning requirements/tests, the candidate options include - Option 1: positioning accuracy: Ground truth vs. reported - only option available for direct positioning - Option 2: CIR/PDP, channel estimation accuracy - Option 3: ToA, RSTD and RSRP, and RSRPP - Option 4: others (e.g., intermediate KPIs, LoS/NLoS)/combinations of the above

Whether the identified test metric is applicable depends on the case considered. In R19 SID on AIML [1], 3 sub use cases are prioritized, which can also be considered as priority in RAN4.

R19 SID on AIML [1]: - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

In Case 1, UE employs AIML model to perform fingerprint positioning. The location is generated inside UE, which is not expected to be reported to NW in many cases. Even if the location may be reported to LMF via LPP signaling for some specific purposes, like model monitoring, how to test the positioning accuracy is unclear. Due to the limitation of generalization, the test environment needs to be informed to the DUT for model selection. Since model training/update procedure is not involved during testing, the test environment should be specified and available at the DUT.

Proposal 1: For UE-sided direct AI/ML positioning (Case 1), positioning accuracy is not testable, if the positioning result derived by UE is not reported to NW.

For Case 2b, whether defining new measurements or enhancement of existing measurements are still under discussion in other WGs. RAN4 can start investigating the related testability after achieving sufficient input from other WGs.

Even if CIR/PDP may be specified by other WGs, how to test is still an open issue. On the one hand, the TE cannot achieve the ground truth of CIR/PDP based on existing testing method. Even though a PRU like terminal can be used to measure the CIR/PDP as ground truth, how to eliminate the impact of possible variations/differences in the PRU is not clear. On the other hand, the CIR/PDP is used as model input in the LMF. The measurement accuracy degrades to what extent will have an impact on the final positioning accuracy is difficult to define. Without a closed-form expression, how to specify the test requirement or measurement tolerance is an open issue.

Observation 1: For UE-assisted positioning with LMF-side positioning (Case 2b), the relationship between measurement accuracy and positioning accuracy is unavailable, which has an impact on the test requirement definition.

Proposal 2: If RAN4 studies how to test Case 2b in AIML for positioning, the relationship between measurement accuracy provided by UE and the eventual positioning accuracy at LMF needs to be investigated firstly.

# Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: For UE-sided direct AI/ML positioning (Case 1), positioning accuracy is not testable, if the positioning result derived by UE is not reported to NW.

Observation 1: For UE-assisted positioning with LMF-side positioning (Case 2b), the relationship between measurement accuracy and positioning accuracy is unavailable, which has an impact on the test requirement definition.

Proposal 2: If RAN4 studies how to test Case 2b in AIML for positioning, the relationship between measurement accuracy provided by UE and the eventual positioning accuracy at LMF needs to be investigated firstly.

# References

- RP-234039, “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP RANP #102, December 11-15, 2023.

- RP-233133, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, Release 18, 3GPP TR 38.843, V2.0.0, December 11-15, 2023.