3GPP TSG-RAN WG4 Meeting # 110bis	R4-2405153

# Changsha, China, April 15th-19th, 2024

Title: 	Simulation results on UE 8Rx performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	6.1.3.1.1

Document for:	Discussion

Background

In RAN4#110 meeting, a WF for UE 8Rx performance requirements was approved [1]. It’s agreed to define two sets of requirements for two UE receiver types for Rank2 and Rank4. This contribution provides our simulation results.

- Simulation results for single carrier

We provide our simulation results as follows:

Table 2-1: Simulation results for 8Rx PDSCH with Rank2

| Duplex mode | Bandwidth (MHz) /SCS(kHz) | MCS | Rank | Propagation condition | Correlation matrix and antenna configuration | Receiver Type | SNR@70% of maxTP(dB) (Ideal results) | SNR@70% of maxTP(dB) (Impairment results) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | 10/15 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | Baseline Receiver | 10.5 | 12.0 |
|  |  |  |  |  |  | Simplified Receiver | 12.6 | 14.1 |
| TDD | 40/30 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | Baseline Receiver | 10.7 | 12.2 |
|  |  |  |  |  |  | Simplified Receiver | 12.8 | 14.3 |

Table 2-2: Simulation results for 8Rx PDSCH with Rank4

| Duplex mode | Bandwidth (MHz) /SCS(kHz) | MCS | Rank | Propagation condition | Correlation matrix and antenna configuration | Receiver Type | SNR@70% of maxTP(dB) (Ideal results) | SNR@70% of maxTP(dB) (Impairment results) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | 10/15 | 17 | 4 | TDLA30-10 | 4x8, ULA Low | Baseline Receiver | 8.9 | 10.4 |
|  |  |  |  |  |  | Simplified Receiver | 12.3 | 13.8 |
| TDD | 40/30 | 17 | 4 | TDLA30-10 | 4x8, ULA Low | Baseline Receiver | 9.2 | 10.7 |
|  |  |  |  |  |  | Simplified Receiver | 12.6 | 14.1 |

Table 2-3: Simulation results for 8Rx PDSCH with Rank8

| Duplex mode | Bandwidth (MHz) /SCS(kHz) | MCS | Rank | Propagation condition | Correlation matrix and antenna configuration | Receiver Type | SNR@70% of maxTP(dB) (Ideal results) | SNR@70% of maxTP(dB) (Impairment results) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FDD | 10/15 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | Baseline Receiver | 19.2 | 20.7 |
| TDD | 40/30 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | Baseline Receiver | 18.9 | 20.4 |

- Simulation results for CA

We provide our simulation results as follows:

Table 3-1: Simulation results for 8Rx PDSCH CA with Rank2 (FDD 15kHz SCS)

| Bandwidth (MHz) | MCS | Rank | Propagation condition | Correlation matrix and antenna configuration | Receiver Type | SNR@70% of maxTP(dB) (Ideal results) | SNR@70% of maxTP(dB) (Impairment results) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | Baseline Receiver | 10.3 | 11.8 |
|  |  |  |  |  | Simplified Receiver | 12.3 | 13.8 |
| 10 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | Baseline Receiver | 10.5 | 12.0 |
|  |  |  |  |  | Simplified Receiver | 12.6 | 14.1 |
| 15 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | Baseline Receiver | 10.8 | 12.3 |
|  |  |  |  |  | Simplified Receiver | 12.6 | 14.1 |
| 20 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | Baseline Receiver | 11.2 | 12.7 |
|  |  |  |  |  | Simplified Receiver | 12.9 | 14.4 |
| 25 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | Baseline Receiver | 10.9 | 12.4 |
|  |  |  |  |  | Simplified Receiver | 12.7 | 14.2 |
| 30 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | Baseline Receiver | 11.1 | 12.6 |
|  |  |  |  |  | Simplified Receiver | 12.8 | 14.3 |
| 35 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | Baseline Receiver | 10.9 | 12.4 |
|  |  |  |  |  | Simplified Receiver | 12.4 | 13.9 |
| 40 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | Baseline Receiver | 11.0 | 12.5 |
|  |  |  |  |  | Simplified Receiver | 12.8 | 14.3 |
| 45 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | Baseline Receiver | 11.2 | 12.7 |
|  |  |  |  |  | Simplified Receiver | 12.9 | 14.4 |
| 50 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | Baseline Receiver | 11.2 | 12.7 |
|  |  |  |  |  | Simplified Receiver | 13.0 | 14.5 |

Table 3-2: Simulation results for 8Rx PDSCH CA with Rank2 (TDD 30kHz SCS)

| Bandwidth (MHz) | MCS | Rank | Propagation condition | Correlation matrix and antenna configuration | Receiver Type | SNR@70% of maxTP(dB) (Ideal results) | SNR@70% of maxTP(dB) (Impairment results) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | Baseline Receiver | 10.0 | 11.5 |
|  |  |  |  |  | Simplified Receiver | 12.3 | 13.8 |
| 10 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | Baseline Receiver | 10.3 | 11.8 |
|  |  |  |  |  | Simplified Receiver | 12.5 | 14.0 |
| 15 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | Baseline Receiver | 10.3 | 11.8 |
|  |  |  |  |  | Simplified Receiver | 12.5 | 14.0 |
| 20 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | Baseline Receiver | 10.3 | 11.8 |
|  |  |  |  |  | Simplified Receiver | 12.5 | 14.0 |
| 25 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | Baseline Receiver | 10.4 | 11.9 |
|  |  |  |  |  | Simplified Receiver | 12.6 | 14.1 |
| 30 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | Baseline Receiver | 10.3 | 11.8 |
|  |  |  |  |  | Simplified Receiver | 12.6 | 14.1 |
| 40 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | Baseline Receiver | 10.7 | 12.2 |
|  |  |  |  |  | Simplified Receiver | 12.8 | 14.3 |
| 50 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | Baseline Receiver | 10.5 | 12.0 |
|  |  |  |  |  | Simplified Receiver | 12.7 | 14.2 |
| 60 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | Baseline Receiver | 10.4 | 11.9 |
|  |  |  |  |  | Simplified Receiver | 12.5 | 14.0 |
| 80 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | Baseline Receiver | 11.1 | 12.6 |
|  |  |  |  |  | Simplified Receiver | 13.2 | 14.7 |
| 90 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | Baseline Receiver | 10.8 | 12.3 |
|  |  |  |  |  | Simplified Receiver | 13.0 | 14.5 |
| 100 | 19 | 2 | TDLC300-100 | 2x8, ULA medium B | Baseline Receiver | 11.0 | 12.5 |
|  |  |  |  |  | Simplified Receiver | 13.2 | 14.7 |

Table 3-3: Simulation results for 8Rx PDSCH CA with Rank8 (FDD 15kHz SCS)

| Bandwidth (MHz) | MCS | Rank | Propagation condition | Correlation matrix and antenna configuration | Receiver Type | Target SNR@70% of maxTP(dB) | SNR@70% of maxTP(dB) (Impairment results) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | Baseline Receiver | 18.1 | 19.6 |
| 10 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | Baseline Receiver | 18.3 | 19.8 |
| 15 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | Baseline Receiver | 18.6 | 20.1 |
| 20 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | Baseline Receiver | 19.1 | 20.6 |
| 25 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | Baseline Receiver | 19.1 | 20.6 |
| 30 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | Baseline Receiver | 19.3 | 20.8 |
| 35 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | Baseline Receiver | 19.3 | 20.8 |
| 40 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | Baseline Receiver | 19.7 | 21.2 |
| 45 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | Baseline Receiver | 19.7 | 21.2 |
| 50 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | Baseline Receiver | 19.4 | 20.9 |

Table 3-4: Simulation results for 8Rx PDSCH CA with Rank8 (TDD 30kHz SCS)

| Bandwidth (MHz) | MCS | Rank | Propagation condition | Correlation matrix and antenna configuration | Receiver Type | Target SNR@70% of maxTP(dB) | SNR@70% of maxTP(dB) (Impairment results) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | Baseline Receiver | 18.4 | 19.9 |
| 10 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | Baseline Receiver | 18.3 | 19.8 |
| 15 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | Baseline Receiver | 18.7 | 20.2 |
| 20 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | Baseline Receiver | 18.7 | 20.2 |
| 25 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | Baseline Receiver | 18.7 | 20.2 |
| 30 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | Baseline Receiver | 19.3 | 20.8 |
| 40 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | Baseline Receiver | 18.9 | 20.4 |
| 50 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | Baseline Receiver | 18.8 | 20.3 |
| 60 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | Baseline Receiver | 19.2 | 20.7 |
| 80 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | Baseline Receiver | 19.6 | 21.1 |
| 90 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | Baseline Receiver | 19.8 | 21.3 |
| 100 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | Baseline Receiver | 20.1 | 21.6 |

- Conclusion

This contribution provides our simulation results for 8Rx PDSCH CA/

- Reference

[1]   R4-2402857 WF for 8Rx UE performance requirements, Huawei, HiSilicon.