3GPP TSG-RAN WG4 Meeting # 110bis	R4-2405161

# Changsha, China, April 15th – 19th , 2024

Title: 	Simulation results on UE performance requirements for Rel-18 eRedCap

Source: 	Huawei, HiSilicon

Agenda item:	6.21.3.1

Document for:	Discussion

Background

In last meeting, WF on [110][125] NR_redcap_enh_demod was agreed, in the contribution we provide our simulation results .

- Simulation results

We provide simulation results for PDSCH requirements as follows:

Table 2-1 to 2-4 provides the summary of simulation results.

Table 2-1: Simulation results for UE not supporting eRedCapNotReducedBB-BW-r18, FDD,15kHz

|  | Antenna Configuration | Modulation | Code rate | Rank | PRB size | TBS | Propagation | SNR@70%of max TP(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 2T1R | MCS4 | 1/3 | Rank 1 | 25 | 1928 | TDLB100-400 | 1.0 |
| QPSK | 2T2R | MCS4 | 1/3 | Rank 1 | 25 | 1928 | TDLB100-400 | -2.7 |
| 16QAM | 2T1R | MCS13 | 0.47 | Rank 1 | 25 | 6272 | TDLC300-100 | 8.9 |
| 16QAM | 2T2R | MCS13 | 0.47 | Rank 1 | 25 | 6272 | TDLC300-100 | 4.9 |

Table 2-2 Simulation results for UE not supporting eRedCapNotReducedBB-BW-r18, TDD, 30kHz

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation | SNR@70%of max TP(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 2T1R | MCS4 | 1/3 | Rank 1 | 12 | 928 | TDLB100-400 | 1.4 |
| QPSK | 2T2R | MCS4 | 1/3 | Rank 1 | 12 | 928 | TDLB100-400 | -2.4 |
| 16QAM | 2T1R | MCS13 | 0.47 | Rank 1 | 12 | 3104 | TDLC300-100 | 9.4 |
| 16QAM | 2T2R | MCS13 | 0.47 | Rank 1 | 12 | 3104 | TDLC300-100 | 5.3 |

Table 2-3: Simulation results for UE supporting eRedCapNotReducedBB-BW-r18, FDD,15kHz

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation | SNR@70%of max TP(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 2T1R | MCS4 | 1/3 | Rank 1 | 52 | 3904 | TDLB100-400 | 0.7 |
| QPSK | 2T2R | MCS4 | 1/3 | Rank 1 | 52 | 3904 | TDLB100-400 | -3.0 |
| 16QAM | 2T1R | MCS13 | 0.47 | Rank 1 | 40 | 9992 | TDLC300-100 | 9.1 |
| 16QAM | 2T2R | MCS13 | 0.47 | Rank 1 | 40 | 9992 | TDLC300-100 | 5.0 |

Table 2-4: Simulation results for UE supporting eRedCapNotReducedBB-BW-r18, TDD,30kHz

|  | Received antennas | Modulation | Code rate | Rank | PRB size | TBS | Propagation | SNR@70%of max TP (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 2T1R | MCS4 | 1/3 | Rank 1 | 51 | 3840 | TDLB100-400 | 0.8 |
| QPSK | 2T2R | MCS4 | 1/3 | Rank 1 | 51 | 3840 | TDLB100-400 | -2.9 |
| 16QAM | 2T1R | MCS13 | 0.47 | Rank 1 | 20 | 4992 | TDLC300-100 | 8.9 |
| 16QAM | 2T2R | MCS13 | 0.47 | Rank 1 | 20 | 4992 | TDLC300-100 | 4.9 |

- Conclusion

In this contribution we provide our simulation results for eRedCap.

- Reference

[1]    RP-2402870 Way forward on [110][325] NR_redcap_enh_demod. Ericsson