3GPP TSG-RAN WG4 Meeting # 110bis	R4-2405162

# Changsha, China, April.15th – April.20st, 2024

Title: 	Discussions on demodulation requirements for NES

Source: 	Huawei, HiSilicon

Agenda item:	6.24.4

Document for:	Discussion

Background

In last meeting, a WF on demodulation and CSI requirements for NES was approved:

Issue 1-1: Whether to introduce SSB less Scell requirements for inter-band CA - Agreement - RAN4 to evaluate the performance of SSB-less SCells CA performance, if the observed performance constitutes a practical operating point of the feature, or if SSB-less SCell compliant UE shall be tested with normal CA requirements. Issue 2-1: Whether to introduce CSI requirements for power/spatial domain adaption - Agreement - Not to introduce CSI requirements power/spatial domain adaption

In this contribution we provide our views on the SSB less Scell requirements for inter-band CA.

Discussions

SSB less Scell requirements

In order to evaluate the SSB less SCell performance with different time offset between PCell and Scell, we did some simulations with simulation assumptions in Table 5-1. The simulation results are shown in Figure 2-1.

Figure 2-1: Simulation results for SSB less Scell

Table 2-1 summarizes the target SNR for each cases:

Table 2-1: SNR(dB) @ 70% of MaxTP

| TO | SNR(dB) @ 70% of Max TP |
| --- | --- |
| 0 | 4.7 |
| 0.4CP | 4.8 |
| -0.4CP | 4.8 |
| 0.9CP | 5.0 |
| -0.9CP | 4.8 |

It’s observed that performance with different time offset are quite same.

Observation 1: The performance with different time offset are quite same.

Regarding the time offset tracking and compensation, the common approach is detecting the first arrived path to adjust the FFT window. For the TDLA channel model, the delay of first path equals to TO configured between Pcell and Scell. The detectable time offset range supported by TRS is [-4.17us~4.17us], which means 0.9CP TO is within the range and the TO can be completely compensated by adjusting FFT window and no significant performance degradation is expected.

Based on the above analysis and simulations, typical TO estimation and compensation algorithm can handle the SSB less cases and no significant performance degradation is observed. Therefore, we propose to not define SSB less Scell requirements.

Proposal 1: Not define SSB less Scell requirements.

Conclusion

In this contribution we provide our views on demodulation requirements for NES. The observations and proposals are:

Observation 1: The performance with different time offset is quite same.

Proposal 1: Not define SSB less Scell requirements.

Reference

[1]   R4-2402872  WF on [109][330] Netw_Energy_NR_demod

Appendix

Table 5-1: Proposed PDSCH SSB-less SCell FR1 inter-band CA test setup, based on legacy CA demodulation testing

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| Channel model |  |  | {TDLA30-10} |
| Antenna configuration |  |  | 2x2, ULA Low |
| Fraction of maximum throughput (%) |  |  | 70 (Only SCell measured) |
| SCS/CBW |  |  | 30kHz/40MHz (106 PRB) |
| MCS |  |  | {16QAM, 0.48 (MCS13)} |
| Channel estimation |  |  | DM-RS based: non-ideal/practical |
| TO Estimation & Compensation |  |  | As per implementation |
| TO |  |  | {-0.9, -0.4, 0, 0.4, 0.9} CP |
| FFT window shift target |  |  | {As per implementation} |
| SSB/TRS |  |  | PCell: SSB and TRS (optional) SCell: TRS only |
| Precoder |  |  | Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Fill remaining DL symbols in the slot. 12 in normal DL slot 4 in special slot |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
|  | Antenna ports |  | {1000} |
| Number of HARQ Processes |  |  | 8 |
| Maximum number of HARQ transmissions |  |  | 4 |
| TDD UL-DL pattern |  |  | [30kHz SCS: FR1.30-1] |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  | 'No CDM’ for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity | Slots | 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |