3GPP TSG-RAN WG4 Meeting # 110bis	R4-2405164

# ChangSha, China, 15 April – 19 April, 2024

Title: 	Discussions on Rel-18 eDSS performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	6.25.1

Document for:	Discussion

Background

In last meeting, a WF on enhancement of DSS demodulation requirements was approved. There are no open issues left and companies are welcome to update results if any, in the contribution we provide our simulation and views on open issues.

- Simulation results

The simulation results are captured in Table 2-1 with simulation assumptions captured in Appendix.

Table 2-1: PDCCH simulation results for eDSS

| Test number | Duplex | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | FDD | 10 | 48 | 2 | 4 | TDLA30-10 | 2x4 Low | 1 | -3.6 |
| 2 | FDD | 10 | 48 | 2 | 8 | TDLC300-100 | 4x2 Low | 1 | -4.1 |
| 3 | TDD | 10 | 48 | 2 | 4 | TDLA30-10 | 2x4 Low | 1 | -3.6 |
| 4 | TDD | 10 | 48 | 2 | 8 | TDLC300-100 | 4x2 Low | 1 | -4.1 |

- Discussions

Existing demodulation test cases have different test parameters and requirements for FDD and TDD, UE have to pass the tests with both duplex mode if supported. However, for eDSS, it’s very strange that FDD and TDD cases have same test parameters and requirements, which means it's meaningless for UE to pass two duplex mode cases with same test setup repeatedly. We propose to differentiate FDD and TDD test cases with different test parameters, e,g, Aggregation level, or define the test applicability rule that if UE support both FDD and TDD, only cases with FDD are applied.

Observation 1: It’s very strange that FDD and TDD test cases have same test parameters and requirements, which means it's meaningless for UE to pass two duplex mode cases with same test setup repeatedly

Proposal 1: RAN4 to differentiate FDD and TDD test cases with different test parameters, e,g, Aggregation level, or define the test applicability rule that if UE supports both FDD and TDD, only cases with TDD are applied.

- Conclusion

In the contribution we provide our simulation and views on remaining issues. The observation and proposal are:

Observation 1: It’s very strange that FDD and TDD test cases have same test parameters and requirements, which means it's meaningless for UE to pass two duplex mode cases with same test setup repeatedly

Proposal 1: RAN4 to differentiate FDD and TDD test cases with different test parameters, e.g. Aggregation level or define the test applicability rule that if UE supports both FDD and TDD, only cases with TDD are applied.

- Reference

[1]    R1-2402873 Way forward on [110][328] NR_DSS_enh. Ericsson

- Appendix

Table 5-1: Simulation assumptions

| Parameters | Values |
| --- | --- |
| Duplex mode | FDD |
| SCS | 15 kHz |
| Bandwidth | 10 MHz |
| Channel model | TDLA30-10 and TDLC300-100 |
| Antenna configuration | 2x2, 2x4, 4x2, 4x4 |
| CRS | 4 port CRS |
| DCI payload (excluding CRC) | 39 bits |
| Interleaving | Non-interleaved |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 |
| Shift index | 0 |
| CORSET duration | 2 (2nd symbol and 3rd symbol) |
| CORSET RB | 48 |
| AL | 4, 8 |
| - Note 1: The bandwidth of NR CORESET is within the bandwidth of LTE. - Note 2: gNB punctures the PDCCH data and DMRS REs overlapped with LTE CRS. Note 3: PDCCH channel estimation is assumed to use only the clean PDCCH symbol. |  |