3GPP TSG-RAN WG4 Meeting #110bis	R4-2405176

Changsha, China, 15th – 19th April, 2024

Title:	Discussion on the minimum output power of NTN UE

Source:	OPPO

Agenda item:	6.16.5.1

Document for:	Approval

- Introduction

In the last RAN4#110 meeting, the NTN UE RF requirement was discussed, and the agreement was achieved on the minimum peak EIRP requirements as below [1].

However, the minimum output power requirements were left for further discussion.

This contribution provides our proposals on the minimum output power requirement for NTN UEs.

- Discussion

According to the proposals from [2], the EIRP could be derived from the following formula:

EIRP=-174dBm/Hz+10log10BW+NF+SNR+IM-SANAntennaGain+PathlossFreeSpce+Atmosphereloss

Where,

- BW is assumed as 100MHz.

- NF is 2.5dB for fixed VSAT supporting LEO and GSO and mobile VSAT; and NF is 6dB for fixed VSAT supporting LEO only.

- SNR is targeted to be -1dB.

- IM is implementation margin for low modulation order, setting to 0dB temporarily.

- SANAntennaGain is 35.5dB for LEO and 55.5dB for GSO.

- PathlossFreeSpace is 176.6dB for LEO and 212.1dB for GSO.

- Atmosphereloss is supposed to be 2dB.

With the above information, the minimum output powers for NTN UE are listed as below.

- For UE type 1/2/4/5: the minimum output power is -9.4dBm

- For UE type 3: the minimum output power is -5.9dBm

Proposal: The minimum output powers for NTN UE are proposed as below.

- For UE type 1/2/4/5: the minimum output power is -9.4dBm

- For UE type 3: the minimum output power is -5.9dBm

- Conclusions

This contribution provides our proposals on the minimum output power requirement for NTN UEs. And provide the following proposal.

Proposal: The minimum output powers for NTN UE are proposed as below.

- For UE type 1/2/4/5: the minimum output power is -9.4dBm

- For UE type 3: the minimum output power is -5.9dBm

- References

[1] R4-2403649, WF on NTN UE RF requirements, ZTE Corporation

[2] R4-2402521, Further discussion on UE RF Tx requirements for NTN in Ka-band, ZTE Corporation