# 3GPP TSG-RAN WG4 Meeting # 110bis                               R4-2405202

# Changsha, China, 15th – 19th April, 2024

# Title: 	Discussion on NR_cov_enh2 demodulation requirements

# Source: 	ZTE Corporation

# Agenda item:	6.17.2

# Document for:	Approval

# 1 Introduction

In RAN #102 meeting[1], the revised work item on Revised WID on further NR coverage enhancements was approved. In RAN4 #109 and RAN4 #110 meeting[2][3], most issues were reached while some issues about the simulation parameters of multiple PRACH transmission are still FFS.

This contribution shares our views about the multiple PRACH demodulation requirements and simulation results for different SCS and channel models.

# 2	Discussion

Some issues about multiple PRACH requirements are listed below[4].

PRACH repetition number for BS performance requirements for Multiple PRACH transmission Way forward for the next meeting: - Companies to further check the simulation assumptions including TDD pattern, PRACH repetition interval for both FR1 and FR2. - For the repetition number, keep the previous meeting agreement before simulation assumption agreed. Sub Carrier Spacing for BS performance requirements for PRACH repetitions - For FR1: 15kHz and 30kHz - For FR2-1: Agree with 120 kHz and FFS for 60 kHz. Encourage operators to provide input.

With regard to TDD pattern, RRC signaling and DCI 2-0 can configure a very flexible TDD pattern. To align simulation results a same TDD pattern such as 3D1S1U or 7D1S2U can be considered.

Proposal 1. To consider the following TDD pattern for multiple PRACH.

3D1S1U for 15KHz, 60kHz and 120kHz

7D1S2U for 30kHz

With regard to PRACH repetition interval, A2 and C2 can be repeated in one slot but B4 can not be repeated in one slot due to the large Nu, therefore it is proposed to transmit one preamble in one slot for different preamble format. Combined with proplsal 1, the following PRACH repetition interval can be considered.

Proposal 2. To consider the following PRACH repetition interval for multiple PRACH.

5 slots for 15KHz, 60kHz and 120kHz

1 slot for 30kHz

With regard to SCS for FR2-1, the requirements of 60kHz SCS and 120kHz SCS for FR2-1 are defined in 38.104[3]. Table 2-1 shows the updated miss detection performance with 60kHz SCS and 120kHz SCS and the miss detection curves are appended in the annex for reference.

Table 2-1 SNR comparison of different SCS for FR2-1 multiple PRACH

| index | Preamble format | SCS(kHz) | Channel mode | Operating SNR |
| --- | --- | --- | --- | --- |
| 1 | A2 | 120 | AWGN w/o FO | -18.6 |
| 2 | A2 | 120 | TDLA30-300 low w/ 4000Hz FO | -11.3 |
| 3 | A2 | 60 | AWGN w/o FO | -17 |
| 4 | A2 | 60 | TDLA30-300 low w/ 4000Hz FO | -10.5 |
| 5 | B4 | 120 | AWGN w/o FO | -21.7 |
| 6 | B4 | 120 | TDLA30-300 low w/ 4000Hz FO | -14.2 |
| 7 | B4 | 60 | AWGN w/o FO | -21.7 |
| 8 | B4 | 60 | TDLA30-300 low w/ 4000Hz FO | -11.1 |
| 9 | C2 | 120 | AWGN w/o FO | -18.6 |
| 10 | C2 | 120 | TDLA30-300 low w/ 4000Hz FO | -11.3 |
| 11 | C2 | 60 | AWGN w/o FO | -17 |
| 12 | C2 | 60 | TDLA30-300 low w/ 4000Hz FO | -10.6 |

From table 2-1, we can see that the performance difference of preamble format B4 between 120kHz SCS and 60kHz for TDLA30-300 low with 4000Hz frequency offset is about 3dB as PRACH with 60kHz SCS is more susceptible to Doppler shift and fixed frequency offset compared to 120kHz SCS. In addition, 60kHz is also a commonly used SCS for FR2-1 PRACH.

Proposal 3. To consider both 120kHz and 60kHz for FR2-1 multiple PRACH requirements.

# 3 Conclusion

In this contribution, we give our viewpoint about requirements for FR1 and the SCS selection for FR2-1for multiple PRACH .

The conclusions are:

Proposal 1. To consider the following TDD pattern for multiple PRACH.

3D1S1U for 15KHz, 60kHz and 120kHz

7D1S2U for 30kHz

Proposal 2. To consider the following PRACH repetition interval for multiple PRACH.

5 slots for 15KHz, 60kHz and 120kHz

1 slot for 30kHz

Proposal 3. To consider both 120kHz and 60kHz for FR2-1 multiple PRACH requirements.

# 4	Reference

[1]  RP-221858, Revised WID on Further NR coverage enhancements, China Telecom

[2]  R4-2316922, WF on NR_cov_enh2_demod, Nokia

[3]  3GPP TS 38.104 V18.3.0

[4]  R4-2402866 WF on NR_cov_enh2_demod_v01, China Telecom

# Annex