# 3GPP TSG-RAN WG4 Meeting # 110bis                                 R4-2405210

# Changsha, China, 15 April – 19 April, 2024

# Title: 	Simulation results for eDSS demodulation requirements

# Source: 	ZTE Corporation

# Agenda item:	6.25.1

# Document for:	Discussion

# 1 Introduction

This contribution contains initial simulation results for the eDSS requirements[1]. The results are provided to aid in determining where new requirements can/should be introduced.

# 2 Discussion

## 2.1 PDCCH requirements

According previous meeting[2] conclusions, we summarized the following Table 2-1 for PDCCH simulation assumptions.

- Table 2-1. Test parameters for PDCCH

| Parameter | Value |
| --- | --- |
| Cyclic prefix | Normal |
| DM-RS sequence generation | NID=0 |
| Frequency domain resource allocation for CORESET | Start from RB = 0 with contiguous RB allocation |
| CCE to REG mapping type | Not-Interleaved |
| REG bundle size | 6 |
| Shift Index | 0 |
| Slots for PDCCH monitoring | Each slot |
| Number of PDCCH candidates for the tested aggregation level | 1 |
| PDCCH Precoding configuration | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination with REG bundling granularity for number of Tx larger than 1 |
| DCI Format | 1_0 |
| Payload (without CRC) | 39 |
| LTE CRS port | 4 |
| Channel estimation | PDCCH CE only on clean symbols |
| Antenna configuration | 2x4, 4x2 for both FDD and TDD |
| Channel model | TDLA30-10 and TDLC300-100 |

- Further, according to the RAN4#110 meeting’s WF[2], the PDCCH simulation results are shown in following Table 2-2 and Figure 1.

Table 2-2. Simulation results for DSS

| Test number | Duplex | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | FDD | 10 | 48 | 2 | 4 | TDLA30-10 | 2x4 Low | 1 | -1.9 |
| 2 | FDD | 10 | 48 | 2 | 8 | TDLC300-100 | 4x2 Low | 1 | -3.1 |
| 3 | TDD | 10 | 48 | 2 | 4 | TDLA30-10 | 2x4 Low | 1 | -2.1 |
| 4 | TDD | 10 | 48 | 2 | 8 | TDLC300-100 | 4x2 Low | 1 | -4.2 |

| Figure 1-1. FDD | - Figure 1-2. TDD |
| --- | --- |

Figure 1.  BLER-SNR curves for PDCCH

# 3 Conclusion

In this contribution, we give some simulation results for enhancement of NR DSS demodulation requirements.

# 4 References

- RP-221622, Revised WID on Enhancement of NR Dynamic spectrum sharing (DSS), RAN#96e.

- R4-2402873. Way forward on [110][328] NR_DSS_enh. Ericsson.

- R4-2321139. WF for Enhancement of Dynamic Spectrum Sharing demodulation requirements, Ericsson.