# 3GPP TSG-RAN WG4 Meeting # 110bis                                 R4-2405211

# Changsha, China, 15 April – 19 April, 2024

# Title: 	Simulation results for PDSCH demodulation requirements for 8Rx CA

# Source: 	ZTE Corporation

# Agenda item:	6.1.3.1.1

# Document for:	Discussion

# 1 Introduction

- In RAN4#110 meeting, 8Rx UE demodulation and CSI requirements were discussed and a WF was agreed[1]. In this paper, we will provide our simulation results on 8Rx PDSCH CA for FR1.

# 2 Discussion

  - 2.2 Simulation results

  - In this section, we present the simulation results in terms of the normalized PDSCH throughput as function of the signal to noise ratio (SNR). We depict the SNR value corresponding to 70% of the peak throughput. All figures have been provided in following Tables.

  - 2.2.1 PDSCH CA requirements

  - And for 8Rx CA, we also did some simulations as shown in following tables which covered MCS 19(Table 1) for rank 2 and MCS 17(Table 1) for rank 8.

Table 2-1. Single carrier performance for FDD 15 kHz SCS with Rank 2 for CA configurations

| Bandwidth (MHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) | Impairment SNR(dB) |
| 5 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 10.3 | 12.8 |
| 10 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 10.4 | 12.9 |
| 15 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 10.7 | 13.2 |
| 20 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 10.6 | 13.1 |
| 25 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 10.8 | 13.3 |
| 30 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 10.8 | 13.3 |
| 35 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 10.8 | 13.3 |
| 40 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 10.8 | 13.3 |
| 45 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 10.8 | 13.3 |
| 50 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 10.9 | 13.4 |

Table 2-2. Single carrier performance for TDD 30 kHz SCS with Rank 2 for CA configurations

| Bandwidth (MHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) | Impairment SNR(dB) |
| 5 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 9.5 | 12 |
| 10 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 10.4 | 12.9 |
| 15 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 10.3 | 12.8 |
| 20 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 10.5 | 13 |
| 25 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 10.6 | 13.1 |
| 30 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 10.6 | 13.1 |
| 40 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 10.5 | 13 |
| 50 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 10.7 | 13.2 |
| 60 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 10.7 | 13.2 |
| 80 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 10.7 | 13.2 |
| 90 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 10.8 | 13.3 |
| 100 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 10.7 | 13.2 |

Table 2-3. Single carrier performance for FDD 15 kHz SCS with Rank 8 for CA configurations

| Bandwidth (MHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) | Impairment SNR(dB) |
| 5 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 18.7 | 21.2 |
| 10 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 19.3 | 21.8 |
| 15 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.0 | 22.5 |
| 20 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.3 | 22.8 |
| 25 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 19.5 | 22 |
| 30 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.6 | 23.1 |
| 35 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.4 | 22.9 |
| 40 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.6 | 23.1 |
| 50 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.5 | 23 |

Table 2-4. Single carrier performance for TDD 30 kHz SCS with Rank 8 for CA configurations

| Bandwidth (MHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) | Impairment SNR(dB) |
| 5 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 18.5 | 21 |
| 10 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 19.1 | 21.6 |
| 15 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 19.6 | 22.1 |
| 20 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 19.5 | 22 |
| 25 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 19.5 | 22 |
| 30 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 19.6 | 22.1 |
| 40 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.3 | 22.8 |
| 50 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.4 | 22.9 |
| 60 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.7 | 23.2 |
| 80 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.6 | 23.1 |
| 90 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.7 | 23.2 |
| 100 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.0 | 23.5 |

  - 2.2.2 PDSCH single CC requirements

Table 2-5. Summary of PDSCH 8Rx simulation results for FDD

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | Ideal SNR (dB) | Impairment SNR(dB) |
| 1-1 | 10 / 15 | 64QAM, 0.5 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | 10.4 | 12.9 |
| 1-3 | 10 / 15 | 64QAM, 0.43 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 10.2 | 12.7 |
| 1-5 | 10 / 15 | 64QAM, 0.43 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.3 | 21.8 |

Table 2-6. Summary of PDSCH 8Rx simulation results for TDD

| Test num. | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | Ideal SNR (dB) | Impairment SNR(dB) |
| 1-1 | 40 / 30 | 64QAM, 0.5 | 2 | TDLC300-100 | 2x8, ULA Medium B | 70 | 10.5 | 13.0 |
| 1-3 | 40 / 30 | 64QAM, 0.43 | 4 | TDLA30-10 | 4x8, ULA Low | 70 | 10.6 | 13.1 |
| 1-5 | 40 / 30 | 64QAM, 0.43 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 20.3 | 22.8 |

# 3 Conclusion

In this contribution, we provide our simulation results for FR1 PDSCH 8Rx CA requirements.

# 4 References

- R4-2402857, WF for UE 8Rx performance requirements. RAN4#110 meeting.