| 3GPP TR 38.718-02-01 V0.11.0 (2024-4) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for2 bands DL with x bands UL (x=1,2) (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	8

x	the first digit:	8

1	Scope	10

2	References	10

3	Definitions of terms, symbols and abbreviations	10

3.1	Terms	10

3.2	Symbols	10

3.3	Abbreviations	11

4	Background	11

4.1	Introduction	11

4.2	TR Maintenance	11

5	Both bands within FR1 Carrier Aggregation: Specific Band Combination Part	12

5.x	CA_nX-nY	12

5.1	CA_n1-n26	17

5.2	CA_n3-n26	20

5.3	CA_n7-n26	24

5.4	CA_n26-n78	28

5.5	CA_n26-n77	31

5.6	CA_n70-n77	35

5.7	CA_n12-n25	39

5.8	CA_n7-n75	41

5.9	CA_n28-n94	42

5.10	 CA_n78-n94	44

5.11	 CA_n75-n78	46

5.12	 CA_n28-n75	47

5.13	 CA_n1-n75	49

5.14	 CA_n5-n77	51

5.15	 CA_n7-n77	54

5.16	 CA_n25-n77	58

5.17	 CA_n66-n77	61

5.18	 CA_n71-n77	64

5.19	CA_n3-n41	68

5.20	 CA_n25-n85	70

5.21	 CA_n41-n85	73

5.22	 CA_n66-n85	76

5.23	 CA_n77-n85	79

5.24	CA_n46-n78	83

5.25	CA_n1-n46	86

5.26	CA_n7-n67	91

5.27	CA_n67-n78	92

5.28	CA_n18-n40	95

5.29	CA_n7-n102	98

5.30	 CA_n28-n102	101

5.31	 CA_n78-n102	105

5.32	 CA_n1-n102	109

5.33   CA_n26-n28	113

5.34	CA_n7-n8	115

5.35	  CA_n3-n75	118

5.36	CA_n3-n7	119

5.37	CA_n46-n77	120

5.38	CA_n2-n41	125

5.39	CA_n5-n71	128

5.40	CA_n5-n41	131

5.41	CA_n2-n71	134

5.42	CA_n7-n12	138

5.43	CA_n7-n71	139

5.44	CA_n12-n41	143

5.45	CA_n12-n78	144

5.46	 CA_n1-n105	148

5.47	 CA_n3-n105	151

5.48	 CA_n40-n105	154

5.49	 CA_n78-n105	156

5.50	 CA_n7-n105	159

5.51	 CA_n3-n102	163

5.52	 CA_n77-n102	167

5.53	 CA_n8-n79	170

5.54	 CA_n78-n79	173

5.55	 CA_n5-n78	173

5.56	 CA_n25-n78	177

5.57	 CA_n66-n78	179

5.58	CA_n26-n29	180

5.59	CA_n26-n48	182

5.60	CA_n29-n48	185

5.61	 CA_n34-n39	187

5.62	 CA_n2-n71	189

5.63	 CA_n5-n71	190

5.64	CA_n46-n102	191

5.65	 CA_n1-n77	195

5.66	 CA_n28-n77	196

5.67	 CA_n71-n85	197

5.68	 CA_n7-n20	199

5.69	 CA_n20-n78	201

5.70	 CA_n25-n41	203

5.71	 CA_n41-n66	204

5.72	 CA_n41-n71	205

5.73	 CA_n41-n77	206

5.74	CA_n26-n71	208

5.75	CA_n8-n77	210

5.76	 CA_n34-n40	212

5.77	 CA_n1-n78	213

5.78	 CA_n3-n78	216

5.79	 CA_n8-n41	218

5.80	 CA_n8-n78	221

5.81	 CA_n28-n41	224

5.82	 CA_n40-n79	226

5.83	CA_n5-n13	228

5.84	 CA_n34-n79	232

5.85	 CA_n39-n79	234

5.86	CA_n78-n104	236

5.87	CA_n28-n79	243

5.88	CA_n3-n79	244

6	Both bands within FR2 Carrier Aggregation: Specific Band Combination Part	246

7	Dual Connectivity: Specific Band Combination Part	246

7.x	DC_nX-nY	246

7.1	DC_n46-n78	247

7.2	DC_n1-n46	247

7.3	DC_n7-n102	248

7.4	DC_n28-n102	248

7.5	DC_n78-n102	249

7.6	DC_n1-n102	249

7.7	DC_n46-n77	250

7.8	DC_n3-n102	250

7.9	DC_n77-n102	252

7.10	 DC_n7-n20	252

7.11	 DC_n20-n78	253

Annex <X> (informative):Change history	254

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

# x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is a technical report for power class 3 NR inter-band CA and DC for 2 bands DL with up to 2 bands UL under Rel-18 time frame. The purpose is to gather the relevant background information and studies in order to address NR inter-band CA and DC for 2 bands DL with up to 2 bands UL for the Rel-18 band combinations.

This TR contains the RF requirements of band specific combination part. The actual requirements are added to the corresponding technical specifications.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[4]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3:  Range 1 and Range 2 Interworking operation with other radios".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, inter-band NR-DC operation and due to support for SUL operations, for serving cell c

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BS	Base Station

CA	Carrier Aggregation

DC	Dual Connectivity

DL	DownLink

FDD	Frequency Division Duplex

IMD	Inter-modulation

MSD	Maximum Sensitivity Deduction

SCS	Subcarrier spacing

TDD	Time Division Duplex

UE	User Equipment

UL	UpLink

4	Background

4.1	Introduction

The present document is a technical report for NR inter-band CA and DC for 3 bands DL with 2 bands UL under Rel-17 time frame. The document covers the RF requirements for each band combination specific issues (i.e. one sub-clause defined per band combination), including:

1:  Common issues for both 1 band UL and 2 bands UL NR CA, including the impact of UL/DL harmonic/ harmonic mixing associated with REFSEN,  delta Tib and delta Rib, and OOB blocking exception, etc.

2:  2 bands UL NR CA specific issues, including MSD caused by IMD issue, etc.

It shall be noted that no new issue for inter-band NR DC combination, the 2 bands UL NR CA specific issues shall be re-used.

4.2	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

Editor's note: It is not recommended to bring TP to TR for the following cases:

1.   NR CA configurations with additional BCS other than BCS0(such as BCS1) if there is no additional technical issue.

2.   High order DL NR CA configurations, such as DL NR CA configuration CA_nXA-nYC

5	Both bands within FR1 Carrier Aggregation: Specific Band Combination Part

# 5.x	CA_nX-nY

5.x.1	Common for 1 band UL and 2 bands UL CA

5.x.1.1	Operating bands for CA

Table 5.x.1.1-1:  CA band combination of band nX+nY

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| nX |  | – |  |  | – |  |  |
| nY |  | – |  |  | – |  |  |
| NOTE X: Editor's note: Whether or not supporting simultaneous Rx/Tx capability should be identified for TDD-TDD or FDD-TDD band combination |  |  |  |  |  |  |  |

5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination of band nX+nY

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_nXA-nYA | - * or CA_nXA-nYA | nX | 5, 10, 15, 20 | 0 |
|  |  | nY | 5, 10, 15, 20, 25, 30 |  |

Editor's note*: ‘-’ is for 1UL

Editor's note: The table format can be referred to Table 5.5A.3.1-1 in TS38.101-1

5.x.1.3	UE co-existence studies

Table 5.x.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ nX-nY.

Table 5.x.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| nX |  |  |  |  |  |  |  |  |  |  |  |  |
| nY |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.x.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| nX |  |  |  |  |  |  |  |  |  |  |  |  |
| nY |  |  |  |  |  |  |  |  |  |  |  |  |

5.x.1.3.1	UE co-existence studies for 2 Uplink CCs in one Intra-Band CA

Editor's note: This study is only needed in case of 2 CCs uplink transmission in one UL Intra-Band Carrier Aggregation. Applicable to both non-contiguous and contiguous intra-band uplink CA with UL in one band only and also combinations of two UL bands with one band is including 2 contiguous CCs.

Table 5.x.1.3.1-1: Co-existence studies for Uplink Intra-Band Non-Contiguous CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data |  |  |  |  |  | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency |  |  |  |  |  |  |
| 2nd | I fU1L-fU2L I | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges |  |  |  |  |  |  |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges |  |  |  |  |  |  |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Interference ranges |  |  |  |  |  |  |
| 5th | I 3*fU1L-2*fU3L I | I 3*fU1H-2*fU3H I | 4*fU1L-fU3L | 4*fU1H-fU3H | 4*fU1L+fU2L | 4*fU1H+fU2H |
| Interference ranges |  |  |  |  |  |  |
| 6th | I 3*fU1L-3*fU2L I | I 3*fU1H-3*fU3H I | 4*fU1L-2*fU3L | 4*fU1H-2*fU3H | 5*fU1L-fU3L | 5*fUH1-fU3H |
| Interference ranges |  |  |  |  |  |  |
| 7th | I 4*fU1L-3*fU3L I | I 4*fU1H-3*fU3H I | 5*fU1L-2*fU3L | 5*fU1H-2*fU3H | 6*fU1L-fU3L | 6*fU1H-fU3H |
| Interference ranges |  |  |  |  |  |  |

Editor's note (This guidance shall be omitted when drafting TPs):

In the table the following abbreviations is used:

fU1L = minimum frequency of TX aggressor band of ULCC1 lower band range

fU2L = minimum frequency of TX aggressor band of ULCC2 lower band range

fU3L = maximum frequency of TX aggressor band of ULCC2 lower band range

fU1H = maximum frequency of TX aggressor band of ULCC1 higher band range

fU2H = minimum frequency of TX aggressor band of ULCC2 higher band range

fU3H = maximum frequency of TX aggressor band of ULCC2 higher band range

fD1L = minimum frequency of RX victim band of DLCC placed on the lower frequency side of the TX aggressor band

fD1H = maximum frequency of RX victim band of DLCC placed on the lower frequency side of the TX aggressor band

fD2L = minimum frequency of RX victim band of DLCC placed on the higher frequency side of the TX aggressor band

fD2H = maximum frequency of RX victim band of DLCC placed on the higher frequency side of the TX aggressor band

Channel BW = Channel bandwidth of the component carrier. Only equal channel BW is considered

Minimum channel separation = Minimum frequency separation between the two component carriers or the inter CC GB

Maximum channel separation = Maximum frequency separation between the two component carriers or aggregated uplink BW

#### 5.x.1.4	∆TIB,c and ∆RIB,c values

Editor’s note: for the table of ∆TIB,c and ∆RIB,c values, please use the same table format as in the latest TS 38.101-1, the table below is from the latest Rel.17 38.101-1, note that the table format might be changed in Rel.18.

For CA_nX-nY, the TIB,c and RIB,c values are given in the tables below.

Table 5.x.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_nX-nY |  |  |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.x.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_nX-nY |  |  |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.x.1.5	REFSENS requirements

Editor's note 1: Text will be added if harmonics and/or harmonic mixing, cross band isolation, etc. issues are identified, and Reference sensitivity exceptions for bands have these issues need to be provided in the table.

Editor's note 2: The table format shall be the same with the corresponding tables in TS38.101-1, i.e.: Table 7.3A.4-1, Table 7.3A.4-4, Table 7.3A.6-1 for harmonics, harmonic mixing and cross band isolation, respectively.

5.x.1.6	OOB blocking exception requirements

Editor's note:  The necessary analysis on the OOB blocking exception will be needed.

Table 5.x.1.6-1: CA band combination with exceptions allowed

CA band combination

5.x.2	Specific for 2 bands UL CA

Editor's note: Text will be added if 2 bands UL CA are supported, otherwise all the clauses shall be void.

5.x.2.1	Maximum output power for inter-band CA

Table 5.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_nXA-nYA | 23 | +x/-y |

5.x.2.2	UE co-existence studies

Table 5.x.2.2-1 lists Band nX + Band nY 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.x.2.2-1: Band nX and Band nY UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | – |  | – |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | – |  | – |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | – |  | – |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) |  | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | – |  | – |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | – |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |  |
| IMD frequency limits (MHz) | – |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. |  |  |  |  |  |

Based on Table 5.x.2.2-1, nth order IMD may also fall into Rx frequencies of bands nX or band nY.

Table 5.x.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.x.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_nX-nY | E-UTRA Band ..... | FDL_low | - | FDL_high |  |  | x |
|  | Frequency range |  | - |  |  |  |  |
| NOTE x: ..... Editor's note: The NOTE order must keep consistent with the Table 6.5A.3.2.3-1 in TS38.101-1. |  |  |  |  |  |  |  |

Table 5.x.2.3-1 lists Band nX + Band nY 2UL bands CA 1st order triple beat (IMD3) related to 2UL band 3CC (one band support intra-band ULCA) for the UE-to-UE coexistence analysis into the receive band of either Band nX or Band nY, where Band nX is the uplink band supporting two uplink carriers and Band nY is the contiguous single uplink carrier.

In this Release only contiguous UL CA in one band is supported, thus minimum and maximum channel separation in Table 5.x.2.3-1 is equal to minimum and maximum aggregated bandwidth respectively.

If non-contiguous UL-CA in one band is supported, minimum and maximum channel separation in Table 5.x.2.3-1 is equal to minimum and maximum bandwidth separation respectively.

Table 5.x.2.3-1: Band nX and Band nY triple beat IMD products

| CC location | fU1L | fU2L | fU3L | fU1H |  | CBW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | - | - | - | - |  | . |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min ch. separation |
| Frequency | - | - | - | - |  | - |
| 1st order TB | IfU3L -fU1L- fSCCL¦ | IfU2L -fU1L + fSCCL¦ | IfU2L -fU1L- fSCCH¦ | IfU3L -fU1L + fSCCH¦ |  | Max ch. separation |
| Ranges | - | - | - | - |  | - |
| 1st order TB | IfU2L+fU1L-fSCCH¦ | IfU1H+fU2H-fSCCL¦ | IfU2L +fU1L+fSCCL¦ | IfU1H +fU2H+fSCCH¦ |  |  |
| Ranges | - | - | - | - |  |  |

Based on Table 5.x.2.3-1, 1st order triple beat IMD may also fall into Rx frequencies of bands nX or band nY.

If, for the same 2UL bands, an IMD3 also falls in the same Rx frequencies of bands nX or band nY based on Table 5.x.2.2-1 analysis, the corresponding triple beat IMD3 can be ignored.

Guidelines on triple-beat analysis can be found in TR 38.846, clause 7.4.

5.x.2.3	REFSENS requirements

Editor's note: Text will be added if  IMD due to 2 bands UL with 2 UL carriers or triple beat due to 2 band UL with 3 UL carriers issues are identified.

## 5.1	CA_n1-n26

5.1.1	Common for 1 band UL and 2 bands UL CA

5.1.1.1		Operating bands for CA

Table 5.1.1.1-1: CA band combination of band n1 and n26

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |

5.1.1.2		Channel bandwidths per operating band for CA

Table 5.1.1.2-1: Supported bandwidths per CA band combination of band n1+n26

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n26A | CA_n1A-n26A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |

5.1.1.3		UE co-existence studies

Table 5.1.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n1-n26. It is shown that there are no harmonic issues to consider.

Table 5.1.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 3840 | 3960 | 5760 | 5940 | 7680 | 7920 | 9600 | 9900 |
| n26 | 814 | 849 | 859 | 894 | 1628 | 1698 | 2442 | 2547 | 3256 | 3396 | 4070 | 4245 |

Table 5.1.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 4220 | 4340 | 6330 | 6510 | 8440 | 8680 | 10550 | 10850 |
| n26 | 814 | 849 | 859 | 894 | 1718 | 1788 | 2577 | 2682 | 3436 | 3576 | 4295 | 4470 |

5.1.1.4		∆TIB and ∆RIB values

For CA_n1-n26, the TIB,c and RIB,c values are same as for DC_1_n5 and are given in the tables below.

Table 5.1.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n1-n26 | 0.3 | 0.3 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.1.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n1-n26 | - | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.1.1.5 		REFSENS requirements

As can be seen in the co-existence studies in 5.1.1.3 there are no harmonics issues.

5.1.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

Table 5.1.1.6-1: CA band combination with exceptions allowed

CA band combination

5.1.2		Specific for 2 bands UL CA

5.1.2.1	Maximum output power for inter-band CA

Table 5.1.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n1A-n26A | 23 | +2/-3 |

5.1.2.2		UE co-existence studies

Table 5.1.2.2-1 lists Band n1 + Band n26 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.1.2.2-1: Band n1 and Band n26 UL harmonics and IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 814 | 849 | 1920 | 1980 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1166 | 1071 | 2734 | 2829 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 352 | 222 | 2991 | 3146 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3548 | 3678 | 4654 | 4809 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2332 | 2142 | 5468 | 5658 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 462 | 627 | 4911 | 5126 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4362 | 4527 | 6574 | 6789 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7106 | 6831 | 1476 | 1276 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8494 | 8769 | 5176 | 5376 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4312 | 4062 | 1293 | 1518 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7388 | 7638 | 6282 | 6507 |

Based on the table above it can be seen that IMD4 may affect own Rx frequencies of band n1.

Table 5.1.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n26 | E-UTRA Band 1, 3, 5, 7, 11, 18, 19, 21, 26, 31, 40, 42, 43, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |  |

5.1.2.3		REFSENS requirements

Based on the co-existence there are potential IMD4 issues into band n1. But it is not possible to define such testpoints. The potential IMD4 issues is with the lowest range in UL band n1 together with the highest range of UL band n26 which would be a hit in mid-range of DL band n1. But a mid-range UL band 1 does not give IMD4 issues.

Furthermore, CA_1-26 does not have MSD defined, and neither does the equivalent combinations DC_1_n5 or DC_5_n1.  So it is consequential to not have MSD values defined for CA_n1-n26.

## 5.2	CA_n3-n26

5.2.1	Common for 1 band UL and 2 bands UL CA

5.2.1.1		Operating bands for CA

Table 5.2.1.1-1: CA band combination of band n3 and n26

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |

5.2.1.2		Channel bandwidths per operating band for CA

Table 5.2.1.2-1: Supported bandwidths per CA band combination of band n3+n26

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n26A | CA_n3A-n26A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |

5.2.1.3		UE co-existence studies

Table 5.2.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n3-n26. It is shown that there are no harmonic issues to consider.

Table 5.2.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3420 | 3570 | 5130 | 5355 | 6840 | 7140 | 8550 | 8925 |
| n26 | 814 | 849 | 859 | 894 | 1628 | 1698 | 2442 | 2547 | 3256 | 3396 | 4070 | 4245 |

Table 5.2.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3610 | 3760 | 5415 | 5640 | 7220 | 7520 | 9025 | 9400 |
| n26 | 814 | 849 | 859 | 894 | 1718 | 1788 | 2577 | 2682 | 3436 | 3576 | 4295 | 4470 |

5.2.1.4		∆TIB and ∆RIB values

For CA_n3-n26, the TIB,c and RIB,c values are same as for CA_3-26 and are given in the tables below.

Table 5.2.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n3-n26 | 0.3 | 0.3 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.2.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n3-n26 | - | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.2.1.5 		REFSENS requirements

As can be seen in the co-existence studies in 5.2.1.3 there are no harmonics issues.

Based on the co-existence studies there are 2nd harmonic mixing from band n26 DL into band n3 UL. MSD values based on CA_n2-n77 corrected for lower Rx harmonic mixing gain for band n26.

Table 5.2.1.5-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n3 | n26 | 5 | 15 | 25 (RBstart=0) | 5 | 3.7 | NOTE 1 | UL1/DL2 |

5.2.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

Table 5.2.1.6-1: CA band combination with exceptions allowed

CA band combination

5.2.2		Specific for 2 bands UL CA

5.2.2.1	Maximum output power for inter-band CA

Table 5.2.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n3A-n26A | 23 | +2/-3 |

5.2.2.2		UE co-existence studies

Table 5.2.2.2-1 lists Band n3 + Band n26 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.2.2.2-1: Band n3 and Band n26 UL harmonics and IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 814 | 849 | 1710 | 1785 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 971 | 861 | 2524 | 2634 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 157 | 12 | 2571 | 2756 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3338 | 3483 | 4234 | 4419 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1942 | 1722 | 5048 | 5268 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 657 | 837 | 4281 | 4541 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4152 | 4332 | 5944 | 6204 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6326 | 5991 | 1686 | 1471 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7654 | 7989 | 4966 | 5181 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3727 | 3432 | 873 | 1128 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6758 | 7053 | 5862 | 6117 |

Based on the table above it can be seen that IMD4 may affect own Rx frequencies of band n3 and that IMD2 and IMD5 may affect band n26.

Table 5.2.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n3-n26 | E-UTRA Band 1, 5, 7, 11, 18, 19, 21, 26, 34, 39, 40, 43, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA band 22, 41, 42 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 4 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

5.2.2.3		REFSENS requirements

Based on the co-existence there are potential IMD4 issues into band n3 and potential IMD2 and IMD5 issues into band n26. MSD values are reused from CA_3A-26A.

Table 5.2.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n3-n26 | n3 | 1771 | 5 | 25 | 1866 | 4 | FDD | IMD4 |
|  | n26 | 838 | 5 | 25 | 883 | N/A | FDD | N/A |
|  | n3 | 1721 | 5 | 25 | 1816 | N/A | FDD | N/A |
|  | n26 | 838 | 5 | 25 | 883 | 26 | FDD | IMD211 |
| NOTE 11: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

## 5.3	CA_n7-n26

5.3.1	Common for 1 band UL and 2 bands UL CA

5.3.1.1		Operating bands for CA

Table 5.3.1.1-1: CA band combination of band n7 and n26

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |

5.3.1.2		Channel bandwidths per operating band for CA

Table 5.3.1.2-1: Supported bandwidths per CA band combination of band n7+n26

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n7A-n26A | CA_n7A-n26A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |
| CA_n7B-n26A | CA_n7A-n26A CA_n7B | n7 | CA_n7B_BCS0 | 0 |
|  |  | n26 | 5, 10, 15, 20 |  |

5.3.1.3		UE co-existence studies

Table 5.3.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n7-n26. It is shown that there are no harmonic issues to consider.

Table 5.3.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5000 | 5140 | 7500 | 7710 | 10000 | 10280 | 12500 | 12850 |
| n26 | 814 | 849 | 859 | 894 | 1628 | 1698 | 2442 | 2547 | 3256 | 3396 | 4070 | 4245 |

Table 5.3.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 | 10480 | 10760 | 13100 | 13450 |
| n26 | 814 | 849 | 859 | 894 | 1718 | 1788 | 2577 | 2682 | 3436 | 3576 | 4295 | 4470 |

5.3.1.4		∆TIB and ∆RIB values

For CA_n7-n26, the TIB,c and RIB,c values are same as for CA_3-26 and are given in the tables below.

Table 5.3.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n7-n26 | 0.3 | 0.3 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.3.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n7-n26 | - | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.3.1.5 		REFSENS requirements

As can be seen in the co-existence studies in 5.3.1.3 there are no harmonics issues.

Based on the co-existence studies there are near miss3rd harmonic mixing from band n26 DL into band n7 UL. MSD value based on the Skyworks discussion paper R4-2215516: MSD for CA_n7A-n26A.

Table 5.3.1.5-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n7 | n263 | 25 | 15 | 25 (RBstart=104) | 5 | 2.0 | NOTE X | UL1/DL3 near-miss |
| NOTE X: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band |  |  |  |  |  |  |  |  |

5.3.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

Table 5.3.1.6-1: CA band combination with exceptions allowed

CA band combination

5.3.2		Specific for 2 bands UL CA

5.3.2.1	Maximum output power for inter-band CA

Table 5.3.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n7A-n26A | 23 | +2/-3 |

5.3.2.2		UE co-existence studies

Table 5.3.2.2-1 lists Band n7 + Band n26 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.3.2.2-1: Band n7 and Band n26 UL harmonics and IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 814 | 849 | 2500 | 2570 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1756 | 1651 | 3314 | 3419 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 942 | 802 | 4151 | 4326 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4128 | 4268 | 5814 | 5989 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3512 | 3302 | 6628 | 6838 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 128 | 47 | 6651 | 6896 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4942 | 5117 | 8314 | 8559 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9466 | 9151 | 896 | 686 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10814 | 11129 | 5756 | 5966 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6082 | 5802 | 2453 | 2698 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9128 | 9408 | 7442 | 7687 |

Based on the table above it can be seen that IMD5 may affect own Rx frequencies of band n7 and that IMD3 and IMD5 may affect band n26.

Table 5.3.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n7-n26 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 22, 26, 29, 30, 31, 40, 42, 43, 65, 66, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 4, 7, 8 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 7, 8 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 14 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 4 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 8: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. |  |  |  |  |  |  |  |

5.3.2.3		REFSENS requirements

Based on the co-existence there are potential IMD5 issues into band n7 and potential IMD3 and IMD5 issues into band n26. MSD values are reused from CA_3A-26A.

Table 5.3.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n26 | n7 | 2556 | 5 | 25 | 2676 | N/A | FDD | N/A |
|  | n26 | 837 | 5 | 25 | 882 | 16.0 | FDD | IMD311 |
|  | n7 | 2567.5 | 5 | 25 | 2687.5 | 2.5 | FDD | IMD5 |
|  | n26 | 816.5 | 5 | 25 | 861.5 | N/A | FDD | N/A |
| NOTE 11: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

## 5.4	CA_n26-n78

5.4.1	Common for 1 band UL and 2 bands UL CA

5.4.1.1		Operating bands for CA

Table 5.4.1.1-1: CA band combination of band n26 and n78

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

5.4.1.2		Channel bandwidths per operating band for CA

Table 5.4.1.2-1: Supported bandwidths per CA band combination of band n26+n78

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n26A-n78A | CA_n26A-n78A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

5.4.1.3		UE co-existence studies

Table 5.4.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n26-n78.

Table 5.4.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n26 | 814 | 849 | 859 | 894 | 1628 | 1698 | 2442 | 2547 | 3256 | 3396 | 4070 | 4245 |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 | 16500 | 19000 |

Table 5.4.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n26 | 814 | 849 | 859 | 894 | 1718 | 1788 | 2577 | 2682 | 3436 | 3576 | 4295 | 4470 |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 | 16500 | 19000 |

5.4.1.4		∆TIB and ∆RIB values

For CA_n26-n78, the TIB,c and RIB,c values are same as for DC_26_n78 and are given in the tables below.

Table 5.4.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n26-n78 | 0.3 | 0.8 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.4.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n26-n78 | - | 0.5 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.4.1.5 		REFSENS requirements

As can be seen in the co-existence studies in 5.4.1.3 there are 4th harmonics issues to DL n78. Values are reused from DC_26_n78 and CA_n20-n78.

Table 5.4.1.5-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n26 | n78 | 5 | 15 | 16 (RBstart=0) | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n26 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| NOTE 4: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

5.4.1.6	OOB blocking exception requirements

Since band n28 is a low band and n78 is a wide band, the OOBB exception is needed.

Table 5.4.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n26-n78

5.4.2		Specific for 2 bands UL CA

5.4.2.1	Maximum output power for inter-band CA

Table 5.4.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n26A-n78A | 23 | +2/-3 |

5.4.2.2		UE co-existence studies

Table 5.4.2.2-1 lists Band n26 + Band n78 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.4.2.2-1: Band n26 and Band n78 UL harmonics and IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 814 | 849 | 3300 | 3800 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2986 | 2451 | 4114 | 4649 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2172 | 1602 | 5751 | 6786 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4928 | 5498 | 7414 | 8449 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 5972 | 4902 | 8228 | 9298 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1358 | 753 | 9051 | 10586 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5742 | 6347 | 10714 | 12249 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14386 | 12351 | 96 | 544 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14014 | 16049 | 6556 | 7196 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9772 | 8202 | 4053 | 5158 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11528 | 13098 | 9042 | 10147 |

Based on the table above it can be seen that IMD4 may affect own Rx frequencies of band n26.

Table 5.4.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n26-n78 | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 34, 39, 40, 65, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 4 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

5.4.2.3		REFSENS requirements

Based on the co-existence studies the following MSD need to be defined. Values are reused from DC_26_n77.

Table 5.4.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n26-n78 | n26 | 836.5 | 5 | 25 | 881.5 | 11.1 | FDD | IMD4 |
|  | n78 | 3391 | 10 | 50 | 3391 | N/A | TDD | N/A |

## 5.5	CA_n26-n77

5.5.1	Common for 1 band UL and 2 bands UL CA

5.5.1.1		Operating bands for CA

Table 5.5.1.1-1: CA band combination of band n26 and n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

5.5.1.2		Channel bandwidths per operating band for CA

Table 5.5.1.2-1: Supported bandwidths per CA band combination of band n26+n77

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n26A-n77A | CA_n26A-n77A | n26 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

5.5.1.3		UE co-existence studies

Table 5.5.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n26-n77.

Table 5.5.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n26 | 814 | 849 | 859 | 894 | 1628 | 1698 | 2442 | 2547 | 3256 | 3396 | 4070 | 4245 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Table 5.5.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n26 | 814 | 849 | 859 | 894 | 1718 | 1788 | 2577 | 2682 | 3436 | 3576 | 4295 | 4470 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

5.5.1.4		∆TIB and ∆RIB values

For CA_n26-n77, the TIB,c and RIB,c values are same as for DC_26_n77 and are given in the tables below.

Table 5.5.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n26-n77 | 0.3 | 0.8 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.5.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n26-n77 | - | 0.5 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.5.1.5 		REFSENS requirements

As can be seen in the co-existence studies in 5.5.1.3 there are 4th and 5th harmonics issues to DL n77. However, in the USA, n77 band is restricted to 3450 – 3550 MHz and 3700 – 3980 MHz. There is no UL harmonic due to n26 UL in downlink for n77 operating in 3450 – 3550 MHz and 3700 – 3980 MHz.

Table 5.5.1.5-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n26 | n778 | 5 | 15 | 16 (RBstart=0) | 10 | N/A | NOTE 4 | UL4/DL1 direct-hit |
| n26 | n778 | 5 | 15 | 16 (RBstart=0) | 10 | N/A | NOTE 5 | UL5/DL1 direct-hit |
| NOTE 8: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

5.5.1.6	OOB blocking exception requirements

Since band n26 is a low band and n77 is a wide band, the OOBB exception is needed.

Table 5.5.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n26-n77

5.5.2		Specific for 2 bands UL CA

5.5.2.1	Maximum output power for inter-band CA

Table 5.5.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n26A-n77A | 23 | +2/-3 |

5.5.2.2		UE co-existence studies

Table 5.5.2.2-1 lists Band n26 + Band n77 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Since in the USA, n77 band is restricted to 3450 – 3550 MHz and 3700 – 3980 MHz, IMD products are calculated based on those frequencies.

Table 5.5.2.2-1: Band n26 and Band n77 UL harmonics and IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 814 | 849 | 3300 | 4200 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2451 | 3386 | 4114 | 5049 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2572 | 1602 | 5751 | 7586 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4928 | 5898 | 7414 | 9249 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1758 | 753 | 9051 | 11786 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5742 | 6747 | 7414 | 13449 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4902 | 6772 | 8228 | 10098 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 944 | 96 | 12351 | 15986 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6556 | 7596 | 14014 | 17649 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 5958 | 4053 | 8202 | 10972 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11528 | 14298 | 9042 | 10947 |

Based on the table above it can be seen that

IMD4 and IMD5 may affect own Rx frequencies of band n26.

Looking at further on the above IMD impacts, there are no IMD products falling into Rx frequencies of n26 or n77 in the USA.

Table 5.5.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n26-n77 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz |  |  |  |  |  |  |  |

5.5.2.3		REFSENS requirements

Based on the co-existence studies MSD would be defined and values could be reused from CA_n5A-n77A, however, there is no IMD4 or IMD5 product in band n26 downlink for n77 operating in 3450 – 3550 MHz or 3700 – 3980 MHz. Values are therefore marked as N/A as in the current version of 38.101-1 (v17.6.0).

Table 5.5.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n26-n7713 | n26 | N/A | N/A | N/A | N/A | N/A | FDD | IMD4 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n26 | N/A | N/A | N/A | N/A | N/A | FDD | IMD5 |
|  | n77 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| NOTE 13: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

5.6	CA_n70-n77

5.6.1	Common for 1 band UL and 2 bands UL CA

5.6.1.1		Operating bands for CA

Table 5.6.1.1-1: CA band combination of band n70 and n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz | FDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

5.6.1.2		Channel bandwidths per operating band for CA

Table 5.6.1.2-1: Supported bandwidths per CA band combination of band n70+n77

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n70A-n77A | CA_n70A-n77A | n70 | 5, 10, 15, 201, 251 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

The following notes are applied to the above tables:

NOTE 1:	This UE channel bandwidth is applicable only to downlink.

5.6.1.3		UE co-existence studies

Table 5.6.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n70-n77.

Table 5.6.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n70 | 1695 | 1710 | 1995 | 2020 | 3390 | 3420 | 5085 | 5130 | 6780 | 6840 | 8475 | 8550 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Table 5.6.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n70 | 1695 | 1710 | 1995 | 2020 | 3990 | 4040 | 5985 | 6060 | 7980 | 8080 | 9975 | 10100 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

5.6.1.4		∆TIB and ∆RIB values

For CA_n70-n77, the TIB,c and RIB,c values are same as for CA_n70-n78 and are given in the tables below.

Table 5.6.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n70-n77 | 0.6 | 0.8 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.6.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n70-n77 | 0.2 | 0.5 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.6.1.5 		REFSENS requirements

As can be seen in the co-existence studies in 5.6.1.3 there are 2nd harmonics issues to DL n77. Values are reused from CA_n70-n78.

Table 5.6.1.5-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n70 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n70 | n77 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n70 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| NOTE 2: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |

Table 7.3A.4-4: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n776 | n70 | N/A | N/A | N/A | N/A | N/A | NOTE 7 | UL1/DL2 |
| NOTE 6: For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. NOTE 7: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

5.6.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

Table 5.6.1.6-1: CA band combination with exceptions allowed

CA band combination

5.6.2		Specific for 2 bands UL CA

5.6.2.1	Maximum output power for inter-band CA

Table 5.6.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n70A-n77A | 23 | +2/-3 |

5.6.2.2		UE co-existence studies

Table 5.6.2.2-1 list Band n70 + Band n77 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.6.2.2-1: Band n70 and Band n77 UL (3450-3550 MHz) IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 1695 | 1710 | 3300 | 4200 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1740 | 1855 | 4114 | 5049 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 160 | 30 | 5751 | 7586 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6840 | 6970 | 7414 | 9249 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1535 | 1680 | 9051 | 11786 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 8535 | 8680 | 7414 | 13449 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3480 | 3710 | 8228 | 10098 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 3230 | 3390 | 12351 | 15986 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10230 | 10390 | 14014 | 17649 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2015 | 1770 | 8202 | 10972 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 | 14070 | 9042 | 10947 |

Since in the USA, n77 band is restricted to 3450 – 3550 MHz and 3700 – 3980 MHz, IMD products are calculated based on those frequencies. And based on the table above it can be seen that

IMD2 and IMD5 may affect own Rx frequencies of band n70.

Table 5.6.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n70-n77 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 50, 51, 53, 66, 70, 71, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz |  |  |  |  |  |  |  |

5.6.2.3		REFSENS requirements

Based on the co-existence studies the following MSD need to be defined. Values are reused from CA_n70-n78 for IMD2 and IMD5

Table 5.6.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n70-n77 | n70 | 1702.5 | 5 | 25 | 2002.5 | 31 | FDD | IMD2 |
|  | n77 | 3705 | 10 | 50 | 3705 | N/A | TDD | N/A |
|  | n70 | 1697.5 | 5 | 25 | 1997.5 | 5.0 | FDD | IMD5 |
|  | n77 | 3545 | 10 | 50 | 3545 | N/A | TDD | N/A |

5.7	CA_n12-n25

5.7.1	Common for 1 band UL and 2 bands UL CA

5.7.1.1	Operating bands for CA

Table 5.7.1.1-1:  CA band combination of band n12+n25

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n12 | 699 MHz | – | 716MHz | 729 MHz | – | 746 MHz | FDD |
| n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |

5.7.1.2	Channel bandwidths per operating band for CA

Table 5.7.1.2-1: Supported bandwidths per CA band combination of band n12+n25

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n12A-n25A | CA_n12A-n25A | n12 | 5, 10, 15 | 0 |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 |  |

5.7.2	Specific for 2 bands UL CA

5.7.2.1	Maximum output power for inter-band CA

Table 5.7.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n12A-n25A | 23 | +2/-3 |

5.7.2.2	UE co-existence studies

Table 5.7.2.2-1 lists Band n12 + Band n25 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.7.2.2-1: Band n12 and Band n25 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1134 – 1216 |  | 2549 – 2631 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 418 – 517 |  | 2984 – 3131 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3248 –3347 |  | 4399 – 4546 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 182 – 298 |  | 4834 – 5046 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2268 – 2432 |  | 5098 – 5262 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3947 – 4063 |  | 6249 –6461 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6684 – 6961 |  | 881 –1014 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4118 – 4347 |  | 1552 – 1733 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8099 – 8376 |  | 4646 – 4779 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6948 – 7177 |  | 5797 – 5978 |  |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. |  |  |  |  |  |

Based on Table 5.7.2.2-1, there is no IMD issue for CA_n12-n25 with 2UL.

Table 5.7.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.7.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n12-n25 | E-UTRA Band 5, 13, 14, 17, 24, 26, 27, 30, 41, 53, 70, 71, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 66 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 2, 12, 25, 85 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

#### 5.7.2.3	REFSENS requirements

Based on co-existence studies on 5.7.2.2, no need to define exceptional REFSENS requirements.

5.8	CA_n7-n75

5.8.1		Common for 1 band UL and 2 bands UL CA

5.8.1.1			Operating bands for CA

Table 5.8.1.1-1: CA band combination of band n7 + n75

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |

5.8.1.2			Channel bandwidths per operating band for CA

Table 5.8.1.2-1: Supported bandwidths per CA band combination of band n7 + n75

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n7A-n75A | - | n7 |  | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n75 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |

5.8.1.3			Co-existence studies

Tables 5.8.1.3-1 and 5.8.1.3-2 list the band edge harmonics for the UL and DL respectively to allow for inspection of potential DL band REFSENS impact and harmonic mixing.

Table 5.8.1.3-1: UL Harmonics

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5000 | 5140 | 7500 | 7710 | 10000 | 10280 |
| n75 | N/A | N/A | 1432 | 1517 | N/A | N/A | N/A | N/A | N/A | N/A |

Table 5.8.1.3-2: DL Harmonics

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 | 10480 | 10760 |
| n75 | N/A | N/A | 1432 | 1517 | 2864 | 3034 | 4296 | 4551 | 5728 | 6068 |

Based on above tables, there are no harmonic or harmonic mixing concerns for this combination.

5.8.1.4			∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c values proposed in the tables below are the same as those already specified for CA_7-32 in TS 36.101.

Table 5.8.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n7-n75 | 0.7 | 0 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.8.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n7-n75 | - | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.8.1.5			REFSENS requirements

No additional REFSENS requirements are required for this combination.

5.8.1.6	OOB blocking exception requirements

No additional OOB blocking exceptions are required for this CA band combination.

5.9	CA_n28-n94

5.9.1			Common for 1 band UL and 2 bands UL CA

5.9.1.1		Operating bands for CA

Table 5.9.1.1-1: CA band combination of band n28 + n94

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| n94 | 880 MHz | – | 915 MHz | 1432 MHz | – | 1517 MHz | FDD |

5.9.1.2		Channel bandwidths per operating band for CA

Table 5.9.1.2-1: Supported bandwidths per CA band combination of band n28 + n94

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n28A-n94A | - | n28 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 0 |
|  |  | n94 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |

5.9.1.3		Co-existence studies

Tables 5.9.1.3-1 and 5.9.1.3-2 list the band edge harmonics for the UL and DL respectively to allow for inspection of potential DL band REFSENS impact and harmonic mixing.

Table 5.9.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 | 2812 | 2992 |
| n94 | 880 | 915 | 1432 | 1517 | 1760 | 1830 | 2640 | 2745 | 3520 | 3660 |

Table 5.9.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n28 | 703 | 748 | 758 | 803 | 1516 | 1606 | 2274 | 2409 | 3032 | 3212 |
| n94 | 880 | 915 | 1432 | 1517 | 2864 | 3034 | 4296 | 4551 | 5728 | 6068 |

As can be seen from tables 5.9.1.3-1 and 5.9.1.3-2, there is a second harmonic hit from the n28 UL in the n94 DL.

5.9.1.4		∆TIB and ∆RIB values

The ΔTIB,c and ΔRIB,c values proposed in the tables below have been derived from those already specified for CA_28-32 and CA_8-28 in TS 36.101.

Table 5.9.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n28-n94 | 0.5 | 0.6 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.98.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n28-n94 | 0.1 | 0.2 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.9.1.5		REFSENS requirements

Table 5.9.1.5-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n28 | n94 | 5 | 15 | 12 (RBstart=0) | 5 | 28.1 | NOTE 2 | UL2/DL1 direct-hit |
| n28 | n94 | 10 | 15 | 50 (RBstart=0) | 50 | 18.7 | NOTE 2 | UL2/DL1 direct-hit |

5.9.1.6	OOB blocking exception requirements

No additional OOB blocking exceptions are required for this CA band combination.

5.10		CA_n78-n94

5.10.1				Common for 1 band UL and 2 bands UL CA

5.10.1.1			Operating bands for CA

Table 5.10.1.1-1: CA band combination of band n78 + n94

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
| n94 | 880 MHz | – | 915 MHz | 1432 MHz | – | 1517 MHz | FDD |

5.10.1.2			Channel bandwidths per operating band for CA

Table 5.10.1.2-1: Supported bandwidths per CA band combination of band n78 + n94

| NR CA Configuration | UL Configuration | NR Band | Channel bandwidth (MHz) (NOTE 3) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n78A-n94A | - | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 |  | 80 | 90 | 100 | 0 |
|  |  | n94 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |

5.10.1.3			Co-existence studies

Tables 5.10.1.3-1 and 5.10.1.3-2 list the band edge harmonics for the UL and DL respectively to allow for inspection of potential DL band REFSENS impact and harmonic mixing.

Table 5.10.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 10800 | 11400 | 13200 | 15200 |
| n94 | 880 | 915 | 1432 | 1517 | 1760 | 1830 | 2640 | 2745 | 3520 | 3660 |

Table 5.10.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 10800 | 11400 | 13200 | 15200 |
| n94 | 880 | 915 | 1432 | 1517 | 2864 | 3034 | 4296 | 4551 | 5728 | 6068 |

Table 5.10.1.3-1 shows there may be a 4th harmonic impact on the n78 DL from the n94 UL.

5.10.1.4			∆TIB and ∆RIB values

As is the case for DC_8-32_n78 in 38.101-3, zero ΔTIB,c or ΔRIB,c has been proposed in Tables 5.10.1.4-1 and 5.10.1.4-2.

Table 5.10.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n78-n94 | - | - |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.10.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n78-n94 | - | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.10.1.5			REFSENS requirements

Table 7.3A.4-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL A FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n94 | n78 | 5 | 15 | 16 (RBstart=0) | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n94 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| NOTE 4: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

5.10.1.6	OOB blocking exception requirements

No additional OOB blocking exceptions are required for this band combination.

5.11	 CA_n75-n78

5.11.1	Common for 1 band UL and 2 bands UL CA

5.11.1.1	Operating bands for CA

Table 5.11.1.1-1: CA band combination of band n75 and n78

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n75 |  | N/A |  | 1432 MHz | – | 1517 MHz | SDL |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

5.11.1.2	Channel bandwidths per operating band for CA

Table 5.11.1.2-1: Supported bandwidths per CA band combination of band n75 and n78

| NR CA configuration | UL | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n75A-n78A | - | n75 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 1 |
|  |  | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  | - | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |  |  |  |  |  |  |  |  |  |  |  | 4 & 5 |
|  |  | n78 | n78 channel bandwidths in Table 5.3.5-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

5.11.1.3	UE co-existence studies

Table 5.11.1.3-1summarizes frequency ranges where harmonics occur for n75A-n78A which shows that there are no harmonics issues.

Table 5.11.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n75 | N/A | N/A | 1432 | 1517 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 | 16500 | 19000 |

Table 5.11.1.3-2 list harmonic mixing issue for the 2DL bands CA with 1 UL. As can be seen there are no harmonic mixing issues.

Table 5.11.1.3-2 Harmonic mixing for 2DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n75 | N/A | N/A | 1432 | 1517 | 2864 | 3034 | 4296 | 4551 | 5728 | 6068 |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 |

5.11.1.4	∆TIB and ∆RIB values

For CA_n75-n78, the TIB,c and RIB,c values are defined in 38.101-1 and are given in the tables below.

Table 5.11.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n75-n78 | - | 0.8 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.11.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n75-n78 | - | 0.5 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.11.1.5	REFSENS requirements

As can be seen in the co-existence studies in 5.11.1.3 there are no harmonics issues.

5.11.1.6	OOB blocking exception requirements

No need to specify OOB exception requirement for CA_n75-n78

5.12	 CA_n28-n75

5.12.1	Common for 1 band UL and 2 bands UL CA

5.12.1.1	Operating bands for CA

Table 5.12.1.1-1: CA band combination of band n1 and n75

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| n75 |  | N/A |  | 1432 MHz | – | 1517 MHz | SDL |

5.12.1.2	Channel bandwidths per operating band for CA

Table 5.12.1.2-1: Supported bandwidths per CA band combination of band n28 and n75

| NR CA configuration | UL | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n28A-n75A | - | n28 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  | 2 |
|  |  | n75 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |

5.12.1.3	UE co-existence studies

Table 5.12.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n28-n75

Table 5.12.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 | 2812 | 2992 | 3515 | 3740 |
| n75 | N/A | N/A | 1432 | 1517 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

Based on above table, the 2nd harmonic of band n28 falls into the DL of band n75.

Table 5.12.1.3-2 Harmonic mixing for 2DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n28 | 703 | 748 | 758 | 803 | 1516 | 1606 | 2274 | 2409 | 3032 | 3212 |
| n75 | N/A | N/A | 1432 | 1517 | 2864 | 3034 | 4296 | 4551 | 5728 | 6068 |

Based on above table, there is no harmonics mixing issue for the band combination of n28 and n75.

5.12.1.4	∆TIB and ∆RIB values

For CA_n28-n75, the TIB,c and RIB,c values are defined in 38.101-1 and are given in the tables below.

Table 5.12.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n28-n75 | 0.3 | - |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.12.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n28-n78 | 0.2 | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.12.1.5	REFSENS requirements

Reference sensitivity exceptions of CA_n28-n75 due to UL harmonic for the CA combination are defined in TS38.101-1 and are given in the tables below.

Table 5.12.1.5-1: Reference sensitivity exceptions due to UL harmonic for NR CA FR1

| MSD due to harmonic exception for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| n28 | n751,2 | 28.1 | 25.3 | 24.0 | 22.8 | 21.8 | 21.0 | 19.7 | 18.7 |  |  |  |  |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range ∆FHD above and below the edge of this downlink transmission bandwidth. The value ∆FHD depends on the band combination: ∆FHD = 10 MHz for CA_n1-n77, CA_n2-n78, CA_n3-n77, CA_n3-n78, CA_n2-n48, CA_n25-n78, CA_n48-n66, CA_n66-n78. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

5.12.1.6	OOB blocking exception requirements

No need to specify OOB exception requirement for CA_n28-n75

5.13	 CA_n1-n75

5.13.1	Common for 1 band UL and 2 bands UL CA

5.13.1.1	Operating bands for CA

Table 5.13.1.1-1: CA band combination of band n1 and n75

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| n75 |  | N/A |  | 1432 MHz | – | 1517 MHz | SDL |

5.13.1.2	Channel bandwidths per operating band for CA

Table 5.13.1.2-1: Supported bandwidths per CA band combination of band n1 and n75

| NR CA configuration | UL | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | BCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n75A | - | n1 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  | 0 |
|  |  | n75 | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |

5.13.1.3	UE co-existence studies

Table 5.13.1.3-1summarizes frequency ranges where harmonics occur for n1A-n75A which shows that there are no harmonics issues.

Table 5.13.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 3840 | 3960 | 5760 | 5940 | 7680 | 7920 | 9600 | 9900 |
| n75 | N/A | N/A | 1432 | 1517 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

Table 5.13.1.3-2 list harmonic mixing issue for the 2DL bands CA with 1 UL. As can be seen there are no harmonic mixing issues.

Table 5.13.1.3-2 Harmonic mixing for 2DLs/1UL

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 4220 | 4340 | 6330 | 6510 | 8440 | 8680 |
| n75 | N/A | N/A | 1432 | 1517 | 2864 | 3034 | 4296 | 4551 | 5728 | 6068 |

5.13.1.4	∆TIB and ∆RIB values

For CA_n1-n75, the TIB,c and RIB,c values are derived fromCA_n41-n50 and are given in the tables below.

Table 5.13.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n1-n75 | 0.3 | - |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.13.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n1-n75 | - | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.13.1.5	REFSENS requirements

As can be seen in the co-existence studies in 5.13.1.3 there are no harmonics issues.

5.13.1.6	OOB blocking exception requirements

No need to specify OOB exception requirement for CA_n1-n7

5.14		CA_n5-n77

5.14.1	Common for 1 band UL and 2 bands UL CA

5.14.1.1 Operating bands for CA

Table 5.14.1.1-1: CA band combination of band n5+n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

5.14.1.2	Channel bandwidths per operating band for CA

Table 5.14.1.2-1: Supported bandwidths per CA band combination of band n5+n77

| CA operating/channel bandwidth [MHz] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band |  | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n5A-n77(3A) | CA_n77(2A) CA_n5A-n77A | n5 |  | 5, 10, 15, 20 | 0 |
|  |  | n77 |  | CA_n77(3A)_BCS1 |  |
| CA_n5A-n77A | n77 CA_n5A-n77A |  | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n5A-n77(2A) | n77 CA_n5A-n77A CA_n77(2A) |  | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |

5.14.1.3	Co-existence studies

Table 5.14.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n5-n77.

Table 5.14.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 | 4120 | 4245 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table, there is harmonic interference of the 4th and 5th harmonic of n5 falling inside n77 RX.

Table 5.14.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 | 4345 | 4470 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table, there is harmonic issue for CA_n5-n77 having the n77 UL fall inside the 4th harmonic DL of n5.

Both the UL harmonic and the harmonic mixing issue is known and captured in 38.101-1. There’s nothing new to adding n77(2A) to these requirements.

5.14.1.4	∆TIB and ∆RIB values

For CA_n5-n77, the TIB,c and RIB,c values are already captured, since there’s nothing new to adding n77(2A) in UL. The values remains as captured below.

Table 5.14.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n5-n77 | 0.6 | 0.8 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.14.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n5-n77 | 0.2 | 0.5 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.14.1.5	REFSENs requirements

There’s nothing new to adding n77(2A) to these requirements.

5.14.1.6	OOB blocking exception requirements

There’s nothing new to adding n77(2A) to these requirements.

5.14.2		Specific for 2 bands UL CA

5.14.2.1	Maximum output power for inter-band CA

Table 5.14.2A.1.3-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n5A-n77A | 23 | +2/-3 |

5.14.2.2		UE co-existence

Table 5.14.2.2-1 gives IMD interference analysis for CA_ n5-n77 with 2 ULs in n77 having a spectrum restriction of 600MHz separation.

Table 5.14.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low+Ch | fy_low+MaxD | fy_high-MaxD | fy_high-Ch |
| --- | --- | --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 3300 | 4200 | 3400 | 3900 | 3600 | 4100 |
| DL Frequency [MHz] | 3300 | 4200 | 3400 | 3900 | 3600 | 4100 |
| 2nd order IMD products | ¦fy_low+Ch – fx_low¦ | ¦fy_low+MaxD – fx_low¦ | ¦fy_low+Ch + fx_low¦ | ¦fy_high-Ch + fx_high¦ |
| IMD frequency limits (MHz) | 100 | 600 | 6700 | 8300 |
| 3rd order IMD products | ¦2*fx_low + fy_low+Ch¦ | ¦2*fx_high + fy_high-Ch¦ | ¦2*fx_low - fy_low+MaxD¦ | ¦2*fx_high - fy_high-MaxD¦ |
| IMD frequency limits (MHz) | 10000 | 12500 | 2700 | 4800 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low+Ch¦ | ¦3*fx_high + fy_high-Ch¦ | ¦3*fx_low - fy_low+MaxD¦ | ¦3*fx_high - fy_high-MaxD¦ |
| IMD frequency limits (MHz) | 13300 | 16700 | 6000 | 9000 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_low+Ch¦ | ¦2*fx_low - 2*fy_low+MaxD¦ | ¦2*fx_low + 2*fy_low+Ch¦ | ¦2*fx_high + 2*fy_high-Ch¦ |
| IMD frequency limits (MHz) | 200 | 1200 | 13400 | 16600 |
| Two-tone 5th order IMD products | ¦4*fx_low + fy_low+Ch¦ | ¦4*fx_high + fy_high-Ch¦ | ¦4*fx_low – fy_low+MaxD¦ | ¦4*fx_high – fy_high-MaxD¦ |
| IMD frequency limits (MHz) | 16600 | 20900 | 9300 | 13200 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_low+MaxD¦ | ¦3*fx_high – 2*fy_high-MaxD¦ | ¦2*fx_low + 3*fy_low+Ch¦ | ¦3*fx_high + 2*fy_high-Ch¦ |
| IMD frequency limits (MHz) | 2100 | 5400 | 16800 | 20800 |

Based on the table 5.14.2.2-1, there is 4th. order intermodulation products of n77 that fall inside the RX band of n5.

Table 5.14.2.2-2 lists the protected bands required for the 2UL bands CA configuration. There’s nothing new to adding n77(2A) to these requirements.

Table 5.14.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n5-n77 | E-UTRA Band 1, 2, 3, 4, 8, 11, 12, 13, 14, 17, 18, 19, 21, 25, 26, 28, 29, 30, 34, 40, 65, 66, 70, 71, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz |  |  |  |  |  |  |  |

5.14.2.3		REFSENS requirements

Table 5.14.2.3-1 lists the MSD required due to the 4th. IMD for the dual uplink configuration. The MSD value is taken from R4-2217804.

Table 5.14.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n7713 | n5 | N/A | 5 | N/A | 880 | 8.6 | FDD | IMD4 |
|  | n7712 | 3410 | 10 | 1 RBSTART=25 | 3410 | N/A | TDD | N/A |
|  |  | 3850 | 10 | 1 RBSTART=25 | 3850 |  |  |  |

NOTE 12:	This band supports intra-band non-contiguous uplink configuration

NOTE 13:	For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped

5.15		CA_n7-n77

5.15.1	Common for 1 band UL and 2 bands UL CA

5.15.1.1 Operating bands for CA

Table 5.15.1.1-1: CA band combination of band n7+n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

5.15.1.2	Channel bandwidths per operating band for CA

Table 5.15.1.2-1: Supported bandwidths per CA band combination of band n7+n77

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n7(2A)-n77(3A) | CA_n77(2A) CA_n7(2A) CA_n7A-n77A | n7 | See CA_n7(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 | 0 |
|  |  | n77 | See CA_n77(3A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |

5.15.1.3	Co-existence studies

Table 5.15.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n7-n77.

Table 5.15.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5000 | 5140 | 7500 | 7710 | 10000 | 10280 | 12500 | 12850 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table, there is no UL harmonic interference.

Table 5.15.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 | 10480 | 10760 | 13100 | 13450 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table, there is harmonic issue for CA_n7-n77 having the 3rd UL harmonic of n7 fall inside the 2nd harmonic DL of n77.

Based on above table, there is harmonic issue for CA_n7-n77 having the 2nd UL harmonic of n77 fall inside the 3rd harmonic DL of n7.

Both the UL harmonic and the harmonic mixing issue is known and captured in 38.101-1. There’s nothing new to adding n77(2A) to these requirements.

5.15.1.4	∆TIB and ∆RIB values

For CA_n7-n77, the TIB,c and RIB values are already specified in 38.101-1. There’s nothing new to having n77(2A) to these requirements. The values remain as captured below.

Table 5.15.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n7-n77 | 0.5 | 0.8 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.15.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n7-n77 | - | 0.5 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.15.1.5	REFSENs requirements

There’s nothing new to adding n77(2A) to these requirements.

5.15.1.6	OOB blocking exception requirements

There’s nothing new to adding n77(2A) to these requirements.

5.15.2		Specific for 2 bands UL CA

5.15.2.1	Maximum output power for inter-band CA

There’s nothing new to adding n77(2A) to these requirements. The requirements are already covered in 38.101-1.

Table 5.15.2A.1.1-1: UE Power Class for intraband non-contiguous CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n77(2A) | 23 | +2/-3 |

Table 5.15.2A.1.3-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n7A-n77A |  |  |  |  | 23 | +2/-3 |  |  |

5.15.2.2		UE co-existence

Table 5.15.2.2-1 gives IMD interference analysis for CA_ n7-n77 with 2 ULs in n77 having a spectrum restriction of 600MHz separation.

Table 5.15.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low+Ch | fy_low+MaxD | fy_high-MaxD | fy_high-Ch |
| --- | --- | --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 3300 | 4200 | 3400 | 3900 | 3600 | 4100 |
| DL Frequency [MHz] | 3300 | 4200 | 3400 | 3900 | 3600 | 4100 |
| 2nd order IMD products | ¦fy_low+Ch – fx_low¦ | ¦fy_low+MaxD – fx_low¦ | ¦fy_low+Ch + fx_low¦ | ¦fy_high-Ch + fx_high¦ |
| IMD frequency limits (MHz) | 100 | 600 | 6700 | 8300 |
| 3rd order IMD products | ¦2*fx_low + fy_low+Ch¦ | ¦2*fx_high + fy_high-Ch¦ | ¦2*fx_low - fy_low+MaxD¦ | ¦2*fx_high - fy_high-MaxD¦ |
| IMD frequency limits (MHz) | 10000 | 12500 | 2700 | 4800 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low+Ch¦ | ¦3*fx_high + fy_high-Ch¦ | ¦3*fx_low - fy_low+MaxD¦ | ¦3*fx_high - fy_high-MaxD¦ |
| IMD frequency limits (MHz) | 13300 | 16700 | 6000 | 9000 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_low+Ch¦ | ¦2*fx_low - 2*fy_low+MaxD¦ | ¦2*fx_low + 2*fy_low+Ch¦ | ¦2*fx_high + 2*fy_high-Ch¦ |
| IMD frequency limits (MHz) | 200 | 1200 | 13400 | 16600 |
| Two-tone 5th order IMD products | ¦4*fx_low + fy_low+Ch¦ | ¦4*fx_high + fy_high-Ch¦ | ¦4*fx_low – fy_low+MaxD¦ | ¦4*fx_high – fy_high-MaxD¦ |
| IMD frequency limits (MHz) | 16600 | 20900 | 9300 | 13200 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_low+MaxD¦ | ¦3*fx_high – 2*fy_high-MaxD¦ | ¦2*fx_low + 3*fy_low+Ch¦ | ¦3*fx_high + 2*fy_high-Ch¦ |
| IMD frequency limits (MHz) | 2100 | 5400 | 16800 | 20800 |

Based on the table 5.15.2.2-1, there is 5th. order intermodulation products of n77 that fall inside the RX band of n7.

Table 5.15.2.2-2 lists the protected bands required for the 2UL bands CA configuration. There’s nothing new to adding n77(2A) to these requirements.

Table 5.15.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n7-n77 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 40, 50, 51, 65, 66, 67, 68, 72, 74, 75, 76 NR Band n100 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 4, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 6 |
| NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |  |

5.15.2.3		REFSENS requirements

Table 5.15.2.3-1 lists the MSD required due to the 5th. IMD for the dual uplink configuration. The MSD value is taken from R4-2217804.

Table 5.15.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n77 | n7 | N/A | 5 | N/A | 2687.5 | 15 | FDD | IMD5 |
|  | n7712 | 3455 | 10 | 1 (RBSTART=0) | 3455 | N/A | TDD | N/A |
|  |  | 3835 | 10 | 1 (RBSTART=7) | 3835 |  |  |  |

NOTE 12:	This band supports intra-band non-contiguous uplink configuration

5.16		CA_n25-n77

5.16.1	Common for 1 band UL and 2 bands UL CA

5.16.1.1 Operating bands for CA

Table 5.16.1.1-1: CA band combination of band n25+n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

5.16.1.2	Channel bandwidths per operating band for CA

Table 5.16.1.2-1: Supported bandwidths per CA band combination of band n25+n77

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n25A-n77(3A) | CA_n77(2A) CA_n25A-n77A | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 0 |
|  |  | n77 | See CA_n77(3A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |
| CA_n25(2A)-n77(3A) | CA_n25(2A) CA_n77(2A) CA_n25A-n77A | n25 | See CA_n25(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 | 0 |
|  |  | n77 | See CA_n77(3A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |

5.16.1.3	Co-existence studies

Table 5.16.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n25-n77.

Table 5.16.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n25 | 1850 | 1915 | 1930 | 1995 | 3700 | 3830 | 5550 | 5745 | 7400 | 7660 | 9250 | 9575 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table, the 2nd UL harmonic of n25 falls inside the n77 RX band.

Table 5.16.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n25 | 1850 | 1915 | 1930 | 1995 | 3860 | 3990 | 5790 | 5985 | 7720 | 7980 | 9650 | 9975 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table, there is harmonic issue for CA_n25-n77 having the 4th UL harmonic of n25 fall inside the 2nd harmonic of the DL of RX band n77.

Based on above table, there is harmonic issue for CA_n25-n77 having the fundamental TX of n77 fall inside the 2nd harmonic DL of n25 and the 2nd UL harmonic of n77 fall inside the 4th harmonic DL of RX band n25.

Both the UL harmonic and the harmonic mixing issue is known and captured in 38.101-1. There’s nothing new to adding n77(2A) to these requirements

5.16.1.4	∆TIB and ∆RIB values

For CA_n25-n77, the TIB,c and RIB values are already specified in 38.101-1. There’s nothing new to having n77(2A) to these requirements. The values remain as captured below.

Table 5.16.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n25-n77 | 0.6 | 0.8 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.16.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n25-n77 | 0.2 | 0.5 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.16.1.5	REFSENs requirements

There’s nothing new to adding n77(2A) to these requirements.

5.16.1.6	OOB blocking exception requirements

There’s nothing new to adding n77(2A) to these requirements.

5.16.2		Specific for 2 bands UL CA

5.16.2.1	Maximum output power for inter-band CA

There’s nothing new to adding n77(2A) to these requirements. The requirements are already covered in 38.101-1.

Table 5.16.2A.1.1-1: UE Power Class for intraband non-contiguous CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n77(2A) | 23 | +2/-3 |

Table 5.16.2A.1.3-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n77A |  |  |  |  | 23 | +2/-3 |  |  |

5.16.2.2		UE co-existence

Table 5.16.2.2-1 gives IMD interference analysis for CA_ n25-n77 with 2 ULs in n77 having a spectrum restriction of 600MHz separation.

Table 5.16.2.2-1: IMD analysis

| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 | 3320 | 3900 | 4200 | 4180 | 3600 |
| 2nd order | I fU1L-fU2L I | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H |  |  |
| Ranges | 20 | 600 | 6620 | 8380 |  |  |
| 3rd order | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Ranges | 2700 | 4800 | 9920 | 12580 |  |  |
| 4th order | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Ranges | 40 | 1200 | 6000 | 9000 | 13220 | 16780 |
| 5th order | I 3*fUL1-2*fU3L I | I 3*fUH1-2*fU3H I | 4*fUL1-fU3L | 4*fUH1-fU3H | 4*fUL1+fU2L | 4*fUH1+fU2H |
| Ranges | 2100 | 5400 | 9300 | 13200 | 16520 | 20980 |
| 6th order | I 3*fUL1-3*fU2L I | I 3*fUH1-3*fU3H I | 4*fUL1-2*fU3L | 4*fUH1-2*fU3H | 5*fUL1-fU3L | 5*fUH1-fU3H |
| Ranges | 60 | 1800 | 5400 | 9600 | 12600 | 17400 |
| 7th order | I 4*fUL1-3*fU3L I | I 4*fUH1-3*fU3H I | 5*fUL1-2*fU3L | 5*fUH1-2*fU3H | 6*fUL1-fU3L | 6*fUH1-fU3H |
| Ranges | 1500 | 6000 | 8700 | 13800 | 15900 | 21600 |

Based on the table 5.16.2.2-1, the 7th order IMD product of n77 fall inside the RX band of n25.

Table 5.16.2.2-2 lists the protected bands required for the 2UL bands CA configuration. There’s nothing new to adding n77(2A) to these requirements.

Table 5.16.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n25-n77 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

5.16.2.3		REFSENS requirements

Table 5.16.2.3-1 lists the MSD required due to the 7th. IMD for the dual uplink configuration. The MSD value is taken from CA_n2A-n77(2A).

Table 5.16.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n77 | n25 | N/A | 5 | N/A | 1987.5 | 2.7 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 RBSTART=10 | 3455 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 RBSTART=0 | 3945 |  |  |  |

NOTE 12:	This band supports intra-band non-contiguous uplink configuration

5.17		CA_n66-n77

5.17.1	Common for 1 band UL and 2 bands UL CA

5.17.1.1 Operating bands for CA

Table 5.17.1.1-1: CA band combination of band n66+n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

5.17.1.2	Channel bandwidths per operating band for CA

Table 5.17.1.2-1: Supported bandwidths per CA band combination of band n66+n77

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n66A-n77(3A) | CA_n77(2A) CA_n66A-n77A | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 0 |
|  |  | n77 | See CA_n77(3A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |
| CA_n66(2A)-n77(3A) | CA_n66(2A) CA_n77(2A) CA_n66A-n77A | n66 | See CA_n66(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 | 0 |
|  |  | n77 | See CA_n77(3A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |

5.17.1.3	Co-existence studies

Table 5.17.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n66-n77.

Table 5.17.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n66 | 1710 | 1780 | 2110 | 2200 | 3420 | 3560 | 5130 | 5340 | 6840 | 7120 | 8550 | 8900 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table, the 2nd UL harmonic of n66 falls inside the n77 RX band.

Table 5.17.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6600 | 8440 | 8800 | 10550 | 11000 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table, there is harmonic issue for CA_n66-n77 having the 4th UL harmonic of n66 fall inside the 2nd harmonic of the DL of RX band n77.

Based on above table, there is harmonic issue for CA_n66-n77 having the 2nd UL harmonic of n77 fall inside the 3rd harmonic DL of RX band n66.

Both the UL harmonic and the harmonic mixing issue is known and captured in 38.101-1. There’s nothing new to adding n77(2A) to these requirements.

5.17.1.4	∆TIB and ∆RIB values

For CA_n66-n77, the TIB,c and RIB values are already specified in 38.101-1. There’s nothing new to having n77(2A) to these requirements. The values remain as captured below.

Table 5.17.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n66-n77 | 0.6 | 0.8 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.17.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n66-n77 | 0.2 | 0.5 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.17.1.5	REFSENs requirements

There’s nothing new to adding n77(2A) to these requirements.

5.17.1.6	OOB blocking exception requirements

There’s nothing new to adding n77(2A) to these requirements.

5.17.2		Specific for 2 bands UL CA

5.17.2.1	Maximum output power for inter-band CA

Table 5.17.2A.1.3-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n66A-n77A | 23 | +2/-3 |

5.17.2.2		UE co-existence

Table 5.17.2.2-1 gives IMD interference analysis for CA_ n66-n77 with 2 ULs in n77 having a spectrum restriction of 600MHz separation.

Table 5.17.2.2-1: IMD analysis

| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 | 3320 | 3900 | 4200 | 4180 | 3600 |
| 2nd order | I fU1L-fU2L I | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H |  |  |
| Ranges | 20 | 600 | 6620 | 8380 |  |  |
| 3rd order | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Ranges | 2700 | 4800 | 9920 | 12580 |  |  |
| 4th order | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Ranges | 40 | 1200 | 6000 | 9000 | 13220 | 16780 |
| 5th order | I 3*fUL1-2*fU3L I | I 3*fUH1-2*fU3H I | 4*fUL1-fU3L | 4*fUH1-fU3H | 4*fUL1+fU2L | 4*fUH1+fU2H |
| Ranges | 2100 | 5400 | 9300 | 13200 | 16520 | 20980 |
| 6th order | I 3*fUL1-3*fU2L I | I 3*fUH1-3*fU3H I | 4*fUL1-2*fU3L | 4*fUH1-2*fU3H | 5*fUL1-fU3L | 5*fUH1-fU3H |
| Ranges | 60 | 1800 | 5400 | 9600 | 12600 | 17400 |
| 7th order | I 4*fUL1-3*fU3L I | I 4*fUH1-3*fU3H I | 5*fUL1-2*fU3L | 5*fUH1-2*fU3H | 6*fUL1-fU3L | 6*fUH1-fU3H |
| Ranges | 1500 | 6000 | 8700 | 13800 | 15900 | 21600 |

Based on the table 5.17.2.2-1, the 5th and 7th order intermodulation products of n77 fall inside the RX band of n66.

Table 5.17.2.2-2 lists the protected bands required for the 2UL bands CA configuration. There’s nothing new to adding n77(2A) to these requirements.

Table 5.17.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n66-n77 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71, 103 | FDL_low | - | FDL_high | -50 | 1 |  |

5.17.2.3		REFSENS requirements

Table 5.17.2.3-1 lists the MSD required due to the 5th and 7th  IMD for the dual uplink configuration. The MSD value is taken from the agreed WF.

Table 5.17.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n66-n77 | n66 | N/A | 5 | N/A | 2197.5 | 15 | FDD | IMD513 |
|  | n7712 | 3305 | 10 | 1 (RBstart=0) | 3305 | N/A | TDD | N/A |
|  |  | 3855 | 10 | 1 (RBstart=8) | 3855 |  |  |  |
|  | n66 | N/A | 5 | N/A | 2197.5 | 1.7 | FDD | IMD7 |
|  | n7712 | 3455 | 10 | 1 (RBstart=10) | 3455 | N/A | TDD | N/A |
|  |  | 3875 | 10 | 1 (RBstart=0) | 3875 |  |  |  |

NOTE 12:	This band supports intra-band non-contiguous uplink configuration

NOTE 13:	For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped.

5.18		CA_n71-n77

5.18.1	Common for 1 band UL and 2 bands UL CA

5.18.1.1 Operating bands for CA

Table 5.18.1.1-1: CA band combination of band n71+n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

5.18.1.2	Channel bandwidths per operating band for CA

Table 5.18.1.2-1: Supported bandwidths per CA band combination of band n71+n77

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n71A-n77(3A) | CA_n77(2A) CA_n71A-n77A | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 0 |
|  |  | n77 | See CA_n77(3A) Bandwidth Combination Set 1 in Table 5.5A.2-1 |  |

5.18.1.3	Co-existence studies

Table 5.18.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n71-n77.

Table 5.18.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n71 | 663 | 698 | 617 | 652 | 1326 | 1396 | 1989 | 2094 | 2652 | 2792 | 3315 | 3490 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table, the 5th UL harmonic of n71 falls inside the n77 RX band.

Table 5.18.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n71 | 663 | 698 | 617 | 652 | 1234 | 1304 | 1851 | 1956 | 2468 | 2608 | 3085 | 3260 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

There is no harmonic mixing occurring between n71 and n77.

Both the UL harmonic and the harmonic mixing issue is known and captured in 38.101-1. There’s nothing new to adding n77(2A) to these requirements.

5.18.1.4	∆TIB and ∆RIB values

For CA_n66-n77, the TIB,c and RIB values are already specified in 38.101-1. There’s nothing new to having n77(2A) to these requirements. The values remain as captured below.

Table 5.18.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n71-n77 | 0.5 | 0.8 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.18.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n71-n77 | 0.2 | 0.5 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.18.1.5	REFSENs requirements

There’s nothing new to adding n77(2A) to these requirements.

5.18.1.6	OOB blocking exception requirements

There’s nothing new to adding n77(2A) to these requirements.

5.18.2		Specific for 2 bands UL CA

5.18.2.1	Maximum output power for inter-band CA

Table 5.18.2A.1.3-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n71A-n77A | 23 | +2/-3 |

5.18.2.2		UE co-existence

Table 5.18.2.2-1 gives IMD interference analysis for CA_ n71-n77 with 2 ULs in n77 having a spectrum restriction of 600MHz separation.

Table 5.18.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low+Ch | fy_low+MaxD | fy_high-MaxD | fy_high-Ch |
| --- | --- | --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 3300 | 4200 | 3400 | 3900 | 3600 | 4100 |
| DL Frequency [MHz] | 3300 | 4200 | 3400 | 3900 | 3600 | 4100 |
| 2nd order IMD products | ¦fy_low+Ch – fx_low¦ | ¦fy_low+MaxD – fx_low¦ | ¦fy_low+Ch + fx_low¦ | ¦fy_high-Ch + fx_high¦ |
| IMD frequency limits (MHz) | 100 | 600 | 6700 | 8300 |
| 3rd order IMD products | ¦2*fx_low + fy_low+Ch¦ | ¦2*fx_high + fy_high-Ch¦ | ¦2*fx_low - fy_low+MaxD¦ | ¦2*fx_high - fy_high-MaxD¦ |
| IMD frequency limits (MHz) | 10000 | 12500 | 2700 | 4800 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low+Ch¦ | ¦3*fx_high + fy_high-Ch¦ | ¦3*fx_low - fy_low+MaxD¦ | ¦3*fx_high - fy_high-MaxD¦ |
| IMD frequency limits (MHz) | 13300 | 16700 | 6000 | 9000 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_low+Ch¦ | ¦2*fx_low - 2*fy_low+MaxD¦ | ¦2*fx_low + 2*fy_low+Ch¦ | ¦2*fx_high + 2*fy_high-Ch¦ |
| IMD frequency limits (MHz) | 200 | 1200 | 13400 | 16600 |
| Two-tone 5th order IMD products | ¦4*fx_low + fy_low+Ch¦ | ¦4*fx_high + fy_high-Ch¦ | ¦4*fx_low – fy_low+MaxD¦ | ¦4*fx_high – fy_high-MaxD¦ |
| IMD frequency limits (MHz) | 16600 | 20900 | 9300 | 13200 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_low+MaxD¦ | ¦3*fx_high – 2*fy_high-MaxD¦ | ¦2*fx_low + 3*fy_low+Ch¦ | ¦3*fx_high + 2*fy_high-Ch¦ |
| IMD frequency limits (MHz) | 2100 | 5400 | 16800 | 20800 |

Based on the table 5.18.2.2-1, the 4th order intermodulation products of n77 fall inside the RX band of n71.

Table 5.18.2.2-2 lists the protected bands required for the 2UL bands CA configuration. There’s nothing new to adding n77(2A) to these requirements.

Table 5.18.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n71-n77 | E-UTRA Band 1, 3, 4, 5, 8, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 26, 27, 28, 30, 39, 40, 44, 45, 50, 51, 53, 65, 66, 73, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 3 |
|  | E-UTRA Band 2, 7, 25, 41, 34, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

5.18.2.3		REFSENS requirements

Table 5.18.2.3-1 lists the MSD required due to the 4th. IMD for the dual uplink configuration. The MSD value is taken from R4-2217804.

Table 5.18.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n71-n7713 | n71 | N/A | 5 | N/A | 640 | 8.6 | FDD | IMD4 |
|  | n7712 | 3480 | 10 | 1 RBSTART=25 | 3480 | N/A | TDD | N/A |
|  |  | 3800 | 10 | 1 RBSTART=25 | 3800 |  |  |  |

NOTE 12:	This band supports intra-band non-contiguous uplink configuration

NOTE 13:	For a UE which supports this band combination only when the Band n77 frequency range restriction defined in NOTE 12 of Table 5.2-1 applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped.

5.19	CA_n3-n41

5.19.1	Common for 1 band UL and 2 bands UL CA

5.19.1.1 Operating bands for CA

Table 5.19.1.1-1: CA band combination CA_n3-n41

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n41 | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

5.19.1.2	Channel bandwidths per operating band for CA

Table 5.19.1.2-1: Supported bandwidths per CA band combination CA_n3-n41

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n41C | CA_n41C CA_n3A-n41C | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |

5.19.1.3	UE Co-existence studies

The harmonic / harmonic mixing issue have been analyzed in previous release.

IMD order for contiguous UL CA as below.

WGap = lowest band edge of n41 - highest DL band edge of n3 = 2496 – 1880 = 616 MHz.

Min(maxUL aggregated BW, UL band bandwidth) = Min(190, 194) = 190 MHz.

IMD order = 2*ceil(WGap /Min(maxUL aggregated BW, UL band bandwidth) + 1 = 9.

If full n41C bandwidth is considered (190MHz aggregated BW), band n3 is subject to IMD9. But since band n3 is not present in regions where n41 uses the full bandwidth, at current stage we don’t need to consider this IMD9 issue.

If 160MHz aggregated bandwidth is considered for the IMD calculation, e.g. in China (also valid for Japan). lowest IMD9 is in upper 5MHz of band n3, i.e. 1875MHz-1880MHz. but 160MHz n41 spectrum and upper 5MHz of n3 are not hold by one operator by now, so MSD of IMD9 is not applicable for such case.

A test point with N/A MSD is as follows with a note on n41 spectrum

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode |  |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n3-n41 | n3 | N/A | 5 | N/A | 1877.5 | N/AX | FDD | IMD9 |
|  | n41 | 2545 | 60 | 1 RBSTART= 0 | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 RBSTART= 272 | 2625 |  |  |  |
| NOTE X: when n41 spectrum is restricted to 2515-2675MHz and n3 is restricted within 1805-1875MHz for DL. |  |  |  |  |  |  |  |  |

5.19.1.4	∆TIB and ∆RIB values

The TIB,c and RIB have been specified in current spec.

5.19.1.5	REFSENs requirements

As analysied before, the MSD is as below.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode |  |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n3-n41 | n3 | N/A | 5 | N/A | 1877.5 | N/AX | FDD | IMD9 |
|  | n41 | 2545 | 60 | 1 RBSTART= 0 | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 RBSTART= 272 | 2625 |  |  |  |
| NOTE X: Applicable when n41 spectrum is restricted to 2515-2675MHz |  |  |  |  |  |  |  |  |

5.19.1.6	OOB blocking exception requirements

There is no OOB blocking exception requirements

5.19.2		Specific for 2 bands UL CA

5.19.2.1	Maximum output power for inter-band CA

Table 5.19.2A.1.3-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n3A-n41C | 23 | +2/-3 |

5.19.2.2		UE co-existence

The UE co-existence for 2UL CA n3A-n41A have already done and the MSD requirements have already been included in the spec.

For UL CA n3A-n41C, on top of 2UL IMD issue, the triple beat UL IMD should be taken into account.

By referring to the R4-2215525, the 1st order triple beat IMD product is an IMD3 since it is the 1st order product of 3 tones |f1-f2+f3|, and MSD test points are:

For n3A: B3 UL Fc(f1)=1747.5MHz is proposed B3 UL Fc=1747.5MHz, which is the center frequency of band n3.

For n41C: UL n41C: DFT-s-OFDM, 60MHz + 60MHz channel bandwidth (CBW), SCS 30kHz and LCRB= 1 (RBstart=30) + 1(RBstart=127) for CC1 and CC2 respectively. Fc=2590MHz, which are:

The lower edge for the CC1 and CC2 are 2530MHz and  2590MHz, respectively.

f2 is n41 CC1 LRB UL Fc (f2= 2540.98MHz, which is equal to 2530 + (30*12*SCS)+1/2*(SCS));

f3 is n41 CC2 LRB UL Fc (f3= 2635.9MHz, which is equal to 2590 + (127*12*SCS)+1/2*(SCS));

So the B3 DL Fc = 1842.5MHz (i.e. 1747.5 + 95(duplexer gap)), and the triple beat IMD product is expected at 1842.42MHz, i.e. ~100kHz offset from the downlink B3 carrier.

5.19.2.3		REFSENS requirements

R4-2215525 have already given the detail analysis and measurements for the triple beat IMD MSD values for UL DC_3A_n41C in the case of different PCB isolation values,  in general such analysis and measurements and IMD MSD values could be applied to UL CA_n3A-n41C.

Therefore, the MSD for triple UL IMD MSD are defined as:

Table 5.19.2.3-1: MSD due to IMD issue

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode |  |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n3-n41 | n3 | 1747.5 | 5 | 25 RBSTART= 0 | 1842.5 | 15.3 | FDD | IMD3 |
|  | n41 | 2560 | 60 | 1 RBSTART= 30 | 2560 | N/A | TDD | N/A |
|  |  | 2620 | 60 | 1 RBSTART= 127 | 2620 |  |  |  |

5.20		CA_n25-n85

5.20.1	Common for 1 band UL and 2 bands UL CA

5.20.1.1 Operating bands for CA

Table 5.20.1.1-1: CA band combination of band n25+n85

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
| n85 | 698 MHz | – | 716 MHz | 728 MHz | – | 746 MHz | FDD |

5.20.1.2	Channel bandwidths per operating band for CA

Table 5.20.1.2-1: Supported bandwidths per CA band combination of band n25+n85

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n25A-n85A | CA_n25A-n85A | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n25(2A)-n85A | CA_n25A-n85A | n25 | CA_n25(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |

5.20.1.3	Co-existence studies

Table 5.20.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n25-n85.

Table 5.20.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n25 | 1850 | 1915 | 1930 | 1995 | 3700 | 3830 | 5550 | 5745 | 7400 | 7660 | 9250 | 9575 |
| n85 | 698 | 716 | 728 | 746 | 1396 | 1432 | 2094 | 2148 | 2792 | 2864 | 3490 | 3580 |

Based on above table, there is no UL harmonic interference.

Table 5.20.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n25 | 1850 | 1915 | 1930 | 1995 | 3860 | 3990 | 5790 | 5985 | 7720 | 7980 | 9650 | 9975 |
| n85 | 698 | 716 | 728 | 746 | 1456 | 1492 | 2184 | 2238 | 2912 | 2984 | 3640 | 3730 |

Based on above table, there is no harmonic mixing issue for CA_n25-n85.

5.20.1.4	∆TIB and ∆RIB values

For CA_n25-n85, the TIB,c and RIB values are taken from CA_n25-n71.

Table 5.20.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n25-n85 | 0.3 | 0.6 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.20.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n25-n85 | - | 0.3 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.20.1.5	REFSENs requirements

There are no specific REFSENS requirements for 1 band UL

5.20.1.6	OOB blocking exception requirements

There is no OOB exception for the CA combination.

5.20.2		Specific for 2 bands UL CA

5.20.2.1	Maximum output power for inter-band CA

Table 5.20.2A.1.3-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n25A-n85A | 23 | +2/-3 |

5.20.2.2		UE co-existence

Table 5.20.2.2-1 gives IMD interference analysis for CA_ n25-n85 with 2 ULs

Table 5.20.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1850 | 1915 | 698 | 716 |
| DL Frequency [MHz] | 1930 | 1995 | 728 | 746 |
| 2nd order IMD products | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| IMD frequency limits (MHz) | 3700 | 3830 | 1396 | 1432 |
| 3rd order IMD products | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| IMD frequency limits (MHz) | 5550 | 5745 | 2094 | 2148 |
| 3rd order IMD products | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| IMD frequency limits (MHz) | 7400 | 7660 | 2792 | 2864 |
| Two-tone 4th order IMD products | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| IMD frequency limits (MHz) | 9250 | 9575 | 3490 | 3580 |
| Two-tone 4th order IMD products | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| IMD frequency limits (MHz) | 11100 | 11490 | 4188 | 4296 |
| Two-tone 4th order IMD products | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| IMD frequency limits (MHz) | 12950 | 13405 | 4886 | 5012 |
| Two-tone 5th order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1217 | 1134 | 2548 | 2631 |
| Two-tone 5th order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2984 | 3132 | 519 | 418 |
| Two-tone 5th order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4398 | 4546 | 3246 | 3347 |
| Two-tone 5th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4834 | 5047 | 179 | 298 |

Based on the table 5.20.2.2-1, there is no IMD issue for CA_n25-n85.

Table 5.20.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.20.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n25-n85 | E-UTRA Band 5, 12, 13, 14, 17, 24, 26, 30, 53, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band n71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

5.20.2.3		REFSENS requirements

There are no additional MSD requirements for this band combination.

5.21		CA_n41-n85

5.21.1	Common for 1 band UL and 2 bands UL CA

5.21.1.1 Operating bands for CA

Table 5.21.1.1-1: CA band combination of band n41+n85

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| n85 | 698 MHz | – | 716 MHz | 728 MHz | – | 746 MHz | FDD |

5.21.1.2	Channel bandwidths per operating band for CA

Table 5.21.1.2-1: Supported bandwidths per CA band combination of band n41+n85

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n41A-n85A | CA_n41A-n85A | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n85A | CA_n41A-n85A | n25 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  | CA_n41C | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41(2A)-n85A | CA_n41A-n85A | n25 | CA_n41(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |

5.21.1.3	Co-existence studies

Table 5.21.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n41-n85.

Table 5.21.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 | 12480 | 13450 |
| n85 | 698 | 716 | 728 | 746 | 1396 | 1432 | 2094 | 2148 | 2792 | 2864 | 3490 | 3580 |

Based on above table, there is no UL harmonic interference.

Table 5.21.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 | 12480 | 13450 |
| n85 | 698 | 716 | 728 | 746 | 1456 | 1492 | 2184 | 2238 | 2912 | 2984 | 3640 | 3730 |

Based on above table, there is no harmonic mixing issue for CA_n41-n85.

5.21.1.4	∆TIB and ∆RIB values

For CA_n41-n85, the TIB,c and RIB values are based on CA_n41-n71.

Table 5.21.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n41-n85 | 0.3 | 0.6 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.21.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n41-n85 | - | 0.2 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.21.1.5	REFSENs requirements

There are no specific REFSENS requirements for 1 band UL

5.21.1.6	OOB blocking exception requirements

There is no OOB exception for the CA combination.

5.21.2		Specific for 2 bands UL CA

5.21.2.1	Maximum output power for inter-band CA

Table 5.21.2A.1.3-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n41A-n85A | 23 | +2/-3 |

5.21.2.2		UE co-existence

Table 5.21.2.2-1 gives IMD interference analysis for CA_ n41-n85 with 2 ULs

Table 5.21.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 2496 | 2690 | 698 | 716 |
| DL Frequency [MHz] | 2496 | 2690 | 728 | 746 |
| 2nd order IMD products | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| IMD frequency limits (MHz) | 4992 | 5380 | 1396 | 1432 |
| 3rd order IMD products | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| IMD frequency limits (MHz) | 7488 | 8070 | 2094 | 2148 |
| 3rd order IMD products | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| IMD frequency limits (MHz) | 9984 | 10760 | 2792 | 2864 |
| Two-tone 4th order IMD products | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| IMD frequency limits (MHz) | 12480 | 13450 | 3490 | 3580 |
| Two-tone 4th order IMD products | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| IMD frequency limits (MHz) | 14976 | 16140 | 4188 | 4296 |
| Two-tone 4th order IMD products | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| IMD frequency limits (MHz) | 17472 | 18830 | 4886 | 5012 |
| Two-tone 5th order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1992 | 1780 | 3194 | 3406 |
| Two-tone 5th order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4276 | 4682 | 1294 | 1064 |
| Two-tone 5th order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5690 | 6096 | 3892 | 4122 |
| Two-tone 5th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6772 | 7372 | 596 | 348 |

Based on the table 5.21.2.2-1, there is no IMD issue for CA_n41-n85.

Table 5.21.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.21.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n41-n85 | E-UTRA Band 5, 12, 13, 14, 17, 24, 26, 30, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 4, 25, 48, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NR Band n71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

5.21.2.3		REFSENS requirements

There are no additional MSD requirements for this band combination.

## 5.22		CA_n66-n85

5.22.1	Common for 1 band UL and 2 bands UL CA

5.22.1.1 Operating bands for CA

Table 5.22.1.1-1: CA band combination of band n66+n85

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
| n85 | 698 MHz | – | 716 MHz | 728 MHz | – | 746 MHz | FDD |

5.22.1.2	Channel bandwidths per operating band for CA

Table 5.22.1.2-1: Supported bandwidths per CA band combination of band n66+n85

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n66A-n85A | CA_n66A-n85A | n66 | See n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n66(2A)-n85A | CA_n66A-n85A | n66 | CA_n66(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |

5.22.1.3	Co-existence studies

Table 5.22.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n66-n85.

Table 5.22.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n66 | 1710 | 1780 | 2110 | 2200 | 3420 | 3560 | 5130 | 5340 | 6840 | 7120 | 8550 | 8900 |
| n85 | 698 | 716 | 728 | 746 | 1396 | 1432 | 2094 | 2148 | 2792 | 2864 | 3490 | 3580 |

Based on above table, the 3rd harmonic of n85 falls inside n66 RX band causing harmonic interference.

Table 5.22.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6600 | 8440 | 8800 | 10550 | 11000 |
| n85 | 698 | 716 | 728 | 746 | 1456 | 1492 | 2184 | 2238 | 2912 | 2984 | 3640 | 3730 |

Based on above table, there is no harmonic mixing issue for CA_n66-n85.

5.22.1.4	∆TIB and ∆RIB values

For CA_n66-n85, the TIB,c and RIB values are similar to those from CA_n66-n71.

Table 5.22.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n66-n85 | 0.3 | 0.3 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.22.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n66-n85 | - | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.22.1.5	REFSENs requirements

Based on the co-existence studies there is 3rd uplink harmonic from band n85 UL into band n66 DL. MSD value proposed based on similar conditions in CA_n12A-n66A as there is UL spectrum overlap (n12 UL: 699MHz - 716MHz ; n85 UL: 698MHz – 716MHz).

Table 5.22.1.5-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n85 | n66 | 5 | 15 | 8 (RBstart=0) | 10 | 10 | NOTE 3 | UL3/DL1 direct-hit |
| n85 | n66 | 5 | 15 | 20 (RBstart=0) | 40 | 2.4 | NOTE 3 | UL3/DL1 direct-hit |
| NOTE 3: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |

5.22.1.6	OOB blocking exception requirements

No exception needed

5.22.2		Specific for 2 bands UL CA

5.22.2.1	Maximum output power for inter-band CA

Table 5.22.2A.1.3-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n66A-n85A | 23 | +2/-3 |

5.22.2.2		UE co-existence

Table 5.22.2.2-1 gives IMD interference analysis for CA_ n66-n85 with 2 ULs.

Table 5.22.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1710 | 1780 | 698 | 716 |
| DL Frequency [MHz] | 2110 | 2200 | 728 | 746 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1082 | 994 | 2408 | 2496 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2704 | 2862 | 384 | 278 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4118 | 4276 | 3106 | 3212 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4414 | 4642 | 314 | 438 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1988 | 2164 | 4816 | 4992 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5828 | 6056 | 3804 | 3928 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1154 | 1012 | 6422 | 6124 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1272 | 1466 | 3944 | 3698 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4502 | 4644 | 7538 | 7836 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5514 | 5708 | 6526 | 6772 |

Based on the table 6.x.2.2-1, the 4th order IMD product of CA_n66-n85 falls inside the RX of n66.

There are no IMD products produced in Rx of band n85

Table 5.22.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.22.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) |  | MBW (MHz) | NOTE |
| CA_n66-n85 | E-UTRA Band 5, 12, 13, 14, 17, 26, 27, 30, 43, 53, 74, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 2, 4, 25, 41, 42, 48, 50, 51, 66, 70 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 |  | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 |  | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 |  | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |  |

5.22.2.3		REFSENS requirements

As seen in the coexistence study 4th IMD product might fall in own Rx band why MSD is needed for 2 band UL. Values from CA_n66A-n71A is reused due to similar frequency range.

Table 5.22.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n66-n85 | n66 | 1770 | 5 | 25 | 2138 | 5 | FDD | IMD4 |
|  | n85 | 701 | 5 | 25 | 731 | N/A | FDD | N/A |

## 5.23		CA_n77-n85

5.23.1	Common for 1 band UL and 2 bands UL CA

5.23.1.1 Operating bands for CA

Table 5.23.1.1-1: CA band combination of band n77+n85

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
| n85 | 698 MHz | – | 716 MHz | 728 MHz | – | 746 MHz | FDD |

5.23.1.2	Channel bandwidths per operating band for CA

Table 5.23.1.2-1: Supported bandwidths per CA band combination of band n77+n85

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n77A-n85A | CA_n77A-n85A | n77 | See n77 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n77(2A)-n85A | CA_n77A-n85A | n77 | CA_n77(2A)_BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |

5.23.1.3	Co-existence studies

Table 5.23.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n77-n85.

Table 5.23.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |
| n85 | 698 | 716 | 728 | 746 | 1396 | 1432 | 2094 | 2148 | 2792 | 2864 | 3490 | 3580 |

Based on above table, the 5th UL harmonic of n85 causes interference in RX of n77.

Table 5.23.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |
| n85 | 698 | 716 | 728 | 746 | 1456 | 1492 | 2184 | 2238 | 2912 | 2984 | 3640 | 3730 |

Based on above table, the fundamental of n77 causes harmonic mixing at the 5th DL harmonic of n85 for CA_n77-n85.

5.23.1.4	∆TIB and ∆RIB values

For CA_n77-n85, the TIB,c and RIB values are based on CA_n71-n77.

Table 5.23.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n77-n85 | 0.7 | 0.5 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.23.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n77-n85 | 0.5 | 0.2 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.23.1.5	REFSENs requirements

As can be seen in the co-existence studies in 5.23.1.3 there are both UL harmonic and harmonic mixing issues.

Based on the co-existence studies there is 5th uplink harmonic from band n85 UL into band n77 DL. MSD value proposed based on similar conditions in CA_n12A-n77A as there is UL spectrum overlap (n12 UL: 699MHz - 716MHz ; n85 UL: 698MHz – 716MHz).

Table 5.23.1.5-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n85 | n77 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n85 | n77 | 5 | 15 | 20 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

Based on the co-existence studies there are 5th harmonic mixing from band n77 UL into band n85 DL. MSD value proposed based on similar conditions in CA_n12A-n77A as there is UL spectrum overlap (n12 UL: 699MHz - 716MHz ; n85 UL: 698MHz – 716MHz).

Table 5.23.1.5-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n85 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 5 | UL1/DL5 |
| n77 | n85 | 15 | 15 | 75 (RBstart=0) | 15 | 26.2 | NOTE 5 | UL1/DL5 |
| NOTE 5: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |

5.23.1.6	OOB blocking exception requirements

Exceptions added for CA_n77-n85

Table 5.23.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n77-n85

5.23.2		Specific for 2 bands UL CA

5.23.2.1	Maximum output power for inter-band CA

Table 5.23.2A.1.3-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n77A-n85A | 23 | +2/-3 |

5.23.2.2		UE co-existence

Table 5.23.2.2-1 gives IMD interference analysis for CA_ n77-n85 with 2 ULs.

Table 5.23.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 3300 | 4200 | 698 | 716 |
| DL Frequency [MHz] | 3300 | 4200 | 728 | 746 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3502 | 2584 | 3998 | 4916 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 5884 | 7702 | 2804 | 1868 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7298 | 9116 | 4696 | 5632 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 9184 | 11902 | 2106 | 1152 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 5168 | 7004 | 7996 | 9832 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10598 | 13316 | 5394 | 6348 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 436 | 1408 | 16102 | 12484 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4452 | 6306 | 11204 | 8468 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6092 | 7064 | 13898 | 17516 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8694 | 10548 | 11296 | 14032 |

Based on the table 6.x.2.2-1, the 2nd order IMD product of CA_n77-n85 falls inside the RX of n77, but n77 is a TDD band, so there’s no RX during 2UL.

Based on the table 6.x.2.2-1, the 5th order IMD product of CA_n77-n85 falls inside the RX of n85.

Table 5.23.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.23.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) |  | MBW (MHz) | NOTE |
| CA_n77-n85 | E-UTRA Band 3, 5, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 26, 27, 28, 30, 39, 40, 45, 53, 73, 74, 103 | FDL_low | - | FDL_high | -50 | 1 |  |  |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 |  | 3 |
|  | E-UTRA Band 1, 2, 4, 7, 10, 25, 41, 34, 50, 51, 65, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 |  | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 |  | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |  |

5.23.2.3		REFSENS requirements

As seen in the coexistence study 5th IMD product might fall in own Rx band why MSD is needed for 2 band UL. Values from CA_n71A-n77A is reused due to similar frequency range and IMD order.

Table 5.23.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n77-n856 | n77 | 3590 | 10 | 25 | 3590 | N/A | TDD | N/A |
|  | n85 | 712 | 10 | 25 | 742 | 5.5 | FDD | IMD5 |
| NOTE 6: Considering the spectrum holdings of the operator for CA_n77(2A) (when one uplink sub block is assigned within 3300-3400MHz, the other uplink sub block is not assigned within 4000-4200MHz or vice versa), no IMD5 result will fall in Rx frequency range of band n3. Therefore, no MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). |  |  |  |  |  |  |  |  |

5.24	CA_n46-n78

5.24.1	Common for 1 band UL and 2 bands UL CA

5.24.1.1 Operating bands for CA

Table 5.24.1.1-1: CA band combination CA_n46-n78

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n46-n781,6 | n46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. NOTE 6: The PCell is allocated in the licensed band in this combination. |  |  |  |  |  |  |  |  |

5.24.1.2	Channel bandwidths per operating band for CA

Table 5.24.1.2-1: Supported bandwidths per CA band combination CA_n46-n78

| NR CA Configuration | UL CA Configuration | NR Band | Channel bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n46A-n78(2A) | CA_n78(2A) CA_n46A-n78A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n46C-n78(2A) | CA_n78(2A) CA_n46A-n78A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n46D-n78(2A) | CA_n78(2A) CA_n46A-n78A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
| CA_n46(2A)-n78A | CA_n46A-n78A | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n46(2A)-n78(2A) | CA_n78(2A) CA_n46A-n78A | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |

5.24.1.3	UE Co-existence studies

The UL harmonic and harmonic issues have been analysed in TR 38.717-02-01, which shows that harmonic mixing of n46 DL H2 with n78 UL H3 and harmonic mixing of n78 DL H3 with n46 UL H2 could happen. No new requirements are needed for adding n78(2A) to the UL.

5.24.1.4	∆TIB and ∆RIB values

For CA_n46-n78, the TIB,c and RIB are already captured, and no new requirements are needed for adding n78(2A) to the UL.

5.24.1.5	REFSENs requirements

No new requirements are needed for adding n78(2A) to the UL. However, the requirement on the sensitivity degradation caused by harmonic mixing of n46 DL H2 with n78 UL H3 is missing from the Rel-17 TR/TS. Since n46 is a NR-U band, it is proposed to define the exclusion region as below.

The reference sensitivity for the shared access band does not apply when there is at least one individual RE within the shared access downlink transmission bandwidth which falls into the reference sensitivity exclusion region as specified in Table 5.24.1.5-1.

Table 5.24.1.5-1: NR-U reference sensitivity measurement exclusion region in MHz.

| NR Band / Harmonic order / Channel BW (MHz) in UL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL Band | UL/DL Harmonic Order | DL Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| n78 | UL3/DL2 | n46 |  | +/-23 | +/-35 | +/-45 | +/-60 | +/-70 | +/-90 | +/-115 | +/-140 | +/-160 | +/-180 | +/-210 | +/-230 |
| NOTE 1: Even though UL harmonic does not fall directly into NR-U band the exclusion region still applies. NOTE 2: The center of the exclusion region is obtained by multiplying the UL channel center frequency by the ratio of the UL harmonic order over the DL harmonic order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

5.24.1.6	OOB blocking exception requirements

No new requirements are needed for adding n78(2A) to the UL.

5.24.2		Specific for 2 bands UL CA

5.24.2.1	Maximum output power for inter-band CA

Table 5.24.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n46A-n78A | 23 | +2/-3 |

5.24.2.2		UE co-existence studies

Since both band n46 and band n78 are TDD, there are no IMD issues caused by dual-UL transmission with one UL in each band.

#### 5.24.2.2.1	UE co-existence studies for Uplink Intra-Band Non-Contiguous CA

When CA_n78(2A) is configured as UL, the potential IMD interference is calculated as below.

Table 5.24.2.2.1-1: Co-existence studies for Uplink Intra-Band Non-Contiguous CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data | 10 | 10 | 600 | 3300 | 3800 | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency | 3300 | 3320 | 3800 | 3800 | 3780 | 3300 |
| 2nd | I fU1L-fU2LI | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 20 | 500 | 6620 | 7580 | - | - |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H | - | - |
| Interference ranges | 2800 | 4300 | 9920 | 11380 | - | - |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Interference ranges | 40 | 1000 | 6100 | 8100 | 13220 | 15180 |
| 5th | I 3*fUL1-2*fU3L I | I 3*fU1H-2*fU3H I | 4*fU1L-fU3L | 4*fU1H-fU3H | 4*fU1L+fU2L | 4*fU1H+fU2H |
| Interference ranges | 2300 | 4800 | 9400 | 11900 | 3320 | 3780 |
| 6th | I 3*fU1L-3*fU2L I | I 3*fU1H-3*fU3H I | 4*fU1L-2*fU3L | 4*fU1H-2*fU3H | 5*fU1L-fU3L | 5*fU1H-fU3H |
| Interference ranges | 60 | 1500 | 5600 | 8600 | 12700 | 15700 |
| 7th | I 4*fU1L-3*fU3L I | I 4*fU1H-3*fU3H I | 5*fU1L-2*fU3L | 5*fU1H-2*fU3H | 6*fU1L-fU3L | 6*fU1H-fU3H |
| Interference ranges | 1800 | 5300 | 8900 | 12400 | 16000 | 19500 |

It can be seen that there are potential IMD6 and IMD7 interference falling into band n46 DL.

5.24.2.3		REFSENS requirements

Based on the co-existence analysis, there is potential IMD interference caused by UL CA_n78(2A) into band n46. For NR-U bands, MSD is not applicable. And the REFSENS requirement for the NR-U band does not apply when the NR-U DL is affected by the IMD interference. Hence, “N/A” is used for the MSD value.

Table 5.24.2.3-1: MSD due to IMD issue

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode |  |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n46-n78 | n46 | N/A | 10 | N/A | 5660 | N/A | TDD | IMD6 |
|  | n7812 | 3310 | 10 | 1 (RBSTART= 25) | 3310 | N/A | TDD | N/A |
|  |  | 3790 | 10 | 1 (RBSTART= 25) | 3790 |  |  |  |
|  | n46 | N/A | 10 | N/A | 5230 | N/A | TDD | IMD7 |
|  | n7812 | 3310 | 10 | 1 (RBSTART= 25) | 3310 | N/A | TDD | N/A |
|  |  | 3790 | 10 | 1 (RBSTART= 25) | 3790 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. |  |  |  |  |  |  |  |  |

5.25	CA_n1-n46

5.25.1	Common for 1 band UL and 2 bands UL CA

5.25.1.1 Operating bands for CA

Table 5.25.1.1-1: CA band combination CA_n1-n46

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n46 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |

5.25.1.2	Channel bandwidths per operating band for CA

Table 5.25.1.2-1: Supported bandwidths per CA band combination CA_n1-n46

| NR CA Configuration | UL CA Configuration | NR Band | Channel bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n46A | CA_n1A-n46A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | 10, 20, 40, 60, 80 |  |
| CA_n1A-n46C | CA_n1A-n46A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46C_BCS0 |  |
| CA_n1A-n46D | CA_n1A-n46A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46D_BCS0 |  |
| CA_n1A-n46(2A) | CA_n1A-n46A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n46 | CA_n46(2A)_BCS0 |  |

5.25.1.3	UE Co-existence studies

Table 5.25.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n1-n46.

Table 5.25.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 3840 | 3960 | 5760 | 5940 | 7680 | 7920 |
| n46 | 5150 | 5925 | 5150 | 5925 | 10300 | 11850 | 15450 | 17775 | 20600 | 23700 |

Based on above table, the 3rd harmonic of the UL in band n1 may fall into the DL in band n46.

Table 5.25.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 4220 | 4340 | 6330 | 6510 | 8440 | 8680 |
| n46 | 5150 | 5925 | 5150 | 5925 | 10300 | 11850 | 15450 | 17775 | 20600 | 23700 |

Based on above table, there is no harmonic mixing issue.

5.25.1.4	∆TIB and ∆RIB values

For CA_n1-n46, the TIB,c and RIB are given in the tables below.

Table 5.25.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n46 | n1 | 0 |
|  | n46 | 0 |

Table 5.25.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n1-n46 | n1 | 0 |
|  | n46 | 0 |

5.25.1.5	REFSENs requirements

Based on the co-existence analysis, there is potential 3rd harmonic interference into band n46.

The reference sensitivity for the shared access band does not apply when there is at least one individual RE within the shared access downlink transmission bandwidth which falls into the reference sensitivity exclusion region as specified in Table 7.3A.4-1a.

Table 7.3A.4-1a: NR-U reference sensitivity measurement exclusion region in MHz.

| NR Band / Harmonic order / Channel BW in UL |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL Band | Harmonic order | DL Band | 5MHz | 10MHz | 15MHz | 20 MHz | 25 MHz | 30 MHz | 40MHz | 45 MHz | 50 MHz |
| n1 | 3 | n46 | +/- 15 | +/- 23 | +/- 35 | +/- 45 | +/- 60 | +/- 70 | +/- 90 | +/- 100 | +/- 115 |
| NOTE 1: Even though UL harmonic does not fall directly into NR-U band the exclusion region still applies. NOTE 2: The center of the exclusion region is obtained by multiplying the UL channel center frequency by the harmonic order. |  |  |  |  |  |  |  |  |  |  |  |

5.25.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

Table 5.25.1.6-1: CA band combination with exceptions allowed

CA band combination

5.25.2		Specific for 2 bands UL CA

5.25.2.1	Maximum output power for inter-band CA

Table 5.25.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n1A-n46A | 23 | +2/-3 |

5.25.2.2		UE co-existence studies

Table 5.25.2.2-1 gives IMD interference analysis for CA_n1-n46 with 2 ULs.

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 5150 | 5925 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 10300 | 11850 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 15450 | 17775 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 20600 | 23700 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 25750 | 29625 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3170 | 4005 | 7070 | 7905 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2085 | 1190 | 8320 | 9930 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8990 | 9885 | 12220 | 13830 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 165 | 790 | 13470 | 15855 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10910 | 11865 | 17370 | 19755 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 8010 | 6340 | 14140 | 15810 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 21780 | 18620 | 2770 | 1755 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 13935 | 11490 | 4360 | 6090 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 22520 | 25680 | 12830 | 13845 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 19290 | 21735 | 16060 | 17790 |

For band combination CA_n1-n46, the IMD5 product (4*n1-n46) may fall into the DL in band n1.

Table 5.25.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.25.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (Mhz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n46 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, NR Band n78, n79, n100, n104 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 15, 27 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 27: This requirement is applicable for channel bandwidths up to 20 MHz within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when the carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when the carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |

5.25.2.3		REFSENS requirements

Based on the co-existence analysis, there is potential IMD5 interference into band n1. The MSD value is proposed as follows.

Table 5.25.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n46 | n1 | 1950 | 5 | 25 | 2140 | 5 | FDD | IMD5 |
|  | n46 | 5660 | 10 | 50 | 5660 | N/A | TDD | N/A |

5.26	CA_n7-n67

5.26.1	Common for 1 band UL and 2 bands UL CA

5.26.1.1		Operating bands for CA

Table 5.26.1.1-1: CA band combination of band n7 and n67

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| n67 | N/A |  |  | 738 MHz | – | 758 MHz | SDL |

5.26.1.2		Channel bandwidths per operating band for CA

Table 5.26.1.2-1: Supported bandwidths per CA band combination of band n7+n67

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n7A-n67A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n67 | 5, 10, 15, 20 |  |

5.26.1.3		UE co-existence studies

Table 5.26.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n7-n67. It is shown that there are no harmonic issues to consider.

Table 5.26.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5000 | 5140 | 7500 | 7710 | 10000 | 10280 | 12500 | 12850 |
| n67 | N/A | N/A | 738 | 758 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |

Table 5.26.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 | 10480 | 10760 | 13100 | 13450 |
| n67 | N/A | N/A | 738 | 758 | 1476 | 1516 | 2214 | 2274 | 2952 | 3032 | 3690 | 3790 |

5.26.1.4		∆TIB and ∆RIB values

For CA_n7-n67, the TIB,c and RIB,c values are same as for CA_n7-n28 and are given in the tables below.

Table 5.26.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n7-n67 | 0.3 | N/A |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.26.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n7-n67 | - | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.26.1.5 		REFSENS requirements

As can be seen in the co-existence studies in 5.26.1.3 there are no harmonics issues and no harmonic mixing issues.

5.26.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

Table 5.26.1.6-1: CA band combination with exceptions allowed

CA band combination

5.27	CA_n67-n78

5.27.1	Common for 1 band UL and 2 bands UL CA

5.27.1.1		Operating bands for CA

Table 5.27.1.1-1: CA band combination of band n67 and n78

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n67 | N/A |  |  | 738 MHz | – | 758 MHz | SDL |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

5.27.1.2		Channel bandwidths per operating band for CA

Table 5.27.1.2-1: Supported bandwidths per CA band combination of band n67+n78

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n67A-n78A | - | n67 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70 80, 90, 100 |  |
| CA_n67A-n78(2A) | CA_n78(2A) | n67 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |

5.27.1.3		UE co-existence studies

Table 5.27.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n67-n78. It is shown that there are no harmonic issues to consider.

Table 5.27.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n67 | N/A | N/A | 738 | 758 | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 | 16500 | 19000 |

Table 5.27.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n67 | N/A | N/A | 738 | 758 | 1476 | 1516 | 2214 | 2274 | 2952 | 3032 | 3690 | 3790 |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 | 16500 | 19000 |

Table 5.27.1.3-3 lists up to 7th order IMD from UL CA_n78(2A) UE-to-UE coexistence analysis

Table 5.27.1.3-1: CA_n78(2A) IMD products

| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 | 3320 | 3780 | 3800 | 3780 | 3320 |
| 2nd | I fU1L-fU2L I | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H |  |  |
| Ranges | 20 | 480 | 6620 | 7580 |  |  |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Ranges | 2820 | 4280 | 9920 | 11380 |  |  |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Ranges | 40 | 960 | 6120 | 8080 | 13220 | 15180 |
| 5th | I 3*fUL1-2*fU3L I | I 3*fUH1-2*fU3H I | 4*fUL1-fU3L | 4*fUH1-fU3H | 4*fUL1+fU2L | 4*fUH1+fU2H |
| Ranges | 2340 | 4760 | 9420 | 11880 | 16520 | 18980 |
| 6th | I 3*fUL1-3*fU2L I | I 3*fUH1-3*fU3H I | 4*fUL1-2*fU3L | 4*fUH1-2*fU3H | 5*fUL1-fU3L | 5*fUH1-fU3H |
| Ranges | 60 | 1440 | 5640 | 8560 | 12720 | 15680 |
| 7th | I 4*fUL1-3*fU3L I | I 4*fUH1-3*fU3H I | 5*fUL1-2*fU3L | 5*fUH1-2*fU3H | 6*fUL1-fU3L | 6*fUH1-fU3H |
| Ranges | 1860 | 5240 | 8940 | 12360 | 16020 | 19480 |

As can be seen in the co-existence analysis above there are IMD4 and IMD6 products from CA_n78(2A) affecting band n67 DL.

5.27.1.4		∆TIB and ∆RIB values

For CA_n67-n78, the TIB,c and RIB,c values are same as for CA_n13-n77 and are given in the tables below.

Table 5.27.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n67-n78 | N/A | 0.8 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.27.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n67-n78 | 0.2 | 0.5 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.27.1.5 		REFSENS requirements

As can be seen in the co-existence studies in 5.27.1.3 there are no harmonics issues and possible 5th order harmonic mixing issues. MSD value is reused from CA_n13-n77.

Table 5.27.1.5-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n67 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 5 | UL1/DL5 |
| n78 | n67 | 10 | 15 | 50 (RBstart=0) | 10 | 28 | NOTE 5 | UL1/DL5 |
| NOTE 5: The requirements should be verified for DL NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |

Table 5.27.1.5-2 lists the MSD required due to IMD4 for the dual uplink configuration. The MSD value is reused from CA_n5-n77.

Table 5.27.1.5-2: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n67-n78 | n67 | N/A | 5 | N/A | 748 | 8.6 | SDL | IMD4x |
|  | n7812 | 3376 | 10 | 1 RBSTART=25 | 3376 | N/A | TDD | N/A |
|  |  | 3750 | 10 | 1 RBSTART=25 | 3750 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration NOTE X; This band is subject to IMD6 also which MSD is not specified |  |  |  |  |  |  |  |  |

5.27.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

Table 5.27.1.6-1: CA band combination with exceptions allowed

CA band combination

5.28	CA_n18-n40

5.28.1	Common for 1 band UL and 2 bands UL CA

5.28.1.1	Operating bands for CA

Table 5.28.1.1-1:  CA band combination of band n18+n40

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
| n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

5.28.1.2	Channel bandwidths per operating band for CA

Table 5.28.1.2-1: Supported bandwidths per CA band combination of band n18+n40

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n18A-n40A | CA_n18A-n40A | n18 | 5, 10, 15 | 0 |
|  |  | n40 | 10, 15, 20, 30, 40 |  |

5.28.1.3	UE co-existence studies

Table 5.28.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n18-n40.

Table 5.28.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n18 | 815 | 830 | 860 | 875 | 1630 | 1660 | 2445 | 2490 | 3260 | 3320 | 4075 | 4150 |
| n40 | 2300 | 2400 | 2300 | 2400 | 4600 | 4800 | 6900 | 7200 | 9200 | 9600 | 11150 | 12000 |

Table 5.28.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n18 | 815 | 830 | 860 | 875 | 1720 | 1750 | 2580 | 2625 | 3440 | 3500 | 4300 | 4375 |
| n40 | 2300 | 2400 | 2300 | 2400 | 4600 | 4800 | 6900 | 7200 | 9200 | 9600 | 11150 | 12000 |

5.28.1.4	∆TIB,c and ∆RIB,c values

For CA_n18-n40, the TIB,c and RIB,c values are given in the tables below.

Table 5.28.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n18-n40 | 0.3 | 0.3 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.28.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n18-n40 | - | - |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.28.1.5	REFSENS requirements

As analysed in 5.28.1.3, there is no harmonic or harmonic mixing issue.

5.28.1.6	OOB blocking exception requirements

Table 5.28.1.6-1: CA band combination with exceptions allowed

CA band combination

5.28.2	Specific for 2 bands UL CA

5.28.2.1	Maximum output power for inter-band CA

Table 5.28.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n18A-n40A | 23 | +2/-3 |

5.28.2.2	UE co-existence studies

Table 5.28.2.2-1 lists Band n18 + Band n40 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.28.2.2-1: Band n18 and Band n40 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1470 – 1585 |  | 3115 – 3230 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 640 – 770 |  | 3770 – 3985 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 3930 – 4060 |  | 5415 – 5630 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 45 – 190 |  | 6070 – 6385 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2940 – 3170 |  | 6230 – 6460 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4745 – 4890 |  | 7715 – 8030 |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8370 – 8785 |  | 860 – 1020 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5240 – 5570 |  | 2110 – 2355 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10015 – 10430 |  | 5560 – 5720 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8530 – 8860 |  | 7045 – 7290 |  |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. |  |  |  |  |  |

Based on Table 5.28.2.2-1, the IMD5 due to dual UL falls into n18 DL. However, only one test point could be selected to accommodate IMD5, i.e., Fn18_UL= 815MHz, Fn40_UL=2400MHz, and the corresponding victim DL frequency is 860MHz(the lowest frequency of n18 DL), which means there is no possibility to deploy the carriers hence there is no IMD issue for this band combination.

Table 5.28.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.28.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n18-n40 | E-UTRA Band 1, 3, 11, 21, 34, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 22 |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 |  |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 22: As exceptions, for 90 and 100 MHz channel bandwidth, -40 dBm/MHz is applicable in the frequency range of 2496 – 2505 MHz. |  |  |  |  |  |  |  |

5.28.2.3	REFSENS requirements

Based on co-existence studies on 5.28.2.2, the IMD5 due to dual UL falls into n18 DL. However, only one test point could be selected to accomandate IMD5, i.e., Fn18_UL= 815MHz, Fn40_UL=2400MHz, and the corresponding victim DL frequency is 860MHz(the lowest frequency of n18 DL), which means there is no possibility to deploy the carriers to perform test hence there is no IMD issue for this band combination.

Hence no exceptional REFSENS requirements need to be defined.

5.29	CA_n7-n102

5.29.1	Common for 1 band UL and 2 bands UL CA

5.29.1.1 Operating bands for CA

Table 5.29.1.1-1: CA band combination of band n7+n102

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| n102 | 5925 MHz | – | 6425 MHz | 5925 MHz | – | 6425 MHz | TDD |

5.29.1.2	Channel bandwidths per operating band for CA

Table 5.29.1.2-1: Supported bandwidths per CA band combination of band n7+n102

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n7A-n102A | CA_n7A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n7A-n102(2A) | CA_n7A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n7A-n102B | CA_n7A-n102A CA_n7A-n102B | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n7A-n102C | CA_n7A-n102A CA_n7A-n102C | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n7A-n102D | CA_n7A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n7A-n102E | CA_n7A-n102A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |

5.29.1.3	Co-existence studies

Table 5.29.1.3-1/2 summarizes frequency ranges where UL harmonics and/or harmonic mixing occur for CA_n7-n102.

Table 5.29.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5000 | 5140 | 7500 | 7710 | 10000 | 10280 | 12500 | 12850 |
| n102 | 5925 | 6425 | 5925 | 6425 | 11850 | 12850 | 17775 | 19275 | 23700 | 25700 | 29625 | 32125 |

Based on above table, there is no uplink harmonic issues.

Table 5.29.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 | 10480 | 10760 | 13100 | 13450 |
| n102 | 5925 | 6425 | 5925 | 6425 | 11850 | 12850 | 17775 | 19275 | 23700 | 25700 | 29625 | 32125 |

Based on above table, there is no harmonic mixing issues.

5.29.1.4	∆TIB and ∆RIB values

For CA_n7-n102, the TIB,c is based on CA_n7-n79 having n79 as the highest frequency range among similar CA’s. For CA_n7-n102, the RIB values are also based on CA_n7-n79

Table 5.29.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n7-n102 | 0.5 | 0.8 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.29.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n7-n102 | - | 0.5 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.29.1.5	REFSENs requirements

There are no specific REFSENS requirements for 1 band UL

5.29.1.6	OOB blocking exception requirements

There is no OOB exception for the CA combination.

5.29.2		Specific for 2 bands UL CA

5.29.2.1	Maximum output power for inter-band CA

Table 5.29.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n7A-n102A | 23 | +2/-3 |
| CA_n7A-n102B | 23 | +2/-3 |
| CA_n7A-n102C | 23 | +2/-3 |

5.29.2.2		UE co-existence

Table 5.29.2.2-1 gives IMD interference analysis for CA_ n7-n102 with uplink CA.

Table 5.29.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 2500 | 2570 | 5925 | 6425 |
| DL Frequency [MHz] | 2620 | 2690 | 5925 | 6425 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3355 | 3925 | 8425 | 8995 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1425 | 785 | 9280 | 10350 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 10925 | 11565 | 14350 | 15420 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1075 | 1785 | 15205 | 16775 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 7850 | 6710 | 16850 | 17990 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 13425 | 14135 | 20275 | 21845 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 23200 | 21130 | 4355 | 3575 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 14275 | 12635 | 4140 | 5350 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 26200 | 28270 | 15925 | 16705 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 22775 | 24415 | 19350 | 20560 |

Based on the table 5.29.2.2-1, there are no IMD interference issues of this band combination

Table 5.29.2.2-3 gives an IMD interference analysis for CA_ n7-n102 with uplink CA for triple beat using the method of [2].

A triple beat study having n102C transmit in two 20MHz blocks with a 160MHz separation indicates IMD3.

Table 5.29.2.2-3: Triple beat IMD analysis of n102C

| CC location | fU1L | fU2L | fU3L | fU1H |  | Channel BW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 5925 | 5965 | 6085 | 6425 |  | 20 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min channel separation |
| Frequency | 2500 | 2570 | 6385 | 6265 |  | 20 |
| 1st order TB | IfU3L -fU1L- fSCCL¦ | IfU2L -fU1L + fSCCL¦ | IfU2L -fU1L- fSCCH¦ | IfU3L -fU1L + fSCCH¦ |  | Max channel separation |
| Ranges | 2340 | 2540 | 2530 | 2730 |  | 160 |
| 1st order TB | IfU2L+fU1L-fSCCH¦ | IfU1H+fU2H-fSCCL¦ | IfU2L +fU1L+fSCCL¦ | IfU1H +fU2H+fSCCH¦ |  |  |
| Ranges | 9320 | 10310 | 14390 | 15380 |  |  |

The uplink of n7A and n102C has 1st order triple beat falling inside n7 DL.

Since this is including a share spectrum access band there is no additional protected bands as compared to n7.

5.29.2.3		REFSENS requirements

There are no specific REFSENS requirements

As stated for figure 1 in [3] the MSD value is not eligible if the two UL inter-bands aren’t part of the same or adjacent band group.

5.30		CA_n28-n102

5.30.1	Common for 1 band UL and 2 bands UL CA

5.30.1.1 Operating bands for CA

Table 5.30.1.1-1: CA band combination of band n28+n102

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| n102 | 5925 MHz | – | 6425 MHz | 5925 MHz | – | 6425 MHz | TDD |

5.30.1.2	Channel bandwidths per operating band for CA

Table 5.30.1.2-1: Supported bandwidths per CA band combination of band n28+n102

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n28A-n102A | CA_n28A-n102A | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n28A-n102(2A) | CA_n28A-n102A | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n28A-n102B | CA_n28A-n102A CA_n28A-n102B | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n28A-n102C | CA_n28A-n102A CA_n28A-n102B | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n28A-n102D | CA_n28A-n102A | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n28A-n102E | CA_n28A-n102A | n28 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |

5.30.1.3	Co-existence studies

Table 5.30.1.3-1/2 summarizes frequency ranges where UL harmonics and/or harmonic mixing occur for CA_n28-n102.

Table 5.30.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 | 2812 | 2992 | 3515 | 3740 |
| n102 | 5925 | 6425 | 5925 | 6425 | 11850 | 12850 | 17775 | 19275 | 23700 | 25700 | 29625 | 32125 |

Based on above table, there is no uplink harmonic issues.

Table 5.30.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n28 | 703 | 748 | 758 | 803 | 1516 | 1606 | 2274 | 2409 | 3032 | 3212 | 3790 | 4015 |
| n102 | 5925 | 6425 | 5925 | 6425 | 11850 | 12850 | 17775 | 19275 | 23700 | 25700 | 29625 | 32125 |

Based on above table, there is no harmonic mixing issues.

5.30.1.4	∆TIB and ∆RIB values

For CA_n28-n102, the TIB,c is based on CA_n28-n79 having n79 as the highest frequency range among similar CA’s. For CA_n28-n102, the RIB values are also based on CA_n28-n79

Table 5.30.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n28-n102 | 0.5 | 0.8 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.30.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n28-n102 | 0.2 | 0.5 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.30.1.5	REFSENs requirements

There are no specific REFSENS requirements for 1 band UL

5.30.1.6	OOB blocking exception requirements

Table 5.30.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n28-n102

5.30.2		Specific for 2 bands UL CA

5.30.2.1	Maximum output power for inter-band CA

Table 5.30.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n28A-n102A | 23 | +2/-3 |
| CA_n28A-n102B | 23 | +2/-3 |
| CA_n28A-n102C | 23 | +2/-3 |

5.30.2.2		UE co-existence

Table 5.30.2.2-1 gives IMD interference analysis for CA_ n28-n102 with uplink CA.

Table 5.30.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 703 | 748 | 5925 | 6425 |
| DL Frequency [MHz] | 758 | 803 | 5925 | 6425 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5177 | 5722 | 6628 | 7173 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 5019 | 4429 | 11102 | 12147 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7331 | 7921 | 12553 | 13598 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4316 | 3681 | 17027 | 18572 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 11444 | 10354 | 13256 | 14346 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8034 | 8669 | 18478 | 20023 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 24997 | 22952 | 2933 | 3613 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 17869 | 16279 | 9606 | 10741 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 24403 | 26448 | 8737 | 9417 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 19181 | 20771 | 13959 | 15094 |

Based on the table 5.30.2.2-1, there are no IMD interference issues of this band combination

Since this is including a share spectrum access band there is no additional protected bands as compared to n28A.

Table 5.30.2.2-3 gives an IMD interference analysis for CA_n28-n102 with uplink CA for triple beat using the method of [2].

A triple beat study having n102C transmit in two 20MHz blocks with a 160MHz separation indicates IMD3.

Table 5.30.2.2-3: Triple beat IMD analysis of n102C

| CC location | fU1L | fU2L | fU3L | fU1H |  | Channel BW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 5925 | 5965 | 6085 | 6425 |  | 20 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min ch. separation |
| Frequency | 703 | 748 | 6385 | 6265 |  | 20 |
| 1st order TB | IfU3L -fU1L- fSCCL¦ | IfU2L -fU1L + fSCCL¦ | IfU2L -fU1L- fSCCH¦ | IfU3L -fU1L + fSCCH¦ |  | Max ch. separation |
| Ranges | 543 | 743 | 708 | 908 |  | 160 |
| 1st order TB | IfU2L+fU1L-fSCCH¦ | IfU1H+fU2H-fSCCL¦ | IfU2L +fU1L+fSCCL¦ | IfU1H +fU2H+fSCCH¦ |  |  |
| Ranges | 11142 | 12107 | 12593 | 13558 |  |  |

The uplink of n28A and n102C has 1st order triple beat falling inside n28 DL.

Since this is including a share spectrum access band there is no additional protected bands as compared to n28A.

5.30.2.3		REFSENS requirements

There are no specific REFSENS requirements

As stated for figure 1 in [3] the MSD value is not eligible if the two UL inter-bands aren’t part of the same or adjacent band group.

5.31		CA_n78-n102

5.31.1	Common for 1 band UL and 2 bands UL CA

5.31.1.1 Operating bands for CA

Table 5.31.1.1-1: CA band combination of band n78+n102

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
| n102 | 5925 MHz | – | 6425 MHz | 5925 MHz | – | 6425 MHz | TDD |

5.31.1.2	Channel bandwidths per operating band for CA

Table 5.31.1.2-1: Supported bandwidths per CA band combination of band n78+n102

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n78A-n102A | CA_n78A-n102A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n78A-n102(2A) | CA_n78A-n102A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n78A-n102B | CA_n78A-n102A CA_n78A-n102B | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n78A-n102C | CA_n78A-n102A CA_n78A-n102C | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n78A-n102D | CA_n78A-n102A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n78A-n102E | CA_n78A-n102A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n78(2A)-n102A | CA_n78(2A) CA_n78A-n102A | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n78(2A)-n102B | CA_n78(2A) CA_n78A-n102A CA_n78A-n102B | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n78(2A)-n102C | CA_n78A-n102A CA_n78A-n102C | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n78(2A)-n102D | CA_n78(2A) CA_n78A-n102A | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n78(2A)-n102E | CA_n78(2A) CA_n78A-n102A | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n78(2A)-n102(2A) | CA_n78(2A) CA_n78A-n102A | n78 | CA_n78(2A)_BCS2 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |

5.31.1.3	Co-existence studies

Table 5.31.1.3-1/2 summarizes frequency ranges where UL harmonics and/or harmonic mixing occur for CA_n78-n102.

Table 5.31.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 | 16500 | 19000 |
| n102 | 5925 | 6425 | 5925 | 6425 | 11850 | 12850 | 17775 | 19275 | 23700 | 25700 | 29625 | 32125 |

Based on above table, there is no uplink harmonic issues.

Table 5.31.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 | 16500 | 19000 |
| n102 | 5925 | 6425 | 5925 | 6425 | 11850 | 12850 | 17775 | 19275 | 23700 | 25700 | 29625 | 32125 |

Based on above table, there is no harmonic mixing issues.

Table 5.31.1.3-3 gives a non-contiguous uplink IMD interference analysis

Table 5.31.1.3-3: Non-contiguous uplink IMD analysis

| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 | 3320 | 3800 | 3800 | 3780 | 3300 |
| 2nd | I fU1L-fU2L I | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 20 | 500 | 6620 | 7580 |  |  |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 2800 | 4300 | 9920 | 11380 |  |  |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Interference ranges | 40 | 1000 | 6100 | 8100 | 13220 | 15180 |
| 5th | I 3*fUL1-2*fU3L I | I 3*fUH1-2*fU3H I | 4*fUL1-fU3L | 4*fUH1-fU3H | 4*fUL1+fU2L | 4*fUH1+fU2H |
| Interference ranges | 2300 | 4800 | 9400 | 11900 | 16520 | 18980 |
| 6th | I 3*fUL1-3*fU2L I | I 3*fUH1-3*fU3H I | 4*fUL1-2*fU3L | 4*fUH1-2*fU3H | 5*fUL1-fU3L | 5*fUH1-fU3H |
| Interference ranges | 60 | 1500 | 5600 | 8600 | 12700 | 15700 |
| 7th | I 4*fUL1-3*fU3L I | I 4*fUH1-3*fU3H I | 5*fUL1-2*fU3L | 5*fUH1-2*fU3H | 6*fUL1-fU3L | 6*fUH1-fU3H |
| Interference ranges | 1800 | 5300 | 8900 | 12400 | 16000 | 19500 |

Min channel = 10MHz

Min Channel separation = 10MHz

Max channel separation = 500MHz

Based on the table 5.x.2.2-3, there are IMD4 and IMD6 products falling inside n102 DL, which may occur in simultaneous RX/TX configurations.

5.31.1.4	∆TIB and ∆RIB values

For CA_n78-n102, the TIB,c is based on CA_n78-n79 having n79 as the highest frequency range among similar CA’s. Since this case may have simultaneous RX/TX the values with Note 8 have been used, For CA_n78-n102, the RIB values are also based on CA_n46-n78.

Table 5.31.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n78-n102 | 1.5 | 1.5 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.31.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n78-n102 | 0.5 | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.31.1.5	REFSENs requirements

Based on the co-existence studies there are a need to define MSD values. MSD values are made in similar way to the NR-U combination of CA_n46A-n78A with CA_n78(2A) in the UL.

Table 5.31.1.5-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n78-n102 | n78 | 3320 | 10 | 1 (RBstart=25) | 3320 | N/A | TDD | N/A |
|  | n78 | 3680 | 10 | 1 (RBstart=25) | 3680 | N/A | TDD | N/A |
|  | n102 | 6280 | 20 | 100 | 6280 | N/A11 | TDD | IMD4 |
| NOTE 11: This is a share spectrum access band, hence no MSD is defined. |  |  |  |  |  |  |  |  |

5.31.1.6	OOB blocking exception requirements

There is no OOB exception for the CA combination.

5.31.2		Specific for 2 bands UL CA

5.31.2.1	Maximum output power for inter-band CA

Table 5.31.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n78A-n102A | 23 | +2/-3 |
| CA_n78A-n102B | 23 | +2/-3 |
| CA_n78A-n102C | 23 | +2/-3 |

5.31.2.2		UE co-existence

Table 5.31.2.2-1 gives IMD interference analysis for CA_ n78-n102.

Table 5.31.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 3300 | 3800 | 5925 | 6425 |
| DL Frequency [MHz] | 3300 | 3800 | 5925 | 6425 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2125 | 3125 | 9225 | 10225 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 175 | 1675 | 8050 | 9550 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 12525 | 14025 | 15150 | 16650 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3475 | 5475 | 13975 | 15975 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6250 | 4250 | 18450 | 20450 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 15825 | 17825 | 21075 | 23075 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 22400 | 19900 | 9275 | 6775 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 12675 | 10175 | 450 | 2950 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 27000 | 29500 | 19125 | 21625 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 24375 | 26875 | 21750 | 24250 |

Based on the table 5.31.2.2-1, there are no IMD interference issues of this band combination

Since this is including a share spectrum access band there is no additional protected bands as compared to n78.

As this is a TDD-TDD combination the triple beat analysis for CA_n78A-n102B and CA_n78A-n102C will never impact any downlink of the two bands, since there’s no triple beat uplink if either band is in downlink operation.

5.31.2.3		REFSENS requirements

There are no additional MSD requirements for this band combination.

5.32		CA_n1-n102

5.32.1	Common for 1 band UL and 2 bands UL CA

5.32.1.1 Operating bands for CA

Table 5.32.1.1-1: CA band combination of band n1+n102

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| n102 | 5925 MHz | – | 6425 MHz | 5925 MHz | – | 6425 MHz | TDD |

5.32.1.2	Channel bandwidths per operating band for CA

Table 5.32.1.2-1: Supported bandwidths per CA band combination of band n1+n102

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n102A | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n1A-n102(2A) | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n1A-n102B | CA_n1A-n102A CA_n1A-n102B | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n1A-n102C | CA_n1A-n102A CA_n1A-n102C | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n1A-n102D | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n1A-n102E | CA_n1A-n102A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |

5.32.1.3	Co-existence studies

Table 5.32.1.3-1/2 summarizes frequency ranges where UL harmonics and/or harmonic mixing occur for CA_n1-n102.

Table 5.32.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 3840 | 3960 | 5760 | 5940 | 7680 | 7920 | 9600 | 9900 |
| n102 | 5925 | 6425 | 5925 | 6425 | 11850 | 12850 | 17775 | 19275 | 23700 | 25700 | 29625 | 32125 |

Based on above table, the 3rd UL harmonic of n1 falls inside the n102 RX band. Highest channel of n1 with 5MHz BW results in a 3rd order UL harmonic at 5932.5MHz, which will collide with 7.5MHp overlap of the lowest 20MHz channel allocation of n102 centered at 5935MHz.

Table 5.32.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 4220 | 4340 | 6330 | 6510 | 8440 | 8680 | 10550 | 10850 |
| n102 | 5925 | 6425 | 5925 | 6425 | 11850 | 12850 | 17775 | 19275 | 23700 | 25700 | 29625 | 32125 |

Based on above table, the UL n102 falls inside the 3rd order harmonic of n1 RX band (n102 – n1 UL1/DL3). Since CA_n1-n102 is defined with UL CA as well the harmonic mixing case above may occur in the UL CA case, but not the single UL case, since n1 will have the PCell in the 1 UL scenario.

5.32.1.4	∆TIB and ∆RIB values

For CA_n1-n102, the TIB,c is based on CA_n1-n77 having n77 as the highest frequency range among similar CA’s. For CA_n1-n102, the RIB values are also based on CA_n1-n77

Table 5.32.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n1-n102 | 0.6 | 0.8 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.32.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n1-n102 | 0.2 | 0.5 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.32.1.5	REFSENs requirements

Based on the co-existence studies there are 3rd uplink harmonic from band n1 UL that partially falls inside band n102 DL at lowest channel allocation. Since this is a share spectrum access band we apply an exclusion region to mitigate this issue.

Table 7.3A.4-1a: NR-U reference sensitivity measurement exclusion region in MHz.

| NR Band / Harmonic order / Channel BW in UL |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL Band | Harmonic order | DL Band | 5MHz | 10MHz | 15MHz | 20 MHz | 25 MHz | 30 MHz | 40MHz | 50 MHz |
| n1 | 3 | n102 | +/- 10 | +/- 20 | +/- 30 | +/- 40 | +/- 50 | +/- 60 | +/- 80 | +/- 100 |
| NOTE 1: Even though UL harmonic does not fall directly into NR-U band the exclusion region still applies. NOTE 2: The center of the exclusion region is obtained by multiplying the UL channel center frequency by the harmonic order. |  |  |  |  |  |  |  |  |  |  |

Based on the co-existence studies there are 3rd harmonic mixing between band n102 UL1 into band n1 DL3. Similar UL1/DL3 cases show in the range of 20dB of relaxation for the larger UL BW case. Since n102 doesn’t support less than 20MHz UL BW we define only this test point. It is expected that n1 and n102 don’t share antenna, which is not the case for other UL1/DL3 cases. So, we assume higher attenuation of the aggressor and therefore use the minimum value of previous UL1/DL3 cases.

Table 5.32.1.5-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n102 | n1 | 20 | 15 | 100 (RBstart=0) | 5 | 13.5 | NOTE 4 | UL1/DL3 |
| NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

5.32.1.6	OOB blocking exception requirements

Exceptions added for CA_n1-n102

Table 5.32.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n1-n102

5.32.2		Specific for 2 bands UL CA

5.32.2.1	Maximum output power for inter-band CA

Table 5.32.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n1A-n102A | 23 | +2/-3 |
| CA_n1A-n102B | 23 | +2/-3 |
| CA_n1A-n102C | 23 | +2/-3 |

5.32.2.2		UE co-existence

Table 5.32.2.2-1 gives IMD interference analysis for CA_ n1-n102 with uplink CA.

Table 5.32.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1920 | 1980 | 5925 | 6425 |
| DL Frequency [MHz] | 2110 | 2170 | 5925 | 6425 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3945 | 4505 | 7845 | 8405 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2585 | 1965 | 9870 | 10930 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9765 | 10385 | 13770 | 14830 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 665 | 15 | 15795 | 17355 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 9010 | 7890 | 15690 | 16810 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 11685 | 12365 | 19695 | 21255 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 23780 | 21720 | 1995 | 1255 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 15435 | 13815 | 5910 | 7090 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 25620 | 27680 | 13605 | 14345 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 21615 | 23235 | 17610 | 18790 |

Based on the table 5.32.2.2-1, the 3rd order intermodulation product fall inside the RX band of n1.

Table 5.32.2.2-3 gives an IMD interference analysis for CA_n1-n102 with uplink CA for triple beat using the method of [2].

A triple beat study having n102C transmit in two 20MHz blocks with a 160MHz separation indicates IMD3.

Table 5.32.2.2-3: Triple beat IMD analysis of n102C

| CC location | fU1L | fU2L | fU3L | fU1H |  | Channel BW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 5925 | 5965 | 6085 | 6425 |  | 20 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min ch. separation |
| Frequency | 1920 | 1980 | 6385 | 6265 |  | 20 |
| 1st order TB | IfU3L -fU1L- fSCCL¦ | IfU2L -fU1L + fSCCL¦ | IfU2L -fU1L- fSCCH¦ | IfU3L -fU1L + fSCCH¦ |  | Max ch. separation |
| Ranges | 1760 | 1960 | 1940 | 2140 |  | 160 |
| 1st order TB | IfU2L+fU1L-fSCCH¦ | IfU1H+fU2H-fSCCL¦ | IfU2L +fU1L+fSCCL¦ | IfU1H +fU2H+fSCCH¦ |  |  |
| Ranges | 9910 | 10890 | 13810 | 14790 |  |  |

The uplink of n1A and n102C has 1st order triple beat falling inside n1 DL.

Since this is including a share spectrum access band there is no additional protected bands as compared to n1.

5.32.2.3		REFSENS requirements

Table 5.32.2.3-1 lists the MSD required due to the 3rd IMD product for the dual uplink configuration. The product that hits n1 DL is similar to that which hits in CA-n1-n3, which is also an IMD3 case. Looking at the architecture of the RFFE there would be an additional attenuation between the antennas due to increased frequency selectivity at the large spectral distance. We propose a 10dB improved relaxation to the MSD compared to the CA_n1-n3 case considering also the minimum UL CBW of n102.

As stated for figure 1 in [3] the MSD value is not eligible if the two UL inter-bands aren’t part of the same or adjacent band group.

Table 5.32.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n102 | n1 | 1977.5 | 5 | 25 | 2167.5 | 13 | FDD | IMD3 |
|  | n102 | 5935 | 20 | 100 | 5935 | N/A | TDD | N/A |

5.33   CA_n26-n28

5.33.1	Common for 1 band UL and 2 bands UL CA

5.33.1.1		Operating bands for CA

Table 5.33.1.1-1: CA band combination of band n26 and n28

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
| n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | TDD |
| NOTE X: Uplink is only in n28 for CA_n26-n28 |  |  |  |  |  |  |  |

5.33.1.2		Channel bandwidths per operating band for CA

Table 5.33.1.2-1: Supported bandwidths per CA band combination of band n26+n28

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n26A-n28A | - | n26 | 5, 10, 15, 20 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |

5.33.1.3		UE co-existence studies

Table 5.33.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n26-n28.

Table 5.33.1.3-1: Impact of UL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n26 | 814 | 849 | 859 | 894 | 1628 | 1698 | 2442 | 2547 | 3256 | 3396 | 4070 | 4245 |
| n28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 | 2812 | 2992 | 3515 | 3740 |

Table 5.33.1.3-2: Impact of DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n26 | 814 | 849 | 859 | 894 | 1718 | 1788 | 2577 | 2682 | 3436 | 3576 | 4295 | 4470 |
| n28 | 703 | 748 | 758 | 803 | 1516 | 1606 | 2274 | 2409 | 3032 | 3212 | 3790 | 4015 |

There’s no REFSENS impact due to UL harmonic or DL harmonic mixing.

5.33.1.4		∆TIB and ∆RIB values

For CA_n26-n28, the TIB,c and RIB,c values are same as for CA_n5-n28 and are given in the tables below.

Table 5.33.1.4-1: ΔTIB,c

| NR CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n26-n28 | n26 | 0.5 |
|  | n28 | 0.5 |

Table 5.33.1.4-2: ΔRIB,c

| NR CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n26-n28 | n26 | 0.2 |
|  | n28 | 0.2 |

5.33.1.5 		REFSENS requirements

There’s no MSD exception needed due to uplink is only in n28

5.33.1.6	OOB blocking exception requirements

There’s no OOBB exception needed for the combo.

5.33.2		Specific for 2 bands UL CA

N/A since the combo does not target on 2 bands UL CA

5.34	CA_n7-n8

5.34.1	Common for 1 band UL and 2 bands UL CA

5.34.1.1	Operating bands for CA

Table 5.34.1.1-1:  CA band combination of band n7+n8

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | TDD |
| n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |

5.34.1.2	Channel bandwidths per operating band for CA

Table 5.34.1.2-1: Supported bandwidths per CA band combination of band n7+n8

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n7A-n8A | CA_n7A-n8A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n8 | 5, 10, 15, 20 |  |

Note that CA_n7A-n8A 1UL/2DL CA had been introduced in Rel.17 38.101-1, related sections for 1UL/2DL CA can be found in TR 38.717-02-01 section 6.89.

5.34.2	Specific for 2 bands UL CA

5.34.2.1	Maximum output power for inter-band CA

Table 5.34.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n7A-n8A | 23 | +2/-3 |

5.34.2.2	UE co-existence studies

Table 5.34.2.2-1 lists Band n7 + Band n8 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.34.2.2-1: Band n7 and Band n8 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 880 | 915 |
| Two tone 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1690 | 1585 | 3380 | 3485 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4085 | 4260 | 810 | 670 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5880 | 6055 | 4260 | 4400 |
| Two-tone 4th order IMD products | ¦3*fx_low - fy_high¦ | ¦3*fx_high - fy_low¦ | ¦3*fy_low - fx_high¦ | ¦3*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 6585 | 6830 | 70 | 245 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8380 | 8625 | 5140 | 5315 |
| Two-tone 4th order IMD products | ¦2*fx_low - 2*fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 3170 | 3380 | 6760 | 6970 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1160 | 950 | 9400 | 9085 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 6020 | 6230 | 10880 | 11195 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2255 | 2500 | 5950 | 5670 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7640 | 7885 | 9260 | 9540 |

Based on Table 5.34.2.2-1, the 5th order IMD may also fall into Rx frequencies of bands n8.

However, following the analysis for DC_8_n7 in TR 37.717 section 6.1.1, it was proposed not to specify IMD5 as the impact is low.

Table 5.34.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.34.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n7-n8 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 NR Band n100, n101 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 42, 43, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 4, 7, 18 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 7, 18 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 18 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 in TS 36.101 [4] and Table 6.5.3.1-2 in TS 38.101-1 [2] are permitted for each assigned carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [4] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 18: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. . |  |  |  |  |  |  |  |

5.34.2.3	REFSENS requirements

No additional MSD requirement for IMD is needed.

5.35	  CA_n3-n75

5.35.1	Common for 1 band UL and 2 bands UL CA

5.35.1.1	Operating bands for CA

Table 5.35.1.1-1:  CA band combination of band n3+n7

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n75 | NA |  |  | 1432 MHz | – | 1517 MHz | SDL |

5.35.1.2	Channel bandwidths per operating band for CA

Table 5.35.1.2-1: Supported bandwidths per CA band combination of band n3+n7

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n75A | - | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n75 | n75 channel bandwidths in Table 5.3.5-1 |  |

5.35.1.3	UE co-existence studies

The harmonic / harmonic mixing issue have been analyzed in previous release.

5.35.1.4	∆TIB,c and ∆RIB,c values

The TIB,c and RIB have been analyzed in previous BCS 0.

5.35.1.5	REFSENS requirements

For CA_n3A-n75A, there are no harmonics or harmonic mixing issues. Therefore, MSD due to harmonic or harmonic mixing is not needed. Here we discuss whether cross band isolation issue needs to be considered for this band combination.

Based on the cross band isolation analysis in R4-2304218, the MSD exception for CA_n3-n75 are specified below.

Table 5.35.5-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n3 | n75 | 1712.5 | 5 | 15 | 25 (RBstart=0) | 1515.5 | 5 | 4.3 | >ACLR2 |

5.35.1.6	OOB blocking exception requirements

There is no OOB blocking exception requirements

5.36	CA_n3-n7

5.36.1	Common for 1 band UL and 2 bands UL CA

5.36.1.1	Operating bands for CA

Table 5.36.1.1-1:  CA band combination of band n3+n7

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |

5.36.1.2	Channel bandwidths per operating band for CA

Table 5.36.1.2-1: Supported bandwidths per CA band combination of band n3+n7

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n7A | CA_n3A-n7A | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 |  |

5.36.1.3	UE co-existence studies

The harmonic / harmonic mixing issue have been analyzed in previous release.

5.36.1.4	∆TIB,c and ∆RIB,c values

The TIB,c and RIB,c have been specified in current spec.

5.36.1.5	REFSENS requirements

For CA_n3A-n7A, there are no harmonics or harmonic mixing issues. Therefore, MSD due to harmonic or harmonic mixing is not needed. Since n7 is very close to Wi-Fi spectrum, the filter needs to provide enough rejection to Wi-Fi. This may impact the filter rejection to n3. However, in comparison to the n41 RX filter operating within the 2496-2690 MHz, the n7 RX filter operates within the 2620-2690 MHz which is significantly distant from the n3 TX transmission frequency of 1710-1785 MHz. As a result, it is easier for the n7 RX filter to achieve high suppression at the n3 transmission frequency, compared to the n41 filter that operates closer to the n3 TX frequency range.

Commercially available n7 duplexer has the attenuation for band n3 TX frequency range better than 40dB. The cross band isolation MSD have been defined in the Table 7.3A.6-1 within the current TS38.101-1，and the same MSD value can be applied to BCS 4 and 5.

5.36.1.6	OOB blocking exception requirements

There is no OOB blocking exception requirements

5.37	CA_n46-n77

5.37.1	Common for 1 band UL and 2 bands UL CA

5.37.1.1 Operating bands for CA

Table 5.37.1.1-1: CA band combination CA_n46-n77

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n46-n771,6 | n46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz |  |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. NOTE 6: The PCell is allocated in the licensed band in this combination. |  |  |  |  |  |  |  |  |

5.37.1.2	Channel bandwidths per operating band for CA

Table 5.37.1.2-1: Supported bandwidths per CA band combination CA_n46-n77

| NR CA Configuration | UL CA Configuration | NR Band | Channel bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n46A-n77A | CA_n46A-n77A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n46C-n77A | CA_n46A-n77A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n46D-n77A | CA_n46A-n77A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n46A-n77(2A) | CA_n77(2A) CA_n46A-n77A | n46 | 10, 20, 40, 60, 80 | 0 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n46C-n77(2A) | CA_n77(2A) CA_n46A-n77A | n46 | CA_n46C_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n46D-n77(2A) | CA_n77(2A) CA_n46A-n77A | n46 | CA_n46D_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |
| CA_n46(2A)-n77A | CA_n46A-n77A | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n46(2A)-n77(2A) | CA_n77(2A) CA_n46A-n77A | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n77 | CA_n77(2A)_BCS4 and 5 |  |

5.37.1.3	UE Co-existence studies

Table 5.37.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n46-n77.

Table 5.37.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n46 | 5150 | 5925 | 5150 | 5925 | 10300 | 11850 | 15450 | 17775 | 20600 | 23700 | 25750 | 29625 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above table, there is no harmonic interference issue.

Table 5.37.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n46 | 5150 | 5925 | 5150 | 5925 | 10300 | 11850 | 15450 | 17775 | 20600 | 23700 | 25750 | 29625 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

Based on above tables, the following harmonic mixing issues could happen:

- harmonic mixing of n46 DL H2 with n77 UL H3

- harmonic mixing of n77 DL H3 with n46 UL H2

- harmonic mixing of n46 DL H3 with n77 UL H4

- harmonic mixing of n77 DL H4 with n46 UL H3

- harmonic mixing of n46 DL H4 with n77 UL H5

- harmonic mixing of n77 DL H5 with n46 UL H4

5.37.1.4	∆TIB and ∆RIB values

For CA_n46-n77, the TIB,c and RIB are reused from CA_n46-n78 and given in the tables below.

Table 5.37.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n46-n77 | - | 0.8 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.37.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n46-n77 | - | 0.5 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.37.1.5	REFSENs requirements

The MSD due to cross band isolation are reused from CA_n46-n78.

Table 5.37.1.5-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n46 | n77 | 5190 | 80 | 30 | 216 (RBstart=0) | 3975 | 10 | 10.5 | >ACLR2 |
| n46 | n77 | 5190 | 80 | 30 | 216 (RBstart=0) | 3930 | 100 | 5.5 | >ACLR2 |

The MSD for band n77 due to harmonic mixing of UL2/DL3 is reused from CA_n46-n78. The MSD for band n77 due to UL3/DL4 or UL4/DL5 are not specified.

Table 5.37.1.5-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n46 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 19.5 | NOTE 2 | UL2/DL3 |
| n46 | n77 | 20 | 15 | 100 (RBstart=0) | 100 | 12 | NOTE 2 | UL2/DL3 |

For harmonic mixing issues that affect band n46, it is proposed to define the exclusion regions as below, since band n46 is a NR-U band. The harmonic mixing due to UL4/DL3 or UL5/DL4 are not specified.

The reference sensitivity for the shared access band does not apply when there is at least one individual RE within the shared access downlink transmission bandwidth which falls into the reference sensitivity exclusion region as specified in Table 5.37.1.5-3.

Table 5.37.1.5-3: NR-U reference sensitivity measurement exclusion region in MHz.

| NR Band / Harmonic order / Channel BW (MHz) in UL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL Band | UL/DL Harmonic Order | DL Band | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |
| n77 | UL3/DL2 | n46 |  | +/-23 | +/-35 | +/-45 | +/-60 | +/-70 | +/-90 | +/-115 | +/-140 | +/-160 | +/-180 | +/-210 | +/-230 |
| NOTE 1: Even though UL harmonic does not fall directly into NR-U band the exclusion region still applies. NOTE 2: The center of the exclusion region is obtained by multiplying the UL channel center frequency by the ratio of the UL harmonic order over the DL harmonic order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

5.37.2		Specific for 2 bands UL CA

5.37.2.1	Maximum output power for inter-band CA

Table 5.37.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n46-n77 | 23 | +2/-3 |

5.37.2.2		UE co-existence studies

Table 5.37.2.2-1 IMD interference analysis for CA_n46-n77 with 2 ULs.

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3300 | 4200 | 5150 | 5925 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 6600 | 8400 | 10300 | 11850 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 9900 | 12600 | 15450 | 17775 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 13200 | 16800 | 20600 | 23700 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 16500 | 21000 | 25750 | 29625 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 950 | 2625 | 8450 | 10125 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 675 | 3250 | 6100 | 8550 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 11750 | 14325 | 13600 | 16050 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3975 | 7450 | 11250 | 14475 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 15050 | 18525 | 18750 | 21975 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 5250 | 1900 | 16900 | 20250 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 20400 | 16400 | 11650 | 7275 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 11175 | 7050 | 2300 | 1950 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 23900 | 27900 | 18350 | 22725 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 22050 | 26175 | 20200 | 24450 |

For band combination CA_n46-n77, no IMD interference will affect own Rx because both bands are TDD.

Table 5.37.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.37.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected Band | Frequency range (Mhz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n46-n77 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 41, 53, 54, 65, 66, 70, 71, 74, 85, 103 NR Band n100, n101, n105 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n104 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |  |  |  |  |  |  |  |

5.37.2.2.1	UE co-existence studies for Uplink Intra-Band Non-Contiguous CA

When CA_n77(2A) is configured as UL, the potential IMD interference is calculated as below.

Table 5.37.2.2.1-1: Co-existence studies for Uplink Intra-Band Non-Contiguous CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data | 10 | 10 | 600 | 3300 | 4200 | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency | 3300 | 3310 | 3900 | 3980 | 3970 | 3380 |
| 2nd | I fU1L-fU2LI | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H | - | - |
| Interference ranges | 10 | 600 | 6610 | 7950 | - | - |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H | - | - |
| Interference ranges | 2700 | 4580 | 9910 | 11930 | - | - |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Interference ranges | 20 | 1200 | 6000 | 8560 | 13210 | 15910 |
| 5th | I 3*fUL1-2*fU3L I | I 3*fU1H-2*fU3H I | 4*fU1L-fU3L | 4*fU1H-fU3H | 4*fU1L+fU2L | 4*fU1H+fU2H |
| Interference ranges | 2100 | 5180 | 9300 | 12540 | 16510 | 19890 |
| 6th | I 3*fU1L-3*fU2L I | I 3*fU1H-3*fU3H I | 4*fU1L-2*fU3L | 4*fU1H-2*fU3H | 5*fU1L-fU3L | 5*fU1H-fU3H |
| Interference ranges | 30 | 1800 | 5400 | 9160 | 12600 | 16520 |
| 7th | I 4*fU1L-3*fU3L I | I 4*fU1H-3*fU3H I | 5*fU1L-2*fU3L | 5*fU1H-2*fU3H | 6*fU1L-fU3L | 6*fU1H-fU3H |
| Interference ranges | 1500 | 5780 | 8700 | 13140 | 15900 | 20500 |

It can be seen that there are potential IMD5, IMD6 and IMD7 interference falling into band n46 DL.

5.37.2.3		REFSENS requirements

Based on the co-existence analysis, there is potential IMD interference caused by UL CA_n77(2A) into band n46. For NR-U bands, MSD is not applicable. Hence, “N/A” is used for the MSD value for band n46 below.

Table 5.37.2.3-1: MSD due to IMD issue

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode |  |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
|  | n46 | N/A | 10 | N/A | 5155 | N/A | TDD | IMD5 |
|  | n7712 | 3385 | 10 | 1 (RBSTART= 25) | 3385 | N/A | TDD | N/A |
|  |  | 3975 | 10 | 1 (RBSTART= 25) | 3975 |  |  |  |
| CA_n46-n77 | n46 | N/A | 10 | N/A | 5660 | N/A | TDD | IMD6 |
|  | n7712 | 3310 | 10 | 1 (RBSTART= 25) | 3310 | N/A | TDD | N/A |
|  |  | 3790 | 10 | 1 (RBSTART= 25) | 3790 |  |  |  |
|  | n46 | N/A | 10 | N/A | 5230 | N/A | TDD | IMD7 |
|  | n7712 | 3310 | 10 | 1 (RBSTART= 25) | 3310 | N/A | TDD | N/A |
|  |  | 3790 | 10 | 1 (RBSTART= 25) | 3790 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. |  |  |  |  |  |  |  |  |

5.38	CA_n2-n41

5.38.1	Common for 1 band UL and 2 bands UL CA

5.38.1.1		Operating bands for CA

Table 5.38.1.1-1: CA band combination of band n2 and n41

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n2 | 1850MHz | – | 1910 MHz | 1930MHz | – | 1990 MHz | FDD |
| n41 | 2496MHz | – | 2690MHz | 2496MHz | – | 2690MHz | TDD |

5.38.1.2		Channel bandwidths per operating band for CA

Table 5.38.1.2-1: Supported bandwidths per CA band combination of band n2+n41

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n2A-n41A | CA_n2A-n41A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n2(2A)-n41A | CA_n2A-n41A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |

5.38.1.3		UE co-existence studies

Table 5.38.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n2-n41. It is shown: There are no harmonics and harmonics mixing issues for CA_ n2-n41.

Table 5.38.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n2 | 1850 | 1910 | 1930 | 1990 | 3700 | 3820 | 5550 | 5730 | 7400 | 7640 | 9250 | 9550 |
| n41 | 2469 | 2690 | 2469 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 | 12480 | 13450 |

Table 5.38.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n2 | 1850 | 1910 | 1930 | 1990 | 3860 | 3980 | 5790 | 5970 | 7720 | 7960 | 9650 | 9950 |
| n41 | 2469 | 2690 | 2469 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 | 12480 | 13450 |

5.38.1.4		∆TIB and ∆RIB values

For CA_n2-n41, the TIB,c and RIB,c values are reused from DC_2_n41 as specified in TS 38.101-1 and are given in the tables below.

Table 5.38.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n2-n41 | 0.5 | 0.45/0.96 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. NOTE 5: The requirement is applied for UE transmitting on the frequency range of 2545 - 2690 MHz. NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2496 - 2545 MHz. |  |  |

Table 5.38.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n2-n41 | - | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.38.1.5 		REFSENS requirements

As the co-existence studies in 5.38.1.3 show, there is no harmonics and harmonic mixing issues, so no MSD is required.

Since the UE RF architecture for CA_n2-n41 is similar to DC_2-n41, MSD due to cross band isolation defined for DC_2-n41 is reused.

Table 5.38.1.5-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n41 | n2 | 2546 | 100 | 30 | 270 (RBstart=0) | 1987.5 | 5 | 0.6 | >ACLR2 |

5.38.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

5.38.2		Specific for 2 bands UL CA

5.38.2.1	Maximum output power for inter-band CA

Table 5.38.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n2A-n41A | 23 | +2/-3 |

5.38.2.2		UE co-existence studies

Table 5.38.2.2-1 lists Band n2 + Band n41 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.38.2.2-1: Band n5 and Band n41 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 3700 | 3820 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 5550 | 5730 | 7488 | 8070 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 7400 | 7640 | 9984 | 10760 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 9250 | 9550 | 12480 | 13450 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 11100 | 11460 | 14976 | 16140 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 12950 | 13370 | 17472 | 18830 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 840 | 586 | 4346 | 4600 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1010 | 1324 | 3082 | 3530 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6196 | 6510 | 6842 | 7290 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1680 | 1172 | 8692 | 9200 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2860 | 3234 | 5578 | 6220 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8046 | 8420 | 9338 | 9980 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8910 | 8074 | 5144 | 4710 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11834 | 12670 | 9896 | 10330 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4370 | 3668 | 738 | 170 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11188 | 11890 | 10542 | 11110 |

Based on the table above, it can be seen there is no IMD issues.

Table 5.38.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.38.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n2-n41 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 42, 50, 51, 54, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 43, 48 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

5.38.2.3		REFSENS requirements

Based on the co-existence study in 5.38.2.2, there is no IMD issues, and no MSD is required.

5.39	CA_n5-n71

5.39.1	Common for 1 band UL and 2 bands UL CA

5.39.1.1		Operating bands for CA

Table 5.39.1.1-1: CA band combination of band n5 and n71

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

5.39.1.2		Channel bandwidths per operating band for CA

Table 5.39.1.2-1: Supported bandwidths per CA band combination of band n5+n71

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n71A | - | n5 | 5, 10, 15, 20 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |

5.39.1.3		UE co-existence studies

Table 5.39.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n5-n71. It is shown that there are no harmonic issues to consider.

Table 5.39.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 | 4120 | 4245 |
| n71 | 663 | 698 | 617 | 652 | 1326 | 1396 | 1989 | 2094 | 2652 | 2792 | 3315 | 3490 |

Table 5.39.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 | 4345 | 4470 |
| n71 | 663 | 698 | 617 | 652 | 1234 | 1304 | 1851 | 1956 | 2468 | 2608 | 3085 | 3260 |

5.39.1.4		∆TIB and ∆RIB values

For CA_n5-n71, the TIB,c and RIB,c values are same as for DC_5_n71and are given in the tables below.

Table 5.39.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n5-n71 | 0.5 | 0.5 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.39.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n5-n71 | - | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.39.1.5 		REFSENS requirements

Reference sensitivity exceptions due to cross band isolation are specified for CA_n5-n71 in Table 5.39.1.5-1.

Table 5.39.1.5-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n5 | 688 | 20 | 15 | 20 (RBstart=86) | 871.5 | 5 | 2.0 | >ACLR2 |
| n5 | n71 | 834 | 20 | 15 | 20 (RBstart=0) | 649.5 | 5 | 3.9 | >ACLR2 |

5.39.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

Table 5.39.1.6-1: CA band combination with exceptions allowed

CA band combination

5.40	CA_n5-n41

5.40.1	Common for 1 band UL and 2 bands UL CA

5.40.1.1		Operating bands for CA

Table 5.40.1.1-1: CA band combination of band n5 and n41

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
| n41 | 2496 MHz | – | 2690MHz | 2496 MHz | – | 2690MHz | TDD |

5.40.1.2		Channel bandwidths per operating band for CA

Table 5.40.1.2-1: Supported bandwidths per CA band combination of band n5+n41

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n41A | CA_n5A-n41A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |

5.40.1.3		UE co-existence studies

Table 5.40.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n5-n41. It is shown:

-	There is a 3rd order harmonic of band 5 UL in band n41

-     There is a 3rd order harmonic mixing of band 5 DL in band n41

Table 5.40.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 | 4120 | 4245 |
| n41 | 2469 | 2690 | 2469 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 | 12480 | 13450 |

Table 5.40.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 | 4345 | 4470 |
| n41 | 2469 | 2690 | 2469 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 | 12480 | 13450 |

5.40.1.4		∆TIB and ∆RIB values

For CA_n5-n41, the TIB,c and RIB,c values are same as for DC_5_n41and are given in the tables below.

Table 5.40.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n5-n41 | 0.6 | 0.3 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.40.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n5-n41 | 0.2 | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.40.1.5 		REFSENS requirements

As the co-existence studies in 5.40.1.3 show, there are 3rd order harmonics and 3rd harmonic mixing issues and MSD is required. MSD values are reused from DC_5A_n41A.

Table 5.40.1.5-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n5 | n411 | 5 | 15 | 16 (RBstart=0) | 10 | 10.3 | NOTE 3 | UL3/DL1 direct-hit |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 3: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |

Table 5.40.1.5-3: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n41 | n5 | 10 | 15 | 25 (RBstart=0) | 5 | 24.3 | NOTE 4 | UL1/DL3 |
| NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in |  |  |  |  |  |  |  |  |

5.40.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

5.40.2		Specific for 2 bands UL CA

5.40.2.1	Maximum output power for inter-band CA

Table 5.40.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n5A-n41A | 23 | +2/-3 |

5.40.2.2		UE co-existence studies

Table 5.40.2.2-1 lists Band n5 + Band n41 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.40.2.2-1: Band n5 and Band n41 UL harmonics and IMD products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 824 | 849 | 2496 | 2690 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1866 | 1647 | 3320 | 3539 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1042 | 798 | 4143 | 4556 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4144 | 4388 | 5816 | 6229 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3732 | 3294 | 6640 | 7078 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 218 | 51 | 6639 | 7246 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4968 | 5237 | 8312 | 8919 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9936 | 9135 | 900 | 606 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10808 | 11609 | 5792 | 6086 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6422 | 5790 | 2445 | 2908 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9136 | 9768 | 7464 | 7927 |

Based on the table above it can be seen that:

3rd and 5th order IMD products may fall into Rx frequencies of band n5.

5th order IMD products may fall into Rx frequencies of bands n41.

Table 5.40.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.40.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n5-n41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13, 14, 17, 24, 25, 26, 28, 29, 30, 34, 40, 42, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 52, 54 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

5.40.2.3		REFSENS requirements

Based on the co-existence study in 5.40.2.2, MSD is required. MSD values are reused from DC_5_n41.

Table 5.40.2.3-1: MSD test points for PCell due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| CA_n5A_n41A | n5 | 839 | 5 | 25 | 884 | 15.6 | FDD | IMD33 |
|  | n41 | 2562 | 10 | 50 | 2562 | N/A | TDD | N/A |
| NOTE 3: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.41	CA_n2-n71

5.41.1	Common for 1 band UL and 2 bands UL CA

5.41.1.1		Operating bands for CA

Table 5.41.1.1-1: CA band combination of band n2 and n71

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n2 | 1850MHz | – | 1910 MHz | 1930MHz | – | 1990 MHz | FDD |
| n71 | 663MHz | – | 698MHz | 617MHz | – | 652MHz | FDD |

5.41.1.2		Channel bandwidths per operating band for CA

Table 5.41.1.2-1: Supported bandwidths per CA band combination of band n2+n71

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n2A-n71A | CA_n2A-n71A | n2 | 5, 10, 15, 20 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n2(2A)-n71A | CA_n2A-n71A | n2 | CA_n2(2A)_BCS0 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |

5.41.1.3		UE co-existence studies

Table 5.41.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n2-n71. It is shown:

- 3rd order harmonic of Band n71 Tx may fall into the own Rx Band of Band n2.

- 3rd order harmonic mixing for Band n71

Table 5.41.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n2 | 1850 | 1910 | 1930 | 1990 | 3700 | 3820 | 5550 | 5730 | 7400 | 7640 | 9250 | 9550 |
| n71 | 663 | 698 | 617 | 652 | 1326 | 1396 | 1989 | 2094 | 2652 | 2792 | 3315 | 3490 |

Table 5.41.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n2 | 1850 | 1910 | 1930 | 1990 | 3860 | 3980 | 5790 | 5970 | 7720 | 7960 | 9650 | 9950 |
| n71 | 663 | 698 | 617 | 652 | 1234 | 1304 | 1851 | 1956 | 2468 | 2608 | 30851 | 3260 |

5.41.1.4		∆TIB and ∆RIB values

For CA_n2-n71, the TIB,c and RIB,c values are reused from CA_n25_n71 as specified in TS 38.101-1 and are given in the tables below.

Table 5.41.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n2-n71 | 0.3 | 0.6 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.41.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n2-n71 | - | 0.3 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.41.1.5 		REFSENS requirements

As the co-existence studies in 5.41.1.3 show, there is 3rd order harmonics and 3rd order harmonic mixing and MSD is required. MSD values are reused from CA_n25-n71.

Table 5.41.1.5-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n71 | n210 | 5 | 15 | 8 (RBstart=0) | 5 | 10 | NOTE 3 | UL3/DL1 direct-hit |
| NOTE 3: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 10: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz |  |  |  |  |  |  |  |  |

Table 5.41.1.5-3: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n2 | n713 | 5 | 15 | 25 (RBstart=0) | 5 | 26.5 | NOTE 4 | UL1/DL3 |
| NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in band. NOTE 3: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. |  |  |  |  |  |  |  |  |

5.41.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

5.41.2		Specific for 2 bands UL CA

5.41.2.1	Maximum output power for inter-band CA

Table 5.41.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n2A-n71A | 23 | +2/-3 |

5.41.2.2		UE co-existence studies

Table 5.41.2.2-1 lists Band n2 + Band n71 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.41.2.2-1: Band n5 and Band n41 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 1850 | 1910 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits (MHz) | 1326 | 1396 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 5550 | 5730 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 7400 | 7640 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 9250 | 9550 |
| 6th harmonics frequency limits | 6* fy_low | 6* fy_high | 6*fx_low | 6*fx_high |
| 6th harmonics frequency limits (MHz) | 3978 | 4188 | 11100 | 11460 |
| 7th harmonics frequency limits | 7* fy_low | 7* fy_high | 7*fx_low | 7*fx_high |
| 7th harmonics frequency limits (MHz) | 4641 | 4886 | 12950 | 13370 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1247 | 1152 | 2513 | 2608 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 584 | 454 | 3002 | 3157 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3176 | 3306 | 4363 | 4518 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2494 | 2304 | 5026 | 5216 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 79 | 244 | 4852 | 5067 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3839 | 4004 | 6213 | 6428 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6977 | 6702 | 942 | 742 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8063 | 8338 | 4502 | 4702 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4404 | 4154 | 1606 | 1831 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6876 | 7126 | 5689 | 5914 |

Based on the table above, it can be seen there is no IMD issues.

Table 5.41.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.41.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n2-n71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 54, 66 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 25, 41, 48, 70, NR Band n2, n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 71 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 5 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

5.41.2.3		REFSENS requirements

Based on the co-existence study in 5.41.2.2, there is no IMD issues and no MSD is required.

5.42	CA_n7-n12

5.42.1	Common for 1 band UL and 2 bands UL CA

5.42.1.1		Operating bands for CA

Table 5.42.1.1-1: CA band combination of band n7 and n12

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n7 | 2500MHz | – | 2570 MHz | 2620MHz | – | 2690 MHz | FDD |
| n12 | 699MHz | – | 716MHz | 729MHz | – | 746MHz | FDD |

5.42.1.2		Channel bandwidths per operating band for CA

Table 5.42.1.2-1: Supported bandwidths per CA band combination of band n7+n12

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n7A-n12A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n12 | 5, 10, 15 |  |

5.42.1.3		UE co-existence studies

Table 5.42.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n7-n12. It is shown:

There are no harmonic and harmonic mixing issues for CA_n7A-n12A combination.

Table 5.42.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5000 | 5140 | 7500 | 7710 | 10000 | 10280 | 12500 | 12850 |
| n12 | 699 | 716 | 729 | 746 | 1398 | 1432 | 2097 | 2148 | 2796 | 2864 | 3495 | 3580 |

Table 5.42.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 | 10480 | 10760 | 13100 | 13450 |
| n12 | 699 | 716 | 729 | 746 | 1458 | 1492 | 2187 | 2238 | 2918 | 2984 | 3645 | 3730 |

5.42.1.4		∆TIB and ∆RIB values

For CA_n7-n12, the TIB,c and RIB,c values are reused from CA_7-12 as specified in TS 36.101 and are given in the tables below.

Table 5.42.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n7-n12 | 0.3 | 0.3 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.42.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n7-n12 | - | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.42.1.5 		REFSENS requirements

As the co-existence studies in 5.42.1.3 show, there are no harmonics and harmonic mixing issues. So, there is no need to define any MSD values.

5.42.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

5.43	CA_n7-n71

5.43.1	Common for 1 band UL and 2 bands UL CA

5.43.1.1		Operating bands for CA

Table 5.43.1.1-1: CA band combination of band n7 and n71

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n7 | 2500MHz | – | 2570 MHz | 2620MHz | – | 2690 MHz | FDD |
| n71 | 663MHz | – | 698MHz | 617MHz | – | 652MHz | FDD |

5.43.1.2		Channel bandwidths per operating band for CA

Table 5.43.1.2-1: Supported bandwidths per CA band combination of band n7+n71

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n7A-n71A | - | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
| CA_n7A-n71A | CA_n7A-n71A | n7 | See n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 |  |

5.43.1.3		UE co-existence studies

Table 5.43.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n7-n71. It is shown:

- 4th order harmonic of Band n71 Tx may fall into the own Rx Band of Band n7.

- 4th order harmonic mixing for Band n71

Table 5.43.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5000 | 5140 | 7500 | 7710 | 10000 | 10280 | 12500 | 12850 |
| n71 | 663 | 698 | 617 | 652 | 1326 | 1396 | 1989 | 2094 | 2652 | 2792 | 3315 | 3490 |

Table 5.43.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 | 10480 | 10760 | 13100 | 13450 |
| n71 | 663 | 698 | 617 | 652 | 1234 | 1304 | 1851 | 1956 | 2468 | 2608 | 30851 | 3260 |

5.43.1.4		∆TIB and ∆RIB values

For CA_n7-n71, the TIB,c and RIB,c values are reused from DC_7_n71 as specified in TS 38.101-1 and are given in the tables below.

Table 5.43.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n7-n71 | 0.6 | 0.3 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.43.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n7-n71 | 0.2 | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.43.1.5 		REFSENS requirements

As the co-existence studies in 5.43.1.3 show, there are 4th order harmonic and 4th order harmonic mixing, so MSD is required. MSD values are reused from DC_7_n71 and CA_n5-n77.

Table 5.43.1.5-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n71 | n71 | 5 | 15 | 8 (RBstart=0) | 5 | 14.6 | NOTE 4 | UL4/DL1 direct-hit |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

Table 5.43.1.5-3: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n7 | n71 | 5 | 15 | 25 (RBstart=0) | 5 | 5.7 | NOTE 8 | UL1/ DL4 |
| NOTE 8: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

5.43.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

### 5.43.2		Specific for 2 bands UL CA

#### 5.43.2.1	Maximum output power for inter-band CA

Table 5.43.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n7A-n71A | 23 | +2/-3 |

#### 5.43.2.2		UE co-existence studies

Table 5.43.2.2-1 lists Band n7 + Band n71 2UL bands CA 2nd, 3rd, 4th and 5th order IMD analysis.

Table 5.43.2.2-1: Band n7 and Band n71 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 2500 | 2570 | 663 | 698 |
| 2nd order IMD products | ¦fx_low – fy_high¦ | ¦fx_high – fy_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1802 | 1907 | 3163 | 3268 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4302 | 4477 | 1104 | 1244 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5663 | 5838 | 3826 | 3966 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6802 | 7047 | 406 | 581 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8163 | 8408 | 4489 | 4664 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3604 | 3814 | 6326 | 6536 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 82 | 292 | 9302 | 9617 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5152 | 5362 | 10663 | 10978 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2906 | 3151 | 6104 | 6384 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6989 | 7234 | 8826 | 9106 |

Based on the table above it can be seen that there is no IMD due to two UL falls into own DL.

#### 5.43.2.3		REFSENS requirements

Based on co-existence studies on 5.43.2.2, no need to define exceptional REFSENS requirements.

5.44	CA_n12-n41

5.44.1	Common for 1 band UL and 2 bands UL CA

5.44.1.1		Operating bands for CA

Table 5.44.1.1-1: CA band combination of band n12 and n41

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n12 | 699 MHz | – | 819 MHz | 729 MHz | – | 746 MHz | FDD |
| n41 | 2496 MHz | – | 2690MHz | 2496 MHz | – | 2690MHz | TDD |

5.44.1.2		Channel bandwidths per operating band for CA

Table 5.44.1.2-1: Supported bandwidths per CA band combination of band n12+n41

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n12A-n41A | - | n12 | 5, 10, 15 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |

5.44.1.3		UE co-existence studies

Table 5.44.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n12-n41. It is shown: There are no harmonics and harmonics mixing issues.

Table 5.44.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n12 | 699 | 819 | 729 | 746 | 1 398 | 1 638 | 2 097 | 2 457 | 2 796 | 3 276 | 3 495 | 4 095 |
| n41 | 2469 | 2690 | 2469 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 | 12480 | 13450 |

Table 5.44.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n12 | 699 | 819 | 729 | 746 | 1 458 | 1 492 | 2 187 | 2 238 | 2 916 | 2 984 | 3 645 | 3 730 |
| n41 | 2469 | 2690 | 2469 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 | 12480 | 13450 |

5.44.1.4		∆TIB and ∆RIB values

For CA_n12-n41, the TIB,c and RIB,c values are same as for DC_12_n41and are given in the tables below.

Table 5.44.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n12-n41 | 0.3 | 0.3 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.44.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n12-n41 | - | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.44.1.5 		REFSENS requirements

As the co-existence studies in 5.44.1.3 show, there are no harmonics and harmonic mixing issues, and MSD is not required.

5.44.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

5.45	CA_n12-n78

5.45.1	Common for 1 band UL and 2 bands UL CA

5.45.1.1		Operating bands for CA

Table 5.45.1.1-1: CA band combination of band n12 and n78

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

5.45.1.2		Channel bandwidths per operating band for CA

Table 5.45.1.2-1: Supported bandwidths per CA band combination of band n12+n78

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n12A-n78A | CA_n12A-n78A | n12 | 5, 10, 15, 20 | 0 |
|  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n12A-n78(2A) | CA_n12A-n78A | n12 | 5, 10, 15, 20 | 0 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |

5.45.1.3		UE co-existence studies

Table 5.45.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n12-n78. It is shown:

- 5th order harmonic of Band n12 Tx may fall into the own Rx Band of Band n78.

- 5th order harmonic mixing for Band n12

Table 5.45.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n12 | 699 | 716 | 729 | 746 | 1398 | 1432 | 2097 | 2148 | 2796 | 2864 | 3495 | 3580 |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 | 16500 | 1900 |

Table 5.45.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n12 | 699 | 716 | 729 | 746 | 1458 | 1492 | 2187 | 2238 | 2916 | 2984 | 3645 | 3730 |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 | 16500 | 1900 |

5.45.1.4		∆TIB and ∆RIB values

For CA_n12-n78, the TIB,c and RIB,c values are reused from DC_12_n78 as specified in TS 38.101-3 and are given in the tables below.

Table 5.45.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n12-n78 | 0.5 | 0.8 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.45.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n12-n78 | 0.2 | 0.5 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.45.1.5 		REFSENS requirements

As the co-existence studies in 5.45.1.3 show, there is 5th order harmonics and 5th order harmonic mixing, and MSD is required. MSD values are reused from CA_n77-n85.

Table 5.45.1.5-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n12 | n781 | 5 | 15 | 10 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 5: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

Table 5.45.1.5-3: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n12 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 5 | UL1/DL5 |
| NOTE 5: The requirements should be verified for DL NR-ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |

5.45.1.6	OOB blocking exception requirements

Exceptions added for CA_n12-n78

Table 5.23.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n12-n78

5.45.2		Specific for 2 bands UL CA

5.45.2.1	Maximum output power for inter-band CA

Table 5.45.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n12A-n78A | 23 | +2/-3 |

5.45.2.2		UE co-existence studies

Table 5.45.2.2-1 lists Band n12 + Band n78 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.45.2.2-1: Band n5 and Band n41 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1398 | 1432 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 16500 | 19000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 2584 | 3101 | 3999 | 4516 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 2402 | 1868 | 5884 | 6901 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 4698 | 5232 | 7299 | 8316 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1703 | 1152 | 9184 | 10701 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 5397 | 5948 | 10599 | 12099 |
| Two-tone 4th order IMD products | 2*fy_low – 2*fx_high | 2*fy_high – 2*fx_low | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 5168 | 6202 | 7998 | 9032 |
| Two-tone 5th order IMD products | ¦4*fx_low – fy_high¦ | ¦4*fx_high – fy_low¦ | 4*fy_low – fx_high | 4*fy_high – fx_low |
| IMD frequency limits (MHz) | 1004 | 436 | 12484 | 14501 |
| Two-tone 5th order IMD products | 4*fx_low + fy_low | 4*fx_high + fy_high | 4*fy_low + fx_low | 4*fy_high + fx_high |
| IMD frequency limits (MHz) | 6096 | 6664 | 13899 | 15916 |
| Two-tone 5th order IMD products | ¦3*fx_low – 2*fy_high¦ | ¦3*fx_high – 2*fy_low¦ | 3*fy_low – 2*fx_high | 3*fy_high – 2*fx_low |
| IMD frequency limits (MHz) | 5503 | 4452 | 8468 | 10002 |
| Two-tone 5th order IMD products | 2*fx_low + 3*fy_low | 2*fx_high + 3*fy_high | 2*fy_low + 3*fx_low | 2*fy_high + 3*fx_high |
| IMD frequency limits (MHz) | 11298 | 12832 | 8697 | 9748 |

Based on the table above, it can be seen 5th order IMD products may fall into the own Rx Band of Band n12.

Table 5.45.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

Table 5.45.2.2-2: Protected bands for the 2UL bands CA configuration

| NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n12-n78 | E-UTRA Band 2, 5, 7. 13, 17, 25, 26, 41, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 66 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 12 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180 kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

5.45.2.3		REFSENS requirements

Based on the co-existence study in 5.45.2.2, MSD is required. The MSD value for DC_12A_n78A due to IMD5 could reuse the MSD value for DC_28A_n78A,

Table 6.1.63.4-3: Reference sensitivity exceptions for PCell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n12-n78 | 12 | 710 | 5 | 25 | 740 | 5.5 | IMD5 |
|  | n78 | 3580 | 10 | 50 | 3580 | N/A | N/A |

5.46		CA_n1-n105

5.46.1	Common for 1 band UL and 2 bands UL CA

5.46.1.1 Operating bands for CA

Table 5.46.1.1-1: CA band combination of band n1+n105

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

5.46.1.2	Channel bandwidths per operating band for CA

Table 5.46.1.2-1: Supported bandwidths per CA band combination of band n1+n105

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n105A | CA_n1A-n105A | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

5.46.1.3	Co-existence studies

Table 5.46.1.3-1/2 summarizes frequency ranges where UL harmonics and/or harmonic mixing occur for CA_n1-n105.

Table 5.46.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 3840 | 3960 | 5760 | 5940 | 7680 | 7920 | 9600 | 9900 |
| n105 | 663 | 703 | 612 | 652 | 1326 | 1406 | 1989 | 2109 | 2652 | 2812 | 3315 | 3515 |

Based on above table, there is no UL harmonic issues.

Table 5.46.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 4220 | 4340 | 6330 | 6510 | 8440 | 8680 | 10550 | 10850 |
| n105 | 663 | 703 | 612 | 652 | 1224 | 1304 | 1836 | 1956 | 2448 | 2608 | 3060 | 3260 |

Based on above table, the UL n1 falls inside the 3rd order harmonic of n105 RX band (n1 – n105 UL1/DL3).

5.46.1.4	∆TIB and ∆RIB values

For CA_n1-n105, the TIB,c and the RIB are based on CA_n25-n71.

Table 5.46.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n1-n105 | 0.3 | 0.6 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.46.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n1-n105 | - | 0.3 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.46.1.5	REFSENs requirements

Based on the co-existence studies n1 uplink and n105 third harmonic mixes into the n105 downlink. The value is based on DC_1A_n71A, which has similar harmonic issue.

Table 5.46.1.5-2: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n1 | n1053 | 5 | 15 | 25 (RBstart=0) | 5 | 26.8 | NOTE 4 | UL1/DL3 |
| NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

5.46.1.6	OOB blocking exception requirements

No additional OOB blocking exceptions are required for this CA band combination.

5.46.2		Specific for 2 bands UL CA

5.46.2.1	Maximum output power for inter-band CA

Table 5.46.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n1A-n105A | 23 | +2/-3 |

5.46.2.2		UE co-existence

Table 5.46.2.2-1 gives IMD interference analysis for CA_ n1-n105 with uplink CA.

Table 5.46.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1920 | 1980 | 663 | 703 |
| DL Frequency [MHz] | 2110 | 2170 | 612 | 652 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1317 | 1217 | 2583 | 2683 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3137 | 3297 | 654 | 514 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4503 | 4663 | 3246 | 3386 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5057 | 5277 | 9 | 189 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2434 | 2634 | 5166 | 5366 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6423 | 6643 | 3909 | 4089 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 892 | 672 | 7257 | 6977 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1731 | 1971 | 4614 | 4354 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4572 | 4792 | 8343 | 8623 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5829 | 6069 | 7086 | 7346 |

Based on the table 5.46.2.2-1, the 3rd order intermodulation product fall inside the RX band of n105.

Table 5.46.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n105 | E-UTRA Band 5, 26 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | 4, 6 |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | 4, 6, 7 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 4, 6, 7 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 6: This requirement is applicable for any channel bandwidths within the range 1920 – 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 – 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band |  |  |  |  |  |  |  |

5.46.2.3		REFSENS requirements

Table 5.46.2.3-1 lists the MSD required due to the 3rd IMD product for the dual uplink configuration. It is based on the similar case of DC_1A_n71A.

Table 5.46.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n105 | n1 | 1958 | 5 | 25 | 2148 | N/A | FDD | N/A |
|  | n105 | 673 | 5 | 25 | 622 | 15.1 | FDD | IMD3 |

5.47		CA_n3-n105

5.47.1	Common for 1 band UL and 2 bands UL CA

5.47.1.1 Operating bands for CA

Table 5.47.1.1-1: CA band combination of band n3+n105

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

5.47.1.2	Channel bandwidths per operating band for CA

Table 5.47.1.2-1: Supported bandwidths per CA band combination of band n3+n105

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n3A-n105A | CA_n3A-n105A | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

5.47.1.3	Co-existence studies

Table 5.47.1.3-1/2 summarizes frequency ranges where UL harmonics and/or harmonic mixing occur for CA_n3-n105.

Table 5.47.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3420 | 3570 | 5130 | 5355 | 6840 | 7140 | 8550 | 8925 |
| n105 | 663 | 703 | 612 | 652 | 1326 | 1406 | 1989 | 2109 | 2652 | 2812 | 3315 | 3515 |

Based on above table, there is no UL harmonic issues.

Table 5.47.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3610 | 3760 | 5415 | 5640 | 7220 | 7520 | 9025 | 9400 |
| n105 | 663 | 703 | 612 | 652 | 1224 | 1304 | 1836 | 1956 | 2448 | 2608 | 3060 | 3260 |

Based on above table, there is no UL harmonic mixing issues.

5.47.1.4	∆TIB and ∆RIB values

For CA_n3-n105, the TIB,c and the RIB are based on CA_n25-n71.

Table 5.47.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n3-n105 | 0.3 | 0.6 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.47.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n3-n105 | - | 0.3 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.47.1.5	REFSENs requirements

There’s no MSD exception needed.

5.47.1.6	OOB blocking exception requirements

There’s no MSD exception needed.

5.47.2		Specific for 2 bands UL CA

5.47.2.1	Maximum output power for inter-band CA

Table 5.47.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n3A-n105A | 23 | +2/-3 |

5.47.2.2		UE co-existence

Table 5.47.2.2-1 gives IMD interference analysis for CA_ n3-n105 with uplink CA.

Table 5.47.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1710 | 1785 | 663 | 703 |
| DL Frequency [MHz] | 1805 | 1880 | 612 | 652 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1122 | 1007 | 2373 | 2488 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2717 | 2907 | 459 | 304 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4083 | 4273 | 3036 | 3191 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4427 | 4692 | 204 | 399 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2014 | 2244 | 4746 | 4976 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5793 | 6058 | 3699 | 3894 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1102 | 867 | 6477 | 6137 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1311 | 1581 | 4029 | 3724 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4362 | 4597 | 7503 | 7843 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5409 | 5679 | 6456 | 6761 |

Based on the table 5.47.2.2-1, there is no intermodulation product issues.

Table 5.47.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n3-n105 | E-UTRA Band 5, 26 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -41 | 0.3 | 3 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

5.47.2.3		REFSENS requirements

There’s no MSD exception needed.

5.48		CA_n40-n105

5.48.1	Common for 1 band UL and 2 bands UL CA

5.48.1.1 Operating bands for CA

Table 5.48.1.1-1: CA band combination of band n40+n105

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
| n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

5.48.1.2	Channel bandwidths per operating band for CA

Table 5.48.1.2-1: Supported bandwidths per CA band combination of band n40+n105

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n40A-n105A | CA_n40A-n105A | n40 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

5.48.1.3	Co-existence studies

Table 5.48.1.3-1/2 summarizes frequency ranges where UL harmonics and/or harmonic mixing occur for CA_n7-n105.

Table 5.48.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n40 | 2300 | 2400 | 2300 | 2400 | 4600 | 4800 | 6900 | 7200 | 9200 | 9600 | 11500 | 12000 |
| n105 | 663 | 703 | 612 | 652 | 1326 | 1406 | 1989 | 2109 | 2652 | 2812 | 3315 | 3515 |

Based on above table, there is no uplink harmonic issue of CA_n40-n105

Table 5.48.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n40 | 2300 | 2400 | 2300 | 2400 | 4600 | 4800 | 6900 | 7200 | 9200 | 9600 | 11500 | 12000 |
| n105 | 663 | 703 | 612 | 652 | 1224 | 1304 | 1836 | 1956 | 2448 | 2608 | 3060 | 3260 |

Based on above table, there is no harmonic mixing issue of CA_n40-n105

5.48.1.4	∆TIB and ∆RIB values

For CA_n40-n105, the TIB,c and the RIB are based on CA_n41-n71.

Table 5.48.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n40-n105 | 0.3 | 0.6 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.48.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n40-n105 | - | 0.2 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.48.1.5	REFSENs requirements

There’s no MSD exception needed.

5.48.1.6	OOB blocking exception requirements

There’s no OOB exception needed.

5.48.2		Specific for 2 bands UL CA

5.48.2.1	Maximum output power for inter-band CA

Table 5.48.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n40A-n105A | 23 | +2/-3 |

5.48.2.2		UE co-existence

Table 5.48.2.2-1 gives IMD interference analysis for CA_ n40-n105.

Table 5.48.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 2300 | 2400 | 663 | 703 |
| DL Frequency [MHz] | 2300 | 2400 | 612 | 652 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1737 | 1597 | 2963 | 3103 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3897 | 4137 | 1074 | 894 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5263 | 5503 | 3626 | 3806 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6197 | 6537 | 411 | 191 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3194 | 3474 | 5926 | 6206 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7563 | 7903 | 4289 | 4509 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 512 | 252 | 8937 | 8497 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2491 | 2811 | 5874 | 5494 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4952 | 5212 | 9863 | 10303 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6589 | 6909 | 8226 | 8606 |

Based on the table 5.48.2.2-1, there is no IMD issues of this combination.

Table 5.48.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n40-n105 | E-UTRA Band 5, 26 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2,44 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 44: As exceptions, for 90 and 100 MHz channel bandwidth, -40 dBm/MHz is applicable in the frequency range of 2496 – 2505 MHz. |  |  |  |  |  |  |  |

5.48.2.3		REFSENS requirements

There are no additional MSD requirements for this band combination.

## 5.49		CA_n78-n105

5.49.1	Common for 1 band UL and 2 bands UL CA

5.49.1.1 Operating bands for CA

Table 5.49.1.1-1: CA band combination of band n78+n105

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
| n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

5.49.1.2	Channel bandwidths per operating band for CA

Table 5.49.1.2-1: Supported bandwidths per CA band combination of band n78+n105

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n78A-n105A | CA_n78A-n105A | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

5.49.1.3	Co-existence studies

Table 5.49.1.3-1/2 summarizes frequency ranges where UL harmonics and/or harmonic mixing occur for CA_n78-n105.

Table 5.49.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 | 16500 | 19000 |
| n105 | 663 | 703 | 612 | 652 | 1326 | 1406 | 1989 | 2109 | 2652 | 2812 | 3315 | 3515 |

Based on above table, there is a 5th order uplink harmonic issue of CA_n78-n105

Table 5.49.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 | 16500 | 19000 |
| n105 | 663 | 703 | 612 | 652 | 1224 | 1304 | 1836 | 1956 | 2448 | 2608 | 3060 | 3260 |

Based on above table, there is no harmonic mixing issue of CA_n78-n105

5.49.1.4	∆TIB and ∆RIB values

For CA_n78-n105, the TIB,c and the RIB are based on CA_n71-n78.

Table 5.49.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n78-n105 | 0.8 | 0.5 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.49.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n78-n105 | 0.5 | 0.2 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.49.1.5	REFSENs requirements

Based on the co-existence studies n105 uplink harmonic hits n78 downlink. The value is based on DC_71A_n78A, which has similar harmonic issue.

Table 5.49.1.5-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n105 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier |  |  |  |  |  |  |  |  |

5.49.1.6	OOB blocking exception requirements

Exceptions added for CA_n78-n105

Table 5.49.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n78-n105

5.49.2		Specific for 2 bands UL CA

5.49.2.1	Maximum output power for inter-band CA

Table 5.49.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n78A-n105A | 23 | +2/-3 |

5.49.2.2		UE co-existence

Table 5.49.2.2-1 gives IMD interference analysis for CA_ n78-n105.

Table 5.49.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 3300 | 3800 | 663 | 703 |
| DL Frequency [MHz] | 3300 | 3800 | 612 | 652 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3137 | 2597 | 3963 | 4503 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 5897 | 6937 | 2474 | 1894 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7263 | 8303 | 4626 | 5206 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 9197 | 10737 | 1811 | 1191 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 5194 | 6274 | 7926 | 9006 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10563 | 12103 | 5289 | 5909 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 488 | 1148 | 14537 | 12497 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4491 | 5611 | 10074 | 8494 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5952 | 6612 | 13863 | 15903 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8589 | 9709 | 11226 | 12806 |

Based on the table 5.49.2.2-1, there is 5th order IMD issue in n105 of this combination.

Table 5.49.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n78-n105 | E-UTRA Band 5, 26 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

5.49.2.3		REFSENS requirements

Table 5.49.2.3-1 lists the MSD required due to the 5th IMD product for the dual uplink configuration. It is based on the similar case of CA_n71A-n78A.

Table 5.49.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n78-n105 | n78 | 3361.5 | 10 | 50 | 3361.5 | N/A | TDD | N/A |
|  | n105 | 682.5 | 5 | 25 | 631.5 | 5.5 | FDD | IMD5 |

5.50		CA_n7-n105

5.50.1	Common for 1 band UL and 2 bands UL CA

5.50.1.1 Operating bands for CA

Table 5.50.1.1-1: CA band combination of band n7+n105

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| n105 | 663 MHz | – | 703 MHz | 612 MHz | – | 652 MHz | FDD |

5.50.1.2	Channel bandwidths per operating band for CA

Table 5.50.1.2-1: Supported bandwidths per CA band combination of band n7+n105

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n7A-n105A | CA_n7A-n105A | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n105 | 5, 10, 15, 20, 25, 30, 35 |  |

5.50.1.3	Co-existence studies

Table 5.50.1.3-1/2 summarizes frequency ranges where UL harmonics and/or harmonic mixing occur for CA_n7-n105.

Table 5.50.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5000 | 5140 | 7500 | 7710 | 10000 | 10280 | 12500 | 12850 |
| n105 | 663 | 703 | 612 | 652 | 1326 | 1406 | 1989 | 2109 | 2652 | 2812 | 3315 | 3515 |

Based on above table, there is 4th order UL harmonic of n105 falling inside n7 RX band.

Table 5.50.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 | 10480 | 10760 | 13100 | 13450 |
| n105 | 663 | 703 | 612 | 652 | 1224 | 1304 | 1836 | 1956 | 2448 | 2608 | 3060 | 3260 |

Based on above table, there is no harmonic mixing issue of CA_n7-n105

5.50.1.4	∆TIB and ∆RIB values

For CA_n7-n105, the TIB,c and the RIB are based on DC_7_n71.

Table 5.50.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n7-n105 | 0.3 | 0.6 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.50.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n7-n105 | - | 0.2 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.50.1.5	REFSENs requirements

Based on the co-existence studies n105 uplink harmonic hits n7 downlink. The value is based on DC_7A_n71A, which has similar harmonic issue.

Table 5.50.1.5-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n105 | n7 | 5 | 15 | 25 (RBstart=0) | 5 | 14.6 | NOTE 3 | UL3/DL1 direct-hit |
| NOTE 3: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with the carrier |  |  |  |  |  |  |  |  |

5.50.1.6	OOB blocking exception requirements

Exceptions added for CA_n7-n105

Table 5.50.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n7-n105

5.50.2		Specific for 2 bands UL CA

5.50.2.1	Maximum output power for inter-band CA

Table 5.50.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n7A-n105A | 23 | +2/-3 |

5.50.2.2		UE co-existence

Table 5.50.2.2-1 gives IMD interference analysis for CA_ n7-n105.

Table 5.50.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 2500 | 2570 | 663 | 703 |
| DL Frequency [MHz] | 2620 | 2690 | 612 | 652 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1907 | 1797 | 3163 | 3273 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4297 | 4477 | 1244 | 1094 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5663 | 5843 | 3826 | 3976 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6797 | 7047 | 581 | 391 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3594 | 3814 | 6326 | 6546 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8163 | 8413 | 4489 | 4679 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 312 | 82 | 9617 | 9297 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2891 | 3151 | 6384 | 6094 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5152 | 5382 | 10663 | 10983 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6989 | 7249 | 8826 | 9116 |

Based on the table 5.50.2.2-1, there is no IMD issues of this combination.

Table 5.50.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n7-n105 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 30, 66, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 4 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 4, 7, 18 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 4, 7, 18 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 4, 18 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band NOTE 18: This requirement is applicable for any channel bandwidths within the range 2500 – 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 – 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |

5.50.2.3		REFSENS requirements

There are no additional MSD requirements for this band combination.

5.51		CA_n3-n102

5.51.1	Common for 1 band UL and 2 bands UL CA

5.51.1.1 Operating bands for CA

Table 5.51.1.1-1: CA band combination of band n3+n102

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| n102 | 5925 MHz | – | 6425 MHz | 5925 MHz | – | 6425 MHz | TDD |

5.51.1.2	Channel bandwidths per operating band for CA

Table 5.51.1.2-1: Supported bandwidths per CA band combination of band n3+n102

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n3A-n102A | CA_n3A-n102A | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n3A-n102(2A) | CA_n3A-n102A | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n3A-n102B | CA_n3A-n102A CA_n3A-n102B | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n3A-n102C | CA_n3A-n102A CA_n3A-n102C | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n3A-n102D | CA_n3A-n102A | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n3A-n102E | CA_n3A-n102A | n3 | 5, 10, 15, 20, 25, 30, 40, 45, 50 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |

5.51.1.3	Co-existence studies

Table 5.51.1.3-1/2 summarizes frequency ranges where UL harmonics and/or harmonic mixing occur for CA_n3-n102.

Table 5.51.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3420 | 3570 | 5130 | 5355 | 6840 | 7140 | 8550 | 8925 |
| n102 | 5925 | 6425 | 5925 | 6425 | 11850 | 12850 | 17775 | 19275 | 23700 | 25700 | 29625 | 32125 |

Based on the table above there is no UL harmonic issues of this combination.

Table 5.51.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n3 | 1710 | 1785 | 1805 | 1880 | 3610 | 3760 | 5415 | 5640 | 7220 | 7520 | 9025 | 9400 |
| n102 | 5925 | 6425 | 5925 | 6425 | 11850 | 12850 | 17775 | 19275 | 23700 | 25700 | 29625 | 32125 |

Based on the table above there is no Harmonic mixing issues of this combination.

5.51.1.4	∆TIB and ∆RIB values

For CA_n3-n102, the TIB,c is based on CA_n3-n79 having n79 as the highest frequency range among similar CA’s. For CA_n3-n102, the RIB values are also based on CA_n3-n79

Table 5.51.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n3-n102 | 0.3 | 0.8 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.51.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n3-n102 | - | 0.5 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.51.1.5	REFSENs requirements

As can be seen in the co-existence studies in 5.51.1.3 there are no harmonics issues.

5.51.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

5.51.2		Specific for 2 bands UL CA

5.51.2.1	Maximum output power for inter-band CA

Table 5.51.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n3A-n102A | 23 | +2/-3 |
| CA_n3A-n102B | 23 | +2/-3 |
| CA_n3A-n102C | 23 | +2/-3 |

5.51.2.2		UE co-existence

Table 5.51.2.2-1 gives IMD interference analysis for CA_ n3-n102 with uplink CA.

Table 5.51.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1710 | 1785 | 5925 | 6425 |
| DL Frequency [MHz] | 1805 | 1880 | 5925 | 6425 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4140 | 4715 | 7635 | 8210 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3005 | 2355 | 10065 | 11140 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 9345 | 9995 | 13560 | 14635 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1295 | 570 | 15990 | 17565 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 9430 | 8280 | 15270 | 16420 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 11055 | 11780 | 19485 | 21060 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 23990 | 21915 | 1215 | 415 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 15855 | 14205 | 6495 | 7720 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 25410 | 27485 | 12765 | 13565 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 21195 | 22845 | 16980 | 18205 |

Based on the table 5.51.2.2-1 there are no intermodulation products falling inside the RX bands.

Since this is including a share spectrum access band there is no additional protected bands as compared to n3.

Table 5.51.2.2-3 gives an IMD interference analysis for CA_n3-n102 with uplink CA for triple beat using the method of [1].

A triple beat study having n102C transmit in two 20MHz blocks with a 160MHz separation indicates IMD3.

Table 5.51.2.2-3: Triple beat IMD analysis of n102C

| CC location | fU1L | fU2L | fU3L | fU1H |  | Channel BW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 5925 | 5965 | 6085 | 6425 |  | 20 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min ch. separation |
| Frequency | 1710 | 1785 | 6385 | 6265 |  | 20 |
| 1st order TB | IfU3L -fU1L- fSCCL¦ | IfU2L -fU1L + fSCCL¦ | IfU2L -fU1L- fSCCH¦ | IfU3L -fU1L + fSCCH¦ |  | Max ch. separation |
| Ranges | 1550 | 1750 | 1745 | 1945 |  | 160 |
| 1st order TB | IfU2L+fU1L-fSCCH¦ | IfU1H+fU2H-fSCCL¦ | IfU2L +fU1L+fSCCL¦ | IfU1H +fU2H+fSCCH¦ |  |  |
| Ranges | 9910 | 10890 | 13810 | 14790 |  |  |

The uplink of n3A and n102C has 1st order triple beat falling inside n3 DL.

5.51.2.3		REFSENS requirements

As stated for figure 1 in [2] the MSD value is not eligible if the two UL inter-bands aren’t part of the same or adjacent band group.

5.52		CA_n77-n102

5.52.1	Common for 1 band UL and 2 bands UL CA

5.52.1.1 Operating bands for CA

Table 5.52.1.1-1: CA band combination of band n77+n102

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
| n102 | 5925 MHz | – | 6425 MHz | 5925 MHz | – | 6425 MHz | TDD |

5.52.1.2	Channel bandwidths per operating band for CA

Table 5.52.1.2-1: Supported bandwidths per CA band combination of band n77+n102

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n77A-n102A | CA_n77A-n102A | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n77A-n102(2A) | CA_n77A-n102A | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n77A-n102B | CA_n77A-n102A CA_n77A-n102B | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n77A-n102C | CA_n77A-n102A CA_n77A-n102C | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n77A-n102D | CA_n77A-n102A | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n77A-n102E | CA_n77A-n102A | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n77(2A)-n102A | CA_n77(2A) CA_n77A-n102A | n77 | CA_n77(2A)_BCS4&5 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n77(2A)-n102(2A) | CA_n77(2A) CA_n77A-n102A | n77 | CA_n77(2A)_BCS4&5 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n77(2A)-n102B | CA_n77(2A) CA_n77A-n102A CA_n77A-n102B | n77 | CA_n77(2A)_BCS4&5 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n77(2A)-n102C | CA_n77(2A) CA_n77A-n102A CA_n77A-n102C | n77 | CA_n77(2A)_BCS4&5 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n77(2A)-n102D | CA_n77(2A) CA_n77A-n102A | n77 | CA_n77(2A)_BCS4&5 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n77(2A)-n102E | CA_n77(2A) CA_n77A-n102A | n77 | CA_n77(2A)_BCS4&5 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |

5.52.1.3	Co-existence studies

Table 5.52.1.3-1/2 summarizes frequency ranges where UL harmonics and/or harmonic mixing occur for CA_n77-n102.

Table 5.52.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |
| n102 | 5925 | 6425 | 5925 | 6425 | 11850 | 12850 | 17775 | 19275 | 23700 | 25700 | 29625 | 32125 |

Based on the table above there is no UL harmonic issues of this combination.

Table 5.52.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |
| n102 | 5925 | 6425 | 5925 | 6425 | 11850 | 12850 | 17775 | 19275 | 23700 | 25700 | 29625 | 32125 |

Based on the table above there is no Harmonic mixing issues of this combination.

5.52.1.4	∆TIB and ∆RIB values

For CA_n77-n102, the TIB,c is based on CA_n78-n102. For CA_n77-n102, the RIB values are also based on CA_n78-n102.

Table 5.31.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n77-n102 | 1.5 | 1.5 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.31.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n77-n102 | 0.5 | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.52.1.5	REFSENs requirements

As can be seen in the co-existence studies in 5.52.1.3 there are no harmonics issues.

5.52.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

5.52.2		Specific for 2 bands UL CA

5.52.2.1	Maximum output power for inter-band CA

Table 5.52.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n77A-n102A | 23 | +2/-3 |
| CA_n77A-n102B | 23 | +2/-3 |
| CA_n77A-n102C | 23 | +2/-3 |

5.52.2.2		UE co-existence

Table 5.52.2.2-1 gives IMD interference analysis for CA_ n77-n102 with uplink CA.

Table 5.52.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 3300 | 4200 | 5925 | 6425 |
| DL Frequency [MHz] | 3300 | 4200 | 5925 | 6425 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1725 | 3125 | 9225 | 10625 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 175 | 2475 | 7650 | 9550 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 12525 | 14825 | 15150 | 17050 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3475 | 6675 | 13575 | 15975 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6250 | 3450 | 18450 | 21250 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 15825 | 19025 | 21075 | 23475 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 22400 | 19500 | 10875 | 6775 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 12675 | 9375 | 750 | 2950 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 27000 | 29900 | 19125 | 23225 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 24375 | 27675 | 21750 | 25450 |

Based on the table 5.52.2.2-1 there are no intermodulation products falling inside the RX bands.

Since n102 is located as shared spectrum there is simultaneous RX/TX in this TDD-TDD combination, which requires an analysis of non-contiguous uplink operation. The method of [1] is applied. For the non-contiguous intra-band ULCA of n77(2A) the impact on n102 RX is analysed using the method of [1].

Table 5.31.2.3-2: IMD analysis non-contiguous UL in n77

| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |  | Channel BW |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 | 3340 | 3900 | 4200 | 4160 | 3600 |  | 20 |
| 2nd | I fU1L-fU2L I | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H |  |  |  | Min Ch. separation |
| Interference ranges | 40 | 600 | 6640 | 8360 |  |  |  | 20 |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |  | Max. Ch. separation |
| Interference ranges | 2700 | 4800 | 9940 | 12560 |  |  |  | 600 |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |  |  |
| Interference ranges | 80 | 1200 | 6000 | 9000 | 13240 | 16760 |  |  |
| 5th | I 3*fUL1-2*fU3L I | I 3*fUH1-2*fU3H I | 4*fUL1-fU3L | 4*fUH1-fU3H | 4*fUL1+fU2L | 4*fUH1+fU2H |  |  |
| Interference ranges | 2100 | 5400 | 9300 | 13200 | 16540 | 20960 |  |  |
| 6th | I 3*fUL1-3*fU2L I | I 3*fUH1-3*fU3H I | 4*fUL1-2*fU3L | 4*fUH1-2*fU3H | 5*fUL1-fU3L | 5*fUH1-fU3H |  |  |
| Interference ranges | 120 | 1800 | 5400 | 9600 | 12600 | 17400 |  |  |
| 7th | I 4*fUL1-3*fU3L I | I 4*fUH1-3*fU3H I | 5*fUL1-2*fU3L | 5*fUH1-2*fU3H | 6*fUL1-fU3L | 6*fUH1-fU3H |  |  |
| Interference ranges | 1500 | 6000 | 8700 | 13800 | 15900 | 21600 |  |  |

The analysis shows that the 4th, 6th and 7th order IMD product of n77(2A) falls inside n102 during simultaneous RX/TX operation. However, 38.101-1 does not capture IMD into unlicensed or shared spectrum bands. No need to define MSD.

Since this is including a share spectrum access band there is no additional protected bands as compared to n77.

As this is a TDD-TDD combination the triple beat analysis for CA_n77A-n102B and CA_n77A-n102C will never impact any downlink of the two bands, since there’s no triple beat uplink if either band is in downlink operation.

5.52.2.3		REFSENS requirements

No additional MSD requirement for IMD is needed.

  - 5.53		CA_n8-n79

    - 5.53.1	Common for 1 band UL and 2 bands UL CA

      - 5.53.1.1 Operating bands for CA

Table 5.53.1.1-1: CA band combination CA_n8-n79

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n8-n79 | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

      - 5.53.1.2	Channel bandwidths per operating band for CA

Table 5.53.1.2-1: Supported bandwidths per CA band combination CA_n8-n79

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n79C | CA_n8A-n79A CA_n79C | n8 | 5, 10, 15, 20 | 0 |
|  |  | n79 | CA_n79C_BCS0 |  |
|  | CA_n8A-n79C | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |

      - 5.53.1.3	UE Co-existence studies

The harmonic / harmonic mixing issue of UL CA_n8A-n79A have been analyzed in previous release.

For UL CA_n79C, the IMD order can be calculated as below in table as below.

Table 5.53.1.3-1: IMD issue caused by CA_n79C

| Combs | Victim_band (contains single carrier) | ULCA for CA_band | maxUL agg. BW (IMD BW) | Victim_BW_max | Victim_band duplexer | Triple beat condition | issue (0/1) | (Wgap) Aggressor-Victim distance | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n8A_n79C | n8 | n79 | 200 | 20 | 45 | 210 | 1 | 3440 | 37 |

It can be seen that there exist the IMD issue caused by UL CA_n79C falls into band n8 DL, in which the IMD order is 37.

In terms of the previous agreements in the WF R4-2105336, there is no need to analysis the IMD order larger than 13, hence there is no need to define the IMD37 MSD requirements.

      - 5.53.1.4	∆TIB and ∆RIB values

The TIB,c and RIB have been specified in current spec.

      - 5.53.1.5	REFSENs requirements

According to the co-existence study, no additional MSD requirements should be defined.

      - 5.53.1.6	OOB blocking exception requirements

The OOB blocking exception requirements for band combination of CA_n8-n79 have already been included in the spec.

    - 5.53.2		Specific for 2 bands UL CA

      - 5.53.2.1	Maximum output power for inter-band CA

Table 5.53.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n8A-n79C | 23 | +2/-3 |

      - 5.53.2.2		UE co-existence studies

In this section, we only focus on the triple beat IMDs caused by UL CA_n8A-n79C. For double beat IMDs caused by CA_n8A-n79A, it was already covered in the co-existence studies of the fallback configurations of DL CA_n8A-n79A with UL single carrier or UL CA_n8A-n79A.

The analysis of the triple beat IMD is found in Table 5.53.2.3-1:

Table 5.53.2.3-1: Band n8 and Band n79 triple beat IMD products

| CC location | fU1L | fU2L | fU3L | fU1H |  | CBW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 4400 | 4480 | 4600 | 5000 |  | 40 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min ch. separation |
| Frequency | 880 | 915 | 4920 | 4800 |  | 40 |
| 1st order TB | IfU3L -fU1L- fSCCL¦ | IfU2L -fU1L + fSCCL¦ | IfU2L -fU1L- fSCCH¦ | IfU3L -fU1L + fSCCH¦ |  | Max ch. separation |
| Ranges | 680 | 960 | 835 | 1115 |  | 200 |
| 1st order TB | IfU2L+fU1L-fSCCH¦ | IfU1H+fU2H-fSCCL¦ | IfU2L +fU1L+fSCCL¦ | IfU1H +fU2H+fSCCH¦ |  |  |
| Ranges | 7965 | 9040 | 9760 | 10835 |  |  |

It can be seen that 1st order TB IMDs would fall into band n8 DL Rx. However, according to the following agreements in the WF R4-2220556:

For two-band inter-band CA or DC combinations, the precondition is that:

- Only FDD-FDD and TDD-FDD combination need to be considered with 3 uplink (UL) component carriers (CC) in two clusters within FR1. The decision tree in Figure 1 provides guidelines to decide if TB detection analysis is needed.

- The 2 UL bands are part of the same band group or belong to adjacent band groups as defined in Table 1.

In the other words, only same band group or adjacent band group TB IMDs issues shall be considered.

For band n8 and n79, band n8 belongs to FR1-1(LB) group while band n79 belongs to FR1-4(VHB), which means there is no need to define the additional TB IMDs requirements.

      - 5.53.2.3		REFSENS requirements

Based on the co-existence , although there are IMD issues, there is no need to define the additional REFSEN exception requirements.

  - 5.54		CA_n78-n79

    - 5.54.1	Common for 1 band UL and 2 bands UL CA

      - 5.54.1.1 Operating bands for CA

Table 5.54.1.1-1: CA band combination of band n78+n79

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
| n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

      - 5.54.1.2	Channel bandwidths per operating band for CA

Table 5.54.1.2-1: Supported bandwidths per CA band combination of band n78+n79

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n78A-n79A | n78A n79A CA_n78A-n79A | n78 | See n78 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |
| CA_n78(2A)-n79A | CA_n78A-n79A | n78 | CA_n78(2A)_BCS4 and 5 | 4 and 5 |
|  |  | n79 | See n79 channel bandwidths in Table 5.3.5-1 |  |

      - 5.54.1.3	REFSENS exception requirements

Since new minimum channel bandwidth 10MHz was introduced in Rel-18, the new MSD test point due to cross band isolation for band n79 10MHz can be introduced as below for BCS4 and 5 cases to overwrite the case with DL band n79 100MHz.

Table 5.54.1.3-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n783 | n79 | 3750 | 100 | 30 | 270 (RBstart=3) | 4405 | 10 | 2 | >ACLR2 |
| NOTE 3: The requirements only apply for UEs supporting inter-band carrier aggregation with simultaneous Rx/Tx capability. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |  |  |  |  |  |  |  |

5.55		CA_n5-n78

5.55.1		Specific for 1 bands UL CA

5.55.1.1 Operating bands for CA

CA_n5-n78 has already been specified and this section does not need to be revisited.

5.55.1.2	Channel bandwidths per operating band for CA

UL CA_n78(2A) need to be added to DL CA_n5A-n78(2A) in the existing configuration table of 38.101-1. See Table 5.55.1-1 below.

Table 5.55.1.2-2: Supported bandwidths per CA band combination of band n5+n78

| CA_n5A-n78(2A) | CA_n78(2A) CA_n5A-n78A | n5 | 5, 10, 15, 20 | 0 |
| --- | --- | --- | --- | --- |
|  |  | n78 | CA_n78(2A)_BCS2 |  |
|  |  | n5 | See n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |
| CA_n5A-n78C | CA_n78C CA_n5A-n78C | n5 | See n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78C_BCS4 and 5 |  |

5.55.1.3	Co-existence studies

CA_n5-n78 has already been specified and this section does not need to be revisited.

Table 5.55.1.3-1 gives a non-contiguous uplink IMD interference analysis

Table 5.55.1.3-1: Non-contiguous uplink IMD analysis

| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 | 3320 | 3800 | 3800 | 3780 | 3300 |
| 2nd | I fU1L-fU2L I | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 20 | 500 | 6620 | 7580 |  |  |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 2800 | 4300 | 9920 | 11380 |  |  |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Interference ranges | 40 | 1000 | 6100 | 8100 | 13220 | 15180 |
| 5th | I 3*fUL1-2*fU3L I | I 3*fUH1-2*fU3H I | 4*fUL1-fU3L | 4*fUH1-fU3H | 4*fUL1+fU2L | 4*fUH1+fU2H |
| Interference ranges | 2300 | 4800 | 9400 | 11900 | 16520 | 18980 |
| 6th | I 3*fUL1-3*fU2L I | I 3*fUH1-3*fU3H I | 4*fUL1-2*fU3L | 4*fUH1-2*fU3H | 5*fUL1-fU3L | 5*fUH1-fU3H |
| Interference ranges | 60 | 1500 | 5600 | 8600 | 12700 | 15700 |
| 7th | I 4*fUL1-3*fU3L I | I 4*fUH1-3*fU3H I | 5*fUL1-2*fU3L | 5*fUH1-2*fU3H | 6*fUL1-fU3L | 6*fUH1-fU3H |
| Interference ranges | 1800 | 5300 | 8900 | 12400 | 16000 | 19500 |

Min channel = 10MHz

Min Channel separation = 10MHz

Max channel separation = 500MHz

Based on the table 5.55.2.2-3, there are IMD4 and IMD6 products falling inside n5 DL, which may occur in simultaneous RX/TX configurations.

MSD for IMD4 is already defined in Table 7.3A.5-1a, but also need to be added for the PA architecture for n78(2A).

MSD for IMD6 is not needed to be added when lower IMD order MSD is specified.

Additionally, the co-existence studies caused by UL CA_n78C should be analyzed for CA_n5A-n78C, which can be found in table Table 5.55.1.3-2:

Table 5.55.1.3-2: Co-existence studies for Uplink Intra-Band Contiguous CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data | 10 | 10 | 200 | 3300 | 3800 | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency | 3300 | 3320 | 3500 | 3800 | 3780 | 3600 |
| 2nd order IMD products | IfU1L-fU2LI | IfU1L-fU3LI | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 20 | 200 | 6620 | 7580 |  |  |
| 3rd order IMD products | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 3100 | 4000 | 9920 | 11380 |  |  |
| 4th order IMD products | I2*fU1L-2*fU2LI | I2*fU1H-2*fU3HI | 3*fU1L-fU3L | 3*fU1H-fU3H | 3*fU1L+fU2L | 3*fU1H+fU2H |
| Interference ranges | 40 | 400 | 6400 | 7800 | 13220 | 15180 |
| 5th order IMD products | I3*fU1L-2*fU3LI | I3*fU1H-2*fU3HI | 4*fU1L-fU3L | 4*fU1H-fU3H | 4*fU1L+fU2L | 4*fU1H+fU2H |
| Interference ranges | 2900 | 4200 | 9700 | 11600 | 16520 | 18980 |
| 6th order IMD products | I3*fU1L-3*fU2LI | I3*fU1H-3*fU3HI | 4*fU1L-2*fU3L | 4*fU1H-2*fU3H | 5*fU1L-fU3L | 5*fU1H-fU3H |
| Interference ranges | 60 | 600 | 6200 | 8000 | 13000 | 15400 |
| 7th order IMD products | I4*fU1L-3*fU3LI | I4*fU1H-3*fU3HI | 5*fU1L-2*fU3L | 5*fU1H-2*fU3H | 6*fU1L-fU3L | 6*fU1H-fU3H |
| Interference ranges | 2700 | 4400 | 9500 | 11800 | 16300 | 19200 |

It can be seen that there are no co-existence issues caused by CA_n78C impact band n5 DL.

5.55.1.4	∆TIB and ∆RIB values

CA_n5-n78 has already been specified and this section does not need to be revisited.

5.55.1.5	REFSENs requirements

CA_n5-n78 has already been specified and this section does not need to be revisited.

5.55.2		Specific for 2 bands UL CA

5.55.2.1	Maximum output power for inter-band CA

Table 5.55.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n5A-n78A | 23 | +2/-3 |

5.55.2.2		UE co-existence

Table 5.55.2.2-1 gives IMD interference analysis for CA_ n5-n78.

Table 5.55.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 824 | 849 | 3300 | 3800 |
| DL Frequency [MHz] | 869 | 894 | 3300 | 3800 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2976 | 2451 | 4124 | 4649 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2152 | 1602 | 5751 | 6776 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4948 | 5498 | 7424 | 8449 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 5952 | 4902 | 8248 | 9298 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1328 | 753 | 9051 | 10576 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5772 | 6347 | 10724 | 12249 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14376 | 12351 | 96 | 504 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14024 | 16049 | 6596 | 7196 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9752 | 8202 | 4053 | 5128 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11548 | 13098 | 9072 | 10147 |

Based on the table 5.55.2.2-1, there are possible IMD4 interference issues into band n5.

MSD is already defined in Table 7.3A.5-1 of TS 38.101-1.

The analysis of the triple beat IMD is found in Table 5.55.2.2-2:

Table 5.55.2.2-2: Band n5 and Band n78 triple beat IMD products

| CC location | fU1L | fU2L | fU3L | fU1H |  | CBW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 | 3320 | 3500 | 3800 |  | 10 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min ch. separation |
| Frequency | 824 | 849 | 3780 | 3600 |  | 10 |
| 1st order TB | IfU3L -fU1L- fSCCL¦ | IfU2L -fU1L + fSCCL¦ | IfU2L -fU1L- fSCCH¦ | IfU3L -fU1L + fSCCH¦ |  | Max ch. separation |
| Ranges | 624 | 844 | 829 | 1049 |  | 200 |
| 1st order TB | IfU2L+fU1L-fSCCH¦ | IfU1H+fU2H-fSCCL¦ | IfU2L +fU1L+fSCCL¦ | IfU1H +fU2H+fSCCH¦ |  |  |
| Ranges | 5771 | 6756 | 7444 | 8429 |  |  |

It can be seen that 1st order TB IMDs would fall into band n5 DL Rx. However, according to the following agreements in the WF R4-2220556:

For two-band inter-band CA or DC combinations, the precondition is that:

- Only FDD-FDD and TDD-FDD combination need to be considered with 3 uplink (UL) component carriers (CC) in two clusters within FR1. The decision tree in Figure 1 provides guidelines to decide if TB detection analysis is needed.

- The 2 UL bands are part of the same band group or belong to adjacent band groups as defined in Table 1.

In the other words, only same band group or adjacent band group TB IMDs issues shall be considered.

For band n5 and n78, band n5 belongs to FR1-1(LB) group while band n78 belongs to FR1-4(VHB), which means there is no need to define the additional TB IMDs requirements.

5.55.2.3		REFSENS requirements

Based on the co-existence studies there is a need to define additional IMD4 MSD value. MSD value is reused from CA_n5-n77.

Table 5.55.2.3-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n5-n78 | n5 | N/A | 5 | N/A | 880 | 8.6 | FDD | IMD4 |
|  | n7812 | 3340 | 10 | 1 RBSTART=25 | 3340 | N/A | TDD | N/A |
|  |  | 3780 | 10 | 1 RBSTART=25 | 3780 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. |  |  |  |  |  |  |  |  |

5.56		CA_n25-n78

5.56.1		Specific for 1 bands UL CA

5.56.1.1 Operating bands for CA

CA_n25-n78 has already been specified and this section does not need to be revisited.

5.56.1.2	Channel bandwidths per operating band for CA

UL CA_n78(2A) is to be added to DL CA_n25A-n78(2A) in the existing configuration table of 38.101-1. See Table 5.56.1-1 below.

Table 5.56.1.2-2: Supported bandwidths per CA band combination of band n25+n78

| CA_n25A-n78(2A) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | CA_n78(2A) | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |

5.56.1.3	Co-existence studies

Table 5.56.1.3-1 gives a non-contiguous uplink IMD interference analysis.

Table 5.56.1.3-1: Non-contiguous uplink IMD analysis

| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 | 3320 | 3800 | 3800 | 3780 | 3300 |
| 2nd | I fU1L-fU2L I | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 20 | 500 | 6620 | 7580 |  |  |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 2800 | 4300 | 9920 | 11380 |  |  |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Interference ranges | 40 | 1000 | 6100 | 8100 | 13220 | 15180 |
| 5th | I 3*fUL1-2*fU3L I | I 3*fUH1-2*fU3H I | 4*fUL1-fU3L | 4*fUH1-fU3H | 4*fUL1+fU2L | 4*fUH1+fU2H |
| Interference ranges | 2300 | 4800 | 9400 | 11900 | 16520 | 18980 |
| 6th | I 3*fUL1-3*fU2L I | I 3*fUH1-3*fU3H I | 4*fUL1-2*fU3L | 4*fUH1-2*fU3H | 5*fUL1-fU3L | 5*fUH1-fU3H |
| Interference ranges | 60 | 1500 | 5600 | 8600 | 12700 | 15700 |
| 7th | I 4*fUL1-3*fU3L I | I 4*fUH1-3*fU3H I | 5*fUL1-2*fU3L | 5*fUH1-2*fU3H | 6*fUL1-fU3L | 6*fUH1-fU3H |
| Interference ranges | 1800 | 5300 | 8900 | 12400 | 16000 | 19500 |

Min channel = 10MHz

Min Channel separation = 10MHz

Max channel separation = 500MHz

Based on the table 5.56.2.2-3, there are IMD7 products falling inside n25 DL, which may occur in simultaneous RX/TX configurations.

5.56.1.4	∆TIB and ∆RIB values

CA_n25-n78 has already been specified and this section does not need to be revisited.

5.56.1.5	REFSENs requirements

Based on the co-existence studies there is a need to define MSD values. MSD value are reused from CA_n2A-n77A with CA_n77(2A) in the UL.

Table 5.56.1.5-1: 3DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n78 | n25 | N/A | 5 | N/A | 1980 | 2.7 | FDD | IMD7 |
|  | n7812 | 3315 | 10 | 1 RBSTART=7 | 3315 | N/A | TDD | N/A |
|  |  | 3760 | 10 | 1 RBSTART=0 | 3760 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. |  |  |  |  |  |  |  |  |

5.57		CA_n66-n78

5.57.1		Specific for 1 bands UL CA

5.57.1.1 Operating bands for CA

CA_n66-n78 has already been specified and this section does not need to be revisited.

5.57.1.2	Channel bandwidths per operating band for CA

UL CA_n78(2A) need to be added to DL CA_n66A-n78(2A) in the existing configuration table of 38.101-1. See Table 5.57.1-1 below.

Table 5.57.1.2-2: Supported bandwidths per CA band combination of band n66+n78

| CA_n66A-n78(2A) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | CA_n78(2A) | n66 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n78 | CA_n78(2A)_BCS2 |  |

5.57.1.3	Co-existence studies

Table 5.57.2.2-3 gives a non-contiguous uplink IMD interference analysis

Table 5.57.2.2-3: Non-contiguous uplink IMD analysis

| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 | 3320 | 3800 | 3800 | 3780 | 3300 |
| 2nd | I fU1L-fU2L I | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 20 | 500 | 6620 | 7580 |  |  |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 2800 | 4300 | 9920 | 11380 |  |  |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Interference ranges | 40 | 1000 | 6100 | 8100 | 13220 | 15180 |
| 5th | I 3*fUL1-2*fU3L I | I 3*fUH1-2*fU3H I | 4*fUL1-fU3L | 4*fUH1-fU3H | 4*fUL1+fU2L | 4*fUH1+fU2H |
| Interference ranges | 2300 | 4800 | 9400 | 11900 | 16520 | 18980 |
| 6th | I 3*fUL1-3*fU2L I | I 3*fUH1-3*fU3H I | 4*fUL1-2*fU3L | 4*fUH1-2*fU3H | 5*fUL1-fU3L | 5*fUH1-fU3H |
| Interference ranges | 60 | 1500 | 5600 | 8600 | 12700 | 15700 |
| 7th | I 4*fUL1-3*fU3L I | I 4*fUH1-3*fU3H I | 5*fUL1-2*fU3L | 5*fUH1-2*fU3H | 6*fUL1-fU3L | 6*fUH1-fU3H |
| Interference ranges | 1800 | 5300 | 8900 | 12400 | 16000 | 19500 |

Min channel = 10MHz

Min Channel separation = 10MHz

Max channel separation = 500MHz

Based on the table 5.57.2.2-3, there are IMD7 products falling inside n66 DL, which may occur in simultaneous RX/TX configurations.

5.57.1.4	∆TIB and ∆RIB values

CA_n66-n78 has already been specified and this section does not need to be revisited.

5.57.1.5	REFSENs requirements

Based on the co-existence studies there is a need to define MSD values. MSD values are reused from CA_n66A-n77A with CA_n77(2A) in the UL.

Table 5.57.1.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n66-n78 | n66 | N/A | 5 | 25 | 2150 | 1.7 | FDD | IMD7 |
|  | n7812 | 3350 | 10 | 1 RBSTART=7 | 3350 | N/A | TDD | N/A |
|  |  | 3750 | 10 | 1 RBSTART=0 | 3750 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. |  |  |  |  |  |  |  |  |

5.58	CA_n26-n29

5.58.1	Common for 1 band UL

5.58.1.1		Operating bands for CA

Table 5.58.1.1-1: CA band combination of band n26 and n29

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
| n29 | N/A | – | N/A | 717 MHz | – | 728 MHz | SDL |

5.58.1.2		Channel bandwidths per operating band for CA

Table 5.58.1.2-1: Supported bandwidths per CA band combination of band n26+n29

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n26A-n29A | - | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n29 | 5, 10 |  |

5.58.1.3		UE co-existence studies

Table 5.58.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n26-n29 shown that there are no harmonic issues to consider.

Table 5.58.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n26 | 814 | 849 | 859 | 894 | 1628 | 1698 | 2442 | 2547 | 3256 | 3396 | 4070 | 4245 |
| n29 |  |  | 717 | 728 |  |  |  |  |  |  |  |  |

Table 5.58.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n26 | 814 | 849 | 859 | 894 | 1718 | 1788 | 2577 | 2682 | 3436 | 3576 | 4295 | 4470 |
| n29 |  |  | 717 | 728 | 1434 | 1456 | 2151 | 2184 | 2868 | 2912 | 3585 | 3640 |

There is cross band isolation interference from UL n66 to DL n29.

5.58.1.4		∆TIB and ∆RIB values

For CA_n26-n29, the TIB,c and RIB,c values are same as for CA_n5-n29 and are given in the tables below.

Table 5.58.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n26-n29 | 0.5 | - |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.58.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n26-n29 | 0.5 | 0.3 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.58.1.5 		REFSENS requirements

As can be seen in the co-existence studies in 5.58.1.3 there are no harmonics issues. The MSD due to cross band isolation are reused from CA_n5-n71.

Table 5.58.1.5-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n26 | n29 | 824 | 20 | 15 | 25 (RBstart=0) | 719.5 | 5 | 3.9 | >ACLR2 |

5.58.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

Table 5.58.1.6-1: CA band combination with exceptions allowed

CA band combination

5.59	CA_n26-n48

5.59.1	Common for 1 band UL and 2 bands UL CA

5.59.1.1		Operating bands for CA

Table 5.59.1.1-1: CA band combination of band n26 and n48

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
| n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |

5.59.1.2		Channel bandwidths per operating band for CA

Table 5.59.1.2-1: Supported bandwidths per CA band combination of band n26+n48

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n26A-n48A | CA_n26A-n48A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
| CA_n26A-n48(2A) | CA_n26A-n48A | n26 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n48 | CA_n48(2A)_BCS0 |  |

5.59.1.3		UE co-existence studies

Table 5.59.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n26-n48 shown that there is 4th harmonic mixing falling into n26 DL.

Table 5.59.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n26 | 814 | 849 | 859 | 894 | 1628 | 1698 | 2442 | 2547 | 3256 | 3396 | 4070 | 4245 |
| n48 | 3550 | 3700 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 | 14200 | 14800 | 17750 | 18500 |

Table 5.59.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n26 | 814 | 849 | 859 | 894 | 1718 | 1788 | 2577 | 2682 | 3436 | 3576 | 4295 | 4470 |
| n48 | 3550 | 3700 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 | 14200 | 14800 | 17750 | 18500 |

5.59.1.4		∆TIB and ∆RIB values

For CA_n26-n48, the TIB,c and RIB,c values are same as for CA_ n26-n77 and are given in the tables below.

Table 5.59.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n26-n48 | 0.3 | 0.8 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.59.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n26-n48 | - | 0.5 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.59.1.5 		REFSENS requirements

- The 4th harmonic mixing falls into Rx frequencies of band 26.

- For CA_n26-n48, the MSD values due to harmonic mixing are same as for CA_ n5-n77 and are given in the tables below.

Table 5.59.1.5-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n48 | n26 | 10 | 15 | 25 (RBstart=0) | 5 | 5.7 | NOTE 8 | UL1/DL4 |
| n48 | n26 | 20 | 15 | 100 (RBstart=0) | 20 | 2.7 | NOTE 8 | UL1/DL4 |

5.59.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

Table 5.59.1.6-1: CA band combination with exceptions allowed

CA band combination

5.59.2		Specific for 2 bands UL CA

5.59.2.1	Maximum output power for inter-band CA

Table 5.59.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n26A-n48A | 23 | +2/-3 |

5.59.2.2		UE co-existence studies

Table 5.59.2.2-1 lists Band n26 + Band n48 2UL bands CA 2nd, 3rd, 4th and 5th order IMD analysis.

Table 5.59.2.2-1: Band n26 and Band n48 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 814 | 849 | 3550 | 3700 |
| 2nd order IMD products | ¦fx_low – fy_high¦ | ¦fx_high – fy_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2701 | 2886 | 4364 | 4549 |
| Two-tone 3rd order IMD products | ¦2*fx_high – fy_low¦ | ¦2*fx_low – fy_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1852 | 2072 | 6251 | 6586 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5178 | 5398 | 7914 | 8249 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1003 | 1258 | 9801 | 10286 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5992 | 6247 | 11464 | 11949 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 5402 | 5772 | 8728 | 9098 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13351 | 13986 | 154 | 444 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15014 | 15649 | 6806 | 7096 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 8952 | 9472 | 4553 | 4958 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12278 | 12798 | 9542 | 9947 |

Based on the table above it can be seen that there is no IMD due to two UL falls into own DL.

Table 5.59.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n26-n48 | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 4 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. NOTE 4: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

#### 5.59.2.3		REFSENS requirements

Based on co-existence studies on 5.59.2.2, no need to define exceptional REFSENS requirements.

5.60	CA_n29-n48

5.60.1	Common for 1 band UL

5.60.1.1		Operating bands for CA

Table 5.60.1.1-1: CA band combination of band n29 and n48

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n29 | N/A | – | N/A | 717 MHz | – | 728 MHz | SDL |
| n48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |

5.60.1.2		Channel bandwidths per operating band for CA

Table 5.60.1.2-1: Supported bandwidths per CA band combination of band n29+n48

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n29A-n48A | - | n29 | 5, 10 | 0 |
|  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |

5.60.1.3		UE co-existence studies

Table 5.60.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n29-n48 shown that there are 5th harmonic mixing issues to consider.

Table 5.60.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n29 |  |  | 717 | 728 |  |  |  |  |  |  |  |  |
| n48 | 3550 | 3700 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 | 14200 | 14800 | 17750 | 18500 |

Table 5.60.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n29 |  |  | 717 | 728 | 1434 | 1456 | 2151 | 2184 | 2868 | 2912 | 3585 | 3640 |
| n48 | 3550 | 3700 | 3550 | 3700 | 7100 | 7400 | 10650 | 11100 | 14200 | 14800 | 17750 | 18500 |

5.60.1.4		∆TIB and ∆RIB values

For CA_n29-n48, the TIB,c and RIB,c values are same as for CA_ n29-n77 and are given in the tables below.

Table 5.60.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n29-n48 | - | 0.8 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.60.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n29-n48 | 0.2 | 0.5 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.60.1.5 		REFSENS requirements

- The 5th harmonic mixing falls into Rx frequencies of band 29.

- For CA_n29-n48, the MSD values due to harmonic mixing are same as for CA_ n29-n77 and are given in the tables below.

Table 5.60.1.5-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n48 | n29 | 10 | 15 | 25 (RBstart=0) | 5 | 31 | NOTE 5 | UL1/DL5 |
| n48 | n29 | 10 | 15 | 50 (RBstart=0) | 10 | 28 | NOTE 5 | UL1/DL5 |

5.60.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

Table 5.60.1.6-1: CA band combination with exceptions allowed

CA band combination

5.61		CA_n34-n39

    - 5.61.1	Common for 1 band UL and 2 bands UL CA

      - 5.61.1.1 Operating bands for CA

Table 5.61.1.1-1: CA band combination CA_n34-n39

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n34-n399 | n34 | 2010 MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz | TDD |
|  | n39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
| NOTE 9: Only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. |  |  |  |  |  |  |  |  |

      - 5.61.1.2	Channel bandwidths per operating band for CA

Table 5.61.1.2-1: Supported bandwidths per CA band combination CA_n34-n39

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n34A-n39A | CA_n34A-n39A | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n39 | See n39 channel bandwidths in Table 5.3.5-1 |  |

      - 5.61.1.3	UE Co-existence studies

Table 5.61.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n34-n39.

Table 5.67.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n34 | 2010 | 2025 | 2010 | 2025 | 4020 | 4050 | 6030 | 6075 | 8040 | 8100 | 10050 | 10125 |
| n39 | 1880 | 1920 | 1880 | 1920 | 3760 | 3840 | 5640 | 5760 | 7520 | 7680 | 9400 | 9600 |

Based on above table, there is no harmonic issue for the band combination of n34 and n39, also because it is TDD-TDD band combination without simultaneous Rx/Tx.

Table 5.67.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n34 | 2010 | 2025 | 2010 | 2025 | 4020 | 4050 | 6030 | 6075 | 8040 | 8100 | 10050 | 10125 |
| n39 | 1880 | 1920 | 1880 | 1920 | 3760 | 3840 | 5640 | 5760 | 7520 | 7680 | 9400 | 9600 |

Based on above table, there is no harmonic mixing issue for the band combination of n34 and n39, also because it is TDD-TDD band combination without simultaneous Rx/Tx.

5.61.1.4	∆TIB and ∆RIB values

It is proposed to reused the TIB,c and RIB of CA_n39-n40 for CA_n34-n39, which are defined in the following tables.

Table 5.61.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n34-n39 | - | - |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.61.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n34-n39 | 0.3 | 0.3 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

      - 5.61.1.5	REFSENs requirements

According to the co-existence study, no additional MSD requirements should be defined.

      - 5.61.1.6	OOB blocking exception requirements

No OOB blocking exception issue.

    - 5.61.2		Specific for 2 bands UL CA

      - 5.61.2.1	Maximum output power for inter-band CA

Table 5.61.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n34A-n39A | 23 | +2/-3 |

      - 5.61.2.2		UE co-existence studies

No IMD issue for the TDD-TDD CA_n34-n39 supporting inter-band carrier aggregation without simultaneous Rx/Tx

Table 5.61.2.2-1: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n34-n39 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz |  |  |  |  |  |  |  |

      - 5.61.2.3		REFSENS requirements

Based on the co-existence there are no IMD issues, so there is no REFSEN exception requirements needed to be defiend.

5.62		CA_n2-n71

5.62.1	Common for 1 band UL and 2 bands UL CA

5.62.1.1 Operating bands for CA

Table 5.62.1.1-1: CA band combination of band n2+n71

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

5.62.1.2	Channel bandwidths per operating band for CA

Table 5.62.1.2-1: Supported bandwidths per CA band combination of band n2+n71

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n2A-n71A | - | n2 | See n2 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 |  |

5.62.1.3	Co-existence studies

There is no new co-existence study for the new bandwidth combination set.

For the uplink harmonic the study of DC_2_n71 and DC_71_n2 show that beyond 20MHz channel bandwidth the MSD will be even less than the 0.6dB declared in these EN/DC cases, so no need to declare and uplink harmonic MSD for CA_n2A-n71A BSC 4 and 5

For harmonic mixing we propose to add the same entry as for CA_n25A-n71A.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n2 | n713 | 5 | 15 | 25 (RBstart=0) | 5 | 26.5 | NOTE 4 | UL1/DL3 |
| n2 | n713 | 20 | 15 | 100 (RBstart=0) | 20 | 15.3 | NOTE 4 | UL1/DL3 |
| NOTE 3: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel's transmission bandwidth of an aggressor (higher) band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

5.62.1.4	∆TIB and ∆RIB values

These values are already part of the specification. There’s nothing new for the proposed bandwidth combination set.

5.62.1.5	REFSENs requirements

There are no specific REFSENS requirements for 1 band UL

5.62.1.6	OOB blocking exception requirements

There is no OOB exception for the CA combination.

5.63		CA_n5-n71

5.63.1	Common for 1 band UL and 2 bands UL CA

5.63.1.1 Operating bands for CA

Table 5.63.1.1-1: CA band combination of band n5A+n71A

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

5.63.1.2	Channel bandwidths per operating band for CA

Table 5.63.1.2-1: Supported bandwidths per CA band combination of band n5A+n71A

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n5A-n71A | - | n5 | n5 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |

#### 5.63.1.3	Co-existence studies

Table 5.63.1.3-1/2 summarizes frequency ranges where UL harmonics and/or harmonic mixing occur for CA_n5-n71.

Table 5.63.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 | 4120 | 4245 |
| n71 | 663 | 698 | 617 | 652 | 1326 | 1396 | 1989 | 2094 | 2652 | 2792 | 3315 | 3490 |

Based on above table, there is no UL harmonic issues.

Table 5.63.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 | 4345 | 4470 |
| n71 | 663 | 698 | 617 | 652 | 1234 | 1304 | 1851 | 1956 | 2468 | 2608 | 3085 | 3260 |

Based on above table, there is no UL harmonic mixing issues.

Note: 4th DL harmonics of n71 (2468-2680) is equal to 3rd UL harmonic of n5. Since it is a higher order product then the impact can be ignored.

5.63.1.4	∆TIB and ∆RIB values

There is no update for ΔTIB,c and ΔRIB,c due to BCS 4+5 update.

5.63.1.5	REFSENs requirements

There’s no MSD exception needed.

5.63.1.6	OOB blocking exception requirements

There’s no MSD exception needed.

5.64	CA_n46-n102

5.64.1	Common for 1 band UL and 2 bands UL CA

5.64.1.1 Operating bands for CA

Table 5.64.1.1-1: CA band combination CA_n46-n102

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n46-n10215,16,18,20 | n46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |
|  | n102 | 5925 MHz | – | 6425 MHz | 5925 MHz | – | 6425 MHz |  |
| NOTE 15: Simultaneous Rx/Tx capability does not apply for UEs supporting CA_n46-n96 or CA_n46-n102. Same restrictions are applied when applicable NR CA configuration is part of a higher order configurations. NOTE 16: The minimum requirements for intra-band non-contiguous CA/DC apply for CA_n46-n96, CA_n46-n102 and related higher order CA/DC configurations. NOTE 18: The minimum requirements for inter-band CA apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. The power spectral density imbalance condition also applies for these carriers when applicable CA configuration is a subset of a higher order CA configuration. NOTE 20: The combination is not used alone as fall back mode of other band combinations in which UL in Band n78 is not used. |  |  |  |  |  |  |  |  |

Note:  CA_n46-n102 with UL is not specified and it is only used on higher order BC’s. Only 1UL in n46 or n102 will be allowed in higher order combos.

5.64.1.2	Channel bandwidths per operating band for CA

Table 5.64.1.2-1: Supported bandwidths per CA band combination CA_n46-n102

| NR CA Configuration | UL CA Configuration | NR Band | Channel bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n46A-n102A | - | n46 | 10, 20, 40, 60, 80, 100 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n46A-n102(2A) | - | n46 | 10, 20, 40, 60, 80, 100 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n46A-n102B | CA_n102B | n46 | 10, 20, 40, 60, 80, 100 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n46A-n102C | CA_n102C | n46 | 10, 20, 40, 60, 80, 100 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n46A-n102D | - | n46 | 10, 20, 40, 60, 80, 100 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n46A-n102E | - | n46 | 10, 20, 40, 60, 80, 100 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n46(2A)-n102A | - | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n46(2A)-n102(2A) | - | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n46(2A)-n102B | CA_n102B | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n46(2A)-n102C | CA_n102C | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n46(2A)-n102D | - | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n46(2A)-n102E | - | n46 | CA_n46(2A)_BCS0 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n46C-n102A | - | n46 | CA_n46C_BCS0 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n46C-n102(2A) | - | n46 | CA_n46C_BCS0 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n46C-n102B | CA_n102B | n46 | CA_n46C_BCS0 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n46C-n102C | CA_n102C | n46 | CA_n46C_BCS0 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n46C-n102D | - | n46 | CA_n46C_BCS0 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n46C-n102E | - | n46 | CA_n46C_BCS0 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |
| CA_n46D-n102A | - | n46 | CA_n46D_BCS0 | 0 |
|  |  | n102 | 20, 40, 60, 80, 100 |  |
| CA_n46D-n102(2A) | - | n46 | CA_n46D_BCS0 | 0 |
|  |  | n102 | CA_n102(2A)_BCS0 |  |
| CA_n46D-n102B | CA_n102B | n46 | CA_n46D_BCS0 | 0 |
|  |  | n102 | CA_n102B_BCS0 |  |
| CA_n46D-n102C | CA_n102C | n46 | CA_n46D_BCS0 | 0 |
|  |  | n102 | CA_n102C_BCS0 |  |
| CA_n46D-n102D | - | n46 | CA_n46D_BCS0 | 0 |
|  |  | n102 | CA_n102D_BCS0 |  |
| CA_n46D-n102E | - | n46 | CA_n46D_BCS0 | 0 |
|  |  | n102 | CA_n102E_BCS0 |  |

5.64.1.3	UE Co-existence studies

Due to non-simultaneous Tx/Rx operation, there are no harmonic/harmonic mixing issues for CA_n46-n102.

The network may signal NS_30 in the cell on band n46 and NS_58 in the cell on band n102. Hence, the UL transmission on band n102 (or n46) may need to meet both of the additional emission requirements.

The additional requirements for NS_58 are duplicated from TS 38.101-1 as below.

6.5F.3.3.6	Requirements for network signalling value "NS_58"

When "NS_58" is indicated in the cell, the power of any UE emission for channels assigned within 5945-6425 MHz shall not exceed the levels specified in Table 6.5F.3.3.6-1. These requirements also apply for frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5F.3.3.6-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 87.5 ≤ f ≤ 118 | -54 | 100 kHz |
| 174 ≤ f ≤ 230 | -54 | 100 kHz |
| 470 ≤ f ≤ 694 | -54 | 100 kHz |
| f ≤ 5935 | -22 | 1 MHz |

And the additional requirements for NS_30 (the most relevant part) are duplicated as below, too.

Table 6.5F.3.3.3-3: Additional requirements for shared access channels assigned within 5725-5850 MHz

| Protected range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20, 40, 60, 80, [100] MHz |  |
| f < 5650 | -27 | 1 MHz |
| 5650 ≤ f < 5700 | -27 to 10 |  |
| 5700 ≤ f < 5720 | 10 to 15.6 |  |
| 5720 < f ≤ 5725 | 15.6 to 27 |  |
| 5850 ≤ f ≤ 5855 | 27 to 15.6 |  |
| 5855 < f ≤ 5875 | 15.6 to 10 |  |
| 5875 < f ≤ 5925 | 10 to -27 |  |
| 5925 < f | -27 |  |
| NOTE: The minimum requirement when specified as a range denotes the emission requirement at the end points of the protected range. The requirement within the protected range is obtained by linear interpolation between the requirements at the end points. |  |  |

It can be seen that for a UE transmitting on band n102 and compliant with NS_58 will meet -22dBm/1MHz emission requirements for f ≤ 5935MHz. With a further 10MHz gap, it’s expected that the UE will be able to meet the 10 to -27dBm/1MHz emission requirements for 5875MHz < f ≤ 5925MHz as indicated by NS_30.

Since the NS_30 requirements are more stringent (-27dBm/1MHz vs -22dBm/1MHz), it’s expected that a UE transmitting on band n46 will meet the NS_58 requirements.

In summary, no new A-MPR requirements will be needed.

5.64.1.4	∆TIB and ∆RIB values

For CA_n46-n102, the TIB,c and RIB are reused from CA_n46-n96 and given in the tables below.

Table 5.64.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n46-n102 | n46 | 0 |
|  | n102 | 0.5 |

Table 5.64.1.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n46-n102 | n46 | 0 |
|  | n102 | 0 |

5.64.1.5	REFSENs requirements

There is no REFSENS exception requirements for CA_n46-n102.

5.64.1.6	OOB blocking exception requirements

There is no OOB blocking exception requirement for CA_n46-n102.

5.65		CA_n1-n77

5.65.1	Common for 1 band and 2 bands UL CA

5.65.1.1	Configurations

Table 5.65.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands) DL and two bands CA UL

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n77(2A) | CA_n77(2A) | n1 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |

Unaffected parts are omitted for the sake of simplicity.

5.65.1.4	∆TIB and ∆RIB values

There is no change to the current TS [1], so this section is omitted.

5.65.1.5	REFSENS requirements

For inter-band UL CA_n1A-n77A, IMD 2, IMD4 and IMD5 fall into DL n1, and the corresponding MSD values are already specified in Table 7.3A.5-1 of latest version of TS [1] and are omitted here for the sake of simplicity.

Analysis of REFSENS exceptions or MSD requirements are needed due to IMD introduced by UL CA_n77(2A).

5.65.1.5.1	MSD analysis for UL CA_n77(2A)

Based on calculation, IMD 5, IMD6 and IMD7 of UL CA_n77(2A) fall into DLn1. Possible problematic cases are:

- IMD5

- IMD6: For IMD6 to fall on n1 DL, carriers in n77 would need to exceed 600 MHz spacing which is not allowed in current specification [1].

- IMD7: This is not an issue either since it can be covered by IMD5, the lowest odd order in this case. A note should be added to clarify that IMD7 is present but the corresponding MSD does not need to be specified, e.g. NOTE X as in Table below.

As such, the lowest odd and even order cases are IMD5 and IMD6, but only IMD5 is recommended to be specified. The MSD exception is defined as in Table below.

Table 7.3A.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1A-n77A12 | n1 | N/A | 5 | N/A | 2130 | 17 | FDD | IMD5X |
|  | n7712 | 3310 | 10 | 1 RBSTART=25 | 3310 | N/A | TDD | N/A |
|  |  | 3900 | 10 | 1 RBSTART=25 | 3900 | N/A | TDD | N/A |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE X: This band is subject to IMD7 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.66		CA_n28-n77

5.66.1	Common for 1 band UL and 2 bands UL CA

5.66.1.1	Configurations

Table 5.66.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands) DL and two bands CA UL

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n77(2A) | CA_n77(2A) | n28 | 5, 10, 15, 20 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |

Unaffected sessions are omitted for the sake of simplicity.

5.66.1.4	∆TIB and ∆RIB values

There is no change to the current TS [1], so this section is omitted.

5.66.1.5	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements due to IMD introduced by inter-band UL CA_n28A-n77A are already completed: IMD 2 and IMD5 fall into DL n28And the corresponding MSD values are already specified in Table 7.3A.5-1 of latest version of TS [1] and hence are omitted here for the sake of simplicity.

Analysis of REFSENS exceptions or MSD requirements are needed due to IMD introduced by UL CA_n77(2A).

5.66.1.5.1	MSD analysis for UL CA_n77(2A)

Based on calculation, IMD 2, 4, 6, 7 and 8 of dual n77 UL falls into n28 DL. Possible problematic cases are

- IMD2: For IMD2 to fall on n28, carriers in n77 would need to exceed 600 MHz spacing which is not allowed in current specification [1].

- IMD4

- IMD6: Considered to be covered by lower even order case of IMD4. A note should be added to clarify that IMD6 is present but the corresponding MSD does not need to be specified e.g. NOTE X as in Table below.

- IMD7: For IMD7 to fall on n28, carriers in n77 would need to exceed 600 MHz spacing which is not allowed in current specification [1].

- IMD8: Considered to be covered by lower even order case of IMD4.

The MSD exception is defined as below.

Table 7.3A.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n28A-n77A12 | n28 | N/A | 5 | N/A | 780 | 8.5 | FDD | IMD4X |
|  | n7712 | 3510 | 10 | 1 RBSTART=25 | 3510 | N/A | TDD | N/A |
|  |  | 3900 | 10 | 1 RBSTART=25 | 3900 | N/A | TDD | N/A |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE X: This band is subject to IMD6 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.67		CA_n71-n85

5.67.1	Common for 1 band UL and 2 bands UL CA

5.67.1.1 Operating bands for CA

Table 5.67.1.1-1: CA band combination of band n71+n85

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
| n85 | 698 MHz | – | 716 MHz | 728 MHz | – | 746 MHz | FDD |

5.67.1.2	Channel bandwidths per operating band for CA

Table 5.67.1.2-1: Supported bandwidths per CA band combination of band n5+n77

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n71A-n85A | - | n71 | See n71 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n71(2A)-n85A | - | n71 | CA_n71(2A) BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |
| CA_n71B-n85A | - | n71 | CA_n71B BCS 4 and 5 | 4 and 5 |
|  |  | n85 | See n85 channel bandwidths in Table 5.3.5-1 |  |

5.67.1.3	Co-existence studies

Table 5.67.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n71-n85.

Table 5.67.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n71 | 663 | 698 | 617 | 652 | 1326 | 1396 | 1989 | 2094 | 2652 | 2792 | 3315 | 3490 |
| n85 | 698 | 716 | 728 | 746 | 1396 | 1432 | 2094 | 2148 | 2792 | 2864 | 3490 | 3580 |

Based on above table, there is no harmonic interference.

Table 5.67.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n71 | 663 | 698 | 617 | 652 | 1234 | 1304 | 1851 | 1956 | 2468 | 2608 | 3085 | 3260 |
| n85 | 698 | 716 | 728 | 746 | 1456 | 1492 | 2184 | 2238 | 2912 | 2984 | 3640 | 3730 |

Based on above table, there is no harmonic mixing issue for CA_n71-n85.

Based on the analysis of cross-band interference provided by multiple companies the cross-band isolation issue of CA_n71A-n85A is address via MSD as defined in Table 5.67.1.3-3.

Table 5.67.1.3-3: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n85 | 688 | 20 | 15 | 20 (RBstart=86) | 730.5 | 5 | 8.2X | ACLR2 |
|  |  | 680.5 | 35 | 15 | 20 (Rbstart=168) | 730.5 | 5 | 23Y | ACLR1 |
| n85 | n71 | 705.5 | 15 | 15 | 20 (Rbstart=0) | 649.5 | 5 | 3.8 | >ACLR2 |
| NOTE X: applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE y: applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth |  |  |  |  |  |  |  |  |  |

5.67.1.4	∆TIB and ∆RIB values

For CA_n71-n85, the TIB,c and RIB values are as agreed in WF.

Table 5.67.1.4-1: ΔTIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n71-n85 | 1 | 1 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.67.1.4-2: ΔRIB,c due to NR CA (two bands)

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n71-n85 | 0.8 | 0.8 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.67.1.5	REFSENs requirements

There are no specific REFSENS requirements for 1 band UL

5.67.1.6	OOB blocking exception requirements

There is no OOB exception for the CA combination. This since the need of additional OBB exception requirement for the band combinations is only valid for a wideband (e.g. n77/n78/n79) when paired with a low frequency band (< 1 GHz).

Table 5.67.1.6-1: CA band combination with exceptions allowed

CA band combination

No exceptions

5.68		CA_n7-n20

5.68.1		Specific for 1 bands UL CA

5.68.1.1 Operating bands for CA

Table 5.68.1.1-1: CA band combination of band n7 and n20

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| n20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |

5.68.1.2	Channel bandwidths per operating band for CA

Table 5.68.1.2-2: Supported bandwidths per CA band combination of band n7+n20

| CA_n7A-n20A | CA_n7A-n20A | n7 | See n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
| --- | --- | --- | --- | --- |
|  |  | n20 | See n20 channel bandwidths in Table 5.3.5-1 |  |

5.68.1.3	Co-existence studies

Table 5.1.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n7-n20.

Table 5.68.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5000 | 5140 | 7500 | 7710 | 10000 | 10280 | 12500 | 12850 |
| n20 | 832 | 862 | 791 | 821 | 1664 | 1724 | 2496 | 2586 | 3328 | 3448 | 4160 | 4310 |

Table 5.68.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 | 10480 | 10760 | 13100 | 13450 |
| n20 | 832 | 862 | 791 | 821 | 1582 | 1642 | 2373 | 2463 | 3164 | 3284 | 3955 | 4105 |

5.68.1.4	∆TIB and ∆RIB values

For CA_n7-n20, the TIB,c and RIB,c values are reused from DC_7A_n20A and DC_20A_n7A and are given in the tables below.

Table 5.68.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n7-n20 | 0.3 | 0.3 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.68.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n7-n20 | 0.5 | - |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.68.1.5	REFSENs requirements

As can be seen in the co-existence studies in 5.68.1.3 there are no harmonics issues or harmonic mixing issues.

5.68.2		Specific for 2 bands UL CA

5.68.2.1	Maximum output power for inter-band CA

Table 5.68.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n7A-n20A | 23 | +2/-3 |

5.68.2.2		UE co-existence

Table 5.68.2.2-1 gives IMD interference analysis for CA_ n7-n20.

Table 5.68.2.2-1: IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 832 | 862 | 2500 | 2570 |
| DL Frequency [MHz] | 791 | 821 | 2620 | 2690 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1738 | 1638 | 3332 | 3432 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 906 | 776 | 4138 | 4308 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4164 | 4294 | 5832 | 6002 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3476 | 3276 | 6664 | 6864 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 74 | 86 | 6638 | 6878 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4996 | 5156 | 8332 | 8572 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9448 | 9138 | 948 | 758 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10832 | 11142 | 5828 | 6018 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6046 | 5776 | 2414 | 2644 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9164 | 9434 | 7496 | 7726 |

Based on the table 5.68.2.2-1, there could theoretically be IMD5 interference issues into DL band n7.

Based on the table 5.68.2.2-1, there could be IMD3 and IMD5 interference into DL band n20.

5.68.2.3		REFSENS requirements

MSD value due to IMD3 into DL band n20 is reused from DC_7A_n20A and DC_20A_n7A.

With symmetric allocation of DL and UL band n7, it is not possible to define test points that gives IMD5 into band n7. Therefore, IMD5 is not defined which is also aligned with DC_7_n20 and DC_20_n7 that don’t have MSD for band 7 or band n7 defined.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n7-n20 | n7 | 2512 | 10 | 50 | 2632 | N/A | FDD | N/A |
|  | n20 | 851 | 5 | 25 | 810 | 12 | FDD | IMD311 |
| NOTE 11: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.69		CA_n20-n78

5.69.1		Specific for 1 bands UL CA

5.69.1.1 Operating bands for CA

CA_n20-n78 has already been specified and this section does not need to be revisited.

5.69.1.2	Channel bandwidths per operating band for CA

DL CA_n20A-n78(2A) with UL CA_n78(2A) need to be added to in the existing configuration table of 38.101-1, see Table 5.69.1-1 below.

Table 5.69.1.2-2: Supported bandwidths per CA band combination of band n20+n78

| CA_n20A-n78(2A) | CA_n20A-n78A CA_n78(2A) | n20 | See n20 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
| --- | --- | --- | --- | --- |
|  |  | n78 | CA_n78(2A)_BCS4 and 5 |  |

5.69.1.3	Co-existence studies

Table 5.69.2.2-3 gives a non-contiguous uplink IMD interference analysis

Table 5.69.2.2-3: Non-contiguous uplink IMD analysis

| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 | 3320 | 3800 | 3800 | 3780 | 3300 |
| 2nd | I fU1L-fU2L I | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 20 | 500 | 6620 | 7580 |  |  |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 2800 | 4300 | 9920 | 11380 |  |  |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Interference ranges | 40 | 1000 | 6100 | 8100 | 13220 | 15180 |
| 5th | I 3*fUL1-2*fU3L I | I 3*fUH1-2*fU3H I | 4*fUL1-fU3L | 4*fUH1-fU3H | 4*fUL1+fU2L | 4*fUH1+fU2H |
| Interference ranges | 2300 | 4800 | 9400 | 11900 | 16520 | 18980 |
| 6th | I 3*fUL1-3*fU2L I | I 3*fUH1-3*fU3H I | 4*fUL1-2*fU3L | 4*fUH1-2*fU3H | 5*fUL1-fU3L | 5*fUH1-fU3H |
| Interference ranges | 60 | 1500 | 5600 | 8600 | 12700 | 15700 |
| 7th | I 4*fUL1-3*fU3L I | I 4*fUH1-3*fU3H I | 5*fUL1-2*fU3L | 5*fUH1-2*fU3H | 6*fUL1-fU3L | 6*fUH1-fU3H |
| Interference ranges | 1800 | 5300 | 8900 | 12400 | 16000 | 19500 |

Min channel = 10MHz

Min Channel separation = 10MHz

Max channel separation = 500MHz

Based on the table 5.69.2.2-3, there are IMD4 and IMD6 products falling inside n20 DL, which may occur in simultaneous RX/TX configurations.

5.69.1.4	∆TIB and ∆RIB values

CA_n20-n78 has already been specified and this section does not need to be revisited.

5.69.1.5	REFSENs requirements

Based on the co-existence studies there is a need to define MSD values. IMD4 MSD value band n20 is reused from CA_n5-n77, CA_n67-n78 and CA_n71-n77.

Table 5.69.1.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n20-n78 | n20 | N/A | 5 | N/A | 800 | 8.6 | FDD | IMD415 |
|  | n7812 | 3350 | 10 | 1 RBSTART=25 | 3350 | N/A | TDD | N/A |
|  |  | 3750 | 10 | 1 RBSTART=25 | 3750 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 15: This band is subject to IMD6 also which MSD is not specified |  |  |  |  |  |  |  |  |

5.70		CA_n25-n41

5.70.1	Common for 1 band UL and 2 bands UL CA

5.70.1.1 Operating bands for CA

Table 5.70.1.1-1: CA band combination of band n25+n41

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

5.70.1.2	Channel bandwidths per operating band for CA

Table 5.70.1.2-1: Supported bandwidths per CA band combination of band n25+n41

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n25A-n41C | CA_n25A-n41C | n25 | See n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS 4 and 5 |  |

5.70.1.3	Co-existence studies

The co-existence study has been completed for this CA except the triple-beat analysis, that this addition brings. The analysis of the IMD is found in Table 5.70.1.3-1

Table 5.70.1.3-1: Triple-beat IMD analysis of band n25+n41

| CC location | fU1L | fU2L | fU3L | fU1H |  | Channel BW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 2496 | 2516 | 2666 | 2690 |  | 10 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min channel separation |
| Frequency | 1850 | 1915 | 2670 | 2520 |  | 10 |
| 1st order triple beat | I fUL3 -fUL1 - fSCCL ¦ | I fUL2 -fUL1 + fSCCL ¦ | I fUL2 -fUL1 - fSCCH ¦ | I fUL3 -fUL1 + fSCCH ¦ |  | Max channel separation |
| Ranges | 1680 | 1870 | 1895 | 2085 |  | 170 |
| 1st order triple beat | I fUL2 +fUL1 - fSCCH ¦ | I fUH1 +fUH2 - fSCCL ¦ | I fUL2 +fUL1 + fSCCL ¦ | I fUH1 +fUH2 + fSCCH ¦ |  |  |
| Ranges | 3097 | 3510 | 6862 | 7275 |  |  |

There is an IMD issue for this combination.

5.70.1.4	∆TIB and ∆RIB values

These values are already part of the specification. There’s nothing new for the proposed bandwidth combination set.

5.70.1.5	REFSENs requirements

Table 5.70.1.5-1 lists the MSD required due to the triple-beat IMD product of CA_n25A-n41C.

Table 5.70.1.5-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25-n41 | n25 | 1860 | 5 | 25 | 1940 | 15.3 | FDD | IMD3 |
|  | n41 | 2501 | 10 | 1 RBstart = 25 | 2501 | N/A | TDD | N/A |
|  |  | 2556 | 100 | 1 RBstart = 208 | 2556 |  |  |  |

5.70.1.6	OOB blocking exception requirements

There is no OOB exception for the CA combination.

5.71		CA_n41-n66

5.71.1	Common for 1 band UL and 2 bands UL CA

5.71.1.1 Operating bands for CA

Table 5.71.1.1-1: CA band combination of band n41+n66

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| n66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

5.71.1.2	Channel bandwidths per operating band for CA

Table 5.71.1.2-1: Supported bandwidths per CA band combination of band n41+n66

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n41C-n66A | CA_n41C-n66A | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n66 | See n66 channel bandwidths in Table 5.3.5-1 |  |

5.71.1.3	Co-existence studies

The co-existence study has been completed for this CA except the triple-beat analysis, that this addition brings. The analysis of the IMD is found in Table 5.71.1.3-1.

Table 5.71.1.3-1: Triple-beat IMD analysis of band n41+n66

| CC location | fU1L | fU2L | fU3L | fU1H |  | Channel BW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 2496 | 2516 | 2666 | 2690 |  | 10 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min channel separation |
| Frequency | 1710 | 1780 | 2670 | 2520 |  | 10 |
| 1st order triple beat | I fUL3 -fUL1 - fSCCL ¦ | I fUL2 -fUL1 + fSCCL ¦ | I fUL2 -fUL1 - fSCCH ¦ | I fUL3 -fUL1 + fSCCH ¦ |  | Max channel separation |
| Ranges | 1540 | 1730 | 1760 | 1950 |  | 170 |
| 1st order triple beat | I fUL2 +fUL1 - fSCCH ¦ | I fUH1 +fUH2 - fSCCL ¦ | I fUL2 +fUL1 + fSCCL ¦ | I fUH1 +fUH2 + fSCCH ¦ |  |  |
| Ranges | 3232 | 3650 | 6722 | 7140 |  |  |

There is no triple-beat issue for this combination.

5.71.1.4	∆TIB and ∆RIB values

These values are already part of the specification. There’s nothing new for the proposed bandwidth combination set.

5.71.1.5	REFSENs requirements

There is no additional sensitivity requirement.

5.71.1.6	OOB blocking exception requirements

There is no OOB exception for the CA combination.

5.72		CA_n41-n71

5.72.1	Common for 1 band UL and 2 bands UL CA

5.72.1.1 Operating bands for CA

Table 5.72.1.1-1: CA band combination of band n41+n71

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

5.72.1.2	Channel bandwidths per operating band for CA

Table 5.72.1.2-1: Supported bandwidths per CA band combination of band n41+n71

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n41C-n71A | CA_n41C-n71A | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 |  |

5.72.1.3	Co-existence studies

The co-existence study has been completed for this CA except the triple-beat analysis, that this addition brings. The analysis of the IMD is found in Table 5.72.1.3-1.

Table 5.72.1.3-1: Triple-beat IMD analysis of band n41+n71

| CC location | fU1L | fU2L | fU3L | fU1H |  | Channel BW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 2496 | 2516 | 2666 | 2690 |  | 10 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min channel separation |
| Frequency | 663 | 698 | 2670 | 2520 |  | 10 |
| 1st order triple beat | I fUL3 -fUL1 - fSCCL ¦ | I fUL2 -fUL1 + fSCCL ¦ | I fUL2 -fUL1 - fSCCH ¦ | I fUL3 -fUL1 + fSCCH ¦ |  | Max channel separation |
| Ranges | 493 | 683 | 678 | 868 |  | 170 |
| 1st order triple beat | I fUL2 +fUL1 - fSCCH ¦ | I fUH1 +fUH2 - fSCCL ¦ | I fUL2 +fUL1 + fSCCL ¦ | I fUH1 +fUH2 + fSCCH ¦ |  |  |
| Ranges | 4314 | 4697 | 5675 | 6058 |  |  |

There is an IMD issue for this combination.

5.72.1.4	∆TIB and ∆RIB values

These values are already part of the specification. There’s nothing new for the proposed bandwidth combination set.

5.72.1.5	REFSENs requirements

Even a triple-beat IMD product of CA_n41C-n71A is identified then according to the agreed WF R4-2220556 the bands belong to different band groups hence no MSD is needed defined.

5.72.1.6	OOB blocking exception requirements

There is no OOB exception for the CA combination.

5.73		CA_n41-n77

5.73.1	Common for 1 band UL and 2 bands UL CA

5.73.1.1 Operating bands for CA

Table 5.73.1.1-1: CA band combination of band n41+n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

5.73.1.2	Channel bandwidths per operating band for CA

Table 5.73.1.2-1: Supported bandwidths per CA band combination of band n41+n77

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n41C-n77A | CA_n41C-n77A | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n77 | See n77 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41A-n77(2A) | CA_n77(2A) CA_n41A-n77A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |

5.73.1.3	Co-existence studies

Triple beat is only present when both TDD bands have active uplink, which means in a 2CA combination there will never be active downlink simultaneously. No study required.

5.73.1.4	∆TIB and ∆RIB values

These values are already part of the specification. There’s nothing new for the proposed bandwidth combination set.

5.73.1.5	REFSENs requirements

For inter-band UL CA_n41A-n77A, IMD5, IMD7 and IMD9 fall into DL n77, and the corresponding MSD value for IMD9 has already been specified in Table 7.3A.5-1 of latest version of TS [2] .

For UL CA_n77(2A), IMD3, IMD5, and IMD7 fall in n41 downlink. Yellow highlighted those cells contain the new PC3 test point for n77(2A) in Table 7.3A.5-1. IMD5 test point was chosen instead of IMD3, as for IMD3 to fall on n41 the frequency separation would need to be over 600 MHz which is not allowed based on clause 5.3A.5 of 38.101-1.

Analysis of REFSENS exceptions or MSD requirements are needed due to IMD5 introduced by UL CA_n77(2A). The MSD exception is defined as in Table below.

Table 7.3A.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n77 | n4112 | 2545 | 60 | 1 (RBSTART=0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBSTART=272) | 2625 |  |  |  |
|  | n77 | N/A | 10 | N/A | 3305 | 2.7 | TDD | IMD9 |
|  | n41 | N/A | 10 | N/A | 2565 | 17 | TDD | IMD516 |
|  | n7712 | 3485 | 10 | 1 (RBSTART=25) | 3485 | N/A | TDD | N/A |
|  |  | 3945 | 10 | 1 (RBSTART=25) | 3945 |  |  |  |
| NOTE 12: This band supports intra-band non-contiguous uplink configuration. NOTE 16: This band is subject to IMD7 also which MSD is not specified. |  |  |  |  |  |  |  |  |

5.73.1.6	OOB blocking exception requirements

There is no OOB exception for the CA combination.

5.74	CA_n26-n71

5.74.1	Common for 1 band UL

5.74.1.1		Operating bands for CA

Table 5.74.1.1-1: CA band combination of band n26+n71

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

5.74.1.2		Channel bandwidths per operating band for CA

Table 5.74.1.2-1: Supported bandwidths per CA band combination of band n26+n71

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n26A-n71A | - | n26 | 5, 10, 15, 20 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |

5.74.1.3		UE co-existence studies

Table 5.74.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n26-n71 shown that there are no harmonic issues to consider.

Table 5.74.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n26 | 814 | 849 | 859 | 894 | 1628 | 1698 | 2442 | 2547 | 3256 | 3396 | 4070 | 4245 |
| n71 | 663 | 698 | 617 | 652 | 1326 | 1396 | 1989 | 2094 | 2652 | 2792 | 3315 | 3490 |

Table 5.74.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n26 | 814 | 849 | 859 | 894 | 1718 | 1788 | 2577 | 2682 | 3436 | 3576 | 4295 | 4470 |
| n71 | 663 | 698 | 617 | 652 | 1234 | 1304 | 1851 | 1956 | 2468 | 2608 | 3085 | 3260 |

There are cross band isolation interferences from UL n26 to DL n71 and UL n71 to DL n26.

5.74.1.4		∆TIB and ∆RIB values

For CA_n26-n71, the TIB,c and RIB,c values are given in the tables below.

Table 5.74.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)9 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration10 |  |
| CA_n26-n71 | 0.5 | 0.5 |
| NOTE 9: “-” denotes ΔTIB,c = 0. NOTE 10: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.74.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)8 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration9 |  |
| CA_n26-n71 | 0.5 | 0.3 |
| NOTE 8: “-” denotes ΔRIB,c = 0. NOTE 9: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

5.74.1.5 		REFSENS requirements

As can be seen in the co-existence studies in 5.74.1.3 there are no harmonics issues. The MSD due to cross band isolation are reused from CA_n5-n71.

Table 5.74.1.5-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n26 | n71 | 824 | 20 | 15 | 20 (RBstart=0) | 649.5 | 5 | 3.9 | >ACLR2 |
| n71 | n26 | 688 | 20 | 15 | 20 (RBstart=86) | 861.5 | 5 | 2.0 | >ACLR2 |

5.74.1.6	OOB blocking exception requirements

There is no OOB exception for this CA combination.

Table 5.74.1.6-1: CA band combination with exceptions allowed

CA band combination

## 5.75	CA_n8-n77

### 5.75.1	Common for 1 band UL and 2 bands UL CA

#### 5.75.1.1	Operating bands for CA

Table 5.75.1.1-1: CA band combination of band n8 and n77

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

#### 5.75.1.2	Channel bandwidths per operating band for CA

Table 5.75.1.2-1: Supported bandwidths per CA band combination of band n8+n77

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n77A | CA_n8A-n77A | n8 | 5, 10, 15, 20 | 0 |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |

#### 5.75.1.3	UE co-existence studies

The harmonic / harmonic mixing issue for 1UL/2DL of CA_n8A-n77A have been analysed in the previous release.

#### 5.75.1.4	∆TIB and ∆RIB values

The TIB,c and RIB,c have been specified in current spec.

#### 5.75.1.5 	REFSENS requirements

The REFSENS requirements for 1UL/2DL of CA_n8A-n77A have been specified in current spec.

#### 5.75.1.6	OOB blocking exception requirements

Since band n8 is a low band and n77 is a wide band, the OOBB exception is needed.

Table 5.75.1.6-1: CA band combination with exceptions allowed

CA band combination

CA_n8-n77

### 5.75.2	Specific for 2 bands UL CA

#### 5.75.2.1	Maximum output power for inter-band CA

Table 5.75.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n8A-n77A | 23 | +2/-3 |

#### 5.75.2.2	UE co-existence studies

Table 5.75.2.2-1 lists Band n8 + Band n77 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.75.2.2-1: Band n8 and Band n77 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequencies (MHz) | 880 | 915 | 3300 | 4200 |
| 2nd order IMD products | ¦fy_low - fx_high¦ | ¦fy_high - fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2385 | 3320 | 4180 | 5115 |
| 3rd order IMD products | ¦fy_high - 2*fx_low¦ | ¦fy_low - 2*fx_high¦ | ¦2*fy_low - fx_high¦ | ¦2*fy_high - fx_low¦ |
| IMD frequency limits (MHz) | 2440 | 1470 | 5685 | 7520 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5060 | 6030 | 7480 | 9315 |
| Two-tone 4th order IMD products | ¦2*fx_low -2* fy_high¦ | ¦2*fx_high - 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 6640 | 4770 | 8360 | 10230 |
| Two-tone 4th order IMD products | ¦3*fx_low -1* fy_high¦ | ¦3*fx_high - 1*fy_low¦ | ¦3*fy_low - 1*fx_high¦ | ¦3*fy_high - 1*fx_low¦ |
| IMD frequency limits (MHz) | 1560 | 555 | 8985 | 11720 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5940 | 6945 | 10780 | 13515 |
| Two-tone 5th order IMD products | ¦fx_low - 4*fy_high¦ | ¦fx_high - 4*fy_low¦ | ¦fy_low - 4*fx_high¦ | ¦fy_high - 4*fx_low¦ |
| IMD frequency limits (MHz) | 15920 | 12285 | 360 | 680 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14080 | 17715 | 6820 | 7860 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high - 3*fx_low¦ |
| IMD frequency limits (MHz) | 10840 | 8070 | 3855 | 5760 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11660 | 14430 | 9240 | 11145 |

Based on the table above it can be seen that

- IMD4 may affect own frequencies of band n8.

The Protected bands for the 2UL bands CA configuration is same for CA_n8-n78 as below.

Table 5.75.2.2-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n8-n77 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7 MHz |  |  |  |  |  |  |  |

#### 5.75.2.3		REFSENS requirements

Based on the co-existence studies MSD would be defined and values could be reused from CA_n8A-n78A.

Table 5.75.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n8-n77 | n8 | 897.5 | 5 | 25 | 942.5 | 8.3 | FDD | IMD4 |
|  | n77 | 3635 | 10 | 50 | 3635 | N/A | TDD | N/A |

5.76		CA_n34-n40

5.76.1	Common for 1 band UL and 2 bands UL CA

5.76.1.1 Operating bands for CA

Table 5.76.1.1-1: CA band combination of band n34+n40

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n34 | 2010 MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz | TDD |
| n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

5.76.1.2	Channel bandwidths per operating band for CA

Table 5.14.1.2-1: Supported bandwidths per CA band combination of band n34+n40

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n34A-n40A | CA_n34A-n40A | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n40 | See n40 channel bandwidths in Table 5.3.5-1 |  |

5.76.1.3	REFSENS exception requirements

The MSD test point due to cross band isolation are introduced as below for BCS4 and 5 cases.

Table 5.76.1.3-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n34 | n40 | 2017.5 | 15 | 15 | 75 (RBstart=4) | 2302.5 | 5 | 6 | >ACLR2 |
| n40 | n34 | 2350 | 100 | 30 | 270 (RBstart=0) | 2022.5 | 5 | 17.9 | >ACLR2 |

  - 5.77		CA_n1-n78

    - 5.77.1	Common for 1 band UL and 2 bands UL CA

      - 5.77.1.1 Operating bands for CA

Table 5.77.1.1-1: CA band combination CA_n1-n78

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n78 | n1 | 1920 MHz | – | 1980 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3400 MHz | – | 3800 MHz | TDD |

      - 5.77.1.2	Channel bandwidths per operating band for CA

Table 5.77.1.2-1: Supported bandwidths per CA band combination CA_n1-n78

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n78C | CA_n1A-n78C | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78C_BCS4 and 5 |  |

      - 5.77.1.3	UE Co-existence studies

The co-existence studies for UL CA_n78C are studied as in the table 5.77.1.3.1-1:

Table 5.77.1.3.1-1: Co-existence studies for Uplink Intra-Band Contiguous CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data | 10 | 10 | 200 | 3300 | 3800 | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency | 3300 | 3320 | 3500 | 3800 | 3780 | 3600 |
| 2nd order IMD products | IfU1L-fU2LI | IfU1L-fU3LI | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 20 | 200 | 6620 | 7580 |  |  |
| 3rd order IMD products | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 3100 | 4000 | 9920 | 11380 |  |  |
| 4th order IMD products | I2*fU1L-2*fU2LI | I2*fU1H-2*fU3H I | 3*fU1L-fU3L | 3*fU1H-fU3H | 3*fU1L+fU2L | 3*fU1H+fU2H |
| Interference ranges | 40 | 400 | 6400 | 7800 | 13220 | 15180 |
| 5th order IMD products | I3*fU1L-2*fU3LI | I3*fU1H-2*fU3H I | 4*fU1L-fU3L | 4*fU1H-fU3H | 4*fU1L+fU2L | 4*fU1H+fU2H |
| Interference ranges | 2900 | 4200 | 9700 | 11600 | 16520 | 18980 |
| 6th order IMD products | I3*fU1L-3*fU2LI | I3*fU1H-3*fU3H I | 4*fU1L-2*fU3L | 4*fU1H-2*fU3H | 5*fU1L-fU3L | 5*fU1H-fU3H |
| Interference ranges | 60 | 600 | 6200 | 8000 | 13000 | 15400 |
| 7th order IMD products | I4*fU1L-3*fU3LI | I4*fU1H-3*fU3HI | 5*fU1L-2*fU3L | 5*fU1H-2*fU3H | 6*fU1L-fU3L | 6*fU1H-fU3H |
| Interference ranges | 2700 | 4400 | 9500 | 11800 | 16300 | 19200 |

It can be seen that there are no co-existence issues caused by CA_n78C impact band n1 DL.

      - 5.77.1.4	∆TIB and ∆RIB values

The TIB,c and RIB of CA_n1-n78 are already defined in the spec TS38.101-1.

      - 5.77.1.5	REFSENs requirements

According to the co-existence study, no additional MSD requirements should be defined.

      - 5.77.1.6	OOB blocking exception requirements

No additional OOB blocking exception issue.

    - 5.77.2		Specific for 2 bands UL CA

      - 5.77.2.1	Maximum output power for inter-band CA

Table 5.77.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n1A-n78C | 23 | +2/-3 |

      - 5.77.2.2		UE co-existence studies

In this section, we only focus on the triple beat IMDs caused by UL CA_n1A-n78C. For double beat IMDs caused by CA_n1A-n78A, it was already covered in the co-existence studies of the fallback configurations of DL CA_n1A-n78A with UL single carrier or UL CA_n1A-n78A.

The analysis of the triple beat IMD is found in Table 5.77.2.3-1:

Table 5.77.2.3-1: Band n1 and Band n78 triple beat IMD products

| CC location | fU1L | fU2L | fU3L | fU1H |  | CBW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 | 3320 | 3500 | 3800 |  | 10 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min ch. separation |
| Frequency | 1920 | 1980 | 3780 | 3600 |  | 10 |
| 1st order TB | IfU3L -fU1L- fSCCL¦ | IfU2L -fU1L + fSCCL¦ | IfU2L -fU1L- fSCCH¦ | IfU3L -fU1L + fSCCH¦ |  | Max ch. separation |
| Ranges | 1720 | 1940 | 1960 | 2180 |  | 200 |
| 1st order TB | IfU2L+fU1L-fSCCH¦ | IfU1H+fU2H-fSCCL¦ | IfU2L +fU1L+fSCCL¦ | IfU1H +fU2H+fSCCH¦ |  |  |
| Ranges | 4640 | 5660 | 8540 | 9560 |  |  |

It can be seen that 1st order TB IMDs would fall into band n1 DL Rx. However, according to the following agreements in the WF R4-2220556:

For two-band inter-band CA or DC combinations, the precondition is that:

- Only FDD-FDD and TDD-FDD combination need to be considered with 3 uplink (UL) component carriers (CC) in two clusters within FR1. The decision tree in Figure 1 provides guidelines to decide if TB detection analysis is needed.

- The 2 UL bands are part of the same band group or belong to adjacent band groups as defined in Table 1.

In the other words, only same band group or adjacent band group TB IMDs issues shall be considered.

For band n1 and n78, band n1 belongs to FR1-2(MB) group while band n78 belongs to FR1-4(VHB), which means there is no need to define the additional TB IMDs requirements.

      - 5.77.2.3		REFSENS requirements

Based on the co-existence , although there are IMD issues, there is no need to define the additional REFSEN exception requirements.

  - 5.78		CA_n3-n78

    - 5.78.1	Common for 1 band UL and 2 bands UL CA

      - 5.78.1.1 Operating bands for CA

Table 5.78.1.1-1: CA band combination CA_n3-n78

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n78 | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

      - 5.78.1.2	Channel bandwidths per operating band for CA

Table 5.78.1.2-1: Supported bandwidths per CA band combination CA_n3-n78

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n78C | CA_n78C CA_n3A-n78C | n3 | See n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78C_BCS4 and 5 |  |

      - 5.78.1.3	UE Co-existence studies

The co-existence studies for UL CA_n78C are studied as in the table 5.78.1.3.1-1:

Table 5.78.1.3.1-1: Co-existence studies for Uplink Intra-Band Contiguous CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data | 10 | 10 | 200 | 3300 | 3800 | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency | 3300 | 3320 | 3500 | 3800 | 3780 | 3600 |
| 2nd order IMD products | IfU1L-fU2LI | IfU1L-fU3LI | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 20 | 200 | 6620 | 7580 |  |  |
| 3rd order IMD products | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 3100 | 4000 | 9920 | 11380 |  |  |
| 4th order IMD products | I2*fU1L-2*fU2LI | I2*fU1H-2*fU3H I | 3*fU1L-fU3L | 3*fU1H-fU3H | 3*fU1L+fU2L | 3*fU1H+fU2H |
| Interference ranges | 40 | 400 | 6400 | 7800 | 13220 | 15180 |
| 5th order IMD products | I3*fU1L-2*fU3LI | I3*fU1H-2*fU3H I | 4*fU1L-fU3L | 4*fU1H-fU3H | 4*fU1L+fU2L | 4*fU1H+fU2H |
| Interference ranges | 2900 | 4200 | 9700 | 11600 | 16520 | 18980 |
| 6th order IMD products | I3*fU1L-3*fU2LI | I3*fU1H-3*fU3H I | 4*fU1L-2*fU3L | 4*fU1H-2*fU3H | 5*fU1L-fU3L | 5*fU1H-fU3H |
| Interference ranges | 60 | 600 | 6200 | 8000 | 13000 | 15400 |
| 7th order IMD products | I4*fU1L-3*fU3LI | I4*fU1H-3*fU3HI | 5*fU1L-2*fU3L | 5*fU1H-2*fU3H | 6*fU1L-fU3L | 6*fU1H-fU3H |
| Interference ranges | 2700 | 4400 | 9500 | 11800 | 16300 | 19200 |

It can be seen that there are no co-existence issues caused by CA_n78C impact band n3 DL.

      - 5.78.1.4	∆TIB and ∆RIB values

The TIB,c and RIB of CA_n3-n78 are already defined in the spec TS38.101-1.

      - 5.78.1.5	REFSENs requirements

According to the co-existence study, no additional MSD requirements should be defined.

      - 5.78.1.6	OOB blocking exception requirements

No additional OOB blocking exception issue.

    - 5.78.2		Specific for 2 bands UL CA

      - 5.78.2.1	Maximum output power for inter-band CA

Table 5.78.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n3A-n78C | 23 | +2/-3 |

      - 5.78.2.2		UE co-existence studies

In this section, we only focus on the triple beat IMDs caused by UL CA_n3A-n78C. For double beat IMDs caused by CA_n3A-n78A, it was already covered in the co-existence studies of the fallback configurations of DL CA_n3A-n78A with UL single carrier or UL CA_n3A-n78A.

The analysis of the triple beat IMD is found in Table 5.78.2.3-1:

Table 5.78.2.3-1: Band n3 and Band n78 triple beat IMD products

| CC location | fU1L | fU2L | fU3L | fU1H |  | CBW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 | 3320 | 3500 | 3800 |  | 10 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min ch. separation |
| Frequency | 1710 | 1785 | 3780 | 3600 |  | 10 |
| 1st order TB | IfU3L -fU1L- fSCCL¦ | IfU2L -fU1L + fSCCL¦ | IfU2L -fU1L- fSCCH¦ | IfU3L -fU1L + fSCCH¦ |  | Max ch. separation |
| Ranges | 1510 | 1730 | 1765 | 1985 |  | 200 |
| 1st order TB | IfU2L+fU1L-fSCCH¦ | IfU1H+fU2H-fSCCL¦ | IfU2L +fU1L+fSCCL¦ | IfU1H +fU2H+fSCCH¦ |  |  |
| Ranges | 4835 | 5870 | 8330 | 9365 |  |  |

It can be seen that 1st order TB IMDs would fall into band n3 DL Rx. However, according to the following agreements in the WF R4-2220556:

For two-band inter-band CA or DC combinations, the precondition is that:

Only FDD-FDD and TDD-FDD combination need to be considered with 3 uplink (UL) component carriers (CC) in two clusters within FR1. The decision tree in Figure 1 provides guidelines to decide if TB detection analysis is needed.

The 2 UL bands are part of the same band group or belong to adjacent band groups as defined in Table 1.

In the other words, only same band group or adjacent band group TB IMDs issues shall be considered.

For band n3 and n78, band n3 belongs to FR1-2(MB) group while band n78 belongs to FR1-4(VHB), which means there is no need to define the additional TB IMDs requirements.

      - 5.78.2.3		REFSENS requirements

Based on the co-existence , although there are IMD issues, there is no need to define the additional REFSEN exception requirements.

  - 5.79		CA_n8-n41

    - 5.79.1	Common for 1 band UL and 2 bands UL CA

      - 5.79.1.1 Operating bands for CA

Table 5.79.1.1-1: CA band combination CA_n8-n41

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n8-n41 | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

      - 5.79.1.2	Channel bandwidths per operating band for CA

Table 5.79.1.2-1: Supported bandwidths per CA band combination CA_n8-n41

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n41C | CA_n41C CA_n8A-n41C | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |

      - 5.79.1.3	UE Co-existence studies

For the UL/DL harmonic and harmonic mixing caused by single band n8 or single band n41, it was already analyzed in the fallback configurations of DL CA_n8A-n41A with UL single carrier, and it can be applied for DL CA_n8A-n41C with UL single carrier.

The co-existence studies for UL CA_n41C are studied as in the table 5.79.1.3.1-1:

Table 5.79.1.3.1-1: Co-existence studies for Uplink Intra-Band Contiguous CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data | 10 | 10 | 190 | 2496 | 2690 | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency | 2496 | 2516 | 2686 | 2690 | 2670 | 2500 |
| 2nd order IMD products | IfU1L-fU2LI | IfU1L-fU3LI | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 20 | 190 | 5012 | 5360 |  |  |
| 3rd order IMD products | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 2306 | 2880 | 7508 | 8050 |  |  |
| 4th order IMD products | I2*fU1L-2*fU2LI | I2*fU1H-2*fU3H I | 3*fU1L-fU3L | 3*fU1H-fU3H | 3*fU1L+fU2L | 3*fU1H+fU2H |
| Interference ranges | 40 | 380 | 4802 | 5570 | 10004 | 10740 |
| 5th order IMD products | I3*fU1L-2*fU3LI | I3*fU1H-2*fU3H I | 4*fU1L-fU3L | 4*fU1H-fU3H | 4*fU1L+fU2L | 4*fU1H+fU2H |
| Interference ranges | 2116 | 3070 | 7298 | 8260 | 12500 | 13430 |
| 6th order IMD products | I3*fU1L-3*fU2LI | I3*fU1H-3*fU3H I | 4*fU1L-2*fU3L | 4*fU1H-2*fU3H | 5*fU1L-fU3L | 5*fU1H-fU3H |
| Interference ranges | 60 | 570 | 4612 | 5760 | 9794 | 10950 |
| 7th order IMD products | I4*fU1L-3*fU3LI | I4*fU1H-3*fU3HI | 5*fU1L-2*fU3L | 5*fU1H-2*fU3H | 6*fU1L-fU3L | 6*fU1H-fU3H |
| Interference ranges | 1926 | 3260 | 7108 | 8450 | 12290 | 13640 |

It can be seen that there are no co-existence issues caused by CA_n41C impact band n8 DL.

      - 5.79.1.3.1	UE co-existence studies for 2 Uplink CCs in one Intra-Band CA

Additionally, the co-existence studies caused by UL CA_n41C should be analyzed for CA_n8A-n41C, which can be found in table Table 5.79.1.3.1-1:

Table 5.79.1.3.1-1: Co-existence studies for Uplink Intra-Band Contiguous CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data | 10 | 10 | 190 | 2496 | 2690 | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency | 2496 | 2516 | 2686 | 2690 | 2670 | 2500 |
| 2nd order IMD products | IfU1L-fU2LI | IfU1L-fU3LI | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 20 | 190 | 5012 | 5360 |  |  |
| 3rd order IMD products | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 2306 | 2880 | 7508 | 8050 |  |  |
| 4th order IMD products | I2*fU1L-2*fU2LI | I2*fU1H-2*fU3H I | 3*fU1L-fU3L | 3*fU1H-fU3H | 3*fU1L+fU2L | 3*fU1H+fU2H |
| Interference ranges | 40 | 380 | 4802 | 5570 | 10004 | 10740 |
| 5th order IMD products | I3*fU1L-2*fU3LI | I3*fU1H-2*fU3H I | 4*fU1L-fU3L | 4*fU1H-fU3H | 4*fU1L+fU2L | 4*fU1H+fU2H |
| Interference ranges | 2116 | 3070 | 7298 | 8260 | 12500 | 13430 |
| 6th order IMD products | I3*fU1L-3*fU2LI | I3*fU1H-3*fU3H I | 4*fU1L-2*fU3L | 4*fU1H-2*fU3H | 5*fU1L-fU3L | 5*fU1H-fU3H |
| Interference ranges | 60 | 570 | 4612 | 5760 | 9794 | 10950 |
| 7th order IMD products | I4*fU1L-3*fU3LI | I4*fU1H-3*fU3HI | 5*fU1L-2*fU3L | 5*fU1H-2*fU3H | 6*fU1L-fU3L | 6*fU1H-fU3H |
| Interference ranges | 1926 | 3260 | 7108 | 8450 | 12290 | 13640 |

It can be seen that there are no co-existence issues caused by CA_n41C impact band n8 DL.

      - 5.79.1.4	∆TIB and ∆RIB values

The TIB,c and RIB of CA_n8-n41 are already defined in the spec TS38.101-1.

      - 5.79.1.5	REFSENs requirements

According to the co-existence study, no additional MSD requirements should be defined.

      - 5.79.1.6	OOB blocking exception requirements

No additional OOB blocking exception issue.

    - 5.79.2		Specific for 2 bands UL CA

      - 5.79.2.1	Maximum output power for inter-band CA

Table 5.79.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n8A-n41C | 23 | +2/-3 |

      - 5.79.2.2		UE co-existence studies

In this section, we only focus on the triple beat IMDs caused by UL CA_n8A-n41C. For double beat IMDs caused by CA_n8A-n41A, it was already covered in the co-existence studies of the fallback configurations of DL CA_n8A-n41A with UL single carrier or UL CA_n8A-n41A.

The analysis of the triple beat IMD is found in Table 5.79.2.3-1:

Table 5.79.2.3-1: Band n8 and Band n41 triple beat IMD products

| CC location | fU1L | fU2L | fU3L | fU1H |  | CBW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 2496 | 2516 | 2686 | 2690 |  | 10 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min ch. separation |
| Frequency | 880 | 915 | 2670 | 2500 |  | 10 |
| 1st order TB | IfU3L -fU1L- fSCCL¦ | IfU2L -fU1L + fSCCL¦ | IfU2L -fU1L- fSCCH¦ | IfU3L -fU1L + fSCCH¦ |  | Max ch. separation |
| Ranges | 690 | 900 | 895 | 1105 |  | 190 |
| 1st order TB | IfU2L+fU1L-fSCCH¦ | IfU1H+fU2H-fSCCL¦ | IfU2L +fU1L+fSCCL¦ | IfU1H +fU2H+fSCCH¦ |  |  |
| Ranges | 4097 | 4480 | 5892 | 6275 |  |  |

It can be seen that 1st order TB IMDs would fall into band n8 DL Rx. However, according to the following agreements in the WF R4-2220556:

For two-band inter-band CA or DC combinations, the precondition is that:

- Only FDD-FDD and TDD-FDD combination need to be considered with 3 uplink (UL) component carriers (CC) in two clusters within FR1. The decision tree in Figure 1 provides guidelines to decide if TB detection analysis is needed.

- The 2 UL bands are part of the same band group or belong to adjacent band groups as defined in Table 1.

In the other words, only same band group or adjacent band group TB IMDs issues shall be considered.

For band n8 and n41, band n8 belongs to FR1-1(LB) group while band n41 belongs to FR1-3(HB), which means there is no need to define the additional TB IMDs requirements.

      - 5.79.2.3		REFSENS requirements

Based on the co-existence , although there are IMD issues, there is no need to define the additional REFSEN exception requirements.

  - 5.80		CA_n8-n78

    - 5.80.1	Common for 1 band UL and 2 bands UL CA

      - 5.80.1.1 Operating bands for CA

Table 5.80.1.1-1: CA band combination CA_n8-n78

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n8-n78 | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

      - 5.80.1.2	Channel bandwidths per operating band for CA

Table 5.80.1.2-1: Supported bandwidths per CA band combination CA_n8-n78

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n8A-n78C | CA_n78C CA_n8A-n78A CA_n8A-n78C | n8 | See n8 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78C_BCS4 and 5 |  |

      - 5.80.1.3	UE Co-existence studies

For the UL/DL harmonic and harmonic mixing caused by single band n8 or single band n78, it was already analyzed in the fallback configurations of DL CA_n8A-n78A with UL single carrier, and it can be applied for DL CA_n8A-n78C with UL single carrier.

The co-existence studies for UL CA_n78C are studied as in the table 5.80.1.3.1-1:

Table 5.80.1.3.1-1: Co-existence studies for Uplink Intra-Band Contiguous CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data | 10 | 10 | 200 | 3300 | 3800 | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency | 3300 | 3320 | 3500 | 3800 | 3780 | 3600 |
| 2nd order IMD products | IfU1L-fU2LI | IfU1L-fU3LI | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 20 | 200 | 6620 | 7580 |  |  |
| 3rd order IMD products | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 3100 | 4000 | 9920 | 11380 |  |  |
| 4th order IMD products | I2*fU1L-2*fU2LI | I2*fU1H-2*fU3H I | 3*fU1L-fU3L | 3*fU1H-fU3H | 3*fU1L+fU2L | 3*fU1H+fU2H |
| Interference ranges | 40 | 400 | 6400 | 7800 | 13220 | 15180 |
| 5th order IMD products | I3*fU1L-2*fU3LI | I3*fU1H-2*fU3H I | 4*fU1L-fU3L | 4*fU1H-fU3H | 4*fU1L+fU2L | 4*fU1H+fU2H |
| Interference ranges | 2900 | 4200 | 9700 | 11600 | 16520 | 18980 |
| 6th order IMD products | I3*fU1L-3*fU2LI | I3*fU1H-3*fU3H I | 4*fU1L-2*fU3L | 4*fU1H-2*fU3H | 5*fU1L-fU3L | 5*fU1H-fU3H |
| Interference ranges | 60 | 600 | 6200 | 8000 | 13000 | 15400 |
| 7th order IMD products | I4*fU1L-3*fU3LI | I4*fU1H-3*fU3HI | 5*fU1L-2*fU3L | 5*fU1H-2*fU3H | 6*fU1L-fU3L | 6*fU1H-fU3H |
| Interference ranges | 2700 | 4400 | 9500 | 11800 | 16300 | 19200 |

It can be seen that there are no co-existence issues caused by CA_n78C impact band n8 DL

      - 5.80.1.3.1	UE co-existence studies for 2 Uplink CCs in one Intra-Band CA

Additionally, the co-existence studies caused by UL CA_n78C should be analyzed for CA_n8A-n78C, which can be found in table Table 5.80.1.3.1-1:

Table 5.80.1.3.1-1: Co-existence studies for Uplink Intra-Band Contiguous CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data | 10 | 10 | 200 | 3300 | 3800 | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency | 3300 | 3320 | 3500 | 3800 | 3780 | 3600 |
| 2nd order IMD products | IfU1L-fU2LI | IfU1L-fU3LI | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 20 | 200 | 6620 | 7580 |  |  |
| 3rd order IMD products | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 3100 | 4000 | 9920 | 11380 |  |  |
| 4th order IMD products | I2*fU1L-2*fU2LI | I2*fU1H-2*fU3HI | 3*fU1L-fU3L | 3*fU1H-fU3H | 3*fU1L+fU2L | 3*fU1H+fU2H |
| Interference ranges | 40 | 400 | 6400 | 7800 | 13220 | 15180 |
| 5th order IMD products | I3*fU1L-2*fU3LI | I3*fU1H-2*fU3HI | 4*fU1L-fU3L | 4*fU1H-fU3H | 4*fU1L+fU2L | 4*fU1H+fU2H |
| Interference ranges | 2900 | 4200 | 9700 | 11600 | 16520 | 18980 |
| 6th order IMD products | I3*fU1L-3*fU2LI | I3*fU1H-3*fU3HI | 4*fU1L-2*fU3L | 4*fU1H-2*fU3H | 5*fU1L-fU3L | 5*fU1H-fU3H |
| Interference ranges | 60 | 600 | 6200 | 8000 | 13000 | 15400 |
| 7th order IMD products | I4*fU1L-3*fU3LI | I4*fU1H-3*fU3HI | 5*fU1L-2*fU3L | 5*fU1H-2*fU3H | 6*fU1L-fU3L | 6*fU1H-fU3H |
| Interference ranges | 2700 | 4400 | 9500 | 11800 | 16300 | 19200 |

It can be seen that there are no co-existence issues caused by CA_n78C impact band n8 DL.

      - 5.80.1.4	∆TIB and ∆RIB values

The TIB,c and RIB of CA_n8-n78 are already defined in the spec TS38.101-1.

      - 5.80.1.5	REFSENs requirements

According to the co-existence study, no additional MSD requirements should be defined.

      - 5.80.1.6	OOB blocking exception requirements

No additional OOB blocking exception issue.

    - 5.80.2		Specific for 2 bands UL CA

      - 5.80.2.1	Maximum output power for inter-band CA

Table 5.80.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n8A-n78C | 23 | +2/-3 |

      - 5.80.2.2		UE co-existence studies

In this section, we only focus on the triple beat IMDs caused by UL CA_n8A-n78C. For double beat IMDs caused by CA_n8A-n78A, it was already covered in the co-existence studies of the fallback configurations of DL CA_n8A-n78A with UL single carrier or UL CA_n8A-n78A.

The analysis of the triple beat IMD is found in Table 5.80.2.3-1:

Table 5.80.2.3-1: Band n8 and Band n78 triple beat IMD products

| CC location | fU1L | fU2L | fU3L | fU1H |  | CBW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 | 3320 | 3500 | 3800 |  | 10 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min ch. separation |
| Frequency | 880 | 915 | 3780 | 3600 |  | 10 |
| 1st order TB | IfU3L -fU1L- fSCCL¦ | IfU2L -fU1L + fSCCL¦ | IfU2L -fU1L- fSCCH¦ | IfU3L -fU1L + fSCCH¦ |  | Max ch. separation |
| Ranges | 680 | 900 | 895 | 1115 |  | 200 |
| 1st order TB | IfU2L+fU1L-fSCCH¦ | IfU1H+fU2H-fSCCL¦ | IfU2L +fU1L+fSCCL¦ | IfU1H +fU2H+fSCCH¦ |  |  |
| Ranges | 5705 | 6700 | 7500 | 8495 |  |  |

It can be seen that 1st order TB IMDs would fall into band n8 DL Rx. However, according to the following agreements in the WF R4-2220556:

For two-band inter-band CA or DC combinations, the precondition is that:

- Only FDD-FDD and TDD-FDD combination need to be considered with 3 uplink (UL) component carriers (CC) in two clusters within FR1. The decision tree in Figure 1 provides guidelines to decide if TB detection analysis is needed.

- The 2 UL bands are part of the same band group or belong to adjacent band groups as defined in Table 1.

In the other words, only same band group or adjacent band group TB IMDs issues shall be considered.

For band n8 and n78, band n8 belongs to FR1-1(LB) group while band n78 belongs to FR1-4(VHB), which means there is no need to define the additional TB IMDs requirements.

      - 5.80.2.3		REFSENS requirements

Based on the co-existence , although there are IMD issues, there is no need to define the additional REFSEN exception requirements.

  - 5.81		CA_n28-n41

    - 5.81.1	Common for 1 band UL and 2 bands UL CA

      - 5.81.1.1 Operating bands for CA

Table 5.81.1.1-1: CA band combination CA_n28-n41

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n28-n41 | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

      - 5.81.1.2	Channel bandwidths per operating band for CA

Table 5.81.1.2-1: Supported bandwidths per CA band combination CA_n28-n41

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n41C | CA_n28A-n41C | n28 | See n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | CA_n41C_BCS4 and 5 |  |

      - 5.81.1.3	UE Co-existence studies

The co-existence studies for UL CA_n41C are studied as in the table 5.81.1.3.1-1:

Table 5.81.1.3.1-1: Co-existence studies for Uplink Intra-Band Contiguous CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data | 10 | 10 | 190 | 2496 | 2690 | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency | 2496 | 2516 | 2686 | 2690 | 2670 | 2500 |
| 2nd order IMD products | IfU1L-fU2LI | IfU1L-fU3LI | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 20 | 190 | 5012 | 5360 |  |  |
| 3rd order IMD products | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 2306 | 2880 | 7508 | 8050 |  |  |
| 4th order IMD products | I2*fU1L-2*fU2LI | I2*fU1H-2*fU3H I | 3*fU1L-fU3L | 3*fU1H-fU3H | 3*fU1L+fU2L | 3*fU1H+fU2H |
| Interference ranges | 40 | 380 | 4802 | 5570 | 10004 | 10740 |
| 5th order IMD products | I3*fU1L-2*fU3LI | I3*fU1H-2*fU3H I | 4*fU1L-fU3L | 4*fU1H-fU3H | 4*fU1L+fU2L | 4*fU1H+fU2H |
| Interference ranges | 2116 | 3070 | 7298 | 8260 | 12500 | 13430 |
| 6th order IMD products | I3*fU1L-3*fU2LI | I3*fU1H-3*fU3H I | 4*fU1L-2*fU3L | 4*fU1H-2*fU3H | 5*fU1L-fU3L | 5*fU1H-fU3H |
| Interference ranges | 60 | 570 | 4612 | 5760 | 9794 | 10950 |
| 7th order IMD products | I4*fU1L-3*fU3LI | I4*fU1H-3*fU3HI | 5*fU1L-2*fU3L | 5*fU1H-2*fU3H | 6*fU1L-fU3L | 6*fU1H-fU3H |
| Interference ranges | 1926 | 3260 | 7108 | 8450 | 12290 | 13640 |

It can be seen that there are no co-existence issues caused by CA_n41C impact band n28 DL.

      - 5.81.1.4	∆TIB and ∆RIB values

The TIB,c and RIB of CA_n28-n41 are already defined in the spec TS38.101-1.

      - 5.81.1.5	REFSENs requirements

According to the co-existence study, no additional MSD requirements should be defined.

      - 5.81.1.6	OOB blocking exception requirements

No additional OOB blocking exception issue.

    - 5.81.2		Specific for 2 bands UL CA

      - 5.81.2.1	Maximum output power for inter-band CA

Table 5.81.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n28A-n41C | 23 | +2/-3 |

      - 5.81.2.2		UE co-existence studies

In this section, we only focus on the triple beat IMDs caused by UL CA_n28A-n41C. For double beat IMDs caused by CA_n28A-n41A, it was already covered in the co-existence studies of the fallback configurations of DL CA_n28A-n41A with UL single carrier or UL CA_n28A-n41A.

The analysis of the triple beat IMD is found in Table 5.81.2.3-1:

Table 5.81.2.3-1: Band n28 and Band n41 triple beat IMD products

| CC location | fU1L | fU2L | fU3L | fU1H |  | CBW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 2496 | 2516 | 2686 | 2690 |  | 10 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min ch. separation |
| Frequency | 703 | 748 | 2670 | 2500 |  | 10 |
| 1st order TB | IfU3L -fU1L- fSCCL¦ | IfU2L -fU1L + fSCCL¦ | IfU2L -fU1L- fSCCH¦ | IfU3L -fU1L + fSCCH¦ |  | Max ch. separation |
| Ranges | 513 | 723 | 728 | 938 |  | 170 |
| 1st order TB | IfU2L+fU1L-fSCCH¦ | IfU1H+fU2H-fSCCL¦ | IfU2L +fU1L+fSCCL¦ | IfU1H +fU2H+fSCCH¦ |  |  |
| Ranges | 4264 | 4657 | 5715 | 6108 |  |  |

It can be seen that 1st order TB IMDs would fall into band n28 DL Rx. However, according to the following agreements in the WF R4-2220556:

For two-band inter-band CA or DC combinations, the precondition is that:

- Only FDD-FDD and TDD-FDD combination need to be considered with 3 uplink (UL) component carriers (CC) in two clusters within FR1. The decision tree in Figure 1 provides guidelines to decide if TB detection analysis is needed.

- The 2 UL bands are part of the same band group or belong to adjacent band groups as defined in Table 1.

In the other words, only same band group or adjacent band group TB IMDs issues shall be considered.

For band n28 and n41, band n28 belongs to FR1-1(LB) group while band n41 belongs to FR1-3(HB), which means there is no need to define the additional TB IMDs requirements.

      - 5.81.2.3		REFSENS requirements

Based on the co-existence , although there are IMD issues, there is no need to define the additional REFSEN exception requirements.

  - 5.82		CA_n40-n79

    - 5.82.1	Common for 1 band UL and 2 bands UL CA

      - 5.82.1.1 Operating bands for CA

Table 5.82.1.1-1: CA band combination CA_n40-n79

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n40-n79 | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4000 MHz | – | 5000 MHz | TDD |

      - 5.82.1.2	Channel bandwidths per operating band for CA

Table 5.82.1.2-1: Supported bandwidths per CA band combination CA_n40-n79

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n40A-n79C | CA_n79C CA_n40A-n79C | n40 | See n40 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |

      - 5.82.1.3	UE Co-existence studies

For the UL/DL harmonic and harmonic mixing caused by single band n40 or single band n79, it was already analyzed in the fallback configurations of DL CA_n40A-n79A with UL single carrier, and it can be applied for DL CA_n40A-n79C with UL single carrier.

      - 5.82.1.3.1	UE co-existence studies for 2 Uplink CCs in one Intra-Band CA

Additionally, the co-existence studies caused by UL CA_n79C should be analyzed for CA_n40A-n79C, which can be found in table Table 5.82.1.3.1-1:

Table 5.82.1.3.1-1: Co-existence studies for Uplink Intra-Band Contiguous CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data | 40 | 40 | 200 | 4400 | 5000 | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency | 4400 | 4480 | 4600 | 5000 | 4920 | 4800 |
| 2nd order IMD products | IfU1L-fU2LI | IfU1L-fU3LI | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 80 | 200 | 8880 | 9920 |  |  |
| 3rd order IMD products | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 4200 | 5200 | 13280 | 14920 |  |  |
| 4th order IMD products | I2*fU1L-2*fU2LI | I2*fU1H-2*fU3H I | 3*fU1L-fU3L | 3*fU1H-fU3H | 3*fU1L+fU2L | 3*fU1H+fU2H |
| Interference ranges | 160 | 400 | 8600 | 10200 | 17680 | 19920 |
| 5th order IMD products | I3*fU1L-2*fU3LI | I3*fU1H-2*fU3H I | 4*fU1L-fU3L | 4*fU1H-fU3H | 4*fU1L+fU2L | 4*fU1H+fU2H |
| Interference ranges | 4000 | 5400 | 13000 | 15200 | 22080 | 24920 |
| 6th order IMD products | I3*fU1L-3*fU2LI | I3*fU1H-3*fU3H I | 4*fU1L-2*fU3L | 4*fU1H-2*fU3H | 5*fU1L-fU3L | 5*fU1H-fU3H |
| Interference ranges | 240 | 600 | 8400 | 10400 | 17400 | 20200 |
| 7th order IMD products | I4*fU1L-3*fU3LI | I4*fU1H-3*fU3HI | 5*fU1L-2*fU3L | 5*fU1H-2*fU3H | 6*fU1L-fU3L | 6*fU1H-fU3H |
| Interference ranges | 3800 | 5600 | 12800 | 15400 | 21800 | 25200 |

It can be seen that there are no co-existence issues caused by CA_n79C impact band n40 DL.

      - 5.82.1.4	∆TIB and ∆RIB values

The TIB,c and RIB of CA_n40-n79 are already defined in the spec TS38.101-1.

      - 5.82.1.5	REFSENs requirements

According to the co-existence study, no additional MSD requirements should be defined. Thus there are no changes for the current specification.

      - 5.82.1.6	OOB blocking exception requirements

No additional OOB blocking exception issue.

    - 5.82.2		Specific for 2 bands UL CA

      - 5.82.2.1	Maximum output power for inter-band CA

Table 5.82.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n40A-n79C | 23 | +2/-3 |

      - 5.82.2.2		UE co-existence studies

Due to both band n40 and n79 are TDD bands, so there are no need to check the TB IMDs caused by UL CA_n40A-n79C since the Tx and Rx for either n40 or n79 will not be operation at the same time.

      - 5.82.2.3		REFSENS requirements

Based on the co-existence , there is no need to define the additional REFSEN exception requirements.

5.83	CA_n5-n13

5.83.1	Common for 1 band UL and 2 bands UL CA

5.83.1.1	Operating bands for CA

Table 5.83.1.1-1:  CA band combination of band n5+n13

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n5 | 824 MHz | – | 849MHz | 869MHz | – | 894MHz | FDD |
| n13 | 777MHz | – | 787MHz | 746MHz | – | 756MHz | FDD |

5.83.1.2	Channel bandwidths per operating band for CA

Table 5.83.1.2-1: Supported bandwidths per CA band combination of band n5+n13

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n5A-n13A | CA_n5A-n13A | n5 | 5, 10, 15, 20 | 0 |
|  |  | n13 | 5, 10 |  |

5.83.1.3	UE co-existence studies

Table 5.83.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ n5-n13.

Table 5.83.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1648 | 1698 | 2472 | 2547 | 3296 | 3396 | 4120 | 4245 |
| n13 | 777 | 787 | 746 | 756 | 1554 | 1574 | 2331 | 2361 | 3108 | 3148 | 3885 | 3935 |

Table 5.83.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 | 4345 | 4470 |
| n13 | 777 | 787 | 746 | 756 | 1492 | 1512 | 2238 | 2268 | 2984 | 3024 | 3730 | 3780 |

Based on the analysis of cross-band interference provided, the cross-band isolation issue of CA_n5A-n13A is addressed via MSD as defined in Table 5.83.1.3-3 (R4-2405454).

Table 5.83.1.3-3: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n5 | n13 | 834 | 20 | 15 | 20 (RBstart=0) | 753.5 | 5 | 2.4 | >ACLR2 |
| n13 | n5 | 782 | 10 | 15 | 20 (RBstart=32) | 871.5 | 5 | 2.1 | >ACLR2 |

5.83.1.4	∆TIB,c and ∆RIB,c values

For CA_n5-n13, the TIB,c and RIB,c values are reused from CA_5_13 due to exact same frequency range, and are given in the tables below.

Table 5.83.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n5-n13 | n5 | 0.5 |
|  | n13 | 0.5 |

Table 5.83.1.4-2: ΔRIB.c

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n5-n13 | n5 | 0 |
|  | n13 | 0 |

5.83.1.5	REFSENS requirements

Based on UE co-existence studies in section 5.83.1.3, there is no harmonic issue for the band combination CA_n5-n13.

Based on UE co-existence studies in section 5.83.1.3, there is no harmonic mixing issue for the band combination CA_n5-n13.

5.83.1.6	OOB blocking exception requirements

There is no OOB exception for CA_n5-n13 combination.

Table 5.83.1.6-1: CA band combination with exceptions allowed

CA band combination

No exceptions

5.83.2	Specific for 2 bands UL CA

Editor's note: Text will be added if 2 bands UL CA are supported, otherwise all the clauses shall be void.

5.83.2.1	Maximum output power for inter-band CA

Table 5.83.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n5A-n13A | 23 | +2/-32 |
| NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

5.83.2.2	UE co-existence studies

Table 5.83.2.2-1 lists Band n5 + Band n13 2UL bands CA 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.83.2.2-1: Band n5 and Band n13 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 777 | 787 | 824 | 849 |
| 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 37 | 72 | 1601 | 1636 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 705 | 750 | 861 | 921 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 2378 | 2423 | 2425 | 2485 |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) | (fx_high + max BW fy) | (fy_low – max BW fx) | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 767 | 797 | 804 | 868 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 1482 | 1537 | 1685 | 1770 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 3155 | 3210 | 3249 | 3334 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 144 | 74 | 3202 | 3272 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2619 | 2509 | 2324 | 2259 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4073 | 4183 | 3932 | 3997 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 993 | 898 | 713 | 633 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4026 | 4121 | 3979 | 4059 |

Based on the table above:

- 3rd IMD products might fall into Rx frequencies of band n5.

- 3rd IMD products might fall into Rx frequencies of band n13.

Table 5.83.2.2-2 lists the protected bands required for the 2UL bands CA configuration.

5.83.2.3	REFSENS requirements

Based on the co-existence studies for CA_n5-n13, MSD is needed, and CA_n5A_n14A is used as baseline due to similarity in frequency.

For the products of IMD3 falling into band n5 Rx frequencies, the MSD due to IMD3 is proposed below.

For the products of IMD3 falling into band n13 Rx frequencies, it falls on the edges of both band n5 and n13, and there is no a valid test point available for a 5MHz channel bandwidth. Thus, the MSD due to IMD3 falling into band n13 cannot be defined.

Table 5.83.2.3-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  | Source of IMD |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |  |
| CA_n5-n13 | n5 | 828 | 5 | 25 | 873 | 25 | FDD | IMD3 |  |
|  | n13 | 783 | 5 | 25 | 752 | N/A | FDD | N/A |  |

Even if a test point cannot be found for band n13 based on RAN4 guidelines, the band n13 may be affected by 2UL IMD3. And the MSD may be up to 15dB for 10MHz (R-2405954). This information is kept in this TR as reference of implementation.

  - 5.84		CA_n34-n79

    - 5.84.1	Common for 1 band UL and 2 bands UL CA

      - 5.84.1.1 Operating bands for CA

Table 5.84.1.1-1: CA band combination CA_n34-n79

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n34-n79 | n34 | 2010 MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4000 MHz | – | 5000 MHz | TDD |

      - 5.84.1.2	Channel bandwidths per operating band for CA

Table 5.84.1.2-1: Supported bandwidths per CA band combination CA_n34-n79

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n34A-n79C | CA_n79C CA_n34A-n79C | n34 | See n34 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |

Noted that the DL_CA_n34A-n79C with UL_CA_n34A-n79A_BCS 4 and 5 have already been included in the current spec.

      - 5.84.1.3	UE Co-existence studies

For the UL/DL harmonic and harmonic mixing caused by single band n34 or single band n79, it was already analyzed in the fallback configurations of DL CA_n34A-n79A with UL single carrier, and it can be applied for DL CA_n34A-n79C with UL single carrier.

      - 5.84.1.3.1	UE co-existence studies for 2 Uplink CCs in one Intra-Band CA

Additionally, the co-existence studies caused by UL CA_n79C should be analyzed for CA_n34A-n79C, which can be found in table Table 5.84.1.3.1-1:

Table 5.84.1.3.1-1: Co-existence studies for Uplink Intra-Band Contiguous CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data | 40 | 40 | 200 | 4400 | 5000 | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency | 4400 | 4480 | 4600 | 5000 | 4920 | 4800 |
| 2nd order IMD products | IfU1L-fU2LI | IfU1L-fU3LI | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 80 | 200 | 8880 | 9920 |  |  |
| 3rd order IMD products | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 4200 | 5200 | 13280 | 14920 |  |  |
| 4th order IMD products | I2*fU1L-2*fU2LI | I2*fU1H-2*fU3H I | 3*fU1L-fU3L | 3*fU1H-fU3H | 3*fU1L+fU2L | 3*fU1H+fU2H |
| Interference ranges | 160 | 400 | 8600 | 10200 | 17680 | 19920 |
| 5th order IMD products | I3*fU1L-2*fU3LI | I3*fU1H-2*fU3H I | 4*fU1L-fU3L | 4*fU1H-fU3H | 4*fU1L+fU2L | 4*fU1H+fU2H |
| Interference ranges | 4000 | 5400 | 13000 | 15200 | 22080 | 24920 |
| 6th order IMD products | I3*fU1L-3*fU2LI | I3*fU1H-3*fU3H I | 4*fU1L-2*fU3L | 4*fU1H-2*fU3H | 5*fU1L-fU3L | 5*fU1H-fU3H |
| Interference ranges | 240 | 600 | 8400 | 10400 | 17400 | 20200 |
| 7th order IMD products | I4*fU1L-3*fU3LI | I4*fU1H-3*fU3HI | 5*fU1L-2*fU3L | 5*fU1H-2*fU3H | 6*fU1L-fU3L | 6*fU1H-fU3H |
| Interference ranges | 3800 | 5600 | 12800 | 15400 | 21800 | 25200 |

It can be seen that there are no co-existence issues caused by CA_n79C impact band n34 DL.

      - 5.84.1.4	∆TIB and ∆RIB values

The TIB,c and RIB of CA_n34-n79 are already defined in the spec TS38.101-1.

      - 5.84.1.5	REFSENs requirements

According to the co-existence study, no additional MSD requirements should be defined. Thus there are no changes for the current specification.

      - 5.84.1.6	OOB blocking exception requirements

No additional OOB blocking exception issue.

    - 5.84.2		Specific for 2 bands UL CA

      - 5.84.2.1	Maximum output power for inter-band CA

Table 5.84.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n34A-n79C | 23 | +2/-3 |

      - 5.84.2.2		UE co-existence studies

Due to both band n34 and n79 are TDD bands, so there are no need to check the TB IMDs caused by UL CA_n34A-n79C since the Tx and Rx for either n34 or n79 will not be operation at the same time.

      - 5.84.2.3		REFSENS requirements

Based on the co-existence , there is no need to define the additional REFSEN exception requirements.

  - 5.85		CA_n39-n79

    - 5.85.1	Common for 1 band UL and 2 bands UL CA

      - 5.85.1.1 Operating bands for CA

Table 5.85.1.1-1: CA band combination CA_n39-n79

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n39-n79 | n39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4000 MHz | – | 5000 MHz | TDD |

      - 5.85.1.2	Channel bandwidths per operating band for CA

Table 5.85.1.2-1: Supported bandwidths per CA band combination CA_n39-n79

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n39A-n79C | CA_n79C CA_n39A-n79C | n39 | See n39 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |

Noted that the DL_CA_n39A-n79C with UL_CA_n39A-n79A_BCS 4 and 5 have already been included in the current spec.

      - 5.85.1.3	UE Co-existence studies

For the UL/DL harmonic and harmonic mixing caused by single band n39 or single band n79, it was already analyzed in the fallback configurations of DL CA_n39A-n79A with UL single carrier, and it can be applied for DL CA_n39A-n79C with UL single carrier.

      - 5.85.1.3.1	UE co-existence studies for 2 Uplink CCs in one Intra-Band CA

Additionally, the co-existence studies caused by UL CA_n79C should be analyzed for CA_n39A-n79C, which can be found in table Table 5.85.1.3.1-1:

Table 5.85.1.3.1-1: Co-existence studies for Uplink Intra-Band Contiguous CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data | 40 | 40 | 200 | 4400 | 5000 | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency | 4400 | 4480 | 4600 | 5000 | 4920 | 4800 |
| 2nd order IMD products | IfU1L-fU2LI | IfU1L-fU3LI | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 80 | 200 | 8880 | 9920 |  |  |
| 3rd order IMD products | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 4200 | 5200 | 13280 | 14920 |  |  |
| 4th order IMD products | I2*fU1L-2*fU2LI | I2*fU1H-2*fU3H I | 3*fU1L-fU3L | 3*fU1H-fU3H | 3*fU1L+fU2L | 3*fU1H+fU2H |
| Interference ranges | 160 | 400 | 8600 | 10200 | 17680 | 19920 |
| 5th order IMD products | I3*fU1L-2*fU3LI | I3*fU1H-2*fU3H I | 4*fU1L-fU3L | 4*fU1H-fU3H | 4*fU1L+fU2L | 4*fU1H+fU2H |
| Interference ranges | 4000 | 5400 | 13000 | 15200 | 22080 | 24920 |
| 6th order IMD products | I3*fU1L-3*fU2LI | I3*fU1H-3*fU3H I | 4*fU1L-2*fU3L | 4*fU1H-2*fU3H | 5*fU1L-fU3L | 5*fU1H-fU3H |
| Interference ranges | 240 | 600 | 8400 | 10400 | 17400 | 20200 |
| 7th order IMD products | I4*fU1L-3*fU3LI | I4*fU1H-3*fU3HI | 5*fU1L-2*fU3L | 5*fU1H-2*fU3H | 6*fU1L-fU3L | 6*fU1H-fU3H |
| Interference ranges | 3800 | 5600 | 12800 | 15400 | 21800 | 25200 |

It can be seen that there are no co-existence issues caused by CA_n79C impact band n39 DL.

      - 5.85.1.4	∆TIB and ∆RIB values

The TIB,c and RIB of CA_n39-n79 are already defined in the spec TS38.101-1.

      - 5.85.1.5	REFSENs requirements

According to the co-existence study, no additional MSD requirements should be defined. Thus there are no changes for the current specification.

      - 5.85.1.6	OOB blocking exception requirements

No additional OOB blocking exception issue.

    - 5.85.2		Specific for 2 bands UL CA

      - 5.85.2.1	Maximum output power for inter-band CA

Table 5.85.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n39A-n79C | 23 | +2/-3 |

      - 5.85.2.2		UE co-existence studies

Due to both band n39 and n79 are TDD bands, so there are no need to check the TB IMDs caused by UL CA_n39A-n79C since the Tx and Rx for either n39 or n79 will not be operation at the same time.

      - 5.85.2.3		REFSENS requirements

Based on the co-existence , there is no need to define the additional REFSEN exception requirements.

## 5.86	CA_n78-n104

### 5.86.1	Common for 1 band UL and 2 bands UL CA

#### 5.86.1.1	Operating bands for CA

Table 5.86.1.1-1:  CA band combination CA_n78-n104

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n78 | 3300 | – | 3800 | 3300 | – | 3800 | TDD |
| n104 | 6425 | – | 7125 | 6425 | – | 7125 | TDD |

#### 5.86.1.2	Channel bandwidths per operating band for CA

Table 5.86.1.2-1: Supported bandwidths per CA band combination CA_n78-n104

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n78A-n104A | CA_n78A-n104A | n78 | n78 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n104 | n104 channel bandwidths in Table 5.3.5-1 |  |

#### 5.86.1.3	UE co-existence studies

Table 5.86.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n78-n104.

Table 5.86.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 | 16500 | 19000 |
| n104 | 6425 | 7125 | 6425 | 7125 | 12850 | 14250 | 19275 | 21375 | 25700 | 28500 | 32125 | 35625 |

Table 5.86.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 | 16500 | 19000 |
| n104 | 6425 | 7125 | 6425 | 7125 | 12850 | 14250 | 19275 | 21375 | 25700 | 28500 | 32125 | 35625 |

The 2nd harmonic interference of UL band n78 may fall into the DL frequency range of band n104.

The UL band n104 frequency range may have a overlapping with 2nd harmonic mixing of DL band n78.

#### 5.86.1.4	∆TIB,c and ∆RIB,c values

For CA_n78-n104, the TIB,c and RIB,c values are given in the tables below.

The following inputs are provided by companies.

Company 1: Additional 1.6dB insertion loss is assumed for the potential combiner. And the TIB,c and RIB,c values were derived for both bands based on the following formula referring to paper R4-162926 below.

∆TIB = (Average Tx)/2 = 0.8

∆RIB = (Average Rx – 0.6)/2 = 0.5

Company 2: ∆TIB: 1.0dB for n78 and n104, ∆RIB: 1.2dB for n78 and n104.

Company 3: ∆TIB: 1.0dB for n78 and 1.2dB for n104, ∆RIB: 0.5dB for n78 and 0.7dB for n104.

After averaging companies’ proposal, the values are calculated below.

Table 5.86.1.4-1: ΔTIB,c

| Inter-band CA combination | ΔTIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n78-n104 | 0.9 | 1.0 |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n3 the band order from left to right is n1 and n3. |  |  |

Table 5.86.1.4-2: ΔRIB,c

| Inter-band CA combination | ΔRIB,c for NR bands (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| CA_n78-n104 | 0.7 | 0.8 |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR bands, such as for CA_n1-n77 the band order from left to right is n1 and n77. |  |  |

#### 5.86.1.5	REFSENS requirements

MSD due to cross band isolation:

Referring to the contributions R4-2405325/R4-2405698/R4-2405450/R4-2405876, the calculations summary for MSD due to cross band isolation are summarised in Table 5.86.1.5-1 for different RF architectures.

Table 5.86.1.5-1: calculation summary for CA_n78-n104 MSD due to cross band isolation

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | Cross-band Interference source | Huawei option 1 | Murata option 2 | Qualcomm option 3 | Skyworks option 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) |  | MSD (dB) |  |  |  |
| n78 | n104 | 3750 | 100 | 30 | 270 (RBstart=0) | 6435 | 20 | >ACLR2 | 4.78 ~ 12.82 | [10.3] | 4.8 | 17.2 |
| n104 | n78 | 6475 | 100 | 30 | 270 (RBstart=0) | 3795 | 10 | >ACLR2 | 26.29 ~ 17.12 | [17.2] | 4.3 | 10.1 |

The following test configurations for CA_n78-n104 MSD due to cross band isolation are proposed after averaging companies’ values below.

Table 5.86.1.5-2: MSD due to cross band isolation for CA_n78-n104

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n78 | n104 | 3750 | 100 | 30 | 270 (RBstart=0) | 6435 | 20 | [14.4] | >ACLR2 |
| n104 | n78 | 6475 | 100 | 30 | 270 (RBstart=0) | 3795 | 10 | [15.8] | >ACLR2 |

MSD due to 2nd harmonic interference:

Referring to the contribution R4-2405325/R4-2405698/R4-2405450/R4-2405876, the calculations for MSD due to 2nd harmonic interference are summarised in Table 5.86.1.5-3 for different band n78 PA H2 Attenuation and Post PA Filter performance.

Table 5.86.1.5-3: calculation summary for CA_n78-n104 MSD due to 2nd harmonic interference

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | UL/DL fc condition | UL/DL harmonic order | Huawei option 1 | Murata option 2 | Qualcomm option 3 | Skyworks option 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) |  |  | MSD (dB) |  |  |  |
| n78 | n104 | 10 | [15] | 50 (RBstart=0) | 20 | NOTE 2 | UL2/DL1 direct-hit | 23.6~32.9 dB Lcrb=24RB | [38.7] | 44.9 | 38.8 |
| n78 | n104 | 10 | [15] | 50 (RBstart=0) | 20 | NOTE 6 | UL2/DL1 near-miss |  | [10.6] | 16.2 |  |

The following test configurations for CA_n78-n104 MSD due to 2nd harmonic interference are proposed below. This DL band may be affected by near-miss interference for which the MSD is not specified

Table 5.86.1.5-4: MSD due to 2nd harmonic interference for CA_n78-n104

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n78 | n104 | 10 | 15 | 50 (RBstart=0) | 20 | [37.5] | NOTE 2 | UL2/DL1 direct-hit |

MSD due to 2nd harmonic mixing interference:

Referring to the contribution R4-2405325/R4-2405698/R4-2405450/R4-2405876, the calculations for MSD due to 2nd harmonic mixing interference are summarised below for different DL channel bandwidths and 2nd harmonic mixing attenuation in LO.

  - Option 1: Huawei

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n104 | n78 | 20 | 30 | 50 (RBstart=0) | 10 | 5.9~24.8dB | NOTE 1 | UL1/DL2 |
| n104 | n78 | 20 | 30 | 50 (RBstart=0) | 100 | 0.8~15.1dB | NOTE 1 | UL1/DL2 |

  - Option 2: Murata

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n104 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | [17.6] | NOTE 7 | UL1/DL2 |
| n104 | n78 | 20 | 15 | 100 (RBstart=0) | 100 | [9.9] | NOTE 7 | UL1/DL2 |
| NOTE 7: The requirements should be verified for UL NR-ARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. |  |  |  |  |  |  |  |  |

  - Option 3: Qualcomm

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n104 | n78 | 20 | 15 | 50 (RBstart=0) | 10 | 24.5 | NOTE 7 | UL1/DL2 |

  - Option 4: Skyworks

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n104 | n78 | 20 | 15 | 50 (RBstart=0) | 10 | 34.1 | NOTE 7 | UL1/DL2 |

The following test configurations for CA_n78-n104 MSD due to 2nd harmonic mixing interference are proposed below.

Table 5.86.1.5-6: MSD due to 2nd harmonic mixing interference for CA_n78-n104

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n104 | n78 | 20 | 15 | 50 (RBstart=0) | 10 | [29] | NOTE 1 | UL1/DL2 |
| n104 | n78 | 20 | 15 | 50 (RBstart=0) | 100 | [18.8] | NOTE 1 | UL1/DL2 |

#### 5.86.1.6	OOB blocking exception requirements

There is no need to specify OOB blocking exception for CA_n78-n104.

### 5.86.2	Specific for 2 bands UL CA

#### 5.86.2.1	Maximum output power for inter-band CA

Table 5.86.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n78A-n104A | 23 | +2/-3 |

#### 5.86.2.2	UE co-existence studies

Since both band n78 and n104 are TDD bands, the IMD interference from both UL band n78 and n104 will not fall into the UE DL receiver of both bands due to no DL slots at this time.

The intersection of the requirements for the individual bands specified in clause 6.5.3.2 from TS 38.101-1 shall also apply for the specified uplink carrier aggregation configurations. Intersection of a requirement means that both UL constituent bands have the same protected band requirement specified and if one or both protected bands have note(s) associated those note(s) also apply. No special protected frequency range for UL CA_n78A-n104A should be highlighted.

#### 5.86.2.3	REFSENS requirements

Since both band n78 and n104 are TDD bands, the IMD interference from both UL band n78 and n104 will not fall into the UE DL receiver of both bands due to no DL slots at this time. There is no RFESENS exception for UL CA_n78-n104.

### 5.86.3	General techinical analysis

#### 5.86.3.1	UE RF architecture

For antennas sharing, generally there are three main cases:

Case 1: RF chains and antennas in band n78 and n104 are fully separated for both main and diversity paths. If 1T4R/2T4R is supported for each band, the total numbers of antennas is up to 8.

Case 2: RF chains and antennas in band n78 and n104 are separated for main paths, but the antennas are shared for diversity paths. If 1T4R is supported for each band, the total number of antennas is up to 5. If 2T4R is supported for each band, the total number of antennas is up to 6.

Case 3: The antennas in band n78 and n104 are fully shared for both main and diversity paths. If 1T4R/2T4R is supported for each band, the total number of antennas is up to 4.

The schematic diagrams for each RF architecture case are shown below.

Figure 5.86.3.1-1: RF architecture case 1: Separate antenna for CA_n78-n104

Figure 5.86.3.1-2: RF architecture case 2: common antenna in diversity chain for CA_n78-n104

Figure 5.86.3.1-3: RF architecture case 3: common antenna in both main and diversity chain for CA_n78-n104

#### 5.86.3.2	UE RFFE filters assumptions

Currently, both band n78 and n77 shared the same full frequency filter (3300~4200MHz) in the real UE implementation. Thus, full frequency filter (3300~4200MHz) can be assumed for band n78 implementation.

For band n104 filters, three alternatives were proposed in way forward R4-2403794. The relative bandwidths are listed below. Quality factor (Q) values of filters for 6GHz will dominate the performance of filters. However, for a specific Quality factor (Q) value, the less relative bandwidth it is, the bettter filter performance can be achieved.

Table 5.86.3.2-1: relative bandwidth analysis for different filter alternative

| Filter frequency range | 5150-7125 MHz | 5925-7125 MHz | 6425-7125 MHz | 3300~4200MHz |
| --- | --- | --- | --- | --- |
| Relative bandwidth | 32% | 18.3% | 10.3% | 24% |

Currently, 3GPP specified band n104 (6425-7125 MHz) for IMT licensed service and band n96/n102 for unlicensed service. However, the maximum output power for band n104 and n96 are different. Generally, 20dBm/23dBm maximum output power for unlicensed band was specified (for indoor scenario). But for band n104, at least 26dBm will be implemented same as band n78/n77, which will become the basic capability for the industry (licensed bands are used for wide area). Based on the analysis above, the difference of MOP between licensed and unlicensed service will result the difference of power tolerance in the whole RF chain. Thus, it’s hard to share the same filter between licensed band n104 and unlicensed band n96.

So, from standard/implementation perspective, it’s not easy to coordinate these differences between licensed bands and unlicensed bands. One example is that 2.4GHz unlicensed band is never shared with either band n40 or band n41 due to different performance demands.

5.87	CA_n28-n79

5.87.1	Common for 1 band UL and 2 bands UL CA

5.87.1.1	Operating bands for CA

Table 5.87.1.1-1:  CA band combination CA_n78-n104

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n28 | 703 | – | 748 | 758 | – | 803 | FDD |
| n79 | 4400 | – | 5000 | 4400 | – | 5000 | TDD |

5.87.1.2	Channel bandwidths per operating band for CA

Table 5.87.1.2-1: Supported bandwidths per CA band combination CA_n78-n104

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n28A-n79C | CA_n79C CA_n28A-n79A CA_n28A-n79C | n28 | n28 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |

5.87.1.3	UE co-existence studies

The harmonic / harmonic mixing issue have been analysed in previous release.

IMD order for contiguous UL CA_n79C is calculated as below.

WGap = lowest band edge of n79 - highest DL band edge of n28 = 4400 – 803 = 3597 MHz.

Min(maxUL aggregated BW, UL band bandwidth) = Min(200, 200) = 200 MHz.

IMD order = 2*ceil(WGap /Min(maxUL aggregated BW, UL band bandwidth) + 1 = 37.

There is no need to consider IMD37.

5.87.1.4	∆TIB,c and ∆RIB,c values

The TIB,c and RIB have been specified in current spec.

5.87.1.5	REFSENS requirements

For UL CA_n79C configuration, there is no additional REFSENS exception.

5.87.1.6	OOB blocking exception requirements

It has been specified in current spec.

5.87.2	Specific for 2 bands UL CA

5.87.2.1	Maximum output power for inter-band CA

Table 5.87.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n28A-n79C | 23 | +2/-3 |

5.87.2.2	UE co-existence studies

The UE co-existence for 2UL CA n28A-n79A have already been analysed.

The bands n28 and n79 are not of same or adjacent band groups, so no requirements to be captured on MSD for TB (Triple beat UL CA_n28A-n79C).

5.87.2.3	REFSENS requirements

For UL CA_n28A-n79C configuration, there is no additional REFSENS exception.

## 5.88	CA_n3-n79

### 5.88.1	Common for 1 band UL and 2 bands UL CA

#### 5.88.1.1	Operating bands for CA

Table 5.88.1.1-1:  CA band combination CA_n3-n79

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 | – | 1775 | 1815 | – | 1880 | FDD |
| n79 | 4400 | – | 5000 | 4400 | – | 5000 | TDD |

#### 5.88.1.2	Channel bandwidths per operating band for CA

Table 5.88.1.2-1: Supported bandwidths per CA band combination CA_n3-n79

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n79C | CA_n79C CA_n3A-n79A CA_n3A-n79C | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |

#### 5.88.1.3	UE co-existence studies

The harmonic / harmonic mixing issue have been analysed in previous release.

IMD order for contiguous UL CA_n79C is calculated as below.

WGap = lowest band edge of n79 - highest DL band edge of n3 = 4400 – 1880 = 2520 MHz.

Min(maxUL aggregated BW, UL band bandwidth) = Min(200, 200) = 200 MHz.

IMD order = 2*ceil(WGap /Min(maxUL aggregated BW, UL band bandwidth) + 1 = 27.

There is no need to consider IMD27.

#### 5.88.1.4	∆TIB,c and ∆RIB,c values

The TIB,c and RIB have been specified in current spec.

#### 5.88.1.5	REFSENS requirements

For UL CA_n79C configuration, there is no additional REFSENS exception.

#### 5.88.1.6	OOB blocking exception requirements

It has been specified in current spec.

### 5.88.2	Specific for 2 bands UL CA

#### 5.88.2.1	Maximum output power for inter-band CA

Table 5.88.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n3A-n79C | 23 | +2/-3 |

#### 5.88.2.2	UE co-existence studies

The UE co-existence for 2UL CA n3A-n79A have already been analysed.

For UL CA_n3A-n79C, the triple beats are analysed below.

Table 5.88.2.2-1: Triple beat analysis for UL CA_n3A-n79C

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 4400 | 5000 | 1710 | 1775 |  |  |
| UE UL carriers | fx1_low | fx2_low | fx1_high | fx2_high | fy_low | fy_high |
| UL frequency (MHz) | 4400 | 4500 | 4900 | 5000 | 1710 | 1775 |
| TB1 | ¦fx1_low-fx2_low+fy_low ¦ | ¦fx1_low-fx2_low+fy_high ¦ | ¦fx1_high-fx2_high+fy_low ¦ | ¦fx1_high-fx2_high+fy_high ¦ | minFrequency limits | maxFrequency limits |
| TB IMD frequency limits (MHz) | 1610 | 1675 | 1610 | 1675 | 1610 | 1675 |
| TB2 | ¦-fx1_low+fx2_low+fy_low ¦ | ¦-fx1_low+fx2_low+fy_high ¦ | ¦-fx1_high+fx2_high+fy_low ¦ | ¦-fx1_high+fx2_high+fy_high ¦ | minFrequency limits | maxFrequency limits |
| TB IMD frequency limits (MHz) | 1810 | 1875 | 1810 | 1875 | 1810 | 1875 |
| TB3 | ¦-fx1_low-fx2_low+fy_low ¦ | ¦-fx1_low-fx2_low+fy_high ¦ | ¦-fx1_high-fx2_high+fy_low ¦ | ¦-fx1_high-fx2_high+fy_high ¦ | minFrequency limits | maxFrequency limits |
| TB IMD frequency limits (MHz) | 7190 | 7125 | 8190 | 8125 | 7125 | 8190 |
| TB4 | ¦fx1_low+fx2_low+fy_low ¦ | ¦fx1_low+fx2_low+fy_high ¦ | ¦fx1_high+fx2_high+fy_low ¦ | ¦fx1_high+fx2_high+fy_high ¦ | minFrequency limits | maxFrequency limits |
| TB IMD frequency limits (MHz) | 10610 | 10675 | 11610 | 11675 | 10610 | 11675 |

The 1st order triple beats interference may fall into the Rx band n3.

But based on the table 1 from WF R4-2220556, as band n3 belongs to FR1-2 (MB) and band n79 belongs to FR1-4 (VHB), this case is not eligible for the triple beat case (The two UL bands are not part of the same or adjacent band group).

Table 5.88.2.2-2 : Band group definition for adjacent band-group criterion

| FR1 band group range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | FR1-1 (LB) | FR1-2 (MB) | FR1-3 (HB) | FR1-4 (VHB) | FR1-5 (UHB) |
| Range (MHz) | 600-1000 | 1400-2200 | 2300-2700 | 3300-5000 | 5250-7125 |
| Duplex mode | Mostly FDD | Mostly FDD | FDD and TDD | TDD only | TDD only |

#### 5.88.2.3	REFSENS requirements

For UL CA_n3A-n79C configuration, there is no additional REFSENS exception.

6	Both bands within FR2 Carrier Aggregation: Specific Band Combination Part

Editor's note: This section is reserved for future used. The template for inter-band FR2 NR CA combination can be referred to TR38.851

7	Dual Connectivity: Specific Band Combination Part

7.x	DC_nX-nY

Editor's note: The texts for NR DC can only be added associated with the texts for the corresponding inter-band 2 bands UL CA above, which means pure TP to TR to included NR DC configuration is not allowed.

7.x.1	Configurations for DC_nX-nY

Table 7.x.1-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_nXA-nYA | DC_nXA-nYA |

7.x.2	Maximum output power for NR-DC

Table 7.x.2-1: UE Power Class for uplink inter-band DC

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_nXA-nYA | 23 | +x/-y |

7.1	DC_n46-n78

7.1.1	Operating bands for DC_n46-n78

Table 7.1.1-1: Inter-band NR DC operating bands

| NR DC Band | NR Band |
| --- | --- |
| DC_n46-n78 | n46, n78 |

7.1.2   Configurations for DC_n46-n78

Table 7.1.2-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n46A-n78(2A) DC_n46C-n78(2A) DC_n46D-n78(2A) | DC_n46A-n78A |
| DC_n46(2A)-n78A DC_n46(2A)-n78(2A) | DC_n46A-n78A |

7.1.3   Maximum output power for NR-DC

Table 7.1.3-1: UE Power Class for uplink inter-band DC

| Uplink DC Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n46A-n78A | 23 | +2/-31 |
| NOTE 1: refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

7.2	DC_n1-n46

7.2.1	Operating bands for DC_n1-n46

Table 7.2.1-1: Inter-band NR DC operating bands

| NR DC Band | NR Band |
| --- | --- |
| DC_n1-n46 | n1, n46 |

7.2.2   Configurations for DC_n1-n46

Table 7.2.2-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n1A-n46A DC_n1A-n46C DC_n1A-n46D DC_n1A-n46(2A) | DC_n1A-n46A |

7.2.3   Maximum output power for NR-DC

Table 7.2.3-1: UE Power Class for uplink inter-band DC

| Uplink DC Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n1A-n46A | 23 | +2/-31 |
| NOTE 1: refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |

7.3	DC_n7-n102

7.3.1	Configurations for DC_n7-n102

Table 7.3.1-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC Configuration |
| --- | --- |
| DC_n7A-n102A | DC_n7A-n102A DC_n7A-n102B DC_n7A-n102C |
| DC_n7A-n102B |  |
| DC_n7A-n102C |  |
| DC_n7A-n102D |  |
| DC_n7A-n102E |  |
| DC_n7A-n102(2A) | DC_n7A-n102A |

7.3.2	Maximum output power for NR-DC

Table 7.3.2-1: UE Power Class for uplink inter-band DC

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n7A-n102A | 23 | +2/-3 |
| DC_n7A-n102B | 23 | +2/-3 |
| DC_n7A-n102C | 23 | +2/-3 |

7.4	DC_n28-n102

7.4.1	Configurations for DC_n28-n102

Table 7.4.1-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC Configuration |
| --- | --- |
| DC_n28A-n102A | DC_n28A-n102A DC_n28A-n102B DC_n28A-n102C |
| DC_n28A-n102B |  |
| DC_n28A-n102C |  |
| DC_n28A-n102D |  |
| DC_n28A-n102E |  |
| DC_n28A-n102(2A) | DC_n28A-n102A |

7.4.2	Maximum output power for NR-DC

Table 7.4.2-1: UE Power Class for uplink inter-band DC

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n28A-n102A | 23 | +2/-3 |
| DC_n28A-n102B | 23 | +2/-3 |
| DC_n28A-n102C | 23 | +2/-3 |

7.5	DC_n78-n102

7.5.1	Configurations for DC_n78-n102

Table 7.5.1-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC Configuration |
| --- | --- |
| DC_n78A-n102A | DC_n78A-n102A DC_n78A-n102B DC_n78A-n102C |
| DC_n78A-n102B |  |
| DC_n78A-n102C |  |
| DC_n78A-n102D |  |
| DC_n78A-n102E |  |
| DC_n78A-n102(2A) | DC_n78A-n102A |
| DC_n78(2A)-n102A | DC_n78A-n102A DC_n78A-n102B DC_n78A-n102C |
| DC_n78(2A)-n102B |  |
| DC_n78(2A)-n102C |  |
| DC_n78(2A)-n102D |  |
| DC_n78(2A)-n102E |  |
| DC_n78(2A)-n102(2A) | DC_n78A-n102A |

7.5.2	Maximum output power for NR-DC

Table 7.5.2-1: UE Power Class for uplink inter-band DC

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n78A-n102A | 23 | +2/-3 |

7.6	DC_n1-n102

7.6.1	Configurations for DC_n1-n102

Table 7.6.1-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC Configuration |
| --- | --- |
| DC_n1A-n102A | DC_n1A-n102A DC_n1A-n102B DC_n1A-n102C |
| DC_n1A-n102B |  |
| DC_n1A-n102C |  |
| DC_n1A-n102D |  |
| DC_n1A-n102E |  |
| DC_n1A-n102(2A) | DC_n1A-n102A |

7.6.2	Maximum output power for NR-DC

Table 7.6.2-1: UE Power Class for uplink inter-band DC

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n1A-n102A | 23 | +2/-3 |
| DC_n1A-n102B | 23 | +2/-3 |
| DC_n1A-n102C | 23 | +2/-3 |

## 7.7	DC_n46-n77

### 7.7.1	Operating bands for DC_n46-n77

Table 7.7.1-1: Inter-band NR DC operating bands

| NR DC Band | NR Band |
| --- | --- |
| DC_n46-n77 | n46, n77 |

### 7.7.2   Configurations for DC_n46-n77

Table 7.7.2-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n46A-n77A DC_n46C-n77A DC_n46D-n77A | DC_n46A-n77A |
| DC_n46A-n77(2A) DC_n46C-n77(2A) DC_n46D-n77(2A) DC_n46(2A)-n77A DC_n46(2A)-n77(2A) | DC_n46A-n77A |

7.7.3   Maximum output power for NR-DC

Table 7.7.3-1: UE Power Class for uplink inter-band DC

| Uplink DC Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n46A-n77A | 23 | +2/-3 |

## 7.8	DC_n3-n102

7.8.1	Configurations for DC_n3-n102

Table 7.8.1-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC Configuration |
| --- | --- |
| DC_n3A-n102A | DC_n3A-n102A DC_n3A-n102B DC_n3A-n102C |
| DC_n3A-n102B |  |
| DC_n3A-n102C |  |
| DC_n3A-n102D |  |
| DC_n3A-n102E |  |
| DC_n3A-n102(2A) | DC_n3A-n102A |

7.8.2	Maximum output power for NR-DC

Table 7.8.2-1: UE Power Class for uplink inter-band DC

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n3A-n102A | 23 | +2/-3 |
| DC_n3A-n102B | 23 | +2/-3 |
| DC_n3A-n102C | 23 | +2/-3 |

## 7.9	DC_n77-n102

7.9.1	Configurations for DC_n77-n102

Table 7.9.1-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC Configuration |
| --- | --- |
| DC_n77A-n102A | DC_n77A-n102A DC_n77A-n102B DC_n77A-n102C |
| DC_n77A-n102B |  |
| DC_n77A-n102C |  |
| DC_n77A-n102D |  |
| DC_n77A-n102E |  |
| DC_n77A-n102(2A) | DC_n77A-n102A |
| DC_n77(2A)-n102A | DC_n77A-n102A DC_n77A-n102B DC_n77A-n102C |
| DC_n77(2A)-n102B |  |
| DC_n77(2A)-n102C |  |
| DC_n77(2A)-n102D |  |
| DC_n77(2A)-n102E |  |
| DC_n77(2A)-n102(2A) | DC_n77A-n102A |

7.9.2	Maximum output power for NR-DC

Table 7.9.2-1: UE Power Class for uplink inter-band DC

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n77A-n102A | 23 | +2/-3 |
| DC_n77A-n102B | 23 | +2/-3 |
| DC_n77A-n102C | 23 | +2/-3 |

## 7.10		DC_n7-n20

### 7.10.1	Operating bands for DC_n7-n20

Table 7.10.1-1: Inter-band NR DC operating bands

| NR DC Band | NR Band |
| --- | --- |
| DC_n7-n20 | n7, n20 |

### 7.10.2   Configurations for DC_n7-n20

Table 7.10.2-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n7A-n20A | DC_n7A-n20A |

7.10.3   Maximum output power for NR-DC

Table 7.10.3-1: UE Power Class for uplink inter-band DC

| Uplink DC Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n7A-n20A | 23 | +2/-3 |

7.11		DC_n20-n78

7.11.1	Operating bands for DC_n20-n78

Table 7.11.1-1: Inter-band NR DC operating bands

| NR DC Band | NR Band |
| --- | --- |
| DC_n20-n78 | n20, n78 |

7.11.2   Configurations for DC_n20-n78

Table 7.11.2-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n20A-n78(2A) | DC_n20A-n78A |

7.11.3   Maximum output power for NR-DC

Table 7.11.3-1: UE Power Class for uplink inter-band DC

| Uplink DC Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n20A-n78A | 23 | +2/-3 |

Annex <X> (informative):Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-08 | RAN4 #104e | R4-2212740 |  |  |  | TR skeleton | 0.0.0 |
| 2022-08 | RAN4 #104e | R4-2212741 |  |  |  | Include the approved TP in #104 meeting: R4-2215016 TP for TR 38.818-02-01 to include CA_n1-n26 R4-2215017 TP for TR 38.818-02-01 to include CA_n3-n26 R4-2215018 TP for TR 38.818-02-01 to include CA_n7-n26 R4-2215019 TP for TR 38.818-02-01 to include CA_n26-n78 | 0.1.0 |
| 2022-10 | RAN4 #104-bis-e | R4-2215909 |  |  |  | Include the approved TP in #104bis meeting: R4-2217034 TP for TR 38.718-02-01: CA_n26-n77 R4-2217035 TP for TR 38.718-02-01: CA_n70-n77 R4-2215742 TP for TR 38.718-02-01 CA_n12A-n25A R4-2216142 TP for TR 38.718-02-01: CA_n7-n75 R4-2217040 TP for TR 38.718-02-01: CA_n28-n94 R4-2217041 TP for TR 38.718-02-01: CA_n78-n94 R4-2216626 TP for TR 38.718-02-01 on table templates and error corrections R4-2216629 TP for TR 38.718-02-01 on corrections to REFSENS requirements for configurations CA_n3-n26 and CA_n7-n26 R4-2217042 TP for TR 38.718-02-01 CA_n75A-n78A R4-2216662 TP for TR 38.718-02-01 CA_n28A-n75A BCS2 R4-2216663 TP for TR 38.718-02-01 CA_n1A-n75A R4-2216087 TP for TR 38.718-02-01 to update CA_n3-n26 R4-2217100 TP for TR 38.718-02-01 to update CA_n7-n26 | 0.2.0 |
| 2022-11 | RAN4 #105 | R4-2219215 |  |  |  | Include the approved TP in #105 meeting: R4-2219085 TP to TR 38.718-02-01 Addition of CA_n5-n77 R4-2219086 TP to TR 38.718-02-01 Addition of CA_n7-n77 R4-2220562 TP to TR 38.718-02-01 Addition of CA_n25-n77 R4-2220563 TP to TR 38.718-02-01 Addition of CA_n66-n77 R4-2219089 TP to TR 38.718-02-01 Addition of CA_n71-n77 R4-2218646 TP for 38.718-02-01 CA_n3A-n41C with UL_n41C R4-2218967 TP for TR 38.718-02-01 to update CA_n7-n26 R4-2220784 TP to TR 38.718-02-01 Addition of CA_n25-n85 R4-2220785 TP to TR 38.718-02-01 Addition of CA_n41-n85 R4-2220786 TP to TR 38.718-02-01 Addition of CA_n66-n85 R4-2220787 TP to TR 38.718-02-01 Addition of CA_n77-n85 | 0.3.0 |
| 2023-03 | RAN4 #106 | R4-2301271 |  |  |  | Include the approved TP in #106 meeting: R4-2303578 TP to TR 38.718-02-01 for CA_n46-n78 and DC_n46-n78 R4-2303579 TP to TR 38.718-02-01 for CA_n1-n46 and DC_n1-n46 R4-2301081 TP for 38.718-02-01 to include CA_n7-n67 R4-2303592 TP for 38.718-02-01 to include CA_n67-n78 R4-2301128 TP for TR 38.718-02-01 CA_n18A-n40A R4-2303580 TP to TR 38.718-02-01 Addition of CA_n7-n102 and DC_n7-n102 R4-2303581 TP to TR 38.718-02-01 Addition of CA_n28-n102 and DC_n28-n102 R4-2303582 TP to TR 38.718-02-01 Addition of CA_n1-n102 and DC_n1-n102 R4-2303583 TP to TR 38.718-02-01 Addition of CA_n78-n102 and DC_n78-n102 R4-2303576 TP for TR 38.718-02-01 to include CA_n26-n28 R4-2302483 TP for TR 38.718-02-01: support of dual uplink CA_n7A-n8A | 0.4.0 |
| 2023-04 | RAN4 #106bis | R4-2306473 |  |  |  | Include the approved TP in #106 meeting: R4-2306503 TP to TR 38.718-02-01 Addition of CA_n3A-n7A R4-2306504 TP to TR 38.718-02-01 Addition of CA_n3A-n75A R4-2306505 TP to TR 38.718-02-01 for CA_n46-n77 and DC_n46-n77 R4-2306506 TP for TR 38.718-02-01 to include CA_n2A-n41A, CA_n2(2A)-n41A R4-2306486 TP for TR 38.718-02-01 to include CA_n5A-n71A R4-2306507 TP for TR 38.718-02-01 to include CA_n5A-n41A). R4-2306508 TP for TR 38.718-02-01 to include CA_n2A-n71A, CA_n2(2A)-n71A R4-2304752 TP for TR 38.718-02-01 to include CA_n7A-n12A R4-2306509 TP for TR 38.718-02-01 to include CA_n7A-n71A R4-2304754 TP for TR 38.718-02-01 to include CA_n12A-n41A R4-2306510 TP for TR 38.718-02-01 to include CA_n12A-n78A, CA_n12A-n78(2A) R4-2306511 TP to TR 38.718-02-01 Addition of CA_n1-n105 R4-2304976 TP to TR 38.718-02-01 Addition of CA_n3-n105 R4-2306512 TP to TR 38.718-02-01 Addition of CA_n40-n105 R4-2306513 TP to TR 38.718-02-01 Addition of CA_n78-n105 R4-2306514 TP to TR 38.718-02-01 Addition of CA_n7-n105 R4-2304980 TP to TR 38.718-02-01 Addition of CA_n3-n102 and DC_n3-n102 R4-2306515 TP to TR 38.718-02-01 Addition of CA_n77-n102 and DC_n77-n102 R4-2305143 TP for TR38.718-02-01_removing the intra-band UL CA and high power limits information R4-2306516 TP for TR38.718-02-01_CA_n3A-n41C with UL CA_n3A-n41C | 0.5.0 |
| 2023-05 | RAN4 #107 | R4-2308181 |  |  |  | Include the approved TP in #106bis and #107 meetings: R4-2310349 TP to TR 38.718-02-01 Addition of UE co-existence studies for 2 Uplink in one Intra-Band CA R4-2310355 TP to TR 38.718-02-01 Addition of CA_n78(2A) UL to CA_n78-n102 R4-2308175 TP for TR38.718-02-01_CA_n8A-n79C with n79C R4-2310411 Updated TP for TR 38.718-02-01 to intoduce BCS4 and BCS5 for CA_n5-n77 R4-2310364 TP for TR 38.718-02-01 to intoduce BCS4 and BCS5 for CA_n78-n79 R4-2310317 TP for 38.718-02-01 on MSD value due to cross band isolation for CA_n5A-n71A combination | 0.6.0 |
| 2023-08 | RAN4 #108 | R4-2312479 |  |  |  | Include the approved TP in #108 meeting: R4-2314768 TP for 38718-02-01 adding UL CA_n78(2A) to DL CA_n5A-n78(2A) R4-2314769 TP for 38718-02-01 adding UL CA_n78(2A) to DL CA_n25A-n78(2A) R4-2314770 TP for 38718-02-01 adding UL CA_n78(2A) to DL CA_n66A-n78(2A) R4-2311462 TP for 38.718-02-01 on correct CA_n3A-n75A R4-2314777 TP for TR 38.718-02-01 CA_n26-n29 R4-2314778 TP for TR 38.718-02-01 CA_n26-n48 R4-2311959 TP for TR 38.718-02-01 CA_n29-n48 R4-2314780 TP for TR38.718-02-01_CA_n34A-n39A R4-2313046 TP to TR 38.718-02-01 Addition of CA_n2A-n71A BCS4_5 R4-2313047 TP to TR 38.718-02-01 Addition of CA_n5-n71A BCS4_5 R4-2314812 TP to TR 38.718-02-01 for CA_n46-n102 R4-2314813 [NR_CADC_R18_2BDL_xBUL-Core] TP for TR 38718-02-01 to add UL CA_n77(2A) to DL CA_n1A-n77(2A) R4-2314904 [NR_CADC_R18_2BDL_xBUL-Core] TP for TR 38718-02-01 to add UL CA_n77(2A) to DL CA_n28A-n77(2A) R4-2314850 TP to 38.718-02-01 to add CA_n71-n85 | 0.7.0 |
| 2023-10 | RAN4 #108bis | R4-2316344 |  |  |  | Include the approved TP in #108bis meeting: R4-2317708 TP for 38.718-02-01 adding CA_n7-n20 and DC_n7-n20 R4-2317709 TP for 38.718-02-01 adding CA_n20A-n78(2A) and DC_n20A-n78(2A) R4-2317700 TP to TR 38.718-02-01 Addition to CA_n25-n41 of ULCA n25A-n41C R4-2316114 TP to TR 38.718-02-01 Addition to CA_n41-n66 of ULCA n41C-n66A R4-2317701 TP to TR 38.718-02-01 Addition to CA_n41-n71 of ULCA n41C-n71A R4-2317702 TP to TR 38.718-02-01 Addition to CA_n41-n77 of ULCA n41C-n77A R4-2317750 TP for TR 38.718-02-01 CA_n26-n71 | 0.8.0 |
| 2023-11 | RAN4 #109 | R4-2320107 |  |  |  | Include the approved TP in #109 meeting: R4-2321832 TP for TR38.718-02-01 Support of CA_n8-n77 R4-2321853 TP to TR 38.718-02-01 Addition to CA_n1-n102 and DC_n1-n102 R4-2321854 TP to TR 38.718-02-01 Addition to CA_n28-n102 and DC_n28-n102 R4-2321855 TP to TR 38.718-02-01 Addition to CA_n3-n102 and DC_n3-n102 R4-2321856 TP to TR 38.718-02-01 Addition to CA_n78-n102 and DC_n78-n102 R4-2321857 TP to TR 38.718-02-01 Addition to CA_n7-n102 and DC_n7-n102 R4-2321850 TP to TR 38.718-02-01 Addition of UE co-existence studies for triple beat R4-2321852 TP to TR 38.718-02-01 Addition of CA_n77-n102 and DC_n77-n102 R4-2321939 [NR_CADC_R18_2BDL_xBUL-Core] TP for TR 38718-02-01 to add UL CA_n77(2A) to DL CA_n41A-n77(2A) R4-2321925 TP for TR 38.718-02-01 to introduce CA_n34A-n40A_BCS4 and 5 with MSD analysis | 0.9.0 |
| 2024-03 | RAN4#110 | R4-2401281 |  |  |  | Include the approved TP in #110 meeting: R4-2403746 TP for TR38.718-02-01_CA_n1A-n78C R4-2403747 TP for TR38.718-02-01_CA_n3A-n78C R4-2403748 TP for TR38.718-02-01_CA_n8A-n41C R4-2403749 TP for TR38.718-02-01_CA_n8A-n78C R4-2401270 TP for TR38.718-02-01_CA_n8A-n79C R4-2403750 TP for TR38.718-02-01_CA_n28A-n41C R4-2401273 TP for TR38.718-02-01_CA_n40A-n79C R4-2403753 TP for addition of UL Intra-band CA to NR CA band combinations | 0.10.0 |
| 2024-04 | RAN4#110bis | R4-2405246 |  |  |  | Include the approved TP in #110 meeting: R4-2406692 TP for TR 38.718-02-01 CA_n7-n71 R4-2406673 TP for TR 38.718: PC3 inter-band CA_n5-n13 R4-2405237 TP for TR38.718-02-01_CA_n5A-n78C R4-2405238 TP for TR38.718-02-01_CA_n34A-n79C R4-2405239 TP for TR38.718-02-01_CA_n39A-n79C R4-2406634 TP for TR 38.718-02-01 to introduce CA_n28A-n79C with BCS4&5 R4-2405328 TP for TR 38.718-02-01 to introduce CA_n3A-n79C with BCS4&5 R4-2406684 TP for TR 38.718-02-01 to introduce CA_n78A-n104A | 0.11.0 |