# 3GPP TSG-RAN WG4 Meeting # 110bis	R4-2405342

# Changsha, China, April 15 – April 19, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rx requirement for Ka band NTN UE

Agenda Item:	6.16.5.2

Document for:	Approval

Introduction

In the RAN4#110 meeting, although the draft big CR [1] was endorsed in RAN4#110 meeting, there are a lot of open issues identified in this draft CR.

In this paper, we’d like to discuss Polarization characteristics, the maximum input level and in-band blocking requirements for VSAT Rx requirements.

Discussion

Polarization characteristics

Based on current RRC signaling design, only either RHCP or LHCP can be indicated by SAN. Thus, there is no need to specify the requirements which can support both RHCP and LHCP simultaneously together.

Proposal 1: Either RHCP or LHCP is supported by Ka band VSAT in Rel-18.

Maximum input level (WF R4-2403649)

If we assume 66dbm EIRP for SAN LEO400, the DL link budget is shown below for LEO400.

Table 5 the calculation of DL link budgets for LEO600

|  | LEO600 |
| --- | --- |
| SAN EIRP (dBm) | 66 |
| Distance (m) | 400000 |
| center frequency (GHz) | 17 |
| freePathloss | 169.1 |
| Received signal level at OTA (dBw) | -133.09 |
| Received signal level at OTA (dBm) | -103.09 |

Considering some margins, it’s proposed to specify -101dBm as OTA maximum input level for Ka band NTN UE.

Proposal 2: it’s proposed to specify -101dBm as OTA maximum input level for (type 3 UE) fixed VSAT supporting LEO only with electronical steering antenna.

As the SNR can be higher than 10dB, the 64QAM can be used for the maximum input level tests.

Proposal 3: it’s proposed to define 64QAM for maximum input level tests.

Summary

Proposal 1: Either RHCP or LHCP is supported by Ka band VSAT in Rel-18.

Proposal 2: it’s proposed to specify -101dBm as OTA maximum input level for (type 3 UE) fixed VSAT supporting LEO only with electronical steering antenna.

Proposal 3: it’s proposed to define 64QAM for maximum input level tests.

- References

[1] R4-2401116, Big CR on TS38.101-5 for UE RF Requirements, Samsung Electronics France SA