3GPP TSG-RAN WG4 Meeting #110bis 	R4-2405359

# Changsha, China, 15th – 19th April, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR on UL transmit timing requirements for R18 LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 17) Rel-17 (Release 18) Rel-18 (Release 19) Rel-19 (Release 20) |  |
| Reason for change: |  | 1. As discussed in P5-P8 and P12 of R4-2405357, the Te requirements and the related compontents are specified here. 2. As discussed in P9 of R4-2405357, introduce uplink timing acurracy requirements for UE based TA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Capture the Te requirements for PDCCH-ordered RACH, and the related conditions to determine the RACH occasion. 2. Specify RRM requirements for UE-based TA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. The Te requirements for PDCCH-ordered RACH is missing. 2. No RRM requirements for UE-based TA. The feature is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

7.1	UE transmit timing

7.1.1	Introduction

The UE shall have capability to follow the frame timing change of the reference cell in connected state or when transmiting PUSCH on CG resources for SDT in RRC_Inactive. The uplink frame transmission takes place  before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. For serving cell(s) in pTAG, UE shall use the SpCell as the reference cell for deriving the UE transmit timing for cells in the pTAG. For serving cell(s) in sTAG, UE shall use any of the activated SCells as the reference cell for deriving the UE transmit timing for the cells in the sTAG. UE initial transmit timing accuracy and gradual timing adjustment requirements are defined in the following requirements.

In the requirements of clause 7.1.2, the term reference cell on a carrier frequency subject to CCA is not available at the UE refers to when at least one SSB is configured by gNB, but the first two successive candidate SSB positions for the same SSB index within the discovery burst transmission window are not available during at least one discovery burst transmission window, at the UE due to DL CCA failures at gNB during the last 1280 ms; otherwise the reference cell on the carrier frequency subject to CCA is considered as available at the UE.

For UE supporting [RACH-based early TA acquisition] for LTM, and if the candidate cell is a neighbor cell or a secondary serving cell without uplink carrier,

-	UE shall have capability to follow the frame timing of the reference cell. The PRACH transmission for early TA acquisition takes place  before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. UE shall use this candidate cell as the reference cell for deriving transmit timing. UE initial transmit timing accuracy is defined in the following requirements.

For multi-DCI based multi-TRP operation with two TAs, for each TAG, the uplink transmission timing takes place (NTA+NTA offset)×Tc before the reception of the first detected path (in time) of the corresponding downlink reference signal associated with UL-TCIState or DLorJointTCIState (if unifiedTCI-StateType is indicated as Joint) , where NTA  is commanded by the network independently for each TAG [TS 38.331].

In the requirements of clause 7.1.2, the SSB applies for both CD-SSB and NCD-SSB if the UE supports [UE capability IE for FG 53-3].

7.1.2	Requirements

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.1.2-1. This requirement applies:

-	when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS, or it is the PRACH transmission, or it is the msgA transmission, or it is the first transmission sent on the PSCell for activating the deactivated SCG without RACH.

-	when it is the transmission for PUSCH on CG resources for SDT in RRC_Inactive.

-	when it is the first transmission on target cell after UE receives LTM cell switch command.

In case UE supports [UE-based TA capability] for LTM, if UE-MeasuredTA-ID is configured, UE shall be able to autonomously adjust the uplink timing of the first transmission on target cell after LTM cell switch. The transmit timing adjustment requirement is defined in 7.1.2.5, and the requirements apply assuming the TAE between cells with the same UE-MeasuredTA-ID is less than 260ns as defined TS 38.104.

When the UL SCS is 120 kHz or smaller,

- If UE supports [RACH-based early TA acquisition, 45-5 in RAN1 feature list] for LTM, after receving the PDCCH order, the UE shall meet the Te requirement for an initial transmission to a candidate cell at the PRACH occasion defined in clause 8.1 of TS 38.213, provided:

  - The SSB of the candidate cell that associated to the PDCCH-ordered RACH is known, and

  - TSSB in clause 8.1 of TS 38.213 is

    - TSSB = 0, if UE supports [Early TCI activation, 45-3a or 45-4a in RAN1 feature list] for LTM, and the TCI activation command for candidate cell is received at least [THARQ + 3Nslotsubframe,µ+ (Tfirst-SSB + TSSB-proc) / NR slot length] before the PDCCH order is received, and measurement period of L1-RSRP for this SSB is no longer than 160ms

    - Otherwise, TSSB = Tfirst_SSB + TSSB_proc

  - where

    - Tfirst_SSB is

      - the SSB periodicity of the candidate cell if SSB is within active BWP of the UE, or

      - the measurement gap periodicity when SSB is outside active BWP of the UE

    - TSSB_proc is the delay of the SSB processing, which is 2ms.

    - An SSB associated to PDCCH-ordered RACH to candidate cell is known, if it is indicated based on the L1-RSRP report or the L3-RSRP report with SSB index for this candidate cell within [5] seconds.

  - and ∆RF/BB_preparation in clause 8.1 of TS 38.213 is

    - ∆RF/BB_preparation = 0, for the case of PRACH bandwidth within active UL BWP

    - ∆RF/BB_preparation is DCI based BWP switching delay specified in clause 8.6, for the case of PRACH bandwidth outside active UL BWP but within one of configured UL BWPs of any active serving cell.

    - ∆RF/BB_preparation is based on the reported value in [UE capability for RF/BB retuning, 39-5 in RAN4 UE feature list], for the case of PRACH bandwidth not within any of the configured UL BWPs of any active serving cell.

- Otherwise, the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms.

When the UL SCS is 480 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available in the last 80 ms. When the UL SCS is 960 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available in the last 40 ms.

The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the reference cell minus . The downlink timing is defined as the time when the first path (in time) of the corresponding downlink frame used by the UE to determine downlink timing is received from the reference cell at the UE antenna. NTA for PRACH is defined as 0.

(in Tc units) for other channels is the difference between UE transmission timing and the downlink timing immediately after when the last timing advance in clause 7.3 was applied. NTA for other channels is not changed until next timing advance is received. The value ofdepends on the duplex mode of the cell in which the uplink transmission takes place and the frequency range (FR). is defined in Table 7.1.2-2.

For multi-DCI based multi-TRP operation with two TAs, UE initial transmission timing error requirements specified in this clause is applicable for each TAG and shall be met for each TAG separetely. The reference point described in this clause for each TAG is the the first detected path (in time) of the corresponding downlink reference signal associated with UL-TCIState or DLorJointTCIState (if unifiedTCI-StateType is indicated as Joint) [TS 38.331].

Table 7.1.2-1: Te Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te |
| --- | --- | --- | --- |
| 1 | 15 | 15 | 12*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 10*64*Tc |
|  | 30 | 15 | 8*64*Tc |
|  |  | 30 | 8*64*Tc |
|  |  | 60 | 7*64*Tc |
| 2-1 | 120 | 60 | 3.5*64*Tc |
|  |  | 120 | 3.5*64*Tc |
|  | 240 | 60 | 3*64*Tc |
|  |  | 120 | 3*64*Tc |
| 2-2 | 120 | 120 | 3.5*64*Tc |
|  |  | 480 | 1.58*64*Tc |
|  | 480 | 120 | 2.86*64*Tc |
|  |  | 480 | 1.35*64*Tc |
|  |  | 960 | 0.90*64*Tc |
|  | 960 | 120 | 2.80*64*Tc |
|  |  | 480 | 1.13*64*Tc |
|  |  | 960 | 0.86*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Table 7.1.2-2: The Value of

| Frequency range and band of cell used for uplink transmission | (Unit: TC) |
| --- | --- |
| FR1 FDD or TDD band with neither E-UTRA–NR nor NB-IoT–NR coexistence case | 25600 (Note 1) |
| FR1 FDD band with E-UTRA–NR and/or NB-IoT–NR coexistence case | 0 (Note 1) |
| FR1 TDD band with E-UTRA–NR and/or NB-IoT–NR coexistence case | 39936 (Note 1) |
| FR2 | 13792 |
| Note 1: The UE identifies based on the information n-TimingAdvanceOffset as specified in TS 38.331 [2]. If UE is not provided with the information n-TimingAdvanceOffset, the default value of is set as 25600 for FR1 band. In case of multiple UL carriers in the same TAG, UE expects that the same value of n-TimingAdvanceOffset is provided for all the UL carriers according to clause 4.2 in TS 38.213 [3] and the value 39936 of can also be provided for a FDD serving cell. Note 2: Void |  |

When it is not the first transmission in a DRX cycle or there is no DRX cycle, and when it is the transmission for PUCCH, PUSCH and SRS transmission, the UE shall be capable of changing the transmission timing according to the received downlink frame of the reference cell except when the timing advance in clause 7.3 is applied.

Table 7.1.2-3: void

If the UE uses a reference cell on a carrier frequency subject to CCA for deriving the UE transmit timing, then the UE shall meet all the transmit timing requirements defined in clause 7.1.2 provided that the reference cell is available at the UE. If the reference cell is not available at the UE on a carrier frequency subject to CCA, then the UE is allowed to transmit in the uplink provided that the UE meets all the transmit timing requirements defined in clause 7.1.2; otherwise the UE shall not transmit any uplink signal.

If a reference cell on a carrier frequency belonging to the PTAG, which is subject to CCA, is not available at the UE then the UE is allowed to use any of available activated SCell(s) at the UE in PTAG as a new reference cell. If the SCell used as reference cell is deactivated, or becomes not available, the UE is allowed to use another active serving cell in PTAG as new reference cell.

If a reference cell on a carrier frequency belonging to the STAG, which is subject to CCA is not available at the UE then the UE is allowed to use any of available activated SCell(s) at the UE in STAG as a new reference cell.

7.1.2.1	Gradual timing adjustment

Requirements in this section shall apply regardless of whether the reference cell is on a carrier frequency subject to CCA or not.

When the transmission timing error between the UE and the reference timing exceeds Te then the UE shall adjust the timing such that timing error is to within Te. The reference timing shall be  before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change in one adjustment shall be Tq.

2)	The minimum aggregate adjustment rate shall be Tp per second.

3)	The maximum aggregate adjustment rate shall be Tq per 200 ms for SCS of UL signals smaller or equal to 120 kHz and 100 ms for SCS of upling signals larger or equal to 480 kHz.

where the maximum autonomous time adjustment step Tq and the aggregate adjustment rate Tp are specified in Table 7.1.2.1-1.

For multi-DCI based multi-TRP operation with two TAs, gradual timing adjustment specified in this clause applies for each TAG and shall be met for each TAG. The reference timing described in this clause for each TAG is the the first detected path (in time) of the corresponding downlink reference signal associated with UL-TCIState or DLorJointTCIState (if unifiedTCI-StateType is indicated as Joint) [TS 38.331].

Table 7.1.2.1-1: Tq Maximum Autonomous Time Adjustment Step and Tp Minimum Aggregate Adjustment rate

| Frequency Range | SCS of uplink signals (kHz) | Tq | Tp |
| --- | --- | --- | --- |
| 1 | 15 | 5.5*64*Tc | 5.5*64*Tc |
|  | 30 | 5.5*64*Tc | 5.5*64*Tc |
|  | 60 | 5.5*64*Tc | 5.5*64*Tc |
| 2-1 | 60 | K*64*Tc | 2.5*64*Tc |
|  | 120 | K*64*Tc | 2.5*64*Tc |
| 2-2 | 120 | 2.5*64*Tc | 2.5*64*Tc |
|  | 480 | 0.8*64*Tc | 0.8*64*Tc |
|  | 960 | 0.8*64*Tc | 0.8*64*Tc |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6] NOTE 2: When highSpeedMeasFlagFR2-r17 is configured for UE supporting power class 6, K = 4.5; otherwise, K = 2.5. |  |  |  |

7.1.2.2	Void

Table 7.1.2.2-1: Void

7.1.2.3	One shot large UL timing adjustment for FR2 Power Class 6 UE

When highSpeedMeasFlagFR2-r17 is configured and highSpeedLargeOneStepUL-TimingFR2-r17 is enabled for UE supporting FR2 power class 6 and ue-OneShotUL-TimingAdj-r17 capability, the following requirements apply to the UE:

If the UE also indicates to support [highSpeedTCISwitchEnhMAC-CE-FR2-r18] and [R18 enhanced MAC-CE indication] is indicated as ‘1’ for the TCI state switch

-	the UE transmit timing immediately after TCI state switch shall be Tnew-(NTA+NTA offset)+2 (Told-Tnew) and clause 7.1.2.1 requirements don’t apply.

-	The UE UL transmission timing error after the TCI state switching procedure shall be less than or equal to ±Te as specified in clause 7.1.2 if the new target TCI state is within active TCI state list, otherwise ±7*64*Tc, and the reference point is Tnew-(NTA+NTA offset)+2 (Told-Tnew).

If the UE also indicates to support [highSpeedTCISwitchEnhMAC-CE-FR2-r18] and [R18 enhanced MAC-CE indication] is indicated as ‘0’ for the TCI state switch

-	the requirement in clause 7.1.2.1 apply to the first UL transmission after a TCI state switch.

If the UE does not indicate to support [highSpeedTCISwitchEnhMAC-CE-FR2-r18] or [R18 enhanced MAC-CE indication] is not indicated for the TCI state switch

-	If the absolute value Told-Tnew≤CP/4, the requirement in clause 7.1.2.1 apply to the first UL transmission after a TCI state switch.

-	Otherwise, the UE transmit timing immediately after TCI state switch shall be Tnew-(NTA+NTA offset)+2 (Told-Tnew) and clause 7.1.2.1 requirements don’t apply.

-	The UE UL transmission timing error after the TCI state switching procedure shall be less than or equal to ±Te as specified in clause 7.1.2 if the new target TCI state is within active TCI state list, otherwise ±7*64*Tc, and the reference point is Tnew-(NTA+NTA offset)+2 (Told-Tnew).

Above,

-	Tnew (in Tc units) is the DL timing defined as the time when UE receives downlink frame with new target TCI state.

-	Told  (in Tc units) is the DL timing defined as the time when UE receives downlink frame with old source TCI state.

7.1.2.4	UE transmit timing for positioning measurements

If cell reselection occurs in RRC_Inactive within srs-PosRRC-InactiveValidityArea area, the UE shall have capability to follow the frame timing change of the reference cell when transmiting SRS for positioning. The uplink frame transmission takes place NTA+NTA offset×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. The UE shall use the current camping cell as reference cell for deriving the UE transmit timing. Gradual timing adjustment requirements are defined in the following requirements.

When ueAotonomousTaAdjustment is configured, the following requirements apply to the UE supporting [AutonomousAdjustOneStepUL-Timing-r18]:

If the DL timing difference is ≥ CP/4, UE autonomously adjusts the TA based on twice of the DL timing difference

-	The UE  UL transmission timing error after autonomous TA adjustment shall be less than or equal to Te in clause 7.1.2.3.

If the DL timing difference is < CP/4, UE follows the DL timing of the new camping cell by performing gradual timing adjustment as defined in clause 7.1.2.1.

The above DL timing difference is timing difference between the last camping cell and current camping cell.

7.1.2.5	UE transmit timing for UE-based TA adjustment in LTM

If UE supports [UE-based TA capability] for LTM, and UE-MeasuredTA-ID is configured, the UE shall follow the frame timing change of the target cell when transmiting the first the uplink transmission to target cell after LTM cell switch. The uplink frame transmission takes place NTA+NTA offset×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the target cell. The UE shall also use the source SpCell as one of the reference cells for deriving DL timing difference. Then, UE autonomously adjusts the TA based on twice of the DL timing difference, and the UL transmission timing error after autonomous TA adjustment shall be less than or equal to Te

The above DL timing difference is timing difference between the source SpCell and the target cell.

<End of Change #1>