3GPP TSG-RAN WG4 Meeting #110bis 	R4-2405363

# Changsha, China, 15th – 19th April, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR on inter-frequency RACH based Cell switch from FR1 to FR1 for R18 LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 17) Rel-17 (Release 18) Rel-18 (Release 19) Rel-19 (Release 20) |  |
| Reason for change: |  | Introduce test case for inter-frequency RACH based Cell switch from FR1 to FR1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce test case for inter-frequency RACH based Cell switch from FR1 to FR1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case is not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.3.x.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

A.6.3.x.5	Inter-frequency LTM RACH based cell switch from FR1 to FR1

A.6.3.x.5.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 inter-frequency LTM RACH based cell switch delay requirements specified in clause 6.3.1. This test is for UE supporting inter-frequency L1 measurement with gaps.

A.6.3.x.5.2	Test Parameters

Supported test configurations are shown in table A.6.3.X.5.2-1. LTM cell switch delay, interruption length and Te requirements for the first uplink transmission are tested by using the parameters in table A.6.3.X.5.2-2, A.6.3.X.5.2-3, and A.6.3.X.5.2-4.

The test consists of a preparation period T0 for the repeated test, and 6 successive time periods, with time durations of T1, T2, T3, T4, T5 and T6 respectively.

Before the start of time duration T1, UE has received Preconfiguration of candidate cells, L3 measurement configuration, L1 measurement configuration of Cell 1. Cell 2 is configured as one candidate SpCell in the LTM-config of Cell 1. The UE has sent one A3-event based L3 report to Cell 1 which triggeres the L1 measurement on Cell 2. A L1 report from UE implies the start of T1.

During T1, UE shall continuously send L1 reports to TE. Based on L1 reports, TE sends the MAC CE based LTM cell switch command indicates one of the configured TCI as the TCI used in Cell 2. This implies the start of T2.

During T2, UE shall perform LTM cell switch from Cell 1 to Cell 2. The end of T2 is the time UE sends PRACH to the target cell. This implies the start of T3. Based on the decoded configuration for Cell 2, UE receives preconfiguration of candidate cells, L3 measurement configuration, L1 measurement configuration of Cell 2, and Cell 1 is configured as one candidate SpCell in the LTM-config of Cell 2.

During T3, UE sends A3-event based L3 report to Cell 2 which triggeres the L1 measurement on Cell 1. A L1 report from UE implies the start of T4.

During T4, UE shall continuously send L1 reports to TE. Based on L1 reports, TE sends the MAC CE based LTM cell switch command indicates one of the configured TCI as the TCI used in Cell 1. This implies the start of T5.

During T5, UE shall perform LTM cell switch from Cell 2 to Cell 1. The end of T5 is the time UE sends PRACH to the target cell. This implies the start of T6. Based on the decoded configuration for Cell 1, UE receives preconfiguration of candidate cells, L3 measurement configuration, L1 measurement configuration of Cell 1, and Cell 2 is configured as one candidate SpCell in the LTM-config of Cell 2.

During T6, UE sends A3-event based L3 report to Cell 1 which triggeres the L1 measurement on Cell 2. A L1 report from UE implies the start of T1.

Table A.6.3.x.5.2-1: Inter-frequency RACH based cell switch from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.x.5.2-2: General test parameters Inter-frequency RACH based cell switch from FR1 to FR1

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| A3-Offset | dB | 0 |  |
| Hysteresis | dB | 0 |  |
| Time To Trigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| Measurement gap pattern Id |  | #0 | Gaps are configured before T1. |
| Access Barring Information | - | Not Sent | No additional delays in random access procedure. |
| ltm-CSI-SSB-ResourceList |  | SSB.0 of Cell 1 SSB.0 of Cell 2 |  |
| ltm-ReportConfigType |  | periodic |  |
| ltm-ReportContent |  | Number of reported cells: 1 Number of reported RSs per Cell: 1 |  |
| Time offset between cells |  | < 3 s | Synchronous cells |
| TA |  | Not present | In cell switch command |
| T0 | ms | 5s |  |
| T1 | ms | < 200ms |  |
| T2 | ms | DLTM | Cell switch from Cell 1 to Cell 2, DLTM is defined in 6.3.1.2 |
| T3 | ms | < 200ms |  |
| T4 | ms | < 200ms |  |
| T5 | ms | DLTM | Cell switch from Cell 2 to Cell 1, DLTM is defined in 6.3.1.2 |
| T6 | ms | < 200ms | In repeated test, the end of T6 is the start of T1. |

Table A.6.3.x.5.2-3: Cell specific test parameters for NR FR1-FR1 Inter frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1-T2 | T3-T5 | T6 | T1-T2 | T3-T5 | T6 |
| NR RF Channel Number |  |  |  | 1 |  |  | 1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |
| measurement channel |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |
| LTM-TCI-Info |  | Config 1 |  | DLorJointTCI:TBD; ULTCI: TBD |  |  |  |  |  |
|  |  | Config 2 |  | DLorJointTCI:TBD; ULTCI: TBD |  |  |  |  |  |
|  |  | Config 3 |  | DLorJointTCI:TBD; ULTCI: TBD |  |  |  |  |  |
| TRS configuration for serving cell |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |
| [TRS configuration for candidate cell] |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |
|  |  |  | dB | 8 | 4 | 8 | 4 | 8 | 4 |
|  |  |  | dB | 8 | 4 | 8 | 4 | 8 | 4 |
| SSB_RP | Config 1,2 |  | dBm/SCS | -87 | -90 | -87 | -90 | -87 | -90 |
|  | Config 3 |  | dBm/SCS | -84 | -87 | -84 | -87 | -84 | -87 |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -57.06 | -61.41 | -57.06 | -61.41 | -57.06 | -61.41 |
|  | Config 3 |  | dBm/ 9.36MHz | -50.96 | -55.31 | -50.96 | -55.31 | -50.96 | -55.31 |
| Propagation condition |  |  | - | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

A.6.3.x.5.3 Test Requirements

The UE shall start to transmit the first uplink transmission, i.e. PRACH, to Cell 2 less than DLTM from the beginning of time period T2, while the first uplink transmission to Cell 2 shall meet the Te requirements to target cell.

The UE shall start to transmit the first uplink transmission, i.e. PRACH, to Cell 1 less than DLTM from the beginning of time period T5, while the first uplink transmission to Cell 1 shall meet the Te requirements to target cell.

DLTM is specified in 6.3.1.2 and Te requirements is specified in 7.1 for LTM.

The rate of correct cell switch observed during repeated tests shall be at least 90%.

NOTE:	The cell switch delay can be expressed as: MAC CE decoding delay + Tinterrupt, where:

MAC CE decoding delay = 3 ms and is specified in clause 6.3.1.2.

Tinterrupt = TLTM-RRC-processing + TLTM-processing + max(Tfirst-RS + TRS-proc, Tfirst_target-PL-RS + [2]*Ttarget_PL-RS + 2ms) + TLTM-IU = 10 + 20 + max(20ms+2ms, 20*3+2ms)+ 10 ms in the test. Tinterrupt is defined in clause 6.3.1.3.

This gives a total of 102 ms.

<End of Change #1>