# 3GPP TSG-RAN WG4 Meeting #110bis	R4-2405378

Changsha, China, 15th  – 19th  April, 2024

Title:	[Draft] draftLS to RAN2 on IE intraBandENDC-supportUL

Response to:

Release:	Rel-18

Work Item:	TEI18

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name: Rui Zhou

E-mail Address: zhourui2@oppo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

During the Rel-17 timeline, the ambiguity issue on IE supportedBandwidthCombinationSetIntraENDC and IE intraBandENDC-Support have been confirmed and an LS [1] has been sent to RAN2 to identify this issue.

For Rel-18, as new band combinations are requested, similar issue for IE intraBandENDC-SupportUL has been confirmed. This kind of inter-band EN-DC band combinations with multiple additional intra-band EN-DC UL configuration will have ambiguity issue as only one IE intraBandENDC-SupportUL cannot correctly indicate the contiguous and non-contiguous capability for multiple intra-band EN-DC band combination.

Hence, RAN4 would like to request RAN2 to check how to solve the ambiguous signaling issues for the inter-band EN-DC band combinations with multiple additional intra-band EN-DC components.

The on-going Rel-18 inter-band EN-DC band combinations are listed below for information.

| Rel-18 inter-band EN-DC band combinations (on-going) |  |
| --- | --- |
| DC_2A-66A_n2A-n66A | DC_2A_n2A DC_2A_n66A DC_66A_n2A DC_66A_n66A |
| DC_2A-5A-66A_n2A-n66A | DC_2A_n2A DC_2A_n66A DC_5A_n2A DC_5A_n66A DC_66A_n2A DC_66A_n66A |
| DC_2A-7A-66A_n2A-n66A | DC_2A_n2A DC_2A_n66A DC_7A_n2A DC_7A_n66A DC_66A_n2A DC_66A_n66A |
| DC_2A-12A-66A_n2A-n66A | DC_2A_n2A DC_2A_n66A DC_12A_n2A DC_12A_n66A DC_66A_n2A DC_66A_n66A |
| DC_2A-66A-71A_n2A-n66A | DC_2A_n2A DC_2A_n66A DC_66A_n2A DC_66A_n66A DC_71A_n2A DC_71A_n66A |
| DC_2A-5A-7A-66A_n2A-n66A | DC_2A_n2A DC_2A_n66A DC_5A_n2A DC_5A_n66A DC_7A_n2A DC_7A_n66A DC_66A_n2A DC_66A_n66A |
| DC_2A-7A-12A-66A_n2A-n66A | DC_2A_n2A DC_2A_n66A DC_7A_n2A DC_7A_n66A DC_12A_n2A DC_12A_n66A DC_66A_n2A DC_66A_n66A |
| DC_2A-7A-66A-71A_n2A-n66A | DC_2A_n2A DC_2A_n66A DC_7A_n2A DC_7A_n66A DC_66A_n2A DC_66A_n66A DC_71A_n2A DC_71A_n66A |

Reference

[1] R4-2403809 LS on IE supportedBandwidthCombinationSetIntraENDC and IE intraBandENDC-Support

2. Actions:

To RAN2:

ACTION: RAN4 respectfully asks RAN2 to take the above decisions into account to update RAN2 specifications.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #111                  20th May – 24th May 2024               Fukuoka City, Japan

TSG-RAN WG4 Meeting #112                  19th August – 23rd  August 2024              Maastricht, NL