3GPP TSG-RAN WG4 Meeting # 110Bbis	R4-2405382

# Changsha, China,  15th April – 19th April, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.786 | CR | draftCR | rev | <> | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR to TR 38.786 for Rel-18 A-MPR simulation results |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-04-08 |
|  |  |  |  |  |  |  |  |  |  | 0 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add the simulation result for NS_28. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The simulation result for NS_28 is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MPR to NS_28 is not defined properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.2.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | . |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<start of change>>

6.1.3.1	A-MPR for SL-U with NS_28

6.1.3.1.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.1.1.1	Qualcomm’s simulation results (R4-2316791)

Table 6.1.3.1.1.1-1: Simulation results for NS_28 Allocation Full and partial allocations.

| NS_28 A-MPR | 20M | 20M | 40M | 40M | 60M | 60M | 80M | 80M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial | Full | Partial | Full | Partial |
| QPSK | ≤8.0 | ≤8.1 | ≤7.7 | ≤8.0 | ≤7.1 | ≤7.8 | ≤6.9 | ≤7.8 |
| 16 QAM | ≤8.0 | ≤8.3 | ≤7.4 | ≤8.3 | ≤7.3 | ≤8.1 | ≤6.9 | ≤8.1 |
| 64 QAM | ≤8.1 | ≤8.7 | ≤7.4 | ≤8.7 | ≤7.3 | ≤8.2 | ≤7.0 | ≤8.2 |
| 256 QAM | ≤8.2 | ≤8.8 | ≤7.4 | ≤8.8 | ≤7.3 | ≤8.2 | ≤7.1 | ≤8.1 |

The power class 5 PSSCH A-MPR in the tables should be considered in the discussion of MPR requirements.

6.1.3.1.1.2	LG Electronics’ simulation results (R4-2401464)

Table 6.1.3.1.1.2-1 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 6.1.3.1.1.2-1: NS_28-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| '20MHz' (5480) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 5.47 | 5.89 | 5.48 | 5.90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 5.47 | 5.89 | 5.48 | 5.90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 5.47 | 5.90 | 5.47 | 5.90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.90 | 5.48 | 5.90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '20MHz' (5500) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.16 | 2.16 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.16 | 2.16 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.16 | 3.17 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.48 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5490) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 5.06 | 5.47 | 5.47 | 5.89 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 5.06 | 5.47 | 5.47 | 5.89 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 5.06 | 5.48 | 5.47 | 5.90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.48 | 5.47 | 5.89 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5510) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 3.53 | 3.91 | 2.81 | 0.39 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 3.53 | 3.90 | 2.81 | 0.40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.53 | 3.91 | 2.81 | 2.15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5550) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.81 | 0.00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.81 | 0.40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.48 | 2.81 | 2.15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '60MHz' (5500) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |
|  | 'QPSK' | 4.67 | 5.07 | 5.47 | 5.90 | 5.06 | 5.48 | 2.13 | 0.11 |
|  | '16QAM' | 4.67 | 5.07 | 5.47 | 5.89 | 5.06 | 5.47 | 2.13 | 0.10 |
|  | '64QAM' | 4.67 | 5.07 | 5.47 | 5.90 | 5.06 | 5.47 | 3.15 | 2.12 |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.89 | 5.47 | 5.47 | 5.47 | 5.05 |
| '60MHz' (5520) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 3.90 | 4.67 | 2.14 | 0.40 | 3.53 | 4.28 | 2.13 | 0.35 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 3.90 | 4.29 | 2.14 | 0.40 | 3.53 | 4.28 | 2.13 | 0.10 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.90 | 4.67 | 3.17 | 2.15 | 3.53 | 4.28 | 3.15 | 2.12 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.88 | 5.05 |  |  |  |  |  |  |  |  |  |  |
| '60MHz' (5540) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.82 | 2.14 | 0.40 | 2.47 | 0.66 | 2.13 | 0.35 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.82 | 2.14 | 0.40 | 2.47 | 0.66 | 2.13 | 0.36 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.17 | 2.82 | 3.17 | 2.15 | 3.17 | 2.48 | 3.16 | 2.12 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.06 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.05 |  |  |  |  |  |  |  |  |  |  |
| '60MHz' (5600) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.14 | 0.39 | 2.47 | 0.66 | 2.13 | 0.10 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.14 | 0.40 | 2.47 | 0.66 | 2.13 | 0.36 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.17 | 2.48 | 3.17 | 2.15 | 3.16 | 2.47 | 3.16 | 2.12 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.88 | 5.05 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' (5510) | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 4.28 | 4.67 | 5.47 | 5.47 | 5.06 | 5.47 | 4.67 | 5.06 | 2.46 | 0.10 | 2.46 | 3.16 |  |  |  |  |  |  |
|  | '16QAM' | 4.28 | 4.67 | 5.47 | 5.89 | 5.06 | 5.47 | 4.67 | 5.07 | 2.46 | 0.09 | 2.46 | 3.16 |  |  |  |  |  |  |
|  | '64QAM' | 4.28 | 4.67 | 5.47 | 5.47 | 5.06 | 5.07 | 4.67 | 5.06 | 3.16 | 2.12 | 3.16 | 3.15 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.90 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.05 | 5.47 | 5.06 |  |  |  |  |  |  |
| '80MHz' (5530)' | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 3.90 | 4.28 | 2.14 | 0.40 | 3.53 | 4.28 | 3.90 | 4.28 | 2.46 | 0.09 | 2.13 | 0.37 |  |  |  |  |  |  |
|  | '16QAM' | 3.53 | 4.28 | 2.15 | 0.39 | 3.53 | 4.28 | 3.90 | 4.28 | 2.46 | 0.36 | 2.13 | 0.63 |  |  |  |  |  |  |
|  | '64QAM' | 3.53 | 4.28 | 3.16 | 2.14 | 3.53 | 4.28 | 3.90 | 4.67 | 3.15 | 2.12 | 3.16 | 2.46 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.06 | 5.47 | 5.08 | 5.47 | 5.07 | 5.47 | 5.06 | 5.46 | 5.05 | 5.47 | 5.06 |  |  |  |  |  |  |
| '80MHz' (5610)' | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |
|  | 'QPSK' | 2.48 | 2.48 | 2.14 | 0.15 | 2.47 | 0.39 | 2.47 | 0.40 | 2.46 | 0.10 | 2.13 | 0.36 |
|  | '16QAM' | 2.48 | 2.48 | 2.14 | 0.40 | 2.47 | 0.39 | 2.47 | 0.66 | 2.46 | 0.09 | 2.13 | 0.64 |
|  | '64QAM' | 2.82 | 2.48 | 3.16 | 2.15 | 2.81 | 2.48 | 3.16 | 2.48 | 3.15 | 2.12 | 3.16 | 2.46 |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.08 | 5.47 | 5.07 | 5.47 | 5.06 | 5.46 | 5.06 | 5.46 | 5.06 |

Table 6.1.3.1.1.2-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 6.1.3.1.1.2-2: NS_28-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | RB Allocation /Centre frequency of CBW (MHz) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz : 5480 40MHz : 5490/5510 60MHz : 5500/5520 80MHz : 5510/5530 |  |  |  | 20MHz : 5500 40MHz : 5550 60MHz : 5540/5600 80MHz : 5610 |  |  |  |
|  |  | Outer RB set configuration |  | Inner RB set configuration |  | Outer RB set configuration |  | Inner RB set configuration |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | 5.48 | 5.90 | 2.46 | 3.16 | 2.81 | 2.82 | 2.46 | 0.36 |
|  | 16 QAM | 5.48 | 5.90 | 2.46 | 3.16 | 2.81 | 2.82 | 2.46 | 0.64 |
|  | 64 QAM | 5.47 | 5.90 | 3.16 | 3.15 | 3.17 | 2.82 | 3.16 | 2.46 |
|  | 256 QAM | 5.48 | 5.90 | 5.47 | 5.06 | 5.48 | 5.08 | 5.88 | 5.06 |

The following centre frequencies for each CBW can be assumed to have same A-MPR in Table 6.1.3.1.1.2-2 (left side A-MPR in Table 6.1.3.1.1.2-2) because the distance from the boundary of regulation ‘-30dBm/MHz’ is effectively the same.

- 20MHz

: A-MPR of 5480 MHz = A-MPR of 5160/5340/5700 MHz

- 40MHz

: A-MPR of 5490 MHz = A-MPR of 5170/5330 MHz

: A-MPR of 5510 MHz = A-MPR of 5190/5310 MHz

- 60MHz

: A-MPR of 5500 MHz = A-MPR of 5180/5320 MHz

: A-MPR of 5520 MHz = A-MPR of 5200/5300/5680 MHz

- 80MHz

: A-MPR of 5510 MHz = A-MPR of 5190/5310 MHz

: A-MPR of 5530 MHz = A-MPR of 5210/5290 MHz

Considering implementation margin and the centre frequencies on the right side of A-MPR in Table Table 6.1.3.1.1.2-2, such as, 5500MHz for CBW 20MHz, 5550MHz for CBW 40MHz, 5540/5600MHz for CBW 60MHz, and 5610MHz for CBW 80MHz, the SL-U PSSCH/PSCCH MPR requirement in Table 6.2E.2F-1 of TS 38.101-1 can be reused as A-MPR requirement.

As a result, considering implementation margin and the centre frequency, Table 6.1.3.1.1.2-3 can be proposed for SL-U NS_28 PSSCH/PSCCH A-MPR.

Table 6.1.3.1.1.2-3. NS_28 PSSCH/PSCCH A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation (Note 3) |  |  |  | RB Allocation (Note 4) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 7.0 | ≤ 4.0 | ≤ 5.0 | Table 6.2E.2F-1 |
|  | 16 QAM | ≤ 6.0 | ≤ 7.5 | ≤ 4.0 | ≤ 5.0 |  |
|  | 64 QAM | ≤ 6.5 | ≤ 7.5 | ≤ 4.5 | ≤ 5.5 |  |
|  | 256 QAM | ≤ 7.5 | ≤ 8.0 | ≤ 7.5 | ≤ 7.5 |  |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, 5480, and 5700 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, 5490, and 5510 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5300, 5320, 5500, 5520, 5680 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 4: Applicable for all valid channels other than those enumerated under NOTE 3. NOTE 5: Contiguous outer sub-band configuration and contiguous inner sub-band configuration in Table 6.1.2.1.1.1-5 apply. NOTE 6: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |  |

6.1.3.1.1.3	OPPO’s simulation results (R4-2)

For NS_28, the PSD requirement is 10dBm/MHz and the ASE is captured as below:

Table 6.1.3.1.1.3-1 ASE for NS_28

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20 MHz |  |
| 47 ≤ f ≤ 74 | -54 | 100 kHz |
| 87.5 ≤ f ≤ 118 | -54 | 100 kHz |
| 174 ≤ f ≤ 230 | -54 | 100 kHz |
| 470 ≤ f ≤ 862 | -54 | 100 kHz |
| 1000 ≤ f ≤ 5150 | -30 | 1 MHz |
| 5350 ≤ f ≤ 5470 | -30 | 1 MHz |
| 5725 ≤ f ≤ 26000 | -30 | 1 MHz |

The band edge of -30dBm/MHz ASE makes the channel next to channel edge should have larger A-MPR while for inner channels that are far from edge can enjoy a little bit lower A-MPR considering also the PSD is 10dBm/MHz which is quite large. The simulation results for PSSCH and PSCCH is shown below. Also the simulation cases are listed.

The simulation cases are captured in sub-clause 6.1.2.1.1.2.

For the A-MPR simulation result, they are captured below for edge channel and non-edge channel.

Table 6.1.3.1.1.3-2 A-MPR for single CC NS_28 edge channel

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 5.3 | 5.4 | 4.4 | 4.4 | 3.9 | 3.5 | 3.2 | 5.5 | 5.8 | 4.4 | 4.4 | 3.9 | 3.5 | 3.2 |
| 16QAM | 5.3 | 5.4 | 4.4 | 4.4 | 3.9 | 3.7 | 3.6 | 5.5 | 5.8 | 4.5 | 4.4 | 3.9 | 3.5 | 3.2 |
| 64QAM | 5.3 | 5.5 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 5.5 | 5.8 | 4.5 | 4.5 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 7.9 | 7.5 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

Table 6.1.3.1.1.3-3 A-MPR for single CC NS_28 non-edge channel

| case | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK | 3.2 | 3.1 | 3.2 | 3.2 | 3.2 | 3.2 |
| 16QAM | 3.7 | 3.6 | 3.7 | 3.7 | 3.7 | 3.7 |
| 64QAM | 5.0 | 5.2 | 5.1 | 5.0 | 5.2 | 5.1 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 |

The wide-band operation simulation result is further provided below for edge channel:

Table 6.1.3.1.1.3-4 A-MPR for Wideband operation NS_28 edge channel

|  | Bitmap | 10 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 | 10000 | 11000 | 11100 | 11110 | 01000 | 01100 | 01110 | 00100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 4.4 | 3.9 | 3.9 | 2.8 | 3.5 | 3.5 | 3.5 | 2.8 | 2.8 | 3.2 | 3.2 | 3.2 | 3.2 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 4.4 | 3.9 | 3.9 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.7 | 3.6 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 5.1 | 4.7 | 4.8 | 5.0 | 5.1 | 4.8 | 5.0 | 5.2 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 7.4 | 7.3 | 7.6 | 8.0 | 7.4 | 7.7 | 8.5 | 8.5 |
| Interlace | QPSK | 4.7 | 4.1 | 3.9 | 2.8 | 3.6 | 3.7 | 3.5 | 2.6 | 2.6 | 3.4 | 3.3 | 3.3 | 3.2 | 2.6 | 2.7 | 2.8 | 2.6 |
|  | 16QAM | 4.7 | 4.1 | 4.0 | 2.9 | 3.6 | 3.7 | 3.5 | 2.9 | 3.0 | 3.4 | 3.3 | 3.3 | 3.2 | 2.9 | 3.0 | 3.0 | 2.8 |
|  | 64QAM | 4.6 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.0 | 4.1 | 4.2 | 4.1 | 4.1 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 | 6.6 | 6.8 | 6.7 | 6.6 | 6.6 | 6.8 | 6.7 | 6.5 |

Table 6.1.3.1.1.3-5 A-MPR for Wideband operation NS_28 non-edge channel

|  | Bitmap | 10 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 |
| Interlace | QPSK | 2.6 | 2.6 | 2.7 | 2.8 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 |
|  | 16QAM | 2.9 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 |
|  | 64QAM | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.1 | 4.1 | 4.2 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 |

Based on the simulations shown above, it can be seen for inner channels, the MPR for QPSK and 16QAM is around 1 to 1.5dB lower compared to the edge channels. For the 64QAM and 256QAM, the dominate factor is mostly still the EVM and hence the difference for inner channel and edge channels are not that different.

<<endof change>>