# 3GPP TSG-RAN WG4 Meeting #110bis	R4-2405384

Changsha, China, 15th – 19th April, 2024

Agenda Item:	6.20.1.1

Source: 	OPPO

Title: 	Further simulation results for up to Rel-17 NS values

Document for:	Approval

- Introduction

The RAN4 Rel-18 sidelink core work has been concluded in the RAN4#109 meeting. During the Rel-18 timeline, due to limited time, 5 of 17 total NS values for up to Rel-18 has been down scoped. In our companion discussion paper, it is proposed to finish the up to Rel-17 defined NR-U NS values in Rel-18 SL-U WID and hence 5 more NS values are proposed.

- Discussion

In our companion paper it is proposed to finish the NS values for NS 28,29,30 54,59 in Rel-18 timeline. Here we give some initial simulation results for NS_28.

For NS_28, the PSD requirement is 10dBm/MHz and the ASE is captured as below:

Table 1 ASE for NS_28

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20 MHz |  |
| 47 ≤ f ≤ 74 | -54 | 100 kHz |
| 87.5 ≤ f ≤ 118 | -54 | 100 kHz |
| 174 ≤ f ≤ 230 | -54 | 100 kHz |
| 470 ≤ f ≤ 862 | -54 | 100 kHz |
| 1000 ≤ f ≤ 5150 | -30 | 1 MHz |
| 5350 ≤ f ≤ 5470 | -30 | 1 MHz |
| 5725 ≤ f ≤ 26000 | -30 | 1 MHz |

The band edge of -30dBm/MHz ASE makes the channel next to channel edge should have larger A-MPR while for inner channels that are far from edge can enjoy a little bit lower A-MPR considering also the PSD is 10dBm/MHz which is quite large. The simulation results for PSSCH and PSCCH is shown below. Also the simulation cases are listed.

Table 2 simulation cases

|  | case | Waveform | BW | RB Setup | SCS |
| --- | --- | --- | --- | --- | --- |
| Full Allocation Single CC | 1 | CP-OFDM | 20 | 105RB0 | 15 |
|  | 2 | CP-OFDM | 20 | 50RB0 | 30 |
|  | 3 | CP-OFDM | 40 | 216RB0 | 15 |
|  | 4 | CP-OFDM | 40 | 105RB0 | 30 |
|  | 5 | CP-OFDM | 60 | 160RB0 | 30 |
|  | 6 | CP-OFDM | 80 | 216RB0 | 30 |
|  | 7 | CP-OFDM | 100 | 270RB0 | 30 |
| Interlaced Allocation Single CC | 8 | CP-OFDM | 20 | 1 RB0 every 10RBs | 15 |
|  | 9 | CP-OFDM | 20 | 1RB0 every 5RBs | 30 |
|  | 10 | CP-OFDM | 40 | 1RB0 every 10RBs | 15 |
|  | 11 | CP-OFDM | 40 | 1RB0 every 5RBs | 30 |
|  | 12 | CP-OFDM | 60 | 1RB0 every 5RBs | 30 |
|  | 13 | CP-OFDM | 80 | 1RB0 every 5RBs | 30 |
|  | 14 | CP-OFDM | 100 | 1RB0 every 5RBs | 30 |
| Wide band operation contiguous RB allocation | 15 | CP-OFDM | 40 | Bitmap 10 | 30 |
|  | 16 | CP-OFDM | 60 | Bitmap 100 | 30 |
|  | 17 | CP-OFDM | 60 | Bitmap 110 | 30 |
|  | 18 | CP-OFDM | 60 | Bitmap 010 | 30 |
|  | 19 | CP-OFDM | 80 | Bitmap 1000 | 30 |
|  | 20 | CP-OFDM | 80 | Bitmap 1100 | 30 |
|  | 21 | CP-OFDM | 80 | Bitmap 1110 | 30 |
|  | 22 | CP-OFDM | 80 | Bitmap 0100 | 30 |
|  | 23 | CP-OFDM | 80 | Bitmap 0110 | 30 |
|  | 24 | CP-OFDM | 100 | Bitmap 10000 | 30 |
|  | 25 | CP-OFDM | 100 | Bitmap 11000 | 30 |
|  | 26 | CP-OFDM | 100 | Bitmap 11100 | 30 |
|  | 27 | CP-OFDM | 100 | Bitmap 11110 | 30 |
|  | 28 | CP-OFDM | 100 | Bitmap 01000 | 30 |
|  | 29 | CP-OFDM | 100 | Bitmap 01100 | 30 |
|  | 30 | CP-OFDM | 100 | Bitmap 01110 | 30 |
|  | 31 | CP-OFDM | 100 | Bitmap 00100 | 30 |
| Wide band operation Interlaced RB allocation | 32 | CP-OFDM | 40 | Bitmap 10 | 30 |
|  | 33 | CP-OFDM | 60 | Bitmap 100 | 30 |
|  | 34 | CP-OFDM | 60 | Bitmap 110 | 30 |
|  | 35 | CP-OFDM | 60 | Bitmap 010 | 30 |
|  | 36 | CP-OFDM | 80 | Bitmap 1000 | 30 |
|  | 37 | CP-OFDM | 80 | Bitmap 1100 | 30 |
|  | 38 | CP-OFDM | 80 | Bitmap 1110 | 30 |
|  | 39 | CP-OFDM | 80 | Bitmap 0100 | 30 |
|  | 40 | CP-OFDM | 80 | Bitmap 0110 | 30 |
|  | 41 | CP-OFDM | 100 | Bitmap 10000 | 30 |
|  | 42 | CP-OFDM | 100 | Bitmap 11000 | 30 |
|  | 43 | CP-OFDM | 100 | Bitmap 11100 | 30 |
|  | 44 | CP-OFDM | 100 | Bitmap 11110 | 30 |
|  | 45 | CP-OFDM | 100 | Bitmap 01000 | 30 |
|  | 46 | CP-OFDM | 100 | Bitmap 01100 | 30 |
|  | 47 | CP-OFDM | 100 | Bitmap 01110 | 30 |
|  | 48 | CP-OFDM | 100 | Bitmap 00100 | 30 |

For the A-MPR simulation result, they are captured below for edge channel and non-edge channel.

Table 3 A-MPR for single CC NS_28 edge channel

| case | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| QPSK | 5.3 | 5.4 | 4.4 | 4.4 | 3.9 | 3.5 | 3.2 | 5.5 | 5.8 | 4.4 | 4.4 | 3.9 | 3.5 | 3.2 |
| 16QAM | 5.3 | 5.4 | 4.4 | 4.4 | 3.9 | 3.7 | 3.6 | 5.5 | 5.8 | 4.5 | 4.4 | 3.9 | 3.5 | 3.2 |
| 64QAM | 5.3 | 5.5 | 5.1 | 5.0 | 5.2 | 5.1 | 5.1 | 5.5 | 5.8 | 4.5 | 4.5 | 4.1 | 3.9 | 4.0 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 | 8.7 | 7.9 | 7.5 | 7.6 | 7.9 | 6.6 | 6.5 | 6.8 |

Table 4 A-MPR for single CC NS_28 non-edge channel

| case | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK | 3.2 | 3.1 | 3.2 | 3.2 | 3.2 | 3.2 |
| 16QAM | 3.7 | 3.6 | 3.7 | 3.7 | 3.7 | 3.7 |
| 64QAM | 5.0 | 5.2 | 5.1 | 5.0 | 5.2 | 5.1 |
| 256QAM | 8.8 | 8.6 | 8.6 | 8.8 | 8.6 | 8.5 |

The wide-band operation simulation result is further provided below for edge channel:

Table 5 A-MPR for Wideband operation NS_28 edge channel

|  | Bitmap | 10 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 | 10000 | 11000 | 11100 | 11110 | 01000 | 01100 | 01110 | 00100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 4.4 | 3.9 | 3.9 | 2.8 | 3.5 | 3.5 | 3.5 | 2.8 | 2.8 | 3.2 | 3.2 | 3.2 | 3.2 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 4.4 | 3.9 | 3.9 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.7 | 3.6 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 | 5.1 | 4.7 | 4.8 | 5.0 | 5.1 | 4.8 | 5.0 | 5.2 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 | 7.4 | 7.3 | 7.6 | 8.0 | 7.4 | 7.7 | 8.5 | 8.5 |
| Interlace | QPSK | 4.7 | 4.1 | 3.9 | 2.8 | 3.6 | 3.7 | 3.5 | 2.6 | 2.6 | 3.4 | 3.3 | 3.3 | 3.2 | 2.6 | 2.7 | 2.8 | 2.6 |
|  | 16QAM | 4.7 | 4.1 | 4.0 | 2.9 | 3.6 | 3.7 | 3.5 | 2.9 | 3.0 | 3.4 | 3.3 | 3.3 | 3.2 | 2.9 | 3.0 | 3.0 | 2.8 |
|  | 64QAM | 4.6 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.0 | 4.1 | 4.2 | 4.1 | 4.1 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 | 6.6 | 6.8 | 6.7 | 6.6 | 6.6 | 6.8 | 6.7 | 6.5 |

Table 6 A-MPR for Wideband operation NS_28 non-edge channel

|  | Bitmap | 10 | 100 | 110 | 010 | 1100 | 1000 | 1110 | 0100 | 0110 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contiguous | QPSK | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 | 2.8 |
|  | 16QAM | 3.5 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.6 | 3.5 | 3.7 |
|  | 64QAM | 5.1 | 5.1 | 4.8 | 5.2 | 4.7 | 5.1 | 4.9 | 5.1 | 4.9 |
|  | 256QAM | 7.4 | 7.5 | 7.7 | 8.5 | 7.3 | 7.4 | 8.0 | 7.5 | 8.5 |
| Interlace | QPSK | 2.6 | 2.6 | 2.7 | 2.8 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 |
|  | 16QAM | 2.9 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 | 2.9 | 3.0 |
|  | 64QAM | 4.1 | 4.1 | 4.2 | 4.1 | 4.2 | 4.1 | 4.1 | 4.1 | 4.2 |
|  | 256QAM | 6.5 | 6.6 | 6.8 | 6.6 | 6.8 | 6.6 | 6.7 | 6.6 | 6.8 |

Based on the simulations shown above, it can be seen for inner channels, the MPR for QPSK and 16QAM is around 1 to 1.5dB lower compared to the edge channels. For the 64QAM and 256QAM, the dominate factor is mostly still the EVM and hence the difference for inner channel and edge channels are not that different.

Proposal 1: It is proposed to capture the simulation results into the TR 38.786.

3	Conclusions

Proposal 1: It is proposed to capture the simulation results into the TR 38.786.

4 References

[1] R4-2403868	 WF on NR sidlink UE RF requirements, LGE

[2] RP-240840 	New WID on NR Sidelink: Intra-band Carrier Aggregation in ITS band