3GPP TSG-RAN WG4 Meeting #110bis    	R4-2405410

Changsha, China, 15th – 19th March, 2024

Title:	Discussion intra-band non-collocated deployment Phase2

Agenda item:	9.7.2

Source:	Nokia

Document for:	Discussion

# 1	Background

This contribution discusses intra-band non-collocated deployment Phase2 receiver architecture type 4.

# 2 	Discussion

The core part objectives relevant for UE RF discussion in [1] are as follows

The core part of the work item includes:

- Specify the core requirements for type 4a/4b capable FWA UE that supports non-co-located scenarios for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and/or FR1 intra-band non-contiguous NR-CA for up to 4-layer DL MIMO per CC, where MRTD>CP is assumed.

  - RF power imbalance requirements of up to 25dB

  - RRM requirements including MTTD and MRTD requirements and other identified requirement such as scheduling restriction/availability, SCell operation related delay and interruption.

  - Work is limited to EN-DC/NR-CA for bands 42, n77/n78

- Discuss and if needed, specify the UE behavior when the UE supports type 4a/b but operates in co-located scenarios depending on whether network signaling is provided. R18 handling of UE behavior can be used as a reference.

Where the UE type 4 is defined in [2]

Table 1: UE types

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 3b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |  |

As can be seen from table 1 UE type 4b and type 2 are very similar in terms of Frequency separation between 2cc, NRCA/ENDC capability and power imbalance. UE type 4a can do EN-DC but not NR CA. However we do not think that special attention is needed whether UE is type 4a or 4b as difference comes from the fact that LTE needs only two receivers hence total number of receivers is six not 8. Both 4a and 4b have same capabilities in terms of power imbalance and MRTD/MTTD and EN-DC and/or NR CA capability is signalled separately.  RAN4 can just use term type 4 in discussions.

As UE type 4b and type 2 as very similar it is natural that type 2 UE requirements should be re-used as much as possible for type 4b except support for MIMOI-layers of course and only with very strong reasons difference can be discussed.

# 3	Conclusion

In this contribution we have discussed intra-band non-collocated deployment Phase2 receiver architecture type 4.

# 4	References

[1]	RP-240838, New WID on Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s)

[2] R4-2217734, WF on NonCol_intraB_ENDC_NR_CA for New Type UE