3GPP TSG-RAN WG4 Meeting # 110bis 	R4-2405426

Changsha, China, April 15 – 19, 2024

Source: 	Huawei, HiSilicon

Title: 	IMT technology related parameters for 7125 to 8400 MHz

Agenda Item:	9.3.3

Document for:	Approval

- Introduction

In the last plenary (RAN#103) the Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved [1].

This paper looks at IMT technology related parameters and propose our views for the range 7125 to 8400 MHz.

- Discussion

The following parameters are listed in the WP5D LS [2],

- Duplex Method

- Channel bandwidth (MHz)

- Signal bandwidth (MHz)

- BS Transmitter characteristics

- BS Receiver characteristics

- UE Transmitter characteristics

- UE Receiver characteristics

Duplex Method

It is a system parameter and should be the same for BS and UE.

The frequency range 7125 to 8400 MHz is unpaired spectrum, hence it’s most likely that TDD should be used. In Release 18, SBFD was studied for TDD BS side. It could be a potential duplex method for this band.

Channel bandwidth, Signal bandwidth (MHz)

It is a system parameter and should be the same for BS and UE.

The supported channel bandwidths are defined per operating band. For the band operation in the range 7125 to 8400 MHz, the total band range is 1275 MHz which is larger than that of existing FR1 bands. 4% the relative channel bandwidth is 310 MHz. We think 200 MHz can be max channel bandwidth for the band. Carrier aggregation will be defined for the operator hold wide spectrum, e.g. 400~800 MHz bandwidth.

The transmission bandwidth configuration defined in BS/UE core spec can be used to derive the signal bandwidth. The signal bandwidth is calculated based on the NR spectrum utilization for each SCS:

Signal bandwidth = NRB x SCS x 12

BS Transmitter characteristics

Power dynamic range (dB):

There is no power control in downlink and fixed power per resource block is assumed in the RAN4 co-existence simulation. Hence 0 dB power dynamic range is proposed

Spectral mask, ACLR:

The frequency range 7125 to 8400 MHz is just adjacent to existing NR band n104. It is proposed that existing spectral mask and ACLR for band n104 is applicable for the range.

Maximum output power:

The maximum output power will be provided in the antenna parameter table which can be discussed with antenna characteristics.

BS Receiver characteristics

Noise figure:

Option 1: Existing assumption for n104 can be reused, i.e. 6 dB (macro), 11 dB (micro) and 14 dB (pico/femto) for base station.

Option 2: in reality the noise figure is much better, hence we can reuse the general FR1 assumption, i.e. 5 dB (macro), 10 dB (micro) and 13 dB (pico/femto) for base station.

Sensitivity:

The sensitivity is not a critical parameter for sharing and compatibility studies. It is proposed to not mention any value for this parameter.

ACS:

The frequency range 7125 to 8400 MHz is just adjacent to existing NR band n104. It is proposed that existing ACS for band n104 is applicable for the range.

Blocking response

The frequency range 7125 to 8400 MHz is just adjacent to existing NR band n104. It is proposed that existing ACS for band n104 is applicable for the range.

UE Transmitter characteristics

Power dynamic range (dB):

The minimum controlled output power of the UE is defined as the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks), when the power is set to a minimum value. The minimum output power is -33/-30 dBm for 100/200 MHz channel bandwidth, i.e. power dynamic range is 56/53 dB for 100/200 MHz channel bandwidth.

Spectral mask, ACLR:

The frequency range 7125 to 8400 MHz is just adjacent to existing NR band n104. It is proposed that existing ACLR for band n104 is applicable for the range.

Maximum output power:

Both PC3 and PC2 are defined for 6 GHz band n104. For the purpose of co-existence analysis, the UE maximum output power for the considered frequency ranges could be 23 dBm.

Receiver characteristics

Noise figure:

The frequency range 7125 to 8400 MHz is just adjacent to existing NR band n104. It is proposed that existing NF for band n104 is applicable for the range, i.e.12 dB

Sensitivity:

The sensitivity is not a critical parameter for sharing and compatibility studies. It was agreed to not mention any value for this parameter.

Blocking response

The blocking characteristic specified in clause 7.6 of TS 38.101-1 [4] for frequency larger than 3300 MHz could be applied for the range.

ACS

The frequency range 7125 to 8400 MHz is just adjacent to existing NR band n104. It is proposed that existing ACS for band n104 is applicable for the range.

- Conclusion

The contribution provides an overview on IMT technology related parameters for frequency range 7125 to 8400 MHz.

Duplex Method

Proposal 1: The frequency range 7125 to 8400 MHz is unpaired spectrum, hence it’s most likely that TDD should be used.

Channel bandwidth, Signal bandwidth (MHz)

Proposal 2: 200 MHz is proposed as the max channel bandwidth for the band.

BS Transmitter characteristics

Power dynamic range (dB):

Proposal 3: 0 dB power dynamic range is proposed.

Spectral mask, ACLR:

Proposal 4: It is proposed that existing spectral mask and ACLR for band n104 is applicable for the range.

Maximum output power:

Proposal 5: The maximum output power will be provided in the antenna parameter table which can be discussed with antenna characteristics.

BS Receiver characteristics

Noise figure:

Proposal 6:

Option 1: Existing assumption for n104 can be reused, i.e. 6 dB (macro), 11 dB (micro) and 14 dB (pico/femto) for base station.

Option 2: To reuse the general FR1 assumption, i.e. 5 dB (macro), 10 dB (micro) and 13 dB (pico/femto) for base station.

Sensitivity:

Proposal 7: The sensitivity is not a critical parameter for sharing and compatibility studies. It is proposed to not mention any value for this parameter.

ACS:

Proposal 8: It is proposed that existing ACS for band n104 is applicable for the range.

Blocking response

Proposal 9: It is proposed that existing blocking for band n104 is applicable for the range.

UE Transmitter characteristics

Power dynamic range (dB):

Proposal 10: The minimum output power is -33/-30 dBm for 100/200 MHz channel bandwidth, i.e. power dynamic range is 56/53 dB for 100/200 MHz channel bandwidth.

Spectral mask, ACLR:

Proposal 11:  It is proposed that existing ACLR for band n104 is applicable for the range.

Maximum output power

Proposal 12: For the purpose of co-existence analysis, the UE maximum output power for the frequency ranges could be 23 dBm.

Receiver characteristics

Noise figure:

Proposal 13:  It is proposed that existing NF for band n104 is applicable for the range, i.e.12 dB

Sensitivity:

Proposal 14: The sensitivity is not a critical parameter for sharing and compatibility studies. It was agreed to not mention any value for this parameter.

Blocking response

Proposal 15: The blocking characteristic specified in clause 7.6 of TS 38.101-1 [4] for frequency larger than 3300 MHz could be applied for the range.

ACS

Proposal 16:  It is proposed that existing ACS for band n104 is applicable for the range.

References

[1]	RP-240787,	New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz, 	Ericsson

[2] R4-2400333, “Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz” ITU-R WP 5D