# 3GPP TSG-RAN WG4 Meeting # 110-bis	R4-2405437

# Changsha, China, 15th – 19th April, 2024

Agenda item:			6.24.4

Source:	MediaTek Inc.

Title:	Discussion on performance part for NR Network energy savings

Document for:	Discussion

# Introduction

Remaining issues on the perfoarmnce part was listed in the prevous meeting [1]. In this contribution, our views on RRM test cases for NR Network energy saving are provided.

# Performance part for NES

Issue 3-1-2: Detailed configurations for  FR1 inter-band SSB-less activation/deactivation

RAN4 has reached a consensus on developing test cases for FR1 inter-band SSB-less activation, with the specific test configurations yet to be finalized and currently listed as FFS in [1] and the test setting for power difference is also discussed in [2].

It has also been agreed that the difference in [EPRE] at the UE side should not exceed [9] dB, a parameter that will be regulated within the test environment. The objective of these tests is to ascertain whether the UE can appropriately adjust its AGC. To this end, a higher power setting is deemed suitable for fulfilling the test requirements. Additionally, an AWGN is preferred for these tests, as using a fading channel would need an extra margin to account for channel variations.

Online session (Thursday Nov 16, 2023) Agreement: - The requirements apply provided that [EPRE] difference at UE side is less than [9] dB. - EPRE difference is based on power difference between TRS symbol on SSB-less SCell and SSB symbol on reference cell - Capture in the WF that RAN4 assumes that UE carries out pre-compensation for AGC considering [BW difference and carrier frequency difference]. - Further discuss whether/how to capture the EPRE after pre-compensation in the spec.

Proposal 1: Configure the target inter-band SSB-less SCell to have a power level that is [< 9 dB] higher than that of the PCell and use an AWGN channel for the setup.

# Summary

The observations and proposals in this contribution are summarized:

Proposal 1: Configure the target inter-band SSB-less SCell to have a power level that is [< 9 dB] higher than that of the PCell and use an AWGN channel for the setup.

# Reference

- R4-2321562, WF on RRM requirements for NR network energy saving, RAN4#109, Nov. 2023.

- R4-2403526, WF on RRM requirements for NR network energy saving, RAN4#110, Feb. 2024.