3GPP TSG-RAN WG4#110bis	R4-2405444

Changsha, China, April 15th – 19th, 2024

Agenda item:	5.1.1.1

Source: 	Qualcomm France

Title: 	Considerations on CA_n40A-n41C

Document for:	Approval

# Introduction

Considerations on CA_n40A-n41C are provided in this contribution.

- Background

WF for CA_n40A-n41C was agreed in RAN4#110 [1]:

Using the following test points to estimate the MSD value:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2353.5 | FFS | TDD | IMD3 |
|  | n41 | 2545 | 60 | 1 (RBSTART= 0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBSTART= 272) | 2625 |  |  |  |

Note: For the band n40 which is subject to IMD5 and IMD7 caused by CA_n41C, the MSD is not specified

- Discussion

Calculating the exact UL RB locations and respective IMD3 frequency, we noticed that actually IMD3 is not hitting on top of the DL channel as specified in WF. Consequently, we analysed IMD3 MSD using the following frequencies

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2358 | FFS | TDD | IMD3 |
|  | n41 | 2545 | 60 | 1 (RBSTART= 0) | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 (RBSTART= 272) | 2625 |  |  |  |

The IMD3 level at PA output is in worst case up to 5dBm. In Spurious Emissions for UE Co-existence, n41C must protect n40 by -40dBm/1MHz. Hence, the Post PA attenuation should be ≥45dB to meet this limit if MPR is not considered.

In CA_n40A-n41A, roughly 40dB Post-PA attenuation at n40 frequency is required to meet ~30dBm cross-band MSD specification. In the analysis below we assumed for simplicity that n41 filter together with other RFFE components provide 40dBm attenuation at n40 RX, so that it is just compliant with UE Co-existence.

The MSD is a whopping 59.3dB! In our view, RAN4 should consider how to handle these kind of cases in specifications; MSD itself is clearly way too high for being pragmatic, yet it exists if RB allocations are such that IMD3 hits directly.

Observation 1: MSD for CA_n40A-n41C is well above 50dB

Proposal 1: RAN4 should consider whether MSD number is captured, or if other ways could be introduced for this kind of combinations were the theoretical worst-case scenario is not pragmatic

- Conclusion

Considerations on CA_n40A-n41C were provided with the following observation and proposal

Observation 1: MSD for CA_n40A-n41C is above 50dB

Proposal 1: RAN4 should consider whether MSD number is captured, or if other ways could be introduced for this kind of combinations were the theoretical worst-case scenario is not pragmatic

- Reference

[1] R4-2403716, “WF on CA_n40A-n41C”, ZTE, Skyworks