3GPP TSG-RAN WG4#110bis	R4-2405451

Changsha, China, April 15th – 19th, 2024

Agenda item:	5.1.1.2

Source: 	Qualcomm France

Title: 	Discussion on MSD for 1TX/2TX PC2

Document for:	Discussion

# Introduction

Possibilities to use some kind framework to derive MSD for 1TX/2TX PC2 cases from respective PC3 MSD is discussed in this contribution.

- Background

Recent introduction of PC2 FDD has been a laborious task in RAN4. Now, and even more in near future the amount of work needed will increase exponentially as it is envisioned most of current PC3 FDD bands combinations will eventually be introduced for PC2 FDD as well.

- Discussion

Over the past ten years or so, Carrier Aggregation has been one of key themes in RAN4 RF. In the beginning everything was sort of new for companies and consequently each combination had its own WI during first years in LTE CA (!). Since that, things have evolved drastically and CA has become so common that in many cases the requirements including MSD exceptions are just copied from already existing similar combinations. While this is in many ways inevitable as no-one has the time to conduct rigorous analysis for each combination including careful assessment of possible filter attenuations etc for the analysis, some kind of consistency should still be maintained - or regained.

FDD PC2 is a bit tricky in the sense that it introduces two architecture options, 1TX PC2 and 2TX PC2.

It is quite straightforward to derive 1TX PC2 MSD for cases where respective PC3 MSD is specified, as the only thing increasing is the aggressor power. 3dB is pretty good estimate, even that does not always fully hold. The amount of TX noise which consists of PA noise and transceiver noise increases, but the exact amount varies a bit. IP2 interference increases two-fold, so 3dB increase to TX power increases receiver IP2 by 6dB. Putting these two aspects together totals everything from ~3dB to a bit above 3dB.

For 2TX case, the situation is much more challenging. Both RX branches see aggressor same aggressor power which consists of the aggressor power of own TX, and the aggressor power coupled from other antenna via antenna isolation. It is, anyhow, possible to derive a consistent method on how to combine the aggressor powers and how to calculate the MSD. Overall, maybe the most challenging topic to agree is what kind of overall difference between PRX and DRX path interference to assume in the analysis. For PC3, the difference has been anything between 0….10dB in the analysis seen from tens of companies in past ten years. The assumed difference has direct impact to the MSD difference between 1TX PC2 and 2TX PC3.

We show some extremely simplified example calculations using 0, 5, 10dB PRX/DRX difference. These are shown just for illustrative purposes. Band which has -97dBm REFSENS with 3dB MSD for PC3 is used as baseline. Please note that in case of 2TX PC2, the interference level used is a simplification mimicking the reality when both receivers see same signal i.e. there is no diversity gain. This is not necessarily the proposed way to make calculations or proposed way how to construct combined interferer for PRX and DRX branches.

Simple illustration above shows the large dependency into 2TX PC2 MSD due the difference between PRX/DRX interference. We note that even it would be easier to use 0dB difference in interference, and that at first sight it looks lucrative from MSD delta point of view, that is not so obvious as PC3 MSD to begin with is higher with 0dB difference than with 5dB difference. Basically to use 0dB PRX/DRX difference would mean all existing PC3 MSD’s would need to be increased.

We note that this observation on 0dB PRX/DRX difference does not generally hold; for instance, in case where there are two MSD test points for different Dl BW’s, the MSD ratio between those two is almost same no matter is equal interference or imbalanced interference is used.

One topic which requires specific consideration and is valid for both 1TX PC2 and 2TX PC2 is how to evaluate combinations which do not have MSD specified. There are many combinations which are on the verge on if small PC3 should be specified or not. For those cases, some MSD for 1TX PC2 and especially for 2TX PC2 is needed. How to find those cases and how to evaluate the need for MSD is to be discussed.

Some observations for further thinking in the group:

- When PC2 1TX PC2 MSD is calculated, the resulting MSD should be aligned with respective PC3 MSD, i.e. using the same architecture, but with PC3 power the MSD should match PC3 MSD reasonably well.

- The main difference in 1TX PC3 vs 2TX PC2 is that in PC2 case both branches see same interference

- RAN4 should discuss if a generic way to derive the PC2 requirements could be agreed, as it would dramatically ease the work

  - 1TX PC2 seems initially quite straightforward, with difference to PC3 being [3]dB increased interference

  - 2TX PC2 case requires more work and internal evaluations from companies. Based on the considerations above, there should be possibilities to find consensus

- RAN4 should discuss how to identify cases which don’t have MSD specified for PC3, but which could need MSD for 1TX PC2 or 2TX PC2

- Conclusion

Possibilities to use some kind framework MSD for 1TX/2TX PC2 cases from respective PC3 MSD is discussed in this contribution, with the following observations:

- When PC2 1TX PC2 MSD is calculated, the resulting MSD should be aligned with respective PC3 MSD, i.e. using the same architecture, but with PC3 power the MSD should match PC3 MSD reasonably well.

- The main difference in 1TX PC3 vs 2TX PC2 is that in PC2 case both branches see same interference

- RAN4 should discuss if a generic way to derive the PC2 requirements could be agreed, as it would dramatically easy the work

  - 1TX PC2 seems initially quite straightforward, with difference to PC3 being [3]dB increased interference

  - 2TX PC2 case requires more work and internal evaluations from companies. Based on the considerations above, there should be possibilities to find consensus

- RAN4 should discuss how to identify cases which don’t have MSD specified for PC3, but which could need MSD for 1TX PC2 or 2TX PC2

- Reference